| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0076 |
| --- | --- |

| Title: | Improving HEVC compression efficiency by intensity dependant spatial quantisation (WD changes) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak David Flynn Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk davidf@rd.bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document illustrates the changes required by the intensity dependant spatial quantisation in the HEVC working draft specification.

# Changes in picture parameter set RBSP syntax

The following changes are required to the syntax which defines the Picture Parameter Set (PPS) RBSP. The changes are highlighted in red.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if(slice_type = =P ¦ ¦ slice_type = = B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| idsq_enabled_flag | u(1) |
| if( idsq_enabled_flag ){ |  |
| idsq_num_intervals_minus1 | ue(v) |
| idsq_init_qp_offset | ue(v) |
| for( i = 0; i < idsq_num_intervals_minus1; i++ ){ |  |
| idsq_interval_width_minus1[ i ] | ue(v) |
| idsq_qp_offset_delta_sign[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

# Picture parameter set RBSP semantics

idsq_enabled_flag equal to 0 specifies that intensity dependent spatial quantisation is disabled. idsq_enabled_flag equal to 1 specifies that intensity dependent spatial quantisation is enabled.

idsq_num_intervals_minus1 plus 1, specifies the number of intervals in the IDSQ histogram.

idsq_init_qp_offset specifies the QP offset of the first IDSQ histogram bin.

idsq_interval_width_minus1[ j ] plus 1, specifies the width of the j’th interval of the IDSQ histogram.

idsq_qp_offset_delta_sign[ j ] specifies the sign of the ±1 difference between the j’th and (j+1)’th intervals of the IDSQ histogram.

The variable idsqProfile][ j ] contains the reconstructed QP offset that is applied to TUs that have a luma predictor with an average sample intensity j. idsqProfile[ j ] is derived as follows:

intervalEnd = 0
i = j = 0
qpOffset = idsq_init_qp_offset
for(; i < idsq_num_intervals_minus1; i++ ){
	intervalEnd += idsq_interval_width_minus1[ i ] + 1
	while( j < intervalEnd && j < 256 )
		idsqProfile[ j++ ] = qpOffset
	qpOffset += 1 – ( idsq_qp_offset_delta_sign[ i ] << 1 )
}
while( j < 256 )
	idsqProfile[ j++ ] = qpOffset

# Changes in the array construction process

### Derivation process for intensity dependant spatial quantisation offset

Inputs of this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable nW specifying the width of the current block,

–	a variable nH specifying the height of the current block,

–	an (nW)x(nH) array predSample containing the predicted samples for the current block.

Outputs of this process are:

–	the value IdsqOffsetQpY for the current block.

If idsq_enabled_flag is equal to zero, the following applies, the value of IdsqOffsetQpY shall be set to zero.

Otherwise, idsq_enabled_flag is equal to one, the value IdsqOffsetQpY is derived as follows:

IdsqOffsetQpY = idsqProfile [ computeIdsqAverage( bit_depth_luma_minus8 ) ]

And computeIdsqAverage( bitDepthMinus8 ):

for( y = yT; y < yT + nH; y++ )
	for( x = xT; x < xT + nW; x++ ){
		sum += predSample[ x, y ]
	}
offset = bitDepthY = = 8 ? 0 : 1 << ( bitDepthY – 9 )
sum = ( sum + offset ) >> (bitDepthY – 8)
return sum / ( nW * nH )

### Derivation process for inverse intensity dependant scaling on reconstructed residues

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	an (nW)x(nH) array r with elements rij containing the residual samples after the scaling and transformation process as specified in subclause XXX,

–	the variable IdsqOffsetQpY computed according to subclause XXX,

–	a variable cIdx specifying the chroma component of the current block.

Output to this process is inverse scaled residuals as a (nW)x(nH) array r’ with elements r’ij.

If idsq_enabled_flag is equal to zero, the following applies: the array r’ is set equal to (nW)x(nH) array of reconstructed residuals

Otherwise, idsq_enabled_flag is equal to one, the following applies.

The variable qpOffset is derived as follows:

qpOffset = IdsqOffsetQpY + 4

The variable shift is derived as follows:

shift = 6

The variable ResidueLimit is derived as follows:

-		If cIdx is equal to 0,

ResidueLimit = 1 << (bitDepthY)

-		Otherwise,

ResidueLimit = 1 << (bitDepthC)

The inverse scaled residuals r’ij with i = 0..nW − 1, j = 0..nH − 1 are derived as follows.

r’ij = Clip3(-ResidueLimit, ResidueLimit-1, ( (cij * mij * levelScale[ qpOffset%6 ] << (qpOffset/6)) + (1 << (shift − 1 )) ) >> shift )