| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0076 |
| --- | --- |

| Title: | Improving HEVC compression efficiency by intensity dependant spatial quantisation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak David Flynn Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk davidf@rd.bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

The properties of the human visual system can be exploited to improve the compression efficiency of the HEVC standard. In this proposal the pixel intensity masking of the human eye is used to apply coarser quantisation in darker and brighter image area. The proposed perceptual quantisation tool performs an Intensity Dependant Spatial Quantisation (IDSQ) similarly to the quantisation performed by the Intensity Dependant Quantisation (IDQ) tool presented at the 9th JCT-VC meeting in contribution I0257. However, the IDSQ tool proposed here is designed to make the inverse quantisation step at the decoder independent from the average pixel data which may introduce latency and pipelining refactoring during decoding. The proposed IDSQ has been implemented in the HM-7.0 and its performance is assessed by measuring the bitrate reduction with respect to the HM-7.0 codec. For the same perceptual quality level bitrate reductions of up to 25% are achieved and on average 3.4% across all tested points.

# Introduction

The human visual system shows space and time varying sensitivity to the distortion introduced by quantisation in lossy coding [1]. This different sensitivity is due to some masking phenomena triggered by spatial and temporal features of videos as textured areas, motion activity, etc. In this contribution, the masking relative to the average intensity is considered; this masking allows performing coarser quantisation in darker and brighter image areas without introducing coding artifacts which are noticeable to any human observer.  In order to find the mapping between the average pixel intensity of the image area being encoded and the quantisation step to be used, an Intensity Dependant (ID) profile [2] is used in this proposal. An example of ID profile is shown in Figure 1 where µ denotes the average pixel intensity and Δ(µ) the relative quantisation step. The average pixel intensity µ should be either communicated or estimated at the decoder to correctly reconstruct compressed pixels. In this proposal the µ value is estimated by computing the average intensity value over the predictor pixels. The main features of the Intensity Dependant Spatial Quantisation (IDSQ) tool described in this proposal can be summarized as follows:

- No need to signal to the decoder the varying quantisation step or Quantisation Parameter (QP) as it can be computed at the decoder.

- Quantisation parameter varied for each Transform Unit (TU).

- ID profile signalled in the Picture Parameter Set (PPS) to increase the flexibility and content adaptation.

More precisely, for each TU the average luma pixels value is used to select a QP offset which is added to the QP used for that TU. The average luma pixel is estimated by the luma average of the predictor (spatial or temporal) pixels while the mapping between the QP and the average luma intensity (i.e. the ID profile [2]) is transmitted to the decoder.

The QP dependency on the predictor data, which are needed to estimate the µ value, may introduce latency and reduce the parallelism for some decoding stages. An example of this parallelisation is for inter coded coding units whereby inverse quantisation and inverse transform are carried out while computing the motion compensated predictor which involves a significant latency due to several reference frame memory accesses. If the quantisation step is dependent on predictor data, the inverse quantisation and transform cannot be carried out in parallel. In this contribution this dependency is addressed by performing the inverse quantisation in the transformed domain independently from the perceptual quantisation step. The final inverse scaling of the reconstructed residuals with respect to the perceptual quantisation step is performed just before the residue addition with predictor data. In the following section the details on the proposed IDSQ tool are presented.

Figure 1: Example of ID profile.

# IDSQ at the encoder

At the encoder, the following steps are carried out:

- Step 1: Estimate the average luma value of the current TU by computing the average over the TU predictor for the luma component. Set this average value to µ.

- Step 2: Look up in the ID profile the QP offset corresponding µ, i.e. qpOffset = idProfile(µ).

- Step 3: Add qpOffset to the QP used for the current TU (QPTU) to obtain the final QP = QPTU + qpOffset.

After the third step, forward quantisation and entropy coding can be carried out as specified in the HEVC standard draft [3]. A schematic representation of IDSQ at the encoder is depicted in Figure 2.

Figure 2: IDSQ at the encoder.

# IDSQ at the decoder

At the decoder side the inverse quantisation step is carried into two stages: one before inverse frequency transform and one before residue addition to the predictor to reconstruct the decoded pixels. More precisely, after entropy decoding, the following steps are carried out (see also Figure 3):

- Step 1: Perform inverse quantisation as specified in the HEVC standard draft [3] with QP = QPTU to obtain the reconstructed coefficients c’.

- Step 2: Over the reconstructed coefficients (c’) perform inverse frequency transform as specified in the HEVC standard draft [3] to obtain the reconstructed residuals r’.

- Step 3: Estimate the average luma value of the current TU by computing the average over the TU predictor for the luma component. Set this average value to µ.

- Step 4: Look up in the ID profile the QP offset corresponding µ, i.e. qpOffset = idProfile(µ).

- Step 5: Perform inverse scaling of residuals r’ with the quantisation step associated to qpOffset(µ) to obtain the final reconstructed residuals r.

More precisely, the inverse scaling described at Step 5 above is carried according to the following procedure:

r[x][y] = (r’[x][y] * (IQ[qpOffset(µ) % 6] << (qpOffset(µ) / 6)) + 32) >> 6,

where IQ denotes the inverse quantisation scale. Since now the inverse quantisation step in the transform domain is independent from the predictor data, the decoder pipelining does not have to be re-designed.

The processing for the luma average computation and ID profile signalling are identical to what proposed in [2] and will not be described in this document. Finally, differently from [2], the perceptual QP modified by IDSQ is not used during the deblocking filter process.

Figure 3: IDSQ at the decoder.

# Experimental results

The IDSQ presented in this proposal has been integrated in the HEVC codec version 7.0 (HM-7.0) and tested over the JCT-VC test contents. The chosen encoder configurations are the ones relative to the common test conditions [4]. The chosen ID profile is the one proposed in [5] whereas the values have been modified by the authors of this proposal after a preliminary subjective viewing on the test contents carried out at the BBC R&D premises. The ID profile values for the different tested QPs are reported in the appendix of this document. To assess the performance brought by the proposed IDSQ tool, the bitrate reductions with respect to the HM-7.0 codec have been measured assuming the same level of perceptual quality. To verify the same level of perceptual quality, an informal viewing has been done by the authors over several tested contents.

Figure 4 reports the Cumulative Distribution Function (CDF) of the bitrate reductions provided by the proposed IDSQ with respect to the HM-7.0 codec. The CDF is reported for each encoder configuration in common test conditions as well as for all the tested points. The 50th percentile (i.e. the median) over all tested points provides a bitrate reduction of 2.2%, while peaks of 25% may be observed, notably for QP 22. Finally, over all contents an average bitrate reduction of -3.4% is provided. Moreover, Table 1 reports some bitrate reductions obtained by sorting in ascendant order the obtained reductions for the different tested contents and coding configurations.

Figure 4: Bitrate reductions with respect to HM-7.0 assuming the same level of perceptual quality.

Table 1: Bitrate and PSNR differences with respect to HM-7.0 for some tested contents.

| Sequence name | Configuration | QP | ΔRate [%] | ΔPSNR [dB] |
| --- | --- | --- | --- | --- |
| SteamLocomotive | ld_he10 | 22 | -25.13 | -0.56 |
| ChinaSpeed | ldP_he10 | 22 | -19.72 | -2.09 |
| Cactus | ld_he10 | 22 | -16.73 | -0.29 |
| Kimono | i_he10 | 22 | -15.39 | -0.72 |
| Nebuta | ld_he10 | 27 | -10.66 | -1.03 |
| ParkScene | i_he10 | 22 | -10.05 | -0.86 |
| BasketballDrive | ld_he10 | 22 | -9.57 | -0.18 |
| Kimono | ld_main | 27 | -9.47 | -0.45 |
| Traffic | i_main | 22 | -8.28 | -1.03 |
| ParkScene | ldP_main | 22 | -8.18 | -0.39 |
| KristenAndSara | ld_he10 | 22 | -7.59 | -0.20 |
| PeopleOnStreet | ld_he10 | 22 | -6.99 | -0.37 |
| BQTerrace | ra_he10 | 22 | -6.79 | -0.19 |
| Johnny | ld_he10 | 22 | -6.58 | -0.11 |

# Conclusions

This proposal has presented an Intensity Dependant Spatial Quantisation (IDSQ) tool which varies the quantisation step for each TU according to the ID profile. The proposed tool does not introduce any additional latency at the decoder as the inverse quantisation step is separated in two different stages. The integration of the proposed perceptual quantisation in the HM-7.0 codec provides a maximum bitrate reduction of up to 25% with respect to the HM-7.0 anchors for the same perceptual quality. Moreover, on average over all the tested content, the bitrate reduction is 3.4% always for the same perceptual quality of the HM-7.0 anchors. Finally, it should be noted that the presented perceptual quantization can be coupled with the default frequency weighting matrices [6] or with others customized matrices to further optimise the perceptual quality for a given target rate.

# Appendix 1: Used ID profiles in the experimental results

As stated in Section 4, the ID profile used in this proposal is the one proposed in [5]; this profile consists in a parabolic piecewise profile whose values are given as follows:

Δ(μ)=k1⋅1−2μ256λ1+1μ≤128k22μ256−1λ2+1otherwise,	(1)

where Δ denotes the quantisation step associated to a given average luma value µ. The parameters k1, k2, λ1 and λ2 are set to 2, 0.8, 3, and 2, respectively [5]. In this proposal, the profile values in (1) have been adopted after some subjective viewing performed at the BBC R&D premises. The derived ID profiles are selected differently for each tested QP. The values selected after this informal subjective viewing for the parameters k1, k2, λ1 and λ2 are reported in Table 2. The values of these profiles are listed from Table 3 to Table 5 in form of (µ, qpOffset) pairs whereby the same signalling method of [2] is assumed. Moreover, a graphical representation of the used ID profiles is also reported in Figure 5.

Table 2: Values for parameters k1, k2, λ1 and λ2 used to obtain the tested ID profiles.

| QP Parameter | 22 | 27 | 32 | 37 |
| --- | --- | --- | --- | --- |
| k1 | 4 | 4 | 1 | 0.4 |
| k2 | 0.8 | 0.8 | 0.4 | 0.2 |
| λ1 | 3 | 3 | 3 | 3 |
| λ2 | 2 | 2 | 2 | 2 |

Table 3: ID profile pairs (µ, qpOffset) for QP=22 and QP=27.

|  | Pair number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0..9] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| µ | 0 | 3 | 9 | 15 | 21 | 27 | 33 | 39 | 45 | 51 |
| qpOffset | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 |
|  | Pair number |  |  |  |  |  |  |  |  |  |
| [10..19] | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| µ | 58 | 65 | 73 | 82 | 97 | 163 | 191 | 211 | 230 | 247 |
| qpOffset | 4 | 3 | 2 | 1 | 0 | 1 | 2 | 3 | 4 | 5 |

Table 4: ID profile pairs (µ, qpOffset) for QP=32.

|  | Pair number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0..9] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| µ | 0 | 5 | 16 | 27 | 40 | 55 | 79 | 178 | 217 | 246 |
| qpOffset | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 1 | 2 | 3 |

Table 5: ID profile pairs (µ, qpOffset) for QP=37.

| [0..9] | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| µ | 0 | 8 | 29 | 61 | 198 | 253 |
| qpOffset | 3 | 2 | 1 | 0 | 1 | 2 |

Figure 5: Used ID profiles in the reported experimental results.

# Appendix 2: Encoder description

The following description is based on [7] and is divided in the following subsections which describe the different processing stages related to the IDSQ.

## ID profile reading and storing in the encoder memory

As stated in Appendix 1, the ID profile values are presented as input to the encoder in form of pairs (µ, qpOffset). These pairs are then used to fill a 1D array of 256 values whereby each array index refers to an average luminance value in the range [0, 255]. Each array cell contains the qpOffset corresponding to average luma value. The values for the idProfile array are computed as follows:

- Set intervaStart = 0

- Set qpOffsetCurrent = 0;

- While the input ID profile file is not finished do

- Read the pair (µ, qpOffset) from the ID profile file

- Set intervalEnd = µ

- for(i = intervalStart; i < intervalEnd; i++)

- {

- idProfile[i] = qpOffsetCurrent;

- }

- intervalStart = intervalEnd

- qpOffsetCurrent += qpOffset

- for(i = intervalStart; i < 256; i++)

- {

- idProfile[i] = qpOffsetCurrent;

- }

## Computation of the average luma value for each TU

For each TU the average luma value is computed/estimated by computing the average over the luma values for the TU predictor (spatial or temporal). The average value is always assumed to be in the rage [0, 255] so if internal bit depth increase is used the average value is shifted down by as many bits as the internal bit depth increment. The average for each TU is stored in the variable averageTU.

## Computation of the final QP for each TU

As stated in Section 2 the QP for each TU is given by QP = QPTU + qpOffset. In the encoder therefore it is computed as QP = QPTU + idProfile[averageTU].

## Transform and quantisation

The same as described in [7].

# References

- H. R. Wu and K. R. Rao, “Digital video image quality and perceptual coding”, CRC press, pages 640, November 2005.

- M. Naccari, M. Mrak, D. Flynn and A. Gabriellini, “On Intensity Dependent Quantization in the HEVC codec”, JCTVC-I0257, 9th meeting, Geneva, CH, April-May 2012.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, 9th meeting, Geneva, CH, April-May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, 9th meeting, Geneva, CH, April-May 2012.

- Y. Jia, W. Lin and A. A. Kassim, “Estimating the just-noticeable distortion for video”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 16, no. 7, pp. 820-829, July 2006.

- M. Haque, A. Tabatabai and Y. Morigami, “HVS model based default quantization matrices”, JCTVC-G880, 7th meeting, Geneva, Switzerland, November 2011.

- K. McCann, B. Bross, I.-K. Kim, K. Sugimoto, W.-J. Han, “HM6: High Efficiency Video Coding (HEVC) Test Model 6 Encoder Description”, JCTVC-H1002, 8th meeting, San José, CA, USA, February 2012.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).