| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0078 |
| --- | --- |

| Title: | AHG12: Non-4:2:0 formats syntax modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon Philippe Bordes 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33-2-99-27-38-07 pierre.andrivon@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents syntax modifications to Draft 7 (JCTVC-I1003_d2) of HEVC in order to prepare the support of non-4:2:0 formats in HEVC profiles. It is reported that proposed Draft 7 syntax modifications are twofold: adaptation of syntax to support non-4:2:0 chroma subsampling formats and simplifications by parsing only chroma syntax elements that are necessary in the decoding process.

# Introduction

It is stated that non-4:2:0 chroma subsampling formats introduction in HEVC through professional profiles is supported by some entities ([5], [8], [10]...) and National Bodies ([3], [4]...). Furthermore, UHDTV draft specification ([13]) and report ([14]) mention 4:2:2 and 4:4:4 as candidates for an enhanced and immersive TV experience.

As of 7th JCT-VC Geneva meeting, a 4:2:2 chroma format contribution ([5]) introduced in Working Draft 5 ([2]) chroma format indicator syntax element to consider non-4:2:0 chroma formats in HEVC. Then, other contributions were related to simplified 4:2:2 or 4:4:4 support in HEVC through “classical” approach ([5], [6], [7]), resampling filters proposals ([10], [11], [12]) or DMC/MCS techniques that allow to use existing 4:2:0 syntax and semantics ([8], [9]).

One of the mandates of the current AHG12: “Support for range extensions” is to assist and advise in the work of removing any implicit assumptions of 4:2:0 formatting from the current draft. Thus, this document proposes syntax modifications to support non-4:2:0 formats in the current Draft 7 ([1]) of HEVC.

# Draft 7 syntax modifications

Two considerations are encompassed in this contribution:

- Support of non-4:2:0 thanks to an adequate syntax through minimal modifications

- Simplifications by parsing only chroma syntax elements that are deemed necessary in the decoding process

ChromaArrayType variable is already defined in the current Draft 7 and used whenever it is possible. Definition is reminded here:

“Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.”

## Semantics modifications remark

It is stated that non-4:2:0 chroma format semantics modifications for support in current Draft is a heavy and invasive process that will affect readability of the Draft as chroma format must be taken into account for intra, inter, weighted prediction, chroma interpolation, in-loop filters, weighted prediction processes etc... It is noticed that special attention should be given to ChromaArrayType = 3 case at the syntax (transform unit)  and semantics level. This case is not particularly addressed in the present document. It is suggested to study the pertinence toward market of particular 4:4:4 profiles from AVC and hence if they make sense for HEVC before modifying semantics (and syntax) relative to enable such profiles later on. Indeed, in AVC, the way to process 4:4:4 data is twofold: either considering each 4:4:4 plane as separate (chroma_format_idc set to 3 and separate_colour_plane_flag set to 1) that implies plane independency (no inter-plane dependency i.e. each plane is coded as a monochrome plane) or 4:4:4 planes as non-separate (chroma_format_idc set to 3 and separate_colour_plane_flag set to 0), then each decision done on a plane is replicated on other planes (same decisions and modes across planes, residuals are different though). It is remarked that JCT-VC should decide whether such modifications should be considered in the current Draft from now on (before FDIS of first version of HEVC).

## Draft 7 (d2)  lightweight text modifications

First of all, a ticket (#542 redundant with part of #328) was addressed to consider limits of cb_qp_offset and cr_qp_offset syntax elements. Two proposals were emitted ([-12; +12] as in AVC or [-25 +26], that is the same range as pic_init_qp_minus26 syntax element). HM7.0 implements [-12; +12] range however it is not reflected in the text Draft 7.

It is noted that chroma interpolation filters have been suggested in several contributions ([5], [10], [11], [12]) however only 4:2:0 ones are defined in the current draft yet.

It is remarked that intra LM chroma mode syntax and semantics will have to be adapted (or conditioned to profile usage) if it is deemed to work with a hypothetical 4:4:4 non-separate colour plane profile because such a profile, if defined as in AVC, naturally replicates decisions performed for one colour plane on the other ones; that implies a paradox either for choosing the intra LM mode for all but one colour plane  (replication paradox) or for defining colour plane to be used as a reference for intra LM mode. Besides, it may be interesting to signal which reconstructed plane(s) is to be used as prediction(s) for other planes. Indeed, intra LM mode may be interesting for colour space with much inter-plane correlation like RGB that is used for very high quality processing in production workflows.

Ticket #469 suggests to remove chroma_format_idc semantics default inference until at least one profile omits the syntax element. Generally speaking, it is then suggested to wonder if non-4:2:0 elements should appear in the first version of HEVC specification text. Authors of this contribution think that such an inference does not disrupt the readability of the syntax element semantic and prepare the ground for future extensions/profiles that are supported by several National Bodies and firms.

It is suggested hereafter to remove references to chroma syntax elements in the bitstream when those piece of information are not necessary for the decoding process.

Below are mainly proposed Draft 7 syntax modifications to take into account considerations chroma format adaptations and simplifications of the text Draft syntax.

#### 6.3 Spatial subdivision of pictures, slices and tiles

The blocks and associated syntax structures are encapsulated in a "unit" as follows.

–	One prediction block (ChromaArrayType is equal to 0) or three prediction blocks (luma and chroma) and associated prediction syntax structures units are encapsulated in a prediction unit.

–	One transform block (ChromaArrayType is equal to 0) or three transform blocks (luma and chroma) and associated transform syntax structures units are encapsulated in a transform unit.

–	One coding block (ChromaArrayType is equal to 0) or three coding blocks (luma and chroma), the associated coding syntax structures and the associated prediction and transform units units are encapsulated in a coding unit.

–	One coding tree block (ChromaArrayType is equal to 0) or three coding tree blocks (luma and chroma), the associated coding tree syntax structures and the associated coding units are encapsulated in a coding tree unit.

#### 7.4.2.1 Sequence parameter set RBSP semantics

log2_diff_max_min_coding_block_size specifies the difference between the maximum and minimum coding block size.

The variables Log2CtbSize, PicWidthInCtbs, PicHeightInCtbs, PicWidthInMinCbs, and PicHeightInMinCbs are derived as follows.

Log2CtbSize = log2_min_coding_block_size_minus 3 + 3 + log2_diff_max_min_coding_block_size	(2-1)
PicWidthInCtbs = Ceil( pic_width_in_luma_samples ÷ ( 1 << Log2CtbSize ) )	(2-2)
PicHeightInCtbs = Ceil( pic_height_in_luma_samples ÷ ( 1 << Log2CtbSize ) )	(2-3)
PicWidthInMinCbs = PicWidthInSamplesL / ( 1 << Log2MinCbSize )	(2-4)
PicHeightInMinCbs = PicHeightInSamplesL / ( 1 << Log2MinCbSize )	(2-5)

The variable CtbSize, which specifies the width and the height, of the square array for each luma coding tree block, is derived as follows.

CtbSize = 1 << Log2CtbSize		(2-6)

The variables CtbWidthC and CtbHeightC, which specify the width and height, respectively, of the array for each chroma coding tree block, are derived as follows.

–	If ChromaArrayType is equal to 0, CtbWidthC and CtbHeightC are both equal to 0.

–	Otherwise, CtbWidthC and CtbHeightC are derived as

CtbWidthC = CtbSize / SubWidthC		(2-7)
CtbHeightC = CtbSize / SubHeightC

#### 7.3.2.1	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| if( ChromaArrayType ! = 0 ) |  |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| if( ChromaArrayType ! = 0 ) |  |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| if( ChromaArrayType ! = 0 ) |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| if( ChromaArrayType ! = 0 ) { |  |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| } |  |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { [Ed. (JB): PPS parsing dependency on syntax element in SPS.] |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if(slice_type = =P ¦ ¦ slice_type = = B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.3	Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3 && chroma_format_idc ! = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

#### 7.3.2.6	Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| if( ChromaArrayType ! = 0 ){ |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| sao_num_lcu_in_width_minus1 | ue(v) |
| sao_num_lcu_in_height_minus1 | ue(v) |
| sao_one_luma_unit_flag | u(1) |
| if( ChromaArrayType ! = 0 ){ |  |
| if(sao_one_luma_unit_flag) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_enable_flag ){ |  |
| sao_one_cb_unit_flag | u(1) |
| if( sao_one_cb_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| } |  |
| for( ry = 0; ry <= sao_num_lcu_in_height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_lcu_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( ChromaArrayType ! = 0 ){ |  |
| if( sao_cb_enable_flag && ! sao_one_cb_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.2	 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if ( ChromaArrayType != 0 ){ |  |
| if(slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

#### 7.3.3.4 	Adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| if( ChromaArrayType ! = 0 ){ |  |
| alf_cb_enable_flag | u(1) |
| alf_cr_enable_flag | u(1) |
| } |  |
| alf_one_luma_unit_per_slice_flag |  |
| if ( ChromaArrayType != 0 ){ |  |
| if( alf_cb_enable_flag ) |  |
| alf_one_cb_unit_per_slice_flag | u(1) |
| if( alf_cr_enable_flag ) |  |
| alf_one_cr_unit_per_slice_flag |  |
| } |  |
| if( !alf_coef_in_slice_flag ) { |  |
| alf_num_lcu_in_width_minus1 | ue(v) |
| alf_num_lcu_in_height_minus1 | ue(v) |
| } else |  |
| alf_num_lcu_in_slice_minus1 | u(v) |
| endCtbrY = (numCtb − 1 + firstCtbAddr) / numCtbInWidth |  |
| endCtbrX = (numCtb − 1 + firstCtbAddr) % numCtbInWidth |  |
| for( i = 0; i < numCtb; i++ ) { |  |
| rx = ( i + firstCtbAddr) % numCtbInWidth |  |
| ry = ( i + firstCtbAddr) / numCtbInWidth |  |
| endrX = ( ry = = endCtbrY ) ? ( endCtbrX ) : ( numCtbInWidth − 1 ) |  |
| if( ( rx = = 0 ) && ( i − numCtbInWidth >= 0 ) && ( alf_one_luma_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[0] | u(1) |
| alf_unit( rx, ry, 0, i, endrX, alf_one_luma_unit_per_slice_flag ) |  |
| if ( ChromaArrayType != 0 ){ |  |
| if( alf_cb_enable_flag ) { |  |
| if( ( rx = = 0 ) && ( i − numCtbInWidth >= 0 ) && ( alf_one_cb_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[1] | u(1) |
| alf_unit(rx, ry, 1, i, endrX, alf_one_cb_unit_per_slice_flag ) |  |
| } |  |
| if( alf_cr_enable_flag ) { |  |
| if( ( rx = = 0) && ( i − numCtbInWidth>= 0) && ( alf_one_cr_unit_per_slice_flag = =0 ) ) |  |
| alf_repeat_row _flag[2] | u(1) |
| alf_unit(rx, ry, 2, i, endrX, alf_one_cr_unit_per_slice_flag ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.3.3.8 	Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ ( slice_type = = B && ref_pic_list_combination_flag = = 0 ) ) { |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( ChromaArrayType != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| if( ref_pic_list_combination_flag = = 0 ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( ChromaArrayType != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| luma_weight_lc_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_lc[ i ] | se(v) |
| luma_offset_lc[ i ] | se(v) |
| } |  |
| if( ChromaArrayType != 0 ) { |  |
| chroma_weight_lc_flag | u(1) |
| if( chroma_weight_lc_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_lc[ i ][ j ] | se(v) |
| delta_chroma_offset_lc[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.3 	Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if ( ChromaArrayType != 0 ){ |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

### 7.3.7 	Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if ( ChromaArrayType == 1 ¦¦ ChromaArrayType == 2 ){ |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_lc_flag[ x0 ][ y0 ] | ae(v) |
| } else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.3.7.2 	PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| if ( ChromaArrayType != 0 ){ |  |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |
| NumPCMBlock-- |  |
| } |  |

### 7.3.8	Transform tree syntax

| transform_tree( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) ) |  |
| split_transform_flag[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| if ( ChromaArrayType == 1 ¦¦ ChromaArrayType == 2 ){ |  |
| if( trafoDepth = = 0 ¦ ¦ log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0C ][ y0C ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| if( split_transform_flag[ x0L ][ y0L ][ trafoDepth ] ) { |  |
| if( interTbSplitDirectionL = = 2 ) { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1L = y0L |  |
| x2L = x0L |  |
| y2L = y0L + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3L = x1L |  |
| y3L = y2L |  |
| } else { |  |
| x1L = x0L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y1L = y0L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL) |  |
| x2L = x1L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y2L = y1L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL) |  |
| x3L = x2L + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionL |  |
| y3L = y2L + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionL) |  |
| } |  |
| if( InterTUSplitDirectionC = = 2 && log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| xC3 = xC1 |  |
| yC3 = yC2 |  |
| } else if ( log2TrafoSize > 3 ) { |  |
| x1C = x0C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y1C = y0C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC) |  |
| x2C = x1C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y2C = y1C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC) |  |
| x3C = x2C + ( ( 1 << log2TrafoWidth ) >> 2 ) * interTbSplitDirectionC |  |
| y3C = y2C + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − interTbSplitDirectionC) |  |
| } else { |  |
| x1C = x0C |  |
| y1C = y0C |  |
| x2C = x0C |  |
| y2C = y0C |  |
| x3C = x0C |  |
| y3C = y0C |  |
| } |  |
| if( interTbSplitDirectionL! = 2 ) { |  |
| log2TrafoWidth = log2TrafoWidth − 2 * interTbSplitDirectionL + 1 |  |
| log2TrafoHeight = log2TrafoHeight + 2 * interTbSplitDirectionL − 1 |  |
| } |  |
| transform_tree( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

### 7.3.9	 Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( (max_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| if( PredMode = = MODE_INTRA ) { |  |
| scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize − 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0 ) |  |
| if( ChromaArrayType == 1 ¦¦ ChromaArrayType == 2){ |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.2.3 	Scaling list data semantics

Table 7-1 – Specification of MatrixID according to SizeID, prediction type and colour component

| - SizeID | - Prediction type | - Colour component | - MatrixID |
| --- | --- | --- | --- |
| - 0, 1, 2 | - Intra | - Y | - 0 |
| - 0, 1, 2 | - Intra | - Cb | - 1 |
| - 0, 1, 2 | - Intra | - Cr | - 2 |
| - 0, 1, 2 | - Inter | - Y | - 3 |
| - 0, 1, 2 | - Inter | - Cb | - 4 |
| - 0, 1, 2 | - Inter | - Cr | - 5 |
| - 3 | - Intra | - Y | - 0 |
| - 3 | - Inter | - Y | - 1 |
| - 3 | - Intra | - Cb | - 2 |
| - 3 | - Inter | - Cb | - 3 |
| - 3 | - Intra | - Cr | - 4 |
| - 3 | - Inter | - Cr | - 5 |

###### 8.5.2.2.2.2 	Chroma sample interpolation process

...

- If chroma_format_idc is equal to 1, the samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples:

ba0,0 = ( −2*B0,−1 + 58*B0,0 + 10*B0,1 − 2*B0,2 ) >> shift1	(8-8)

ca0,0 = ( −4*B0,−1 + 54*B0,0 + 16*B0,1 − 2*B0,2 ) >> shift1	(8-9)

da0,0 = ( −6*B0,−1 + 46*B0,0 + 28*B0,1 − 4*B0,2 ) >> shift1	(8-10)

ea0,0 = ( −4*B0,−1 + 36*B0,0 + 36*B0,1 − 4*B0,2 ) >> shift1	(8-11)

fa0,0 = ( −4*B0,−1 + 28*B0,0 + 46*B0,1 − 6*B0,2 ) >> shift1	(8-12)

ga0,0 = ( −2*B0,−1 + 16*B0,0 + 54*B0,1 − 4*B0,2 ) >> shift1	(8-13)

ha0,0 = ( −2*B0,−1 + 10*B0,0 + 58*B0,1 − 2*B0,2 ) >> shift1	(8-14)

–	If chroma_format_idc is equal to 1, the samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction:

bX0,0 = ( −2*aX0,−1 + 58*aX0,0 + 10*aX0,1 − 2*aX0,2 ) >> shift2	(8-15)

cX0,0 = ( −4*aX0,−1 + 54*aX0,0 + 16*aX0,1 − 2*aX0,2 ) >> shift2	(8-16)

dX0,0 = ( −6*aX0,−1 + 46*aX0,0 + 28*aX0,1 − 4*aX0,2 ) >> shift2	(8-17)

eX0,0 = ( −4*aX0,−1 + 36*aX0,0 + 36*aX0,1 − 4*aX0,2 ) >> shift2	(8-18)

fX0,0 = ( −4*aX0,−1 + 28*aX0,0 + 46*aX0,1 − 6*aX0,2 ) >> shift2	(8-19)

gX0,0 = ( −2*aX0,−1 + 16*aX0,0 + 54*aX0,1 − 4*aX0,2 ) >> shift2	(8-20)

hX0,0 = ( −2*aX0,−1 + 10*aX0,0 + 58*aX0,1 − 2*aX0,2 ) >> shift2

# Conclusion

It is proposed in this contribution lightweight modifications of current HEVC text draft syntax in order to prepare non-4:2:0 profiles support. It is remarked that special attention should be given to ChromaArrayType = 3 case. It is also requested to consider if such modifications (especially for 4:4:4 chroma format related profiles) should be integrated in the first standard version.

# References

- “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d2, Geneva, Switzerland, May 2012.

- “WD5: Working Draft 5 of High-Efficiency Video Coding”, JCT-VC-G1103_d9, Geneva, Switzerland, Nov. 2011.

- Japan National Body, “JNB comments on HEVC scalable extensions to support non-420, n-bit video”, JCTVC-I0496, Geneva, Switzerland, Apr. 2012.

- US National Body, “USNB Comments on HEVC”, ISO/IEC JTC1/SC29/WG11 m24263, Geneva, Switzerland, Apr. 2012.

- K. Sugimoto, A. Minezawa, S. Sekiguchi, “4:2:2 support in HEVC”, JCTVC-G967, Geneva, Switzerland, Nov. 2011.

- K. Sugimoto, A. Minezawa, S. Sekiguchi, “AHG20: 4:2:2 support in HEVC”, JCTVC-H650, San José, CA, USA, Feb. 2012.

- K. Sugimoto, A. Minezawa, S. Sekiguchi, “AHG14: Color format extension for HEVC”, JCTVC-I0521, Geneva, Switzerland, Apr. 2012.

- T. Lin, S.Wang, K.Zhou, “Subjective quality comparison between Mixed Chroma Sampling-Rate coding and Chroma-subsampled 420 coding using YUV444 screen content test sequences”, JCTVC-I0336, Geneva, Switzerland, Apr. 2012.

- T. Lin, P. Zhang, S.Wang, K.Zhou, “4:4:4 Screen content coding using mixed chroma sampling-rate techniques”, JCTVC-I0272, Geneva, Switzerland, Apr. 2012.

- W. Dai, M. Krishnan, P. Topiwala, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, JCTVC-F618, Torino, Italia, July 2011.

- W. Dai, M. Krishnan, P. Topiwala, “Advanced Resampling Filters for HEVC Applications”, JCTVC-G862, Geneva,  Switzerland, Nov. 2011.

- W. Dai, M. Krishnan, P. Topiwala, “Coding in 444: Preferred Conversions in Color and Sampling”, JCTVC-H0394, San José, CA, USA, Feb. 2012.

- ITU-R WP6C/564-E Annex 4, “Preliminary draft new Recommendation ITU-R BT.[IMAGE-UHDTV] – Parameters values for UHDTV systems for production and international programme exchange”, Oct. 2011.

- Report ITU-R. BT. 2246, “The present state of ultra high definition television”, Oct. 2011.

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.