| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0079 |
| --- | --- |

| Title: | AHG12: On beyond 8 bit-depth support in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon Philippe Bordes Philippe Salmon 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33-2-99-27-38-07 pierre.andrivon@technicolor.com philippe.bordes@technicolor.com philippe.salmon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports an analysis of HEVC text Draft 7 as well as experiments results of Test Model 7 (HM7.0 and HM7.1rc1) with regards to beyond 8 bit-depth signals coding support, namely 10, 12 and 14-bit. It is reported that no major issues have been identified for 8, 10 and 12 bit-depths for both HEVC Draft 7 d4 and HM7.0 (RA-HE and AI-HE). It is claimed that several issues appear for 14 bit-depth and fixes are proposed to increase 14 bit-depth support. 14-bit coding reportedly shows consistent results with 8, 10 and 12 bit-depth coding with proposed global patch. Besides, it is stated that HM7.0 picture-level lossless coding for 10, 12, 14 bits reconstruct perfectly pictures.

# Introduction

It was proposed in [2] to start a beyond 8 bit-depth support analysis of both HEVC Test Model and text Draft. Consequently, an existing AHG on non-4:2:0 chroma format support was broadened in order to support this initiative that prepares the ground for professional and/or UHDTV-related (which draft shows 10 and 12 bits considerations) HEVC extensions and profiles. This contribution attempts to  reply to some of “AHG12 Support for range extension” mandates.

# An analysis of HEVC Draft 7 and HM7.0/HM7.1rc1

## Intra transform skipping and (de)quantization

As of last (9th) JCT-VC meeting in Geneva, tickets and suggestions were proposed leading to fixes in HEVC text Draft 7 and HM7.0 accordingly.

It was reported a limitation in intra transform skipping that prevents this tool to work with bit-depth beyond 13 bits because of negative shifting issue (ticket #560). A pending patch for HM7.0 and HM7.1rc1 is proposed and text modifications were incorporated in Draft 7 d3.

Following this issue, it was remarked (ticket #560 yet) that in HM7.0 the variable iTransformShift (in TComTrQuant::xQuant() and TComTrQuant::xDequant()) is actually declared as an unsigned integer (variable misnaming) and may be negative for bit-depth beyond 10 bits leading to underflow (data corruption).

iTransformShift = MAX_TR_DYNAMIC_RANGE - uiBitDepth - uiLog2TrSize;

Indeed, MAX_TR_DYNAMIC_RANGE = 15, uiLog2TrSize = 2 to 5, uiBitDepth = 8 to 14.
So worse case is iTransformShift = 15 – 14 – 5 = -4.

It is noticed that iTransformShift is declared as an integer in:
 TComTrQuant::xRateDistOptQuant()and TComTrQuant::setErrScaleCoeff().

It is suggested in a common patch to declare the variable iTransformShift as an integer value (the least modifications to carry out) or to handle the negative shift by introducing an opposite temporary variable like proposed in intra transform skipping patch. The first proposal is preferred as a code analysis shows that all other variables using iTransformShift when having a negative value (e.g. min(iQbits)=10, min(iQBitsC)=3, min(qBits8)=2, min(iShift)=1...) remains in their variable. It is also claimed that unsigned integer underflow introduced subsequent overflow/underflow cascading that have no influence on results for software platforms so that patch does not change results while fixing a weakness. A patch fixing all those mentioned issued was proposed in ticket #560.

## Deblocking filter process (Draft 7)

In deblocking filter process, scaling of β and tC, that was deservedly performed in HM, is now specified accordingly in text Draft 7 d2 (ticket #529). No other issue with regards to up to 14 bits bit-depth were found both in text Draft 7 d4 (8.7.1) and HM7.0/7.1rc1.

## Weighted prediction

In Draft 7 d4 8.5.2.2.3 Weighted sample prediction process is described the variable offset1 set equal to 1<<(shift1-1) however shift1 is set equal to 14 – bitDepth leading to a negative shift when bitDepth is set to 14.  As proposed in ticket #594, a straightforward lightweight solution would be to set offset1 equal to 0 when shift1 is lesser than 0 (14-bit and beyond case).

In HM7.1rc1, TComWeightPrediction::addWeightBi()and

TComWeightPrediction::addWeightUni() offset issue appearing three times (luma and chroma) for each. As IF_INTERNAL_PREC is set to 14 and shiftNum (14-bit case)  may be null,  a negative shift may appear.  A suggested straightforward fix (ticket #595) would be to set round to 0 when shift is null.

Example for luma with proposed fix in red:
{
…
  Int shiftNum = IF_INTERNAL_PREC - ( g_uiBitDepth + g_uiBitIncrement );

Int shift   = wp0[0].shift + shiftNum;
  Int round   = shift?(1<<(shift-1)) * bRound:0;
…
}

## Interpolation filter (HM7.1rc1)

Same offset issues as indicated in Weighted Prediction process occur also here however with the straightforward solution proposed approach above there would be nothing to change in Draft 7 d4.

As IF_INTERNAL_PREC is fixed to 14 then shift may reach 0 value (14 bit-depth) that is an issue for offset as it begets a negative shift. As mentioned above it is suggested to set offset to IF_INTERNAL_OFFS + 0 when shift is null (14-bit). Patch is also proposed in ticket #595.

In HM7.1rc1 TComInterpolationFilter::filterCopy()

{

…

Int shift = IF_INTERNAL_PREC - ( g_uiBitDepth + g_uiBitIncrement );
Short offset = IF_INTERNAL_OFFS ;//(+ (1 << (shift - 1));)

offset += shift?1<<(shift - 1):0;

…

}

## RDOQ (HM7.1rc1)

Several variables overflows have been identified in HM7.1rc1 for beyond 10 bit-depth when using RDOQ non-normative technique (TComTrQuant::xRateDistOptQuant()). This is addressed in ticket #596. Indeed rdFactor, minCostInc, costUp and costDown integer variables were proven to overflow for 14 bit-depth and are fixed by moving them to 64-bit integer.

It is remarked that ticket #597 also fixes bit-depth scaling for beyond 8 bit-depth thus a merged patch would improve encoder performances for beyond 8 bit-depth. This merged patch was not integrated in provided patch at the time of writing.

## Some other processes compliancy

### Draft 7

Scaling and transformation (8.6) (especially Quantization parameters (8.6.1), Scaling (8.6.3) and transform (8.6.4)), SAO (8.7.2), PCM (8.4), inter (8.5) excluding weighted prediction (8.5.2.2.3) but including luma/chroma sample interpolation (8.5.2.2.2.1/2)  and intra (8.4) except Intra_FromLuma (8.4.3.1.8 not analyzed yet),  prediction processes and parsing process (9) seem to cope with up to 14 bit-depth.

It is noticed that equation (9-5) contains Clip3(0, 51, SliceQPY) and yet SliceQPY range is indeed in the range of –QpBdOffsetY to +51 inclusive meaning that preCtxState initialization does not take bit-depth into account (same as in AVC).

It is remarked that at the time of writing, ALF process (8.7.3) modifications were not all incorporated in Draft text 7 but one limitation was identified and fixed (fixed by JCTVC-I0040 Cf. ticket #466). Further study will be provided when the Draft text will be up-to-date.

### HM7.1rc1

It is claimed that scaling, transformation, quantization, SAO in HM7.0 and HM7.1rc1 as well as fixed modules presented in above sections can support up to 14-bit encoding and decoding.

# 8 to 14 bit-depth experiments in HM7.0/HM7.1rc1

## Setup for lossy coding

Same 6 sequences (SVT) and conditions as those presented in [2] are used. Considering JM18.3, tests were performed in order to check that results match with those presented in [3] with HEVC regular test material. Tests are performed on the 250 first frames (5 seconds). Remark concerning beyond 8 bit-depth PSNR computation in HM is maintained.

Considering lossy compression, test conditions are All Intra – High Efficiency (AI-HE) and Random Access – High Efficiency (RA-HE) as described in [4] and provided with configuration files of HM7.0/HM7.1rc1 software. Minor modifications have been brought in those configuration files in order to take into account bit-depth. Actually InternalBitDepth and InputBitDepth are set to 8, 10, 12, 14 respectively for 8-bit, 10-bit, 12-bit, 14-bit experiment. Colour space used is Y’CbCr according to ITU-R BT.709 with 4:2:0 chroma subsampling. It is noticed that between HM6.0 and HM7.0 tools evolved but one new tool was adopted and is present in HE configuration test set: intra transform skipping that is described in [5].

Both intra transform skipping as well as proposed patches suggested in ticket #560 are applied on top of HM7.0.

## Experimental results for lossy coding

Generally speaking, according to Table 1 and Table 2, results are roughly close from those presented in [2] then differential results are proposed hereafter. One may observe small gains for chroma to the detriment of luma (balance effect). HM6.0 versus HM7.0 results are provided within the contribution package for  information. Results presented here are piecewise-cubic BD-rate.

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | 0.3% | -3.1% | -5.7% |
|  | 10 bits | 0.0% | -3.2% | -5.7% |
|  | 12 bits | 0.1% | -2.3% | -4.0% |
|  | 14 bits | 0.3% | -3.2% | -3.9% |

Table 1 HM6.0 vs HM7.0 All Intra HE

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | 1.3% | -1.1% | -3.5% |
|  | 10 bits | 1.5% | -1.3% | -3.2% |
|  | 12 bits | 1.7% | 0.4% | -1.1% |
|  | 14 bits | 1.7% | 0.0% | -1.3% |

Table 2 HM6.0 vs HM7.0 Random Access HE

It is then decided to present global results between JM18.3 and HM7.0.

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | -18.1% | -31.3% | -27.1% |
|  | 10 bits | -17.7% | -30.2% | -24.8% |
|  | 12 bits | -17.3% | -29.6% | -23.6% |
|  | 14 bits | -12.8% | -23.7% | -15.6% |

Table 3 JM18.3 HP vs HM7.0 All Intra HE

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | -31.3% | -50.6% | -43.4% |
|  | 10 bits | -31.8% | -47.7% | -35.9% |
|  | 12 bits | -31.2% | -46.6% | -33.8% |
|  | 14 bits | * | * | * |

Table 4 JM18.3 HP vs HM7.0 Random Access HE

(*) As a remark, it is claimed that JM18.3 crashes for most of sequences used when considering 14 bits 4:2:0 RA-HE test configuration. This issue was signaled in bug tracker and a fix is under way.

Results drop appears yet for AI-HE 14 bits test configuration.

In order to manage to appreciate RA-HE 14 bits results consistency a comparison across bit-depth is proposed for HM7.0.

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 vs 10 bits | -1.0% | -7.4% | -7.1% |
|  | 8 vs 12 bits | -1.9% | -9.2% | -7.8% |
|  | 8 vs 14 bits | 0.4% | -1.9% | -3.3% |

Table 5 HM7.0 Random Access HE, coding efficiency comparison across bit-depth

It is reported that a result drop is observed for RA-HE 14 bits too confirming the issue observed with AI HE test configuration. 14-bit issues have been indicated (Cf. ticket #560, ticket #594, ticket #595, ticket #596 and subpart 2) and a global patch is provided as well as Draft 7 modifications.

The global ticket #596 patch is applied and results between JM18.3 HP and HM7.1rc1 RA-HE/AI-HE are presented. It was chosen to provide a patch on top of HM7.1rc1 that was released as the latest stable version in order to ease integration within last HM version.

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | -18.1% | -31.3% | -27.1% |
|  | 10 bits | -17.7% | -30.2% | -24.8% |
|  | 12 bits | -17.3% | -29.6% | -23.6% |
|  | 14 bits | -17.1% | -28.7% | -22.2% |

Table 6 JM18.3 HP vs HM7.1rc1 All Intra HE

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 bits | -31.3% | -50.6% | -43.4% |
|  | 10 bits | -31.8% | -47.8% | -35.9% |
|  | 12 bits | -31.4% | -46.7% | -33.9% |
|  | 14 bits | * | * | * |

Table 7 JM18.3 HP vs HM7.1rc1 Random  Access HE

| Class B 1080p SVT sequences | Bit-depth | Y | U | V |
| --- | --- | --- | --- | --- |
|  | 8 vs 10 bits | -1.0% | -7.4% | -7.1% |
|  | 8 vs 12 bits | -2.1% | -9.3% | -8.0% |
|  | 8 vs 14 bits | -2.0% | -7.2% | -5.7% |

Table 8 HM7.1rc1 Random Access HE, coding efficiency comparison across bit-depth

HM7.0 and HM7.1rc1 results are very close except for 14 bit-depth where gains are reported in next table.

| Class B 1080p SVT sequences | GoP | Y | U | V |
| --- | --- | --- | --- | --- |
|  | AI-HE 14-bit | -5.0% | -7.2% | -8.2% |
|  | RA-HE 14-bit | -2.4% | -5.1% | -2.3% |

Table 9 HM7.0 vs HM7.1rc1 with 14-bit proposed patch

HM7.1rc1 with 14-bit patch unveils results that are more consistent with 8, 10, 12 bit-depth behavior.

## Setup for lossless coding

In order to check lossless compression in HM7.0 at the picture level with regards to high bit-depth, AI-HE, RA-HE, LD-HE, LP-HE configuration files were modified as follows:
- QP  6*(8 - bit_depth_minus8) with bit_depth_luma_minus8 = bit_depth_chroma_minus8 = bit_depth_minus8. So QP 0 (8-bit), QP -12 (10-bit), QP -24 (12-bit), QP -36 (14-bit).
- LoopFilterDisable  1 (disable deblocking filter)
- SAO  0 (disable SAO in-loop filter)
- ALF  0 (disable ALF in-loop filter)
- TS  1
- PCMEnabledFlag  1 (enable PCM coding)
- LosslessCuEnabled  1

Typedef.h: SEQUENCE_LEVEL_LOSSLESS macro was set to 1 as well. Other parameters may be switched on or off within the program due to lossless constraint. (e.g. RDOQ makes then not sense for picture-level lossless coding).

## Experimental results for lossless coding

All tests perform (AI-HE, LP-HE, LB-HE, RA-HE for six class B-like sequences (SVT) with 8, 10, 12, 14 bit-depth) for picture-level lossless coding show perfect reconstruction with no mismatch between encoder and decoder (HM7.0).

|  | Compression ratio |  |  | Compression ratio |
| --- | --- | --- | --- | --- |
| All Intra HE | 1.70 |  | All Intra HE | 1.38 |
| Random Access HE | 1.96 |  | Random Access HE | 1.54 |
| Low Delay B HE | 1.96 |  | Low Delay B HE | 1.54 |
| Low Delay P HE | 1.93 |  | Low Delay P HE | 1.52 |

Table 10 HM7.0 lossless 8-bit					Table 11 HM7.0 lossless 10-bit

|  | Compression ratio |  |  | Compression ratio |
| --- | --- | --- | --- | --- |
| All Intra HE | 1.14 |  | All Intra HE | 1.01 |
| Random Access HE | 1.24 |  | Random Access HE | 1.02 |
| Low Delay B HE | 1.24 |  | Low Delay B HE | 1.02 |
| Low Delay P HE | 1.22 |  | Low Delay P HE | 1.01 |

Table 12 HM7.0 lossless 12-bit 					Table 13 HM7.0 lossless 14-bit

Compression ratios reported here are inferior to those presented in [6] however SVT sequences are known to be noisy (scan from silver film). Noise can also explain drop of coding efficiency across bit-depth as for higher bit-depth mostly noise (unlikely correlated) is significant and might be hard to be predicted so hard to be lossless encoded. Further studies such that removing significant LSB bits (set to 0) may confirm that hypothesis. Besides, It is remarked that decrease slope representing the evolution of compression ratio across bit-depth is regular (globally, a loss of about 10% compression ratio per additional bit).

Improvements were added in HM7.1rc1 with respect to lossless coding and further studies should be performed to assess the impact on beyond 8 bit-depth lossless coding.

# Conclusion

This contribution presented an analysis of HEVC Draft 7 d4 and HM7.0/HM7.1rc1 with respect to beyond 8 bit-depth support in HEVC. Proposed modifications address several identified bit-depth issues so that 14-bit support could be increased. It is stated that picture-level lossless coding (HM7.0) reconstructs picture perfectly with a loss in coding efficiency across bit-depth.

# References

- “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d4, Geneva, Switzerland, May 2012.

- P. Andrivon, P. Bordes, “High Bit Depth Considerations in HEVC”, JCTVC-I0108, Geneva, Switzerland, Apr. 2012.

- B. Li, G.J. Sullivan, J. Xu, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile”, JCTVC-I0409, Geneva, Switzerland, Apr. 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, Switzerland, Apr. 2012.

- C. Lan, J. Xu, G.J. Sullivan, F. Wu, “Intra transform skipping”, JCTVC-I0408, Geneva, Switzerland, Apr. 2012.

- M. Zhou, “AHG13: Sample-based angular intra prediction for HEVC lossless coding”, Geneva, Switzerland, Apr. 2012.

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.