| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0089 Mxxxxx |
| --- | --- |

| Title: | Bin reduction for delta QP coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze, Madhukar Budagavi Texas Instruments Vadim Seregin, Joel Sole, Marta Karczewicz Qualcomm | Email: Email: | sze@ti.com vseregin@qualcomm.com |
| Source: | Texas Instruments and Qualcomm |  |  |

_____________________________

# Abstract

To improve the throughput of the CABAC, the number of context coded and bypass bins should be reduced.  Hardware is designed for the worst case, thus it is also important to reduce the worst case number of context coded bins.  In HM-7.0, cu_qp_delta accounts for 18% of the context coded bins in the worst case.  This is due to the fact that it is binarized with truncated unary and all the bins are context coded.  This contribution proposes using a combination of truncated unary and Exp-Golomb coding (TUEGk), similar to what is already used for motion vector difference.  Only the truncated unary portion is context coded and the Exp-Golomb portion is bypass coded. Furthermore, the bypass bins are grouped for throughput improvement; specifically, the sign bin is signaled after the magnitude bins. These modifications were implemented in HM-7.0 for TU with cMax=5 and Exp-Golomb, with k=0. Asserted benefits of this proposal include: 5x reduction of worst case context bins (from 26 to 5), ~2x reduction of total bin (from 27 to 15), grouping of bypass bins, simplified binarization process, and reduction in number of contexts by 1. The worst case number of bins for 8-bits and 10-bits are the same (0.06 context coded bins/pixel and 0.22 total bins/pixel).  Under the test conditions set by the AHG5 on entropy coding improvements, where cu_qp_delta is signaled for every 8x8 CU, and this proposal has overall BD-rate between -0.01 to 0.03.

# Introduction

CABAC is a well known throughput bottleneck of the video codec.  This is primarily due to the data dependencies in context selection which make it difficult to parallelize.  Binary symbols (bins) can be either context coded or bypass coded.  Since bypass coded bins do not require context selection, they can be processed at a higher throughput.  Thus, to improve the throughput of the CABAC, the number of context coded bins should be reduced.  Hardware is designed for the worst case, thus it is also important to reduce the worst case number of context coded bins and overall number of bins.

Truncated unary (TU) binarization is used to map the syntax element value to bins.  Unary involves signaling a bin string of length N+1, where the first N bins are 1 and the last bins is 0.  The decoder searches for a 0 to determine when the syntax element is complete. Truncated unary coding has one less bin that unary coding by setting a maximum on the largest possible value of the syntax element (cMax).   An example of truncated unary coding is shown in Table 1 with cMax =26.

Table 1: Example of truncated unary binarization with cMax=26

| Value | Bin string |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 11110 |
| 5 | 111110 |
| 6 | 1111110 |
| 7 | 11111110 |
| 8 | 111111110 |
| … | … |
| 26 | 1111 1111 1111 1111 1111 1111 1 |

## Number of bins

In HM-7.0, cu_qp_delta uses TU binarization and all the bins are context coded.  When cMax is a large value, the number of context coded bins can be prohibitively large.  For instance, cu_qp_delta has a large range from –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2),  and cMax has a value of ( 26+ QpBdOffsetY / 2 ),  which means in the worst case ( 26+ QpBdOffsetY / 2 ) context coded bins are signalled for each cu_qp_delta.

For Main Profile, where QpBdOffsetY =0, cMax=26 which means in the worst case a maximum of 26 context coded bins are signalled per coding unit (CU), or equivalently 26/8x8 = 0.40625 context coded bins/pixel.  For the worst case, delta QP accounts for 18% of the context coded bins in an LCU.

For HE10, where QpBdOffsetY =12, the number of context coded bins per pixel is 32/8x8=2 context coded bins/pixel

## Sign bin

In addition, in HM-7.0 where only TU is used, the cMax of TU is depends on the sign value of cu_qp_delta.  Thus, the sign must be sent before the magnitude bins.  This results in interleaving of bypass and context coded bins.  To code cu_qp_delta requires switching from context coded bin, to bypass and then to context coded bins again.

This requires a separate method of binarization in section 9.2.2.6. of the spec as shown below:

absV = Abs( synVal ) if( absV = = 0 ) put( 0 ) else { put( 1 ) signV = ( synVal > 0 ) ? 0 : 1 put( signV ) cMax = 24 + ( QpBdOffsetY >> 1 ) + signV cNum = absV − 1 absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1 put( absVGreaterThan1Flag ) if( absVGreaterThan1Flag ) { while( cNum− − ) put( 1 ) if( cMax > absV − 1 ) put( 0 ) } }

# Proposal

Rather than using only truncated unary binarization, this contribution proposes using only truncated unary for the prefix, which a much smaller cMax, and using kth order Exp-Golomb coding as the suffix. Only the bins of the prefix are context coded, while the bins of the suffix are bypass coded.   This is similar to how coeff_abs_level_minus1 was coded in H.264/AVC and how motion vector difference (mvd) is currently coded in HEVC.  Table 2 shows an example of truncated unary combined with Exp-Golomb.  For value 26, rather than having 26 context coded bins when Table 1 (pure truncated unary) is used, the proposed method in Table 2 (truncated unary + Exp-Golomb), only 5 context coded bins are required (plus 9 bypass coded bins).

Table 2: Example of truncated unary binarization with cMax=5 for prefix and 0th order Exp-Golomb for suffix

| Value | Bin string (truncated unary) | Bin string (0th order Exp-Golomb) |
| --- | --- | --- |
| 0 | 0 |  |
| 1 | 10 |  |
| 2 | 110 |  |
| 3 | 1110 |  |
| 4 | 11110 |  |
| 5 | 11111 | 0 |
| 6 | 11111 | 100 |
| 7 | 11111 | 101 |
| 8 | 11111 | 11000 |
| … | … | … |
| 26 | 11111 | 111100110 |

With TU5EG0, there is no dependency on the sign bin.  Thus, the sign is moved to the end of the cu_qp_delta and grouped with the bypass bins of the suffix.

To summarize, the benefits of using TU5EG0 over the HM-7.0 where a separate binarization method using only TU is used are:

- Reduction of total context coded bins in worst case (26 bins  5 bins)

- Reduction of total bins in worst case  (27 bins  15 bins)

- Group bypass bins and fewer switches between context coded bins and bypass

- Removes dependency of cMax on sign value

- Reduction of number of contexts from 3 to 2.

For Main Profile, where QpBdOffsetY =0, cMax=26 which means in the worst case a maximum of 5 context coded bins are signalled per coding unit (CU), or equivalently 5/8x8 = 0.08 context coded bins/pixel.  For HE10, where QpBdOffsetY =12, the number of context coded bins per pixel is also 0.08.

Since delta QP is not signaled under common test conditions, it was tested in HM-7.0 under the conditions specified by AHG5 on entropy coding improvements, where cu_qp_delta is sent for every 8x8 CU.  There is negligible coding efficiency impact between -0.01 to 0.03.

# Experiment Results

The HM-7.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz, 64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed [2].

For cu_qp_delta, HM-7.0 was used under the conditions set by AHG5 on entropy coding improvements.

#======== Quantization ============= 
QP                            : 32          # Quantization parameter(0-51) 
MaxDeltaQP                    : 0           # CU-based multi-QP optimization 
MaxCuDQPDepth                 : 3           # Max depth of a minimum CuDQP for sub-LCU-level delta QP 
DeltaQpRD                     : 0           # Slice-based multi-QP optimization 
RDOQ                          : 1           # RDOQ 
ChromaQpOffset                : 0           # ChromaQPOffset 
ChromaQpOffset2nd             : 0           # ChromaQPOffset2nd 
AdaptiveQP                    : 1           # QP adaptation based on a psycho-visual model (0: None, 1: TM5Step3) 
MaxQPAdaptationRange         : 12          # dQP range of QP adaptation

Table 1: Coding efficiency results for changing cu_qp_delta to TU and Exp-Golomb.

|  |  | MAIN |  |  |  |  |  |  |  |  | HE10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI |  |  | RA |  |  | LB |  |  | AI |  |  | RA |  |  | LB |  |  |  |
| TU4EG0 |  | 0.03 |  |  | 0.04 |  |  | 0.01 |  |  | 0.03 |  |  | 0.03 |  |  | 0.04 |  |  |  |
| TU5EG0 |  | 0.02 |  |  | 0.02 |  |  | -0.01 |  |  | 0.02 |  |  | 0.02 |  |  | 0.03 |  |  |  |
|  | AHG5 Summary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | MAIN |  |  |  |  |  |  |  |  | HE10 |  |  |  |  |  |  |  |  |
|  |  |  | max bins per syntax element |  |  | max context bins per syntax element |  |  | max bypass bins per syntax element |  |  | max bins per syntax element |  |  | max context bins per syntax element |  |  | max bypass bins per syntax element |  |  |
|  |  |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  |
|  | ref_idx_lx |  | 15 |  |  | 15 |  |  | 0 |  |  | 15 |  |  | 15 |  |  | 0 |  |  |
|  | cu_qp_delta |  | 27 | 15 |  | 26 | 5 |  | 1 | 10 |  | 33 | 15 |  | 32 | 5 |  | 1 | 10 |  |
|  | sao_offset |  | 9 |  |  | 8 |  |  | 1 |  |  | 33 |  |  | 32 |  |  | 1 |  |  |
|  |  |  | MAIN |  |  |  |  |  |  |  |  | HE10 |  |  |  |  |  |  |  |  |
|  |  |  | max bins per sample |  |  | max context bins per sample |  |  | max bypass bins per sample |  |  | max bins per sample |  |  | max context bins per sample |  |  | max bypass bins per sample |  |  |
|  |  |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  | HM-7.0 | This Work |  |
|  | ref_idx_lx |  | 0.94 |  |  | 0.94 |  |  | 0.00 |  |  | 0.94 |  |  | 0.94 |  |  | 0.00 |  |  |
|  | cu_qp_delta |  | 0.42 | 0.23 |  | 0.41 | 0.08 |  | 0.02 | 0.16 |  | 0.52 | 0.23 |  | 0.50 | 0.08 |  | 0.02 | 0.16 |  |
|  | sao_offset |  | 0.42 |  |  | 0.38 |  |  | 0.05 |  |  | 1.55 |  |  | 1.50 |  |  | 0.05 |  |  |

# Summary

This contribution proposes using truncated unary as the prefix and Exp-Golomb as the suffix for cu_qp_delta.  In order to reduce the context coded bins in the worst case, only the prefix bins are context coded.  It also proposes moving the sign bin to after the entire magnitude of cu_qp_delta is signaled.  The proposal also provides benefits in terms of reduction in total number of bins, grouping of bypass bins for throughput and simplified binarization. We recommend adopting the proposed method of TU5EG0 for cu_qp_delta for in HEVC.

# References

- V. Sze et al, “Reduction in context coded bins for ref_idx and cu_qp_delta,” JCTVC-I0204, Geneva, Apr. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.

# Working Draft text

This text modification for working draft is based on JCTVC-I1003_d4.

7.3.9. Transform unit syntax

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ¦ ¦ cbf_cb[ x0C ][ y0C ][ trafoDepth ] ¦ ¦ cbf_cr[ x0C ][ y0C ][ trafoDepth ] { |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| abs_cu_qp_delta | ae(v) |
| if (cu_qp_delta > 0 ) |  |
| cu_qp_delta_sign |  |
| CuQpDelta = abs_ cu_qp_delta * ( 1 − 2 * cu_qp_delta_sign ) |  |
| IsCuQpDeltaCoded = 1 |  |
| } |  |

abs_cu_qp_delta specifies the absolute value of the difference between a luma quantization parameter to be used and its prediction for a coding unit quantization group where the coding unit quantization group is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CbSize is greater than or equal to Log2MinCUDQPSize, the coding unit quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CbSize is equal to Log2MinCUDQPSize, the coding unit quantization group includes all coding units split from this coding unit.

The decoded value of CuQPDelta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. abs_cu_qp_delta is inferred to be equal to 0 when it is not present for any coding unit quantization group.

The luma quantization parameter QP’Y and the chroma quantization parameters QP’Cb and QP’Cr are derived by invoking the process specified in subclause Error: Reference source not found with ( x0 ,y0 ) as inputs.

cu_qp_delta_sign specifies the sign of a cu_qp_delta as follows.

- If cu_qp_delta _flag is equal to 0, the corresponding cu_qp_delta has a positive value.

- Otherwise (cu_qp_delta_sign is equal to 1), the corresponding cu_qp_delta has a negative value.

When cu_qp_delta_sign is not present, it is inferred to be equal to 0.

Table 9-1 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | cu_qp_delta | Table 9-2 | 0..3 0..1 | 4..7 2..3 | 8..11 4..5 |

Table 9-2 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 |

| Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initialisationType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| abs_cu_qp_delta | 0 | Prefix: TU, cMax=5 Suffix: EG0 | Prefix: 2 Suffix: na | Prefix: Table 9-2 Suffix: na | Prefix: 0 Suffix: na, (uses Decode Bypass) |
|  | 1 |  | Prefix: 2 Suffix: na | Prefix: Table 9-2 Suffix: na | Prefix: 2 Suffix: na, (uses Decode Bypass) |
|  | 2 |  | Prefix: 2 Suffix: na | Prefix: Table 9-2 Suffix: na | Prefix: 4 Suffix: na, (uses Decode Bypass) |
| cu_qp_delta_siqn | 1/2 | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

9.2.2.6 Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The prefix bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	cNum = min (absV − 1,cMax)
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}

If absVal-1 > =cMax, then the suffix absVal-1-cMax is binarized using Exp-Golomb (EG0) process defined in 9.2.2.4

| Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| abs_cu_qp_delta | Table 9-2 | 0 | 0 | 1 | 1 | 1 | 1 |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |

# Patent rights declaration(s)

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).