| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0090_r1 |
| --- | --- |

| Title: | AHG6: Transform Dependent Deblocking Strength |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Visual quality evaluations have demonstrated that certain video content suffers from severe blocking artifacts that are transform size dependent and are particularly visible for the maximum transform size, as specified in the SPS syntax. This maximum TU size is encoder implementation dependent and, therefore, it is important that the HEVC deblocking filter process is flexible and general enough to reduce this type of blocking artifacts. This contribution proposes to signal specific deblocking adjustment parameters to control the deblocking strength in case that at least one of two adjacent video blocks P and Q is included in a TU of maximum size. The advantage of this method is not only applying stronger deblocking filtering to the edges of the TUs of maximum size, but more importantly, this method avoids oversmoothing in picture areas that are unaffected by these largest blocking artifacts. This contribution also proposes a stronger β threshold curve with increased β values outside of the QP range of the common test conditions (>41), but within the QP range of the current HM7 curve. The advantage is that the deblocking strength can be further increased beyond the HM7 strength without increasing the number of values to be stored in memory. The experimental results report on the maximum TU size deblocking method with the HM7 β threshold curve and the proposed curve. Visual quality examples illustrate the reduction of the blocking artifacts originating from the maximum TU sizes. It is proposed to adopt the maximum TU size deblocking method with the stronger β threshold curve into the HEVC DIS.

# Problem Description

Because of the significantly different psycho-visual effect of blocking artifacts with widely varying sizes (transform unit sizes 4×4 up to 32×32), the application of a single offset to the QP value for looking up tC or β deblocking parameters has a different subjective quality impact as well as BD-rate impact for each block size [2][3].

Visual quality evaluations have demonstrated that certain video content suffers from severe blocking artifacts that are transform size dependent. These observed blocking artifacts are mainly visible at relatively high QP values (> 30) and for the maximum transform size as specified by the SPS syntax parameters log2_diff_max_min_transform_block_size and log2_min_transform_block_size_minus2 [1].

These large blocking artifacts are most visible if the maximum transform size is 32x32, however, they are also visible if the maximum transform size is 16x16 or 8x8, while less visible for size 4x4. Therefore, this solution proposes to signal specific deblocking adjustment parameters to control the deblocking strength in case that at least one of two adjacent video blocks P and Q is included in a transform unit with the maximum transform size.

The advantage of this method is not only applying stronger deblocking filtering to the edges of the TUs of maximum size, but more importantly, this method avoids oversmoothing in picture areas that are unaffected by these largest blocking artifacts as opposed to methods that apply strong filtering to every block edge independent of TU size.

The following figures illustrate strong blocking artifacts in the “Riverbed” and “WestWindEasy” sequences [5] coded according to the Lowdelay-B-Main test condition and with QP set to 37 (no tC or β offset is applied). In Figure 1 and 4 the maximum TU is the default size of 32x32, while in Figure 2 and 5 it is 16x16 and in Figure 3 and 6 it is 8x8. This maximum TU size may be encoder implementation dependent and, therefore, it is important that the HEVC deblocking filter process is flexible and general enough to reduce these maximum TU size artifacts.

Figure 1  Frame 1 (cropped) of the “Riverbed” sequence demonstrating strong 32x32 blocking artifacts.

Figure 2  Frame 1 (cropped) of the “Riverbed” sequence demonstrating strong 16x16 blocking artifacts.

Figure 3  Frame 1 (cropped) of the “Riverbed” sequence demonstrating strong 8x8 blocking artifacts.

Figure 4 Frame 1 (cropped) of the “WestWindEasy” sequence demonstrating strong 32x32 blocking artifacts.

Figure 5 Frame 1 (cropped) of the “WestWindEasy” sequence demonstrating strong 16x16 blocking artifacts.

Figure 6 Frame 1 (cropped) of the “WestWindEasy” sequence demonstrating strong 8x8 blocking artifacts.

# HEVC Deblocking Adjustment Parameters

In the HEVC committee draft [1], deblocking filter adjustment parameters may be signaled in the PPS and may be overridden in the slice header, as specified in Table 1 and Table 2.

Table 1 PPS syntax

| … |  |
| --- | --- |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |

Table 2 Slice header syntax

| … |  |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_overriding_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| slice_header_beta_offset_div2 | se(v) |
| slice_header_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

The deblocking adjustment parameters that offset the QP value (average of QP values of neighboring P and Q blocks [1]) before looking up the tC or β thresholds are: pps_beta_offset_div2, pps_tc_offset_div2, slice_header_beta_offset_div2, slice_header_tc_offset_div2 [1]. Neither of these offset parameters supports transform size dependent control of the deblocking filter strength.

# Proposal Description

## Deblocking Strength Adjustment Parameters for Max TU Size Edges

It is proposed to signal specific deblocking adjustment parameters for offsetting tC and β deblocking thresholds to control the deblocking strength in case that at least one of two adjacent video blocks P and Q is included in a transform unit with the maximum transform size. The common edge between blocks P and Q is subject to deblocking filtering. If neither video block P or Q is included in a maximum sized transform unit, then the current deblocking adjustment parameters apply. In addition to the proposed parameters, a flag is signalled to enable the signalling of the additional offset parameters. The following tables illustrate the syntax changes.

Table 3 PPS syntax including proposed deblocking parameters for maximum TU size edges

| … |  |
| --- | --- |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| deblocking_max_tu_offset_enabled_flag | u(1) |
| if (deblocking_max_tu_offset_enabled_flag) { |  |
| pps_beta_offset_max_tu | se(v) |
| pps_tc_offset_max_tu_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

Table 4 Slice header syntax

| … |  |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_overriding_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| slice_header_beta_offset_div2 | se(v) |
| slice_header_tc_offset_div2 | se(v) |
| if (deblocking_max_tu_offset_enabled_flag) { |  |
| slice_header_beta_offset_max_tu | se(v) |
| slice_header_tc_offset_max_tu_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

The semantics of the proposed syntax parameters is as follows (parameter values are inferred 0 if not present):

deblocking_max_tu_offset_enabled_flag enables the signalling of the beta_offset_max_tu and tc_offset_max_tu_div2 syntax parameters if the flag value is equal to 1, otherwise, these parameters are not signalled and their value is inferred to be equal to the corresponding beta_offset_div2 (times 2) and tc_offset_div2 parameters that are inferred to be equal to 0 if not present.

pps_beta_offset_max_tu / slice_header_beta_offset_max_tu specify the default deblocking parameter offset for β that is applied to edges of the maximum transform unit size in pictures referring to the picture parameter set unless the default deblocking parameter offset for β is overridden by the deblocking parameter offset present in the slice header for pictures referring to the picture parameter set.

pps_tc_offset_max_tu_div2 / slice_header_tc_offset_max_tu_div2 specify the default deblocking parameter offset for tC (divided by 2) that is applied to edges of the maximum transform unit size in pictures referring to the picture parameter set unless the default deblocking parameter offset for tC is overridden by the deblocking parameter offset present in the slice header for pictures referring to the picture parameter set.

The semantics of the current adjustment parameters is modified as follows:

pps_beta_offset_div2 / slice_header_beta_offset_div2 and pps_tc_offset_div2 / slice_header_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to edges of transform units that are not also edges of the maximum transform unit size in pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set.

The usage of the proposed deblocking adjustment parameters for maximum transform unit sizes is illustrated in the following pseudo-code (pps_ or slice_header_ prefixes are omitted for brevity; QP is average value of the QP values of blocks P and Q [1]).

- IF (P or Q are included in maximum transform unit size) THEN TC_OFFSET = 2*tc_offset_max_tu_div2; ELSE TC_OFFSET = 2*tc_offset_div2

- IF (P or Q are included in maximum transform unit size) THEN BETA_OFFSET = beta_offset_max_tu; ELSE BETA_OFFSET = 2*beta_offset_div2

- IF ( P or Q have MODE_INTRA ) THEN TcOffset = DEFAULT_INTRA_TC_OFFSET(=2) + TC_OFFSET; ELSE TcOffset = TC_OFFSET

- BetaOffset = BETA_OFFSET

- tC: Q = Clip3(0, MAX_QP + DEFAULT_INTRA_TC_OFFSET, QP + TcOffset )   (luma and chroma deblocking)

- β: Q = Clip3(0, MAX_QP, QP + BetaOffset )    (luma deblocking only)

Note that the transform units of maximum size may need to contain at least one non-zero coded coefficient before applying the proposed beta_offset_max_tu and tc_offset_max_tu_div2 parameters to adjust the deblocking strength. This check is included in the source code used for producing the results in this proposal.

## Beta Threshold Curve for Increased Deblocking Strength

In Figure 7, four alternative curves with various slopes (4, 8, 12, 16) are depicted to modify β-curve values beyond the QP value 41. The β-curve values are modified beyond the QP value range that is commonly employed by the HM test conditions, and therefore, common test condition coding efficiency results are unaffected by the proposed modification. The advantage of these curves is that they do not extend beyond the HM7 β-curve QP range of 0 to 51, which preserves the number of values to store in memory. It is similar to the adopted modification of the tC-curve [4], which also does not extend the QP range. The values of the proposed beta-curves are enumerated in Table 5.

In this contribution, the strongest curve with slope 16 (prop4) is employed for testing. The purpose of the proposed β threshold curve modification is to increase deblocking strength, for example, by signaling a β offset parameter, such as beta_offset_div2 [1] or the proposed beta_offset_max_tu. The latter is not divided by 2 to achieve finer control over the deblocking strength adjustment using the proposed curve. The simulation results section reports on testing the maximum TU size deblocking method together with the proposed stronger β threshold curve and the HM7 curve for comparison.

Figure 7  β-curve modifications for QP > 41.

Table 5  Values of proposed β-curves as a function of QP. Values are modified with respect to HM7 for QP > 41.

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| beta-HM7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| beta-prop1-d4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| beta-prop2-d8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| beta-prop3-d12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| beta-prop4-d16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

| QP | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| beta-HM7 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 |
| beta-prop1-d4 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 48 | 52 | 56 | 60 | 64 | 68 | 72 | 76 | 80 | 84 |
| beta-prop2-d8 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 52 | 60 | 68 | 76 | 84 | 92 | 100 | 108 | 116 | 124 |
| beta-prop3-d12 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 56 | 68 | 80 | 92 | 104 | 116 | 128 | 140 | 152 | 164 |
| beta-prop4-d16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 60 | 76 | 92 | 108 | 124 | 140 | 156 | 172 | 188 | 204 |

- Experimental Results

## Test Conditions

The following 1080p sequences have been identified as exhibiting significant blocking artifacts and are used in the objective and subjective evaluation:

- “Riverbed”

- “DucksTakeOff”

- “WestWindEasy”

- “ChinaSpeed” (class F)

The test configuration is Lowdelay-B-Main. Optionally, the QuadtreeTULog2MaxSize configuration parameter with default value 5 (32x32 transform) is changed to value 4 (16x16 transform) or value 3 (8x8 transform). The deblocking offset parameters LoopFilterBetaOffset_div2 and LoopFilterTcOffset_div2 are set to their maximum allowed value 13, which configures the HM7 deblocking filter with maximum strength for reference.

The proposal is implemented on top of the HM7.0 source code. The configuration parameters are set as follows:

DeblockingFilterControlPresent	: 1        	# Dbl control params present (0=not present, 1=present)

LoopFilterOffsetInAPS         	: 0        	# Dbl params in APS or SliceHeader (0=SliceHeader, 1=APS)

LoopFilterDisable             	: 0       	# Disable deblocking filter in slice header (0=Filter, 1=No Filter)

LoopFilterBetaOffset_div2     	: 0        	# Range: -13 ~ 13

LoopFilterTcOffset_div2       	: 0        	# Range: -13 ~ 13

LoopFilterOffsetMaxTUEnabled: 1	# Enable max TU offsets

LoopFilterBetaOffsetMaxTU     : 26     	# Range: -26 ~ 26

LoopFilterTcOffsetMaxTUDiv2	: 13   	# Range: -13 ~ 13

## Objective Results

The following tables enumerate the bit rates and PSNRs at QP values 32, 37 and 41 for the four test sequences. It is important for subjective quality evaluation that the bit rates are similar to the bit rates of the reference. The PSNR values are less useful for this test.

Table 6 QP=32

| Sequence | Max TU | Condition | Bitrate [kbps] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 32x32 | Reference | 5852.94 | 34.62 | 39.03 | 41.43 |
|  |  | Prop. & HM7 Beta | 5853.81 | 34.63 | 39.03 | 41.44 |
|  |  | Prop. & New Beta | 5852.45 | 34.56 | 39.03 | 41.43 |
|  | 16x16 | Reference | 5975.89 | 34.36 | 39.03 | 41.28 |
|  |  | Prop. & HM7 Beta | 5974.05 | 34.37 | 39.04 | 41.28 |
|  |  | Prop. & New Beta | 5971.08 | 34.23 | 39.03 | 41.28 |
|  | 8x8 | Reference | 6283.83 | 34.00 | 38.72 | 40.84 |
|  |  | Prop. & HM7 Beta | 6285.04 | 33.99 | 38.73 | 40.85 |
|  |  | Prop. & New Beta | 6288.82 | 33.74 | 38.73 | 40.85 |
| DucksTakeOff | 32x32 | Reference | 18569.83 | 30.29 | 32.63 | 36.60 |
|  |  | Prop. & HM7 Beta | 18569.44 | 30.33 | 32.63 | 36.60 |
|  |  | Prop. & New Beta | 18689.27 | 30.17 | 32.63 | 36.59 |
|  | 16x16 | Reference | 18867.83 | 30.12 | 32.60 | 36.63 |
|  |  | Prop. & HM7 Beta | 18862.21 | 30.15 | 32.60 | 36.63 |
|  |  | Prop. & New Beta | 19075.96 | 29.86 | 32.60 | 36.63 |
|  | 8x8 | Reference | 19743.79 | 29.85 | 32.52 | 36.55 |
|  |  | Prop. & HM7 Beta | 19752.04 | 29.86 | 32.53 | 36.55 |
|  |  | Prop. & New Beta | 20144.60 | 29.38 | 32.53 | 36.55 |
| WestWindEasy | 32x32 | Reference | 3741.14 | 36.14 | 44.77 | 47.56 |
|  |  | Prop. & HM7 Beta | 3746.05 | 36.19 | 44.79 | 47.57 |
|  |  | Prop. & New Beta | 3747.06 | 36.14 | 44.77 | 47.56 |
|  | 16x16 | Reference | 3760.05 | 35.97 | 44.74 | 47.15 |
|  |  | Prop. & HM7 Beta | 3762.94 | 36.01 | 44.75 | 47.16 |
|  |  | Prop. & New Beta | 3765.66 | 35.89 | 44.76 | 47.16 |
|  | 8x8 | Reference | 3814.90 | 35.65 | 44.08 | 46.60 |
|  |  | Prop. & HM7 Beta | 3812.53 | 35.66 | 44.09 | 46.59 |
|  |  | Prop. & New Beta | 3833.03 | 35.42 | 44.09 | 46.60 |
| ChinaSpeed | 32x32 | Reference | 1362.64 | 34.59 | 39.98 | 39.33 |
|  |  | Prop. & HM7 Beta | 1338.93 | 34.92 | 40.18 | 40.07 |
|  |  | Prop. & New Beta | 1351.55 | 34.77 | 40.20 | 40.04 |
|  | 16x16 | Reference | 1374.43 | 34.55 | 39.91 | 39.36 |
|  |  | Prop. & HM7 Beta | 1352.99 | 34.85 | 40.09 | 39.99 |
|  |  | Prop. & New Beta | 1374.70 | 34.61 | 40.10 | 40.01 |
|  | 8x8 | Reference | 1396.20 | 34.40 | 39.78 | 39.26 |
|  |  | Prop. & HM7 Beta | 1376.58 | 34.66 | 39.94 | 39.90 |
|  |  | Prop. & New Beta | 1419.47 | 34.22 | 39.94 | 39.90 |

Table 7 QP=37

| Sequence | Max TU | Condition | Bitrate [kbps] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 32x32 | Reference | 2851.54 | 31.71 | 38.13 | 40.90 |
|  |  | Prop. & HM7 Beta | 2854.03 | 31.71 | 38.13 | 40.90 |
|  |  | Prop. & New Beta | 2852.32 | 31.70 | 38.13 | 40.91 |
|  | 16x16 | Reference | 2926.90 | 31.51 | 38.20 | 40.64 |
|  |  | Prop. & HM7 Beta | 2927.87 | 31.50 | 38.20 | 40.63 |
|  |  | Prop. & New Beta | 2926.26 | 31.47 | 38.20 | 40.62 |
|  | 8x8 | Reference | 2983.93 | 31.04 | 37.64 | 40.37 |
|  |  | Prop. & HM7 Beta | 2988.25 | 31.03 | 37.64 | 40.35 |
|  |  | Prop. & New Beta | 2981.05 | 30.94 | 37.65 | 40.34 |
| DucksTakeOff | 32x32 | Reference | 7590.54 | 27.72 | 31.56 | 35.97 |
|  |  | Prop. & HM7 Beta | 7591.72 | 27.73 | 31.56 | 35.97 |
|  |  | Prop. & New Beta | 7614.43 | 27.67 | 31.56 | 35.97 |
|  | 16x16 | Reference | 7807.94 | 27.59 | 31.52 | 35.97 |
|  |  | Prop. & HM7 Beta | 7808.21 | 27.59 | 31.52 | 35.98 |
|  |  | Prop. & New Beta | 7847.11 | 27.48 | 31.52 | 35.97 |
|  | 8x8 | Reference | 8276.79 | 27.33 | 31.38 | 35.81 |
|  |  | Prop. & HM7 Beta | 8276.92 | 27.33 | 31.38 | 35.81 |
|  |  | Prop. & New Beta | 8341.11 | 27.13 | 31.38 | 35.81 |
| WestWindEasy | 32x32 | Reference | 1666.20 | 32.98 | 43.67 | 46.62 |
|  |  | Prop. & HM7 Beta | 1669.72 | 33.00 | 43.67 | 46.65 |
|  |  | Prop. & New Beta | 1667.93 | 33.00 | 43.67 | 46.64 |
|  | 16x16 | Reference | 1686.54 | 32.87 | 43.40 | 45.83 |
|  |  | Prop. & HM7 Beta | 1687.40 | 32.88 | 43.46 | 45.82 |
|  |  | Prop. & New Beta | 1685.62 | 32.85 | 43.44 | 45.81 |
|  | 8x8 | Reference | 1695.41 | 32.59 | 43.11 | 45.68 |
|  |  | Prop. & HM7 Beta | 1698.58 | 32.59 | 43.09 | 45.64 |
|  |  | Prop. & New Beta | 1697.15 | 32.50 | 43.11 | 45.68 |
| ChinaSpeed | 32x32 | Reference | 607.94 | 31.55 | 38.48 | 37.39 |
|  |  | Prop. & HM7 Beta | 606.44 | 31.68 | 38.67 | 38.00 |
|  |  | Prop. & New Beta | 609.89 | 31.56 | 38.69 | 38.01 |
|  | 16x16 | Reference | 615.19 | 31.49 | 38.39 | 37.33 |
|  |  | Prop. & HM7 Beta | 612.00 | 31.60 | 38.53 | 37.93 |
|  |  | Prop. & New Beta | 619.02 | 31.44 | 38.53 | 37.94 |
|  | 8x8 | Reference | 622.98 | 31.32 | 38.26 | 37.28 |
|  |  | Prop. & HM7 Beta | 620.77 | 31.41 | 38.33 | 37.84 |
|  |  | Prop. & New Beta | 631.00 | 31.11 | 38.33 | 37.84 |

Table 8 QP=41

| Sequence | Max TU | Condition | Bitrate [kbps] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | 32x32 | Reference | 1555.21 | 29.54 | 37.78 | 40.63 |
|  |  | Prop. & HM7 Beta | 1554.13 | 29.53 | 37.78 | 40.61 |
|  |  | Prop. & New Beta | 1552.00 | 29.54 | 37.78 | 40.64 |
|  | 16x16 | Reference | 1607.22 | 29.37 | 37.77 | 40.28 |
|  |  | Prop. & HM7 Beta | 1606.81 | 29.36 | 37.77 | 40.20 |
|  |  | Prop. & New Beta | 1604.98 | 29.35 | 37.77 | 40.23 |
|  | 8x8 | Reference | 1594.68 | 28.83 | 37.19 | 40.05 |
|  |  | Prop. & HM7 Beta | 1596.20 | 28.81 | 37.20 | 40.04 |
|  |  | Prop. & New Beta | 1592.64 | 28.80 | 37.20 | 40.04 |
| DucksTakeOff | 32x32 | Reference | 3713.80 | 25.98 | 30.94 | 35.67 |
|  |  | Prop. & HM7 Beta | 3714.33 | 25.98 | 30.95 | 35.67 |
|  |  | Prop. & New Beta | 3719.16 | 25.95 | 30.94 | 35.66 |
|  | 16x16 | Reference | 3867.83 | 25.85 | 30.88 | 35.62 |
|  |  | Prop. & HM7 Beta | 3870.22 | 25.85 | 30.88 | 35.61 |
|  |  | Prop. & New Beta | 3876.26 | 25.80 | 30.88 | 35.61 |
|  | 8x8 | Reference | 4051.45 | 25.57 | 30.66 | 35.45 |
|  |  | Prop. & HM7 Beta | 4054.41 | 25.57 | 30.67 | 35.45 |
|  |  | Prop. & New Beta | 4065.25 | 25.48 | 30.67 | 35.45 |
| WestWindEasy | 32x32 | Reference | 852.49 | 30.77 | 43.05 | 46.03 |
|  |  | Prop. & HM7 Beta | 852.94 | 30.77 | 43.04 | 46.06 |
|  |  | Prop. & New Beta | 852.79 | 30.78 | 43.05 | 45.98 |
|  | 16x16 | Reference | 864.34 | 30.64 | 42.78 | 45.20 |
|  |  | Prop. & HM7 Beta | 863.97 | 30.65 | 42.72 | 45.13 |
|  |  | Prop. & New Beta | 863.12 | 30.68 | 42.77 | 45.18 |
|  | 8x8 | Reference | 850.71 | 30.35 | 42.54 | 44.61 |
|  |  | Prop. & HM7 Beta | 852.57 | 30.34 | 42.55 | 44.57 |
|  |  | Prop. & New Beta | 849.90 | 30.31 | 42.57 | 44.56 |
| ChinaSpeed | 32x32 | Reference | 308.90 | 29.22 | 37.59 | 36.09 |
|  |  | Prop. & HM7 Beta | 309.55 | 29.27 | 37.77 | 36.61 |
|  |  | Prop. & New Beta | 310.33 | 29.17 | 37.77 | 36.62 |
|  | 16x16 | Reference | 312.47 | 29.11 | 37.48 | 36.09 |
|  |  | Prop. & HM7 Beta | 313.49 | 29.17 | 37.61 | 36.62 |
|  |  | Prop. & New Beta | 313.33 | 29.03 | 37.62 | 36.62 |
|  | 8x8 | Reference | 315.95 | 28.95 | 37.31 | 36.05 |
|  |  | Prop. & HM7 Beta | 315.91 | 28.99 | 37.39 | 36.50 |
|  |  | Prop. & New Beta | 317.03 | 28.78 | 37.39 | 36.49 |

“Riverbed” reference (max TU is 32x32):

“Riverbed” reference (max TU is 16x16):

“DucksTakeOff” reference (max TU is 32x32):

“DucksTakeOff” reference (max TU is 16x16):

### “Riverbed” test (max TU is 32x32):

“Riverbed” test (max TU is 16x16):

“DucksTakeOff” test (max TU is 32x32):

“DucksTakeOff” test (max TU is 16x16):

### “Riverbed” test (max TU is 32x32):

“Riverbed” test (max TU is 16x16):

“DucksTakeOff” test (max TU is 32x32):

“DucksTakeOff” test (max TU is 16x16):

## Visual Quality Examples

### “Riverbed” Sequence (Max TU 32x32; QP 37)

Figure 8  Reference (crop from frame 1)

Figure 9 Proposal with Proposed β-Curve (crop from frame 1)

### “Riverbed” Sequence (Max TU 16x16; QP 37)

Figure 10 Reference (crop from frame 1)

Figure 11 Proposal with Proposed β-Curve (crop from frame 1)

### “WestWindEasy” Sequence (Max TU 32x32; QP 37)

Figure 12 Reference (crop from frame 1)

Figure 13 Proposal with Proposed β-Curve (crop from frame 1)

### “WestWindEasy” Sequence (Max TU 16x16; QP 37)

Figure 14 Reference (crop from frame 1)

Figure 15 Proposal with Proposed β-Curve (crop from frame 1)

# Conclusion

It is suggested to adopt the proposed transform dependent deblocking strength adjustment for the maximum transform size and the proposed stronger β threshold curve into the HEVC DIS. The advantage of the approach is that the severe blocking artifacts that occur in certain content types can be effectively reduced by specifically increasing deblocking filter strength for the edges of the TUs of maximum size, while avoiding oversmoothing of other picture areas.

# HEVC Draft Text Changes

See proposal description in Section 3 for syntax changes.

The following text changes for the deblocking filter are based on [1].

8.7.2.4.3 Decision process for luma block edge

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Outputs of this process are:

–	variables dE, dEp, dEq containing decisions,

–	variables β, tC.

If edgeType is equal to EDGE_VER, the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = recPictureL[ xC + xB – i − 1, yC + yB + k ]		(8-2)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +k, yC + yB + i ]		(8-3)

pi,k = recPictureL[ xC + xB +k, yC + yB – i − 1 ]		(8-4)

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )	(8-5)

The variables log2TrafoSizeQ and log2TrafoSizeP are set equal to the log2TrafoSize values of the transform units which include the sample q0,0 and p0,0, respectively.

A variable BetaOffset is derived as follows:

If log2TrafoSizeP or log2TrafoSizeQ is equal to Log2MaxTrafoSize, then BetaOffset is set equal to beta_offset_max_tu, otherwise, BetaOffset is set equal to (beta_offset_div2 << 1), where beta_offset_max_tu and beta_offset_div2 are the values of the variables beta_offset_max_tu  and beta_offset_div2 for the slice that contains sample q0,0.

The value of the variable β′ is determined as specified in Error: Reference source not found based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) )	(8-6)

where beta_offset_div2 is the value of the variable beta_offset_div2 for the slice that contains sample q0,0.

Q = Clip3( 0, 51, qPL + BetaOffset )	(8-7)

The variable β is derived as:

β = β′*(1<<(BitDepthY − 8))	(8-8)

A variable TcOffset is derived as follows:

If log2TrafoSizeP or log2TrafoSizeQ is equal to Log2MaxTrafoSize, then TcOffset is set equal to (tc_offset_max_tu_div2 << 1), otherwise, TcOffset is set equal to (tc_offset_div2 << 1), where tc_offset_max_tu_div2 and tc_offset_div2 are the values of the variables tc_offset_max_tu_div2  and tc_offset_div2 for the slice that contains sample q0,0.

The value of the variable tC′ is determined as specified as Error: Reference source not found based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) )	(8-9)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0.

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + TcOffset )	(8-10)

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(8-11)

…

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” 9th JCT-VC Meeting, Geneva, Switzerland, Apr.-May 2012, Doc. JCTVC-I1003_d5

[2] G. Van der Auwera, R. Joshi, X. Wang, M. Karczewicz, “CE10 Subtest 4: Transform Dependent Deblocking Filter Parameter Adjustment,” 8th JCT-VC Meeting, San Jose, CA, USA, Feb. 2012, Doc. JCTVC-H0398.

[3] G. Van der Auwera, R. Joshi, X. Wang, M. Karczewicz, “Non-CE12 Subtest 5: Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level,” 7th JCT-VC Meeting, Geneva, Switzerland, Nov. 2011, JCTVC-G291.

[4] A. Norkin, “On deblocking clipping table,” 9th JCT-VC Meeting, Geneva, Switzerland, Apr.-May 2012, Doc. JCTVC-I0258

[5] http://media.xiph.org/video/derf/

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).