| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0091_r1 |
| --- | --- |

| Title: | AHG6: Chroma QP Offsets and Chroma Deblocking Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cb_qp_offset and cr_qp_offset syntax elements are signalled in the PPS and specify the offsets that are added to the luma QP before deriving the corresponding chroma QP values. The HM7 chroma deblocking filter determines the chroma filtering strength without considering the cb_qp_offset and cr_qp_offset values, which can significantly modify the chroma QP values for coding and, therefore, the filtering strength of chroma blocking artifact edges may be too weak or too strong. To resolve this issue, it is proposed to include the cb_qp_offset and cr_qp_offset values into the chroma deblocking filter process. The HM7.0 anchor is reproduced under common test conditions. The chroma deblocking strength correction is illustrated.

# Introduction

The HEVC Committee Draft (CD) [1] includes the signalling of offset values for determining the quantization parameters (QP) that are used for coding the chrominance components Cb and Cr. The cb_qp_offset and cr_qp_offset syntax elements are signaled in the PPS and their semantics are as follows:

“cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr, respectively.”

The HM7 deblocking filter [1] processes the reconstructed chrominance components. In particular, an edge between two blocks P and Q is processed in case at least one of these blocks is intra coded (boundary strength Bs=2).  The deblocking filter typically looks up the tC parameter in a table based on the QPC value that is derived as follows:

QPC = QpUV ( (QPY,P + QPY,Q + 1)/2 ),					(1)

with QpUV a table that converts the luma QPY into the chroma QPCr or QPCb values. The tC parameter is applied in the clipping operation of the chroma filter and determines the filtering strength. The QPC value is identical for both chroma components Cr and Cb.

The problem is that in equation (1) the chroma QP offset values, cb_qp_offset and cr_qp_offset, are not considered and since the range of these offset values is significant (-12 … 12), the deblocking filter may apply a deblocking strength to the chrominance components that is either too weak or too strong, which results in reduced chroma fidelity.

# Proposal

It is proposed to include the cr_qp_offset and cb_qp_offset values in the computation of the QPC values that are used to look up tC parameters for deblocking filtering the Cb and Cr components, as follows:

QPCr = QpUV ( (QPY,P + QPY,Q + 1)/2 + cr_qp_offset ),					(2)

QPCb = QpUV ( (QPY,P + QPY,Q + 1)/2 + cb_qp_offset ).					(3)

Subsequently, separate tC values for Cb and Cr are looked up in a table based on the computed QPCr and QPCb (constant 2 is added for Bs=2).

Fig.1 illustrates the quality improvement based on a test image with radial gradients. The image is coded according to the Intra-Main test conditions with QP=27 and cr_qp_offset = cb_qp_offset = 12. Fig. 1(a) depicts the anchor and Fig. 1(b) illustrates the proposed correction for chroma deblocking. In Fig. 1(a) chroma blockiness is present while in Fig. 1(b) the chroma blockiness is reduced, because the QPC value 35 is used to determine filtering strength. Fig. 2 depicts a second example from the same test image.

| (a) | (b) |
| --- | --- |

Figure 1

| (a) | (b) |
| --- | --- |

Figure 2

# Objective Results

The proposal has no impact on BD-rate results under common test conditions (JCTVC-I1100), because configuration file parameters CbQpOffset and CrQpOffset are equal to 0. In the performed test, the source code is checked by decoding the HM7.0 anchor. The results are identical to the HM7.0 anchor. Decoding times are measured in a cluster computing environment.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |

# Conclusion

It is proposed to include the cr_qp_offset and cb_qp_offset syntax element (PPS) values into the derivation of the QPC values that are used to configure the deblocking filtering strength for the chroma block boundaries. This way too weak or too strong chroma filtering is avoided in case cr_qp_offset or cb_qp_offset are nonzero.

# HEVC Draft Text Changes

The following CD text changes are based on [1].

##### 8.7.2.4.5 Filtering process for chroma block edge

….

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QPC is determined as specified in Error: Reference source not found based on the index qPI derived as:

qPI = ( ( QPQ + QPP + 1 ) >> 1 )

- If s’ represents the chroma sample array recPictureCb of the current picture:

qPI = ( ( ( QPQ + QPP + 1 ) >> 1 ) + cb_qp_offset ).

- Otherwise, if s’ represents the chroma sample array recPictureCr of the current picture:

qPI = ( ( ( QPQ + QPP + 1 ) >> 1 ) + cr_qp_offset ).

The value of the variable tC′ is determined as specified as Error: Reference source not found based on the chroma quantization parameter Q derived as:

Q = Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) )

….

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” 9th JCT-VC Meeting, Geneva, CH, 27 Apr. – 7 May 2012, Doc. JCTVC-I1003_d5

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).