| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0094 |
| --- | --- |

| Title: | Simplification of multiple sign bit hiding criterion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Wang Dake He Xiang Yu Gaelle Martin-Cocher | Tel: Email: | +1-519-597-5923 {wangj, dhe, xiayu, gmartincoher}@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the Multiple Sign Bit Hiding (SBH) by applying the number-of-non-zeros based criterion to coefficient groups that do not contain the last significant coefficient. The proposed method removes the need to find and store the position of the last non-zero coefficient for every CG which translates in removing up to 15 checks in the worst-case scenario. Under HM7.0 common test conditions, on average -0.01%, -0.02%, -0.05% BD-rates are observed for AI-Main, RA-Main, LB-Main settings, and 0.00%, 0.01%, 0.02% BD-rates are observed for AI-HE10, RA-HE10, LB-HE10 settings. When RDOQ is turned off, on average -0.02%, -0.08%, -0.07% BD-rates are observed for AI-Main, RA-Main, LB-Main settings, and -0.03%, -0.07%, -0.01% BD-rates are observed for AI-HE10, RA-HE10, LB-HE10 settings. Further encoder-only changes to related part in RDOQ result in average BD-rates (-0.11%, -0.07%, -0.09%, -0.26%, -0.37%, -0.61%) in common test conditions.

# Introduction

In the Multiple Sign Bit Hiding [1] in HM7.0, the following criterion is tested for each coefficient group (CG) to determine whether SBH is applied or not.

signHidden = lastNZPosInCG - firstNZPosInCG >= 4

where firstNZPosInCG is  the scan position of the first non-zero coefficient in a CG, and lastNZPosInCG is the scan position of the last non-zero coefficient in a CG.

In order to determine whether a sign is hidden, firstNZPosInCG and lastNZPosInCG need to be found and stored for every CG. Especially, to find lastNZPosInCG, every coefficient in a CG needs to be checked to determine if it is the last one or not.

# Proposed method

It is proposed to change the criterion as follows

If (the current CG is the lastCG)

signHidden = lastNZPosInCG - firstNZPosInCG >= 4

Else

signHidden = numNonZero >= 2

where lastCG is the CG that contains the last significant coefficient in a TU, and numNonZero is the total number of non-zero coefficients in a CG.

With the proposed change, there is no need to find and store lastNZPosInCG. This holds for the lastCG, too, because the position of the last significant coefficient in a TU is decoded from the bitstream.

As a result, instead of doing up to 16 checks per CG to determine lastNZPosInCG, only 1 check per CG is needed to determine whether the current CG is the lastCG or not.

# Experimental Results

The performance of the proposed change compared against the HM7.0 anchor with RDOQ on.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.01% | -0.02% | 0.02% | 0.00% | 0.02% | 0.04% |
| Class B | -0.01% | -0.02% | -0.01% | 0.00% | 0.02% | 0.01% |
| Class C | -0.01% | -0.02% | -0.02% | 0.00% | -0.01% | -0.02% |
| Class D | -0.02% | -0.01% | 0.03% | 0.00% | 0.05% | 0.02% |
| Class E | 0.00% | 0.03% | -0.02% | 0.00% | 0.00% | 0.01% |
| Overall | -0.01% | -0.01% | 0.00% | 0.00% | 0.02% | 0.01% |
|  | -0.01% | -0.01% | 0.00% | 0.00% | 0.01% | 0.01% |
| Class F | -0.01% | -0.02% | 0.00% | 0.01% | -0.13% | 0.00% |
| Enc Time[%] | 101% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 92% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.01% | -0.14% | -0.03% | -0.03% | 0.10% | -0.05% |
| Class B | 0.00% | -0.05% | -0.07% | 0.00% | 0.00% | -0.13% |
| Class C | -0.02% | 0.09% | -0.08% | 0.01% | 0.18% | -0.14% |
| Class D | 0.00% | -0.02% | 0.01% | 0.02% | -0.14% | -0.11% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.05% | 0.01% | 0.02% | -0.12% |
|  | 0.00% | 0.00% | -0.05% | 0.01% | 0.00% | -0.12% |
| Class F | 0.06% | 0.08% | 0.12% | -0.01% | 0.14% | -0.05% |
| Enc Time[%] | 103% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.04% | 0.16% | -0.08% | 0.03% | 0.30% | 0.20% |
| Class C | -0.05% | 0.05% | 0.11% | 0.00% | 0.02% | -0.08% |
| Class D | -0.01% | -0.34% | 0.19% | -0.02% | -0.21% | 0.05% |
| Class E | -0.14% | -0.11% | -0.09% | 0.11% | -0.70% | 0.04% |
| Overall | -0.05% | -0.05% | 0.03% | 0.02% | -0.08% | 0.06% |
|  | -0.05% | -0.05% | 0.01% | 0.02% | -0.05% | 0.14% |
| Class F | -0.05% | 0.31% | 0.19% | 0.09% | -0.15% | 0.92% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

The performance of the proposed change compared against the HM7.0 anchor with RDOQ off.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.03% | 0.00% | -0.02% | -0.04% | -0.02% | 0.01% |
| Class B | -0.03% | 0.02% | 0.02% | -0.04% | 0.02% | 0.04% |
| Class C | -0.02% | -0.01% | 0.00% | -0.01% | -0.01% | -0.01% |
| Class D | -0.02% | -0.02% | 0.02% | -0.02% | 0.00% | 0.00% |
| Class E | -0.01% | 0.04% | 0.06% | -0.02% | 0.02% | 0.02% |
| Overall | -0.02% | 0.01% | 0.02% | -0.03% | 0.00% | 0.01% |
|  | -0.02% | 0.01% | 0.01% | -0.03% | 0.01% | 0.01% |
| Class F | -0.01% | 0.05% | 0.00% | 0.00% | -0.04% | 0.06% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.03% | -0.15% | -0.10% | -0.02% | 0.13% | -0.01% |
| Class B | -0.03% | 0.06% | 0.05% | -0.04% | 0.14% | 0.03% |
| Class C | -0.06% | -0.07% | 0.13% | -0.05% | 0.14% | 0.02% |
| Class D | -0.08% | 0.11% | -0.16% | -0.07% | 0.07% | 0.32% |
| Class E |  |  |  |  |  |  |
| Overall | -0.08% | 0.01% | -0.01% | -0.07% | 0.12% | 0.10% |
|  | -0.05% | 0.01% | -0.01% | -0.05% | 0.07% | 0.08% |
| Class F | -0.04% | -0.10% | 0.08% | -0.01% | -0.01% | -0.10% |
| Enc Time[%] | 99% |  |  | 109% |  |  |
| Dec Time[%] | 99% |  |  | 111% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.06% | 0.26% | 0.52% | -0.08% | -0.15% | 0.03% |
| Class C | -0.06% | 0.03% | -0.09% | -0.06% | 0.22% | 0.07% |
| Class D | -0.03% | 0.28% | -0.30% | 0.02% | 0.47% | -0.33% |
| Class E | -0.15% | 0.00% | 0.33% | 0.11% | 0.74% | -0.28% |
| Overall | -0.07% | 0.16% | 0.13% | -0.01% | 0.27% | -0.11% |
|  | -0.07% | 0.12% | 0.08% | -0.02% | 0.22% | -0.07% |
| Class F | -0.28% | -0.16% | -0.64% | 0.06% | -0.64% | 0.14% |
| Enc Time[%] | 104% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 100% |  |  |

Note that the timing results are not reliable as they were obtained in a mixed computing cluster.

HM Ticket #597 [3] reported an encoder-only fix for the cost calculation in RDOQ, which improves the performance for all HE10 settings when RDOQ is enabled. The following table summarizes the results of the proposed method compared against HM7.0 with the fix of HM Ticket #597 for AI-HE10, RA-HE10, and LB-HE10 settings.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.01% | -0.06% | -0.03% |
| Class B | -0.01% | -0.03% | -0.03% |
| Class C | -0.01% | -0.01% | 0.04% |
| Class D | -0.02% | 0.01% | -0.05% |
| Class E | -0.01% | -0.02% | 0.00% |
| Overall | -0.01% | -0.02% | -0.01% |
|  | -0.01% | -0.02% | -0.01% |
| Class F | 0.01% | 0.02% | 0.03% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A (8bit) | 0.00% | 0.09% | -0.15% |
| Class B | -0.03% | 0.08% | -0.04% |
| Class C | 0.01% | -0.02% | 0.04% |
| Class D | -0.02% | -0.35% | 0.24% |
| Class E |  |  |  |
| Overall | -0.02% | -0.06% | 0.04% |
|  | -0.01% | -0.08% | 0.08% |
| Class F | 0.00% | 0.10% | -0.08% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.02% | -0.19% | -0.08% |
| Class C | -0.03% | 0.03% | 0.00% |
| Class D | 0.03% | 0.31% | -0.25% |
| Class E | -0.01% | -0.66% | -1.21% |
| Overall | -0.01% | -0.10% | -0.31% |
|  | -0.01% | 0.01% | -0.36% |
| Class F | -0.09% | -0.10% | 0.50% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |

Further modifications to the SBH related part in RDOQ lead to noticeable improvement in coding efficiency. The following table summarizes the results.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.13% | -0.12% | -0.26% | -1.03% | -0.90% |
| Class B | -0.10% | -0.14% | -0.13% | -0.30% | -1.02% | -1.05% |
| Class C | -0.12% | -0.23% | -0.21% | -0.27% | -0.69% | -0.75% |
| Class D | -0.11% | -0.18% | -0.24% | -0.25% | -0.74% | -0.60% |
| Class E | -0.13% | -0.03% | 0.02% | -0.18% | -0.85% | -0.90% |
| Overall | -0.11% | -0.15% | -0.15% | -0.26% | -0.87% | -0.85% |
|  | -0.11% | -0.15% | -0.15% | -0.27% | -0.86% | -0.83% |
| Class F | -0.15% | -0.21% | -0.23% | -0.27% | -0.80% | -0.51% |
| Enc Time[%] | 108% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.10% | -0.33% | -0.15% | -0.27% | -1.21% | -1.27% |
| Class B | -0.07% | -0.04% | -0.15% | -0.36% | -1.10% | -1.02% |
| Class C | -0.08% | -0.02% | -0.05% | -0.41% | -0.67% | -0.74% |
| Class D | -0.04% | 0.25% | -0.11% | -0.43% | -0.76% | -1.07% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.03% | -0.12% | -0.37% | -0.95% | -1.03% |
|  | -0.06% | -0.04% | -0.10% | -0.44% | -0.95% | -1.08% |
| Class F | -0.05% | -0.08% | -0.27% | -0.31% | -0.13% | -0.82% |
| Enc Time[%] | 105% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.09% | 0.07% | 0.26% | -0.64% | -1.13% | -1.29% |
| Class C | -0.06% | -0.01% | -0.07% | -0.67% | -1.17% | -1.06% |
| Class D | -0.12% | 0.20% | 0.03% | -0.73% | -1.08% | -1.02% |
| Class E | -0.10% | 0.32% | -1.45% | -0.30% | -2.03% | -0.73% |
| Overall | -0.09% | 0.13% | -0.20% | -0.61% | -1.30% | -1.06% |
|  | -0.09% | 0.07% | -0.27% | -0.62% | -1.20% | -0.95% |
| Class F | -0.17% | 0.07% | 0.11% | -0.27% | -0.77% | -0.80% |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |

# Proposed syntax for working draft

This section describes the proposed change to the HEVC specification [2]. The changes are highlighted.

7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| ...... |  |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| ...... |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| lastNZPosInLastCG = numCoeff – 1 – (numLastSubset << 4); |  |
| signHidden = ( ( (i < numLastSubset) ? (numSigCoeff >= 2) : (lastNZPosInLastCG – firstNZPosInCG >= sign_hiding_threshold) ) && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| ...... |  |
| } |  |
| } |  |

# Conclusions

A change to simplify the Multiple Sign Bit Hiding (SBH) that removes the necessity to find and store the position of the last non-zero coefficient for every CG is proposed, which translates in removing up to 15 checks in the worst-case scenario. Results in common test conditions against HM7.0 anchor with and without RDOQ show that this simplification has no impacts on coding efficiency. It is recommended that this change is adopted into the HEVC working draft.

# References

[1] X. Yu, J. Wang, D. He, G. Martin-Cocher, and S. Campbell, “Multiple Sign Bits Hiding”, JCTVC-H0481.

[2] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting.

[3] [JCTVC HM] Ticket #597: encoder-only fix for rdFactor calculation for HE10 settings, http://hevc.kw.bbc.co.uk/trac/ticket/597

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).