| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 July – 20 July 2012 | Document: JCTVC-J0097 M25419 |
| --- | --- |

| Title: | Evaluation of picture-based SAO optimization in HM-7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Picture-based signaling of SAO parameter in APS was removed in the 9th JCT-VC meeting. However, picture-based SAO optimization still remains in HM-7.0 as an encoder technique to improve coding efficiency at the cost of high complexity. In this contribution, the performance of picture-based optimization is evaluated, and the RDO process in HM-7.0 is fixed to improve its performance. Experimental results with common test condition reportedly shows that, when compared to LCU-based SAO optimization, the picture-based SAO optimization in HM-7.0 results in luma BD-rate loss of 0.0% (0.0%), 0.1% (0.2%), 0.7% (0.7%) and 0.3% (0.5%) for AI-Main (AI-HE10), RA-Main (RA-HE10), LB-Main (LB-HE10) and LP-Main (LP-HE10), respectively. After modifying the RDO process, experimental results reportedly shows that the picture-based SAO optimization results in luma BD-rate gain of 0.1% (0.0%), 0.3% (0.2%), -0.1% (-0.1%) and 0.2% (0.0%) for AI-Main (AI-HE10), RA-Main (RA-HE10), LB-Main (LB-HE10) and LP-Main (LP-HE10), respectively, when compared to the LCU-based SAO optimization.

# Introduction

SAO parameters can be estimated either at picture level or at LCU level. In case that SAO parameters are estimated at picture level (i.e., picture-based SAO optimization), it is possible to have a set of partition-based SAO parameters optimized for a whole picture. In case that SAO parameters are estimated at LCU level (i.e. LCU-based SAO optimization), a SAO parameter for each LCU is estimated independently without considering other LCUs. Therefore, in terms of coding efficiency overall performance of the LCU-based optimization could be inferior to the picture-based SAO optimization.

Previously, SAO parameters by picture-based optimization were signaled in APS while those by LCU-based optimization were signaled in LCU. However, as LCU-level SAO parameter signaling along with LCU-based optimization shows comparable performance to picture-based ones (especially when  LCU size is 64x64), normative picture-based SAO signaling in APS was removed in the 9th JCT-VC meeting [1].

Since SAO parameters estimated at picture level can be signaled in LCU, picture-based SAO optimization has been kept in HM-7.0 as a non-normative encoder technique. It has been expected that picture-based SAO optimization would be superior to LCU-based optimization even with LCU-based signaling. In this document, we evaluate the performance of picture-based SAO optimization with LCU-based signaling in HM-7.0. We also modified its RDO process for the performance improvement.

# Picture-based SAO Optimization in HM-7.0

We tested picture-based SAO optimization under the common test condition [2] by simply setting SAOLcuBasedOptimization to 0. The performance of picture-based SAO optimization is summarized in Table 1 with respect to HM-7.0 anchor, i.e., LCU-based SAO optimization. Unlike what was expected, it turned out that its performance was worse than LCU-based SAO optimization. It resulted in luma BD-rate loss of 0.0% (0.0%), 0.1% (0.2%), 0.7% (0.7%) and 0.3% (0.5%) for AI-Main (AI-HE10), RA-Main (RA-HE10), LB-Main (LB-HE10) and LP-Main (LP-HE10), respectively.

Table 1. BD-rate results of picture-based SAO optimization compared to HM-7.0 anchor (Anchor: HM-7.0, i.e. LCU-based SAO optimization, Test: picture-based SAO optimization).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.6% | -0.9% | -0.1% | -1.6% | -2.2% |
| Class B | -0.1% | -0.4% | -0.6% | 0.0% | -0.8% | -1.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.6% |
| Class D | 0.1% | -0.3% | -0.3% | 0.2% | -0.1% | -0.3% |
| Class E | 0.0% | -1.0% | -1.3% | 0.0% | -0.9% | -1.6% |
| Overall | 0.0% | -0.4% | -0.6% | 0.0% | -0.8% | -1.2% |
|  | 0.0% | -0.5% | -0.7% | 0.0% | -0.8% | -1.2% |
| Class F | 0.1% | 0.0% | -0.2% | -0.1% | -1.2% | -1.5% |
| Enc Time[%] | 101% |  |  | 110% |  |  |
| Dec Time[%] | 101% |  |  | 108% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -1.3% | -1.2% | -0.2% | -3.0% | -3.9% |
| Class B | 0.2% | -0.8% | -0.7% | 0.3% | -1.3% | -1.5% |
| Class C | 0.1% | 0.0% | -0.2% | 0.3% | -0.8% | -1.2% |
| Class D | 0.0% | 0.7% | 0.3% | 0.3% | 0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.4% | -0.5% | 0.2% | -1.2% | -1.7% |
|  | 0.1% | -0.4% | -0.6% | 0.2% | -1.2% | -1.7% |
| Class F | 0.0% | -0.4% | -0.9% | 0.1% | -1.4% | -2.6% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 110% |  |  | 108% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -2.2% | -2.9% | 0.1% | -3.3% | -4.4% |
| Class C | 0.4% | -0.4% | -1.0% | 0.5% | -2.5% | -2.9% |
| Class D | 0.4% | 0.7% | 0.6% | 0.8% | -0.4% | -1.1% |
| Class E | 2.2% | -2.2% | -3.2% | 2.0% | -3.4% | -3.2% |
| Overall | 0.7% | -1.1% | -1.6% | 0.7% | -2.4% | -3.0% |
|  | 0.7% | -1.1% | -1.5% | 0.7% | -2.3% | -2.9% |
| Class F | -0.2% | -2.7% | -2.7% | -0.1% | -3.4% | -5.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -3.5% | -4.3% | -0.2% | -3.7% | -5.3% |
| Class C | 0.1% | -1.1% | -1.6% | 0.4% | -2.8% | -3.2% |
| Class D | 0.4% | 0.5% | 0.8% | 0.7% | -1.4% | -2.0% |
| Class E | 2.2% | -3.3% | -3.1% | 1.8% | -4.0% | -3.2% |
| Overall | 0.3% | -1.9% | -2.1% | 0.5% | -3.0% | -3.5% |
|  | 0.3% | -1.8% | -2.1% | 0.6% | -2.9% | -3.4% |
| Class F | -0.4% | -3.1% | -3.1% | -0.3% | -3.4% | -6.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  |

# Modified Picture-based SAO Optimization

It was noticed that the RDO process for picture-based SAO optimization in HM-7.0 was still based on the SAO parameter signalling in APS, by which bits required to signal SAO parameter are counted only once per each partition, and it was the main cause of the poor performance of picture-based SAO optimization in HM-7.0. Therefore, it was fixed by counting required bits to signal SAO parameters for all LCUs in a partition according to LCU-based signalling syntax.

In the RDO process, the SAO CABAC state is updated in raster-scan order within a partition. However, in the actual coding, the SAO CABAC state should be updated in raster-scan order within the whole picture. Note that in our modification, this problem is not resolved, since supporting the CABAC state update in raster-scan order within the whole picture will be too complicated to implement.

We tested the modified picture-based SAO optimization under the common test condition [2] and its performance is summarized in Table 1 with respect to HM-7.0 anchor, i.e., LCU-based SAO optimization. It resulted in luma BD-rate gain of 0.1% (0.0%), 0.3% (0.2%), -0.1% (-0.1%) and 0.2% (0.0%) for AI-Main (AI-HE10), RA-Main (RA-HE10), LB-Main (LB-HE10) and LP-Main (LP-HE10), respectively. We can see that the performance improved significantly. Except for LB cases, it turned out that the modified picture-based SAO optimization is better than LCU-based one. However, considering its complexity, such gains may not be enough. Therefore, we encourage further study in order to justify the usefulness of the picture-based SAO optimization.

Table 2. BD-rate results of the modified picture-based SAO optimization compared to HM-7.0 anchor (Anchor: HM-7.0, i.e. LCU-based SAO optimization, Test: modified picture-based SAO optimization).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.3% | -0.1% | -1.3% | -1.8% |
| Class B | -0.1% | 0.1% | 0.0% | -0.1% | -0.5% | -0.8% |
| Class C | -0.1% | 0.3% | 0.4% | 0.0% | -0.3% | -0.4% |
| Class D | 0.1% | -0.1% | -0.2% | 0.2% | -0.1% | -0.2% |
| Class E | -0.1% | 0.2% | -0.1% | 0.0% | -0.2% | -0.7% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | -0.5% | -0.8% |
|  | -0.1% | 0.0% | -0.1% | 0.0% | -0.5% | -0.8% |
| Class F | 0.0% | 0.3% | 0.1% | -0.2% | -1.0% | -1.1% |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.1% | 0.0% | -0.3% | -2.0% | -2.7% |
| Class B | -0.1% | 0.2% | 0.5% | -0.1% | -0.7% | -0.8% |
| Class C | -0.2% | 0.5% | 0.4% | -0.1% | -0.6% | -0.9% |
| Class D | -0.4% | 0.7% | 0.5% | -0.2% | 0.1% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | 0.3% | 0.3% | -0.2% | -0.8% | -1.2% |
|  | -0.3% | 0.3% | 0.3% | -0.2% | -0.8% | -1.1% |
| Class F | -0.4% | 0.0% | -0.3% | -0.4% | -1.1% | -2.2% |
| Enc Time[%] | 98% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.0% | -1.3% | -0.1% | -2.3% | -3.3% |
| Class C | 0.1% | 0.1% | -0.3% | 0.1% | -1.8% | -2.3% |
| Class D | 0.0% | 0.7% | 1.2% | 0.3% | -0.2% | -0.4% |
| Class E | 0.4% | -1.6% | -2.3% | 0.0% | -3.1% | -3.3% |
| Overall | 0.1% | -0.4% | -0.6% | 0.1% | -1.8% | -2.3% |
|  | 0.1% | -0.4% | -0.6% | 0.1% | -1.7% | -2.3% |
| Class F | -0.6% | -2.0% | -2.3% | -0.5% | -2.6% | -3.8% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -2.1% | -2.3% | -0.3% | -2.7% | -3.6% |
| Class C | -0.1% | -0.5% | -0.8% | 0.2% | -2.3% | -2.6% |
| Class D | 0.0% | 0.4% | 0.7% | 0.2% | -0.6% | -1.5% |
| Class E | 0.7% | -2.0% | -1.8% | 0.1% | -2.6% | -2.8% |
| Overall | -0.2% | -1.1% | -1.1% | 0.0% | -2.1% | -2.7% |
|  | -0.2% | -1.1% | -1.1% | 0.0% | -2.0% | -2.4% |
| Class F | -0.8% | -2.0% | -2.3% | -0.7% | -2.5% | -4.8% |
| Enc Time[%] | 104% |  |  | 100% |  |  |
| Dec Time[%] | 108% |  |  | 101% |  |  |

# Conclusion

In this contribution, we evaluated the performance of picture-based SAO optimization with LCU-based SAO parameter signaling and modified the RDO process to improve its performance. The modified picture-based SAO optimization showed superior performance to LCU-based SAO optimization except for LB cases. However, considering the complexity, the benefit of the picture-based SAO optimization is not enough. Moreover, the similar gain can be observed with more efficient design of SAO parameter signaling for LCU-based SAO optimization [3]. So we recommend picture-based SAO optimization to be removed from HM software. Otherwise we recommend the RDO fix in this contribution to be adopted into HM software for further study on the picture-based SAO optimization.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1003, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.

- W.-S. Kim and D.-K. Kwon, “AHG6: SAO complexity reduction with SAO LCU flag coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0140, July 2012