| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0098 |
| --- | --- |

| Title: | AHG5: Bypass bins for reference index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Joel Sole, Xianglin Wang, Marta Karczewicz (*) Vivienne Sze, Madhukar Budagavi (**) (*) Qualcomm 5775 Morehouse Drive San Diego, CA 92121, USA\ (**) Texas Instruments | Tel: Email: | vseregin@qualcomm.com sze@ti.com |
| Source: | Qualcomm and Texas Instruments |  |  |

_____________________________

# Abstract

Context-coded bins reduction in reference index coding is proposed.  All the bins after the second bin are coded with CABAC bypass mode. The worst-case of context-coded bins is reduced from 15 to 2. Experimental results show no performance change and 8% of bypass bins in average under four common test configurations. Additionally, the proposal also results in one context removal.

# Introduction

In current HM7.0, reference index is binarized using truncated unary code and all bins are context-coded. Totally, three contexts are used, with first two contexts dedicated respectively for bin0 and bin1 of the reference index, and a third context shared by bin2 and subsequent bins. Hence, in the worst-case all 15 bins are context-coded.

# Proposal

## Truncated unary binarization with bypass coded bins

The third context mentioned above might be inefficient since the coded bins sharing the context may have quite different statistical properties. Therefore, instead of having a context for this case, it is proposed to remove the context so that bin2 and later bins are coded with CABAC bypass mode. This method was presented in [1] and [2].

Such simplification of reference index coding was implemented on top of HM7.0. Results based on common test conditions [3] are summarised in the following Table 1. No noticeable performance change was observed.

Table 1. Experimental results for proposed method

Table 2. Bypass bins ratio

## Truncated unary combined with Exponential Golomb binarization with bypass coded bins

In the previous method, number of total bins in the worst-case is same as anchor. However, the total number of context-coded bins is only 2, while 13 bins are bypass coded.

If total number of bins is necessary to be reduced, it can be done by combining the truncated unary code and Exponential Golomb (TU4EG0) code [2] and to reduce number of context coded bins further it is proposed to code only first two bins with context and the rest is bypassed. Such a simplification also removes one context. The performance in common test conditions [3] is exactly same as the first method.

To compare these two methods, truncated unary and TU4EG0 binarizations are summarized in the next table 2.

Table 2. Comparison between TU and TU4EG0.

| Ref index | Truncated unary | TU4EG0 | Bin difference (TU – TU4EG0) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Non-last index | Last index |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 10 | 10 | 0 | 0 |
| 2 | 110 | 110 | 0 | 0 |
| 3 | 1110 | 1110 | 0 | 0 |
| 4 | 11110 | 1111 0 | 0 | 0 |
| 5 | 111110 | 1111 100 | -1 | -2 |
| 6 | 1111110 | 1111 101 | 0 | -1 |
| 7 | 11111110 | 1111 11000 | -1 | -2 |
| 8 | 111111110 | 1111 11001 | 0 | -1 |
| 9 | 1111111110 | 1111 11010 | 1 | 0 |
| 10 | 11111111110 | 1111 11011 | 2 | 1 |
| 11 | 111111111110 | 1111 1110000 | 1 | 0 |
| 12 | 1111111111110 | 1111 1110001 | 2 | 1 |
| 13 | 11111111111110 | 1111 1110010 | 3 | 2 |
| 14 | 111111111111110 | 1111 1110011 | 4 | 3 |
| 15 | 111111111111111 | 1111 1110101 | 4 | 4 |

Since number of reference frames are known exactly and fixed, truncated unary code represents the reference index without extra unused bins and is a better choice in this case.

The numbers in the above table show the difference in number of bins for reference index coding between truncated unary code and Exponential Golomb code TU4EG0. In the fourth column (non-last index), bin difference is summarized when reference index is not the last one and in the fifth column, the same difference is summarized, but for the case when the reference index form the first column is the last one.

For example, let us consider reference index equal to 5 but there are more than six reference frames. The two code words with each under different binarization are the following:

111110 vs 1111  100, truncated unary is one bin shorter and -1 is put in the fourth column.

If reference index is equal to 5 and there are six reference frames, reference index equal to 5 is last possible one. In this case truncated unary will be one bin shorter and related code words are the following:

11111 vs 1111  100, truncated unary is two bins shorter and -2 is put in the fifth column.

In summary, there is a bin saving in the cases from 5 to 8 which are more frequently used than those larger reference indices. So it is better to keep the current truncated unary binarization unchanged for reference index coding, but apply bypass coding after the second bin.

# Conclusion

With the proposed method, reference index coding can be simplified by removing the context shared by bin2 and later bins. Those bins are coded directly using CABAC bypass mode. Experimental results show that the simplification has no noticeable impact on coding performance and at the same time, it results in about 8% bypass bins for reference index coding. We recommend adopting the first method in HEVC.

# References

- V. Seregin et al, “Bypass bins for reference index coding,” JCTVC-I0352, Geneva, Apr. 2012.

- V. Sze et al, “Reduction in context coded bins for ref_idx and cu_qp_delta,” JCTVC-I0204, Geneva, Apr. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-I1100, Geneva, Apr. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-I1003.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Table 9-2 |  | 0..2 0..1 | 3..5 2..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-2 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 2 | 4 3 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 2 1 | Table 9-2 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 2 1 | Table 9-2 | 0 |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |

Table 9-49 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| ref_idx_l0 | Table 9-2 | 0 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |
| ref_idx_l0, ref_idx_l1 | Table 9-2 | 3 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |