| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0099 |
| --- | --- |

| Title: | Sign data hiding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Two methods that allegedly simplify the sign data hiding algorithm are proposed. A first method removes the first non-zero position dependency from the sign hiding criterion. There is no average BD-rate difference on common conditions and -0.1% for RDOQ off case. A second method applies the hiding criterion only once per transform unit (instead of once every 16 coefficients) and doesn’t need to keep track of the position of the first and last non-zero coefficients in the coefficient group. The average BD-rate for all configurations is -0.1% for common conditions and -0.2% for RDOQ off case. Further encoder-only changes related to RDOQ result in average BD-rates in common test conditions of -0.1% for Main profile and  -0.3%, -0.4%, -0.6% for AI, RA, and LB-HE, respectively.

# Introduction

HEVC provides a method to reduce the number of coded signs, called sign data hiding (SDH), which was adopted in [1]. With this method, the coding of the sign of the first non-zero coefficient in a coefficient group (CG) is sometimes omitted. Instead, the sign is embedded in the parity of the sum of the levels of the CG: even corresponds to “+” and odd to “–”. The distance in scan order between the first and last non-zero coefficients of a CG is the criterion used to decide whether to embed the sign for the CG:

lastNZPosInCG – firstNZPosInCG >= 4

If this distance is larger than 3, then SDH is used. On the decoder side, if the criterion is met, the sign of the first non-zero coefficient of each CG is not decoded. Instead, it is inferred from the parity of the sum of the levels in the CG.

SDH can be simplified as it was shown in [2] and [3].

# Proposed Methods

## Method 1: Simpler criterion within a CG

In SDH, the decoder has to keep track of two variables to check the sign hiding criterion: the scan positions of the first and last non-zero in the CG. It is proposed to simplify this criterion to

lastNZPosInCG >= 4

In the proposed method [2], only one variable is necessary, thus simplifying the decoder process.

## Method 2: Once per TU sign hiding criterion

The second method is even simpler. Currently, the criterion is checked once per CG. Instead, it is proposed to check the criterion only once per TU. The check occurs for the CG with the last significant coefficient in the TU. If the last position is

lastNZPosInTU >= 4

then the sign is hidden for that CG. Otherwise, it is not. A sign is always hidden for the remaining coefficient groups. lastNZPosInTU is the last non-zero coefficient in a TU, which is information that HEVC already signals in the bitstream. lastNZPosInTU value is taken with respect to the beginning of the CG (like done for lastNZPosInCG in HM7.0).

With this method, there is no need to track the first and last non-zero coefficients in a CG and up to 15 checks per CG are removed. Also, there is no need to check the condition of sign hiding for every CG. Therefore, the HM7.0 sign data hiding algorithm is greatly simplified.

# Simulation results

The proposed methods are implemented in HM7.0 and simulated under the common test conditions Error: Reference source not found. Software for both methods is provided with this contribution.

The performance is compared to HM7.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate saving. Table 1 and 2 summarize the experimental results for method 1 for common conditions and for RDOQ off, respectively. Table 3 and 4 show the results for method 2 for common conditions and for RDOQ off, respectively. Table 5 show the results for method 2 for common conditions with the improved RDOQ.

Table 1. Method 1 results, common conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.15% | -0.09% | 0.00% | 0.03% | 0.00% |
| Class B | -0.07% | -0.13% | -0.10% | 0.03% | 0.01% | -0.01% |
| Class C | -0.11% | -0.16% | -0.18% | 0.01% | 0.01% | -0.01% |
| Class D | -0.10% | -0.15% | -0.22% | 0.01% | 0.02% | 0.02% |
| Class E | -0.12% | -0.01% | 0.02% | 0.01% | -0.05% | 0.03% |
| Overall | -0.09% | -0.13% | -0.12% | 0.01% | 0.01% | 0.00% |
|  | -0.09% | -0.13% | -0.12% | 0.01% | 0.00% | 0.00% |
| Class F | -0.16% | -0.17% | -0.25% | 0.18% | -0.10% | 0.24% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.20% | 0.13% | 0.00% | 0.51% | 0.72% |
| Class B | -0.05% | 0.29% | 0.10% | 0.03% | 0.35% | 0.16% |
| Class C | -0.02% | 0.09% | -0.09% | 0.06% | 0.37% | 0.02% |
| Class D | -0.05% | 0.32% | 0.16% | 0.03% | 0.17% | -0.10% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | 0.14% | 0.08% | 0.03% | 0.35% | 0.20% |
|  | -0.02% | 0.13% | 0.06% | 0.04% | 0.34% | 0.18% |
| Class F | -0.08% | 0.01% | -0.11% | 0.23% | 0.65% | 0.29% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.61% | 0.50% | 0.07% | 0.80% | 0.49% |
| Class C | -0.08% | 0.43% | 0.17% | 0.07% | 0.29% | 0.31% |
| Class D | -0.09% | 0.65% | 0.72% | 0.03% | 0.58% | -0.13% |
| Class E | -0.14% | 0.59% | -0.78% | 0.20% | -0.42% | 0.17% |
| Overall | -0.09% | 0.57% | 0.23% | 0.09% | 0.39% | 0.23% |
|  | -0.09% | 0.51% | 0.22% | 0.09% | 0.47% | 0.30% |
| Class F | -0.21% | 0.11% | 0.36% | 0.30% | 1.28% | 0.78% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

Table 2. Method 1 results, RDOQ off

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | 0.03% | -0.02% | -0.14% | 0.14% | 0.04% |
| Class B | -0.13% | 0.10% | 0.07% | -0.17% | 0.15% | 0.12% |
| Class C | -0.11% | -0.01% | 0.00% | -0.11% | 0.05% | 0.02% |
| Class D | -0.11% | 0.09% | 0.06% | -0.10% | 0.11% | 0.02% |
| Class E | -0.10% | 0.07% | 0.01% | -0.11% | 0.05% | 0.01% |
| Overall | -0.11% | 0.06% | 0.03% | -0.13% | 0.11% | 0.05% |
|  | -0.11% | 0.05% | 0.03% | -0.13% | 0.11% | 0.05% |
| Class F | -0.08% | 0.04% | 0.06% | 0.10% | 0.08% | 0.18% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | 0.04% | 0.25% | -0.14% | 0.72% | 0.20% |
| Class B | -0.07% | 0.25% | 0.23% | -0.08% | 0.35% | 0.43% |
| Class C | -0.18% | -0.05% | 0.13% | -0.05% | 0.10% | 0.19% |
| Class D | -0.25% | 0.10% | -0.19% | -0.15% | -0.01% | 0.39% |
| Class E |  |  |  |  |  |  |
| Overall | -0.15% | 0.10% | 0.11% | -0.10% | 0.29% | 0.31% |
|  | -0.15% | 0.10% | 0.14% | -0.10% | 0.23% | 0.29% |
| Class F | -0.08% | -0.17% | -0.08% | 0.15% | 0.18% | 0.41% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.12% | 0.48% | 0.48% | -0.08% | 0.55% | 0.32% |
| Class C | -0.21% | 0.23% | 0.20% | -0.18% | 0.27% | 0.09% |
| Class D | -0.21% | 0.16% | 0.32% | -0.09% | 1.04% | 0.15% |
| Class E | -0.10% | -0.46% | 0.43% | 0.08% | 1.18% | -0.24% |
| Overall | -0.16% | 0.16% | 0.36% | -0.08% | 0.72% | 0.11% |
|  | -0.16% | 0.13% | 0.34% | -0.07% | 0.72% | 0.15% |
| Class F | -0.22% | 0.56% | -0.40% | 0.11% | -0.14% | 1.25% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. Method 2 results, common conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06% | -0.14% | -0.11% | -0.01% | 0.00% | 0.03% |
| Class B | -0.07% | -0.15% | -0.14% | 0.03% | 0.05% | 0.02% |
| Class C | -0.11% | -0.18% | -0.19% | 0.01% | 0.01% | 0.00% |
| Class D | -0.11% | -0.13% | -0.24% | 0.01% | 0.04% | 0.03% |
| Class E | -0.13% | -0.05% | 0.01% | 0.02% | -0.02% | 0.02% |
| Overall | -0.09% | -0.13% | -0.14% | 0.01% | 0.02% | 0.02% |
|  | -0.09% | -0.14% | -0.14% | 0.01% | 0.02% | 0.03% |
| Class F | -0.14% | -0.19% | -0.26% | 0.20% | -0.11% | 0.19% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.11% | 0.07% | 0.19% | 0.00% | 0.69% | 0.60% |
| Class B | -0.03% | 0.27% | 0.05% | 0.06% | 0.29% | 0.23% |
| Class C | -0.06% | 0.12% | 0.01% | 0.04% | 0.27% | 0.16% |
| Class D | -0.05% | 0.31% | -0.21% | 0.03% | 0.13% | 0.06% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | 0.20% | 0.01% | 0.03% | 0.34% | 0.26% |
|  | -0.02% | 0.19% | 0.01% | 0.04% | 0.34% | 0.24% |
| Class F | 0.00% | 0.04% | 0.01% | 0.28% | 0.42% | 0.37% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.05% | 0.79% | 0.84% | 0.10% | 0.95% | 0.21% |
| Class C | -0.07% | 0.28% | 0.30% | 0.10% | 0.18% | 0.29% |
| Class D | -0.05% | 0.98% | 0.58% | 0.05% | 0.67% | 0.20% |
| Class E | -0.13% | -0.15% | 0.26% | 0.16% | -0.98% | -1.32% |
| Overall | -0.07% | 0.53% | 0.53% | 0.10% | 0.33% | -0.06% |
|  | -0.07% | 0.42% | 0.48% | 0.10% | 0.42% | 0.06% |
| Class F | -0.18% | -0.36% | 0.14% | 0.17% | 1.07% | 0.34% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |

Table 4. Method 2 results, RDOQ off

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | 0.05% | -0.05% | -0.17% | 0.24% | 0.11% |
| Class B | -0.14% | 0.21% | 0.22% | -0.18% | 0.29% | 0.26% |
| Class C | -0.13% | 0.02% | 0.05% | -0.12% | 0.06% | 0.05% |
| Class D | -0.11% | 0.11% | 0.11% | -0.11% | 0.18% | 0.09% |
| Class E | -0.09% | 0.09% | 0.03% | -0.11% | 0.12% | 0.07% |
| Overall | -0.11% | 0.11% | 0.08% | -0.14% | 0.19% | 0.13% |
|  | -0.11% | 0.10% | 0.08% | -0.14% | 0.19% | 0.13% |
| Class F | -0.08% | 0.08% | -0.02% | 0.12% | 0.07% | 0.25% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | 0.05% | 0.60% | -0.18% | 0.53% | 0.72% |
| Class B | -0.14% | 0.32% | 0.54% | -0.15% | 0.45% | 0.39% |
| Class C | -0.23% | 0.12% | 0.37% | -0.16% | 0.30% | 0.28% |
| Class D | -0.27% | 0.37% | 0.11% | -0.19% | 0.26% | 0.65% |
| Class E |  |  |  |  |  |  |
| Overall | -0.19% | 0.22% | 0.41% | -0.17% | 0.39% | 0.50% |
|  | -0.19% | 0.22% | 0.42% | -0.16% | 0.33% | 0.51% |
| Class F | -0.05% | -0.06% | 0.07% | 0.12% | 0.49% | 0.67% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.19% | 0.44% | 0.84% | -0.19% | 0.71% | 0.63% |
| Class C | -0.34% | -0.10% | 0.00% | -0.29% | 0.23% | 0.21% |
| Class D | -0.37% | 0.12% | 0.41% | -0.21% | 0.76% | -0.15% |
| Class E | -0.16% | 0.11% | 0.26% | -0.05% | 0.63% | -0.09% |
| Overall | -0.27% | 0.16% | 0.41% | -0.19% | 0.59% | 0.19% |
|  | -0.26% | 0.16% | 0.32% | -0.19% | 0.60% | 0.27% |
| Class F | -0.37% | -0.27% | -0.46% | 0.04% | 0.04% | 0.28% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5. Method 2 results, common conditions, RDOQ improved

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.14% | -0.10% | -0.26% | -1.04% | -0.87% |
| Class B | -0.09% | -0.12% | -0.13% | -0.29% | -1.01% | -1.03% |
| Class C | -0.12% | -0.16% | -0.15% | -0.28% | -0.62% | -0.71% |
| Class D | -0.12% | -0.16% | -0.19% | -0.27% | -0.66% | -0.55% |
| Class E | -0.13% | 0.00% | -0.02% | -0.17% | -0.84% | -0.87% |
| Overall | -0.10% | -0.12% | -0.12% | -0.26% | -0.84% | -0.81% |
|  | -0.11% | -0.13% | -0.12% | -0.27% | -0.83% | -0.79% |
| Class F | -0.15% | -0.19% | -0.26% | -0.11% | -0.56% | -0.41% |
| Enc Time[%] | 107% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | -0.02% | 0.06% | -0.37% | -0.98% | -1.11% |
| Class B | -0.06% | 0.28% | -0.10% | -0.37% | -0.69% | -0.63% |
| Class C | -0.08% | 0.13% | 0.12% | -0.36% | -0.40% | -0.64% |
| Class D | -0.08% | 0.43% | 0.17% | -0.39% | -0.58% | -0.74% |
| Class E |  |  |  |  |  |  |
| Overall | -0.08% | 0.21% | 0.05% | -0.37% | -0.67% | -0.77% |
|  | -0.03% | 0.22% | 0.06% | -0.43% | -0.67% | -0.79% |
| Class F | -0.04% | 0.11% | 0.00% | -0.16% | -0.13% | -0.31% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.63% | 0.40% | -0.64% | -0.50% | -0.75% |
| Class C | -0.08% | 0.27% | 0.13% | -0.67% | -0.69% | -0.87% |
| Class D | -0.11% | 0.98% | 0.18% | -0.69% | -0.61% | -1.10% |
| Class E | -0.19% | -0.77% | -0.16% | -0.32% | -1.20% | -1.30% |
| Overall | -0.11% | 0.37% | 0.17% | -0.60% | -0.71% | -0.97% |
|  | -0.11% | 0.30% | 0.15% | -0.62% | -0.66% | -0.91% |
| Class F | -0.17% | -0.39% | 0.14% | -0.20% | 0.46% | -0.25% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

- Conclusions

Two methods improving the HM7.0 sign data hiding algorithm are proposed. They both simplify the current method at no performance impact for common conditions and providing some gains for the RDOQ off case.

# References

[1] X. Yu, J. Wang, D. He, G. Martin-Cocher, S. Campbell, “Multiple Sign Bits Hiding,” JCTVC-H0481, 8th  JCT-VC Meeting, San Jose, USA, February 2012.

[2] J. Sole, J. Chen, W.-J. Chien, M. Karczewicz, “Sign data hiding simplification and enhancement”, JCTVC-I0326, 9th JCT-VC Meeting, Geneva, CH, April/May, 2012.

[3] T. Ikai, “Simplified decision for sign hiding”, JCTVC-I0291, 9th JCT-VC Meeting, Geneva, CH, April/May, 2012.

[4] F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, 9th JCT-VC Meeting, Geneva, CH, April/May, 2012.

# Text for Working Draft

## Method 1

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| … |  |

## Method 2

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| signHidden = ((numCoeff − 1) − (numLastSubset <<4) >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| signHidden = !cu_transquant_bypass_flag ? 1: 0 |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).