| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0101 |
| --- | --- |

| Title: | Splitting contexts for MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Jianle Chen, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | vseregin@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In HM7.0, the first two binarized bins of motion vector difference (MVD) are coded with adaptive context and the remaining bins are coded with bypass mode; and there is no MVD signaling for list L1 if mvd_l1_zero_flag is signaled as true. Contexts for the first two bins are shared for all blocks regardless of their associated inter prediction direction or reference lists. However, MVD from different directions may have very different statistical properties, in which case context sharing may not be suitable. That is true especially if motion estimation is unbalanced at encoder side, for example some MVDs are set to zero for a particular list. This contribution proposes assigning a separate context to the first MVD bin with respect to uni-predicted, bi-predicted list L0 and bi-predicted list L1 MVD.

# Introduction and proposal

In current HM, the first two binarized bins of each motion vector difference (MVD) syntax are coded with two contexts respectively and the remaining bins (binarized with ExpGolomb code) are processed through bypass mode. The context models are shared among MVDs regardless of the MVDs belonging to horizontal or vertical direction, lists L0 or L1 and uni-predicted PU or bi-predicted PU. However, there is one exception. For GPB case, when all reference frames POCs are less than current frame POC, MVD for list L1 may be set to zero at encoder and the mvd_l1_zero_flag is signaled as true to decoder. In such case, L1 zero MVD is not signaled and is derived as zero at decoder side.

This zero MVD mode is controlled by the flag mvd_l1_zero_flag signaled for B slices in slice header. According to current HM, MVD signaling is changed only for list L1 and cannot be done for L0, also this change cannot be applied for non-GPB case. Additionally, the switch can be done only per slice and cannot be applied for more fine structures like CU.

When MVD is restricted at encoder side, statistics for MVD from list L0 and L1 can be unbalanced and context sharing does not work efficient. Following [1], this contribution propose to use separate contexts the first bin of the MVD syntax according to the reference picture list and inter prediction type. A separate context is assigned for the first bin respectively for uni-predicted, bi-predicted list L0 and bi-predicted list L1 MVD; hence two context models are added. The separation of context can adaptively accommodate the unbalanced MVD statistics, thus the zero MVD mode becomes unnecessary. Moreover, the proposed support more generic way for encoder to choose different optimized method.

Suggested context separation scheme was implemented on top of HM7.0 and results are summarised in the next Table 1 based on common test conditions [2].

Table 1. Proposal with two contexts against HM7.0 anchor

For above mentioned method, two additional contexts were introduced. To reduce this number, only one additional context [2] was tested as well and the results are summarised in Table 2. In this case, bin0 of MVD is separated for List 0 and List 1 for both uni- and bi-directional motion vectors.

Table 2. Proposal with one context against HM7.0 anchor

To reduce number of context further, contexts for bin0 and bin1 are shared for B-slices, one context is used for bin0 and bin1 coming from list L0 and one context for bin0 and bin1 coming from list L1. In this case, overall number of context used for MVD coding is same as anchor. Experimental results for context sharing are summarized in the next table.

Table 3. Proposal with context sharing for bin0 and bin1 against HM7.0 anchor

# Conclusion

Three methods for MVD bin coding are considered to adaptively accommodate possible unbalanced statistical characteristics of MVD for both lists L0 and L1. Skipping of MVD coding for list L1 is removed along with mvd_l1_zero_flag in slice header. Proposed method is more flexible since different strategies for MVD setting can be used, and desired strategy can be chosen at any level, for example picture, slice, CU or PU. We recommend adopting it in HEVC.

# References

- V. Seregin et al, “Splitting contexts for MVD coding” JCTVC-I0350, Geneva, Apr. 2012.

- V. Seregin et al, “Non-CE9: Splitting contexts for MVD coding” JCTVC-H0431, San Jose, Feb. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-I1100, Geneva, Apr. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-I1003.

## Two additional MVD contexts

### Slice header syntax

| if( slice_type = = B ) |  |
| --- | --- |
| mvd_l1_zero_flag | u(1) |

7.3.7	Prediction unit syntax

| …… |  |
| --- | --- |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ], 0) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ], 1) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |

7.3.7.1 Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y , RefPicList) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| …… |  |

7.4.3	Slice header semantics

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 2..4 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 1 | 5 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-1 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialisation variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 140 | 198 | 169 | 169 | 31 | 198 |

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 5 |

Table 9-39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| abs_mvd_greater0_flag[ ] | Error: Reference source not found1 | 0 | 0 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | inter_pred_idc != Pred_BI ? 0 : (RefPicList + 1 ) | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Error: Reference source not found1 | 1 | 0 | na | na | na | na |
|  |  | 5 | 0 | na | na | na | na |

## One additional MVD context

### Slice header syntax

| if( slice_type = = B ) |  |
| --- | --- |
| mvd_l1_zero_flag | u(1) |

7.3.7	Prediction unit syntax

| …… |  |
| --- | --- |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ], 0) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ], 1) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |

7.3.7.1 Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y , RefPicList) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| …… |  |

7.4.3	Slice header semantics

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 2..3 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 1 | 4 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-1 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialisation variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| initValue | 140 | 198 | 169 | 31 | 198 |

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 4 |

Table 9-39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| abs_mvd_greater0_flag[ ] | Error: Reference source not found1 | 0 | 0 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | RefPicList | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Error: Reference source not found1 | 1 | 0 | na | na | na | na |
|  |  | 4 | 0 | na | na | na | na |

## Shared MVD context

### Slice header syntax

| if( slice_type = = B ) |  |
| --- | --- |
| mvd_l1_zero_flag | u(1) |

7.3.7	Prediction unit syntax

| …… |  |
| --- | --- |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ], 0) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ], 1) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |

7.3.7.1 Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y , RefPicList) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| …… |  |

7.4.3	Slice header semantics

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc | Error: Reference source not found |  |  | 0..4 |
|  | ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 2..3 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 1 | 2..3 |
|  | mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-1 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialisation variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 140 | 198 | 169 | 31 |

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |

Table 9-39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| abs_mvd_greater0_flag[ ] | Error: Reference source not found1 | 0 | 0 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | RefPicList | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Error: Reference source not found1 | 1 | 0 | na | na | na | na |
|  |  | 2 | RefPicList | na | na | na | na |