| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0103 |
| --- | --- |

| Title: | AHG6: Decoupling SAO LCU on/off from SAO type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Chih-Wei Hsu, Chih-Ming Fu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | In Suk Chong +1-858-658-5673 {ichong, martak} @qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chihming.fu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-7.0, each LCU has one SAO type for each color component, and the SAO type can represent SAO-off, EO, or BO. In this proposal, SAO on/off signaling is decoupled from the SAO type. One SAO LCU on/off flag for each color component is sent before the other syntax elements of the color component. In addition to the new SAO LCU on/off flag coding, constrained merge syntax is also proposed where a merge-left flag is not coded and inferred as zero when the left LCU is SAO-off. Results reportedly show 0.0%/0.3%/0.3%/0.5% luma coding gains for Main-AI/RA/LB/LP, respectively.

# Introduction

In HM-7.0, SAO uses LCU-based syntax. Each LCU has one SAO type (i.e., sao_type_idx) for each color component, and the SAO type can represent SAO-off, EO, or BO (See Table 1). In the Geneva meeting, there were proposals which introduce decoupling of SAO-on/off signaling from sao_type_idx and then send it before merge signaling [1][2], in which the number of allowed values for sao_type_idx is reduced from 6 to 5 (See Table 2). Even though it has notable gains, one of concerns was to introduce dependency on SAO LCU on/off flags of upper LCUs for parsing of merge-up flags, i.e., not sending merge-up of current LCU when the upper LCU is SAO off. In this proposal we propose a modified solution which removes this dependency while providing similar gains.

Table 1.  Original SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Off |
| - 1 | - 1D 0-degree edge |
| - 2 | - 1D 90-degree edge |
| - 3 | - 1D 135-degree edge |
| - 4 | - 1D 45-degree edge |
| - 5 | - Band |

Table 2.  Proposed SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

# Proposed method

In the proposed method, we used the same approach in [2] except removing parsing dependency between upper LCU’s SAO-on/off flags and current LCU’s merge-up flags. But when parsing merge-left, we still propose to check SAO-on/off of the left LCU, i.e., not sending merge-left of current LCU when the left LCU is SAO off as in [2] .

# Simulation results

Experiments were conducted for Main-AI/RA/LB/LP conditions [3]. Results show 0.0%/0.3%/0.3%/0.5% luma coding gains for Main-AI/RA/LB/LP, respectively, as shown in Table 3.

# Conclusions

In this contribution, a simplified SAO LCU on/off flag coding method from JCTVC-I0199 is proposed for SAO signaling in HM-7.0. For each color component, one SAO LCU on/off flag, decoupled from the SAO type, is sent before the other SAO syntax elements of the color component. The proposal is integrated with constrained merge-left syntax to avoid sending merge-left when the left LCU is SAO-off. Results reportedly show 0.0%/0.3%/0.3%/0.5% luma coding gains for Main-AI/RA/LB/LP, respectively. We propose to adopt this proposal for HEVC.

# References

- W.-S. Kim, D.-K. Kwon, I. S. Chong, M. Karczewicz, “Non-CE1: LCU SAO enable flag coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0193, April, 2012.

- C.-W. Hsu, C.-M. Fu, T.-D. Chuang, Y.-W. Huang, S. Lei, W.-S. Kim, D.-K. Kwon, I. S. Chong, M. Karczewicz, “Non-CE1: Decoupling SAO on/off from SAO type with neighbor-based contexts,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0199, April, 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 3.  Results of the proposed method