#### 7.3.4.1 Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| sao_lcu_enable_flag[cIdx][rx][ry] | ae(v) |
| if( sao_lcu_enable_flag[cIdx][rx][ry] == 1 ) { |  |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile && sao_lcu_enable_flag[cIdx][rx - 1][ry]) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =4 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 4 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

sao_lcu_enable_flag[cIdx][rx][ry] equal to 1 sepecifies that SAO processing of current coding tree block is enabled; otherwise, there is no any additional SAO parameters for current coding tree block. The array index cIdx specifies the colour component; cIdx is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices rx and ry specify the location ( rx, ry ) of the considered coding treeblock relative to the top-left coding treeblock of the picture. When sao_lcu_enable_flag is not present, it is inferred to be equal to 0.

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-1Error: Reference source not found of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-1 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |
| - 4 | - Band offset |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the sample range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 4.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding tree block at position rx and ry for the colour component cIdx.

When sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx − 1 ][ ry ][ i ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to 0.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 5.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ]

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal 0.

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 4 and i is greater than 1,offsetSign is set equal to −1.

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is less than 4 and i is less than 2, offsetSign is set equal to 1.

- Otherwise, if sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is equal to 0, offsetSign is set to equal to 1.

- Otherwise, offsetSign is set to equal to −1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-1)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-2)

#### 8.7.2.1 Coding tree block modification process

Inputs to this process are:

–	picture buffer recPicture for the colour component cIdx,

–	a variable cIdx specifying colour component index,

–	a pair of variables ( rx, ry ) specifying the coding tree block location,

–	a coding tree block size nS.

Output of this process is a modified picture buffer saoPicture for the colour component cIdx.

The variable saoTypeIdx is set equal to sao_type_idx[ cIdx ][ rx ][ ry ].

The variable saoLcuEnableFlag is set equal to sao_lcu_encable_flag [ cIdx ][ rx ][ ry ].

The variable bitDepth is derived as follows.

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The variables xC and yC are set equal to rx*nS and ry*nS, respectively.

For i = 0..nS−1 and j = 0..nS−1, depending on the value of saoTypeIdx, pcm_loop_filter_disable_flag, pcm_flag, and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xC + i, yC + j ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- saoLcuEnableFlag is equal to 0.

–	Otherwise, if saoTypeIdx is equal to one of the values of 0, 1, 2 or 3, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k=0..1 are specified in Table 8-2 based on sao_type_idx[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows.

- If one or more of the following conditions for k=0..1 are true, edgeIdx is set equal to 0.

- The sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) is outside picture boundary

- slice_loop_filter_across_slices_enabled_flag is equal to 0 and the sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to a different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to a different tile.

- Otherwise, edgeIdx is dervied as follows.

edgeIdx = 2 + ∑k( Sign1( recPicture[ xC + i, yC + j ] – 
	recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-3)

When edgeIdx is equal to 0, 1, or 2, it is modified as follows.

edgeIdx = (edgeIdx = = 2) ? 0 : (edgeIdx + 1)	(8-4)

- The modified picture buffer saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-5)

- Otherwise (saoTypeIdx is equal to 4), the following ordered steps apply:

- The variable bandShift is set equal to bitDepth − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The vector bandTable is defined with 32 elements and all elements are initially set to 0. Then, four of its elements (indicating the starting position of bands for explicit offsets) are modified as follows.

for( k = 0; k < 4; k++ )
	bandTable[ (k + saoLeftClass) & 31 ] = k + 1	(8-6)

- The variable bandIdx is set equal to bandTable[ recPicture[ xC + i, yC + j ] >> bandShift ].

- The modified picture buffer saoPicture[ xC + i, yC + j ] is derived as follows.

saoPicture[ xC + i, yC + j ] = Clip3( 0, (1 << bitDepth) – 1, recPicture[ xC + i, yC + j ] + 
			SaoOffsetVal[ cIdx ][ rx ][ ry ][ bandIdx ] )	(8-7)

Table 8-2 – Specification of hPos and vPos according to the sample adaptive offset type

| sao_type_idx[ cIdx ][ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 9-X – Values of variable initValue for sao_lcu_enable_flag ctxIdx

| Initialization variable | sao_lcu_enable_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 46 | 79 | 46 | 79 | 63 | 111 |

| Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element |  | initType |  | Type of binarization |  |  | maxBinIdxCtx |  | ctxIdxTable |  | ctxIdxOffset |  |  |
| sao_lcu_enable_flag |  | 0 |  | FL, cMax = 1 |  |  | 0 |  | Table 9- |  | 0 |  |  |
|  |  | 2 |  |  |  |  | 0 |  | Table 9- |  | 2 |  |  |
|  |  | 4 |  |  |  |  | 0 |  | Table 9- |  | 4 |  |  |
| Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Syntax element | ctxIdxTable, ctxIdxOffset |  |  |  | binIdx |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 1 |  | 2 |  | 3 |  | >=4 |  |
| sao_lcu_enable_flag | Table 9- |  | 0 |  | 0,1 (subclause ) | na |  | na |  | na |  | na |  |
|  |  |  | 2 |  | 0,1 (subclause ) | na |  | na |  | na |  | na |  |
|  |  |  | 4 |  | 0,1 (subclause ) | na |  | na |  | na |  | na |  |

###### 9.2.3.1.1.x Derivation process of ctxIdxInc using left syntax element of sao_lcu_enable_flag

Input to this process are the colour component index cIdx, the location (  rx, ry ) specifying the considered coding treeblock relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the location ( rxL, ryL ) specify a location covered by the coding treeblock to the left of the considered coding treeblock with rxL = rx − 1 and ryL = ry, the variable availableL is derived as follows.

- If the coding treeblock covering location ( rxL, ryL ) is available, availableL is set equal to 1.

- Otherwise (the coding treeblock covering location ( rxL, ryL ) is not available), availableL is set equal to 0.

The assignment of ctxIdxInc for the syntax element sao_enable_flag is specified as follow.

ctxIdxInc = ( sao_lcu_enable_flag [ cIdx ][ rxL ][ ryL ] && availableL )	(9-8x)