| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0105_r1 WG11 Number: m25427 |
| --- | --- |

| Title: | Simplification of coefficient scanning order |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

The current HEVC text specification maps 9 Intra prediction modes to the horizontal coefficient scan order and 9 other Intra prediction modes to the vertical coefficient scan order. This contribution proposes reducing the range of modes from 9 to 8. It is reported that by reducing the range in this way to a power of 2, a table-lookup or set of range comparisons can be replaced by a Boolean check on a few bits of the Intra prediction mode value. Three variations of this method are presented. Average BD-Rate changes range from 0.1% - 0.2% for Simplification 1 and 0.0% - 0.1% for Simplifications 2 and 3. Changes to the HEVC text specification and additional results from combining this proposal with JCTVC-J0093 are provided as well.

# Introduction

Table 7-12 of the High Efficiency Video Coding (HEVC) text specification draft 7 [1] defines a mapping between the Intra prediction mode and the coefficient scan order ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]. It maps 9 modes to ScanType 1 (horizontal) and a different set of 9 modes to ScanType 2 (vertical). The rest of the modes are mapped to ScanType 0 (up-right diagonal). This mapping is shown in Figure 1.

Figure 1. Current HEVC scan order mapping for Intra prediction modes

The mapping from 9 values, however, typically requires a table-lookup or range comparisons to see if the Intra prediction mode is less than or greater than a set of values. If this range of mapped values is reduced to 8, then the table-lookup or range check can be replaced by a binary check on a few bits of the Intra prediction mode value.

# Scan order map simplifications

Three versions of this simplification are tested as shown in Figure 2.

Figure 2. Three simplifications for coefficient scan order mapping

## Simplification 1

This simplification reduces the number of horizontal and vertical scan mappings both from 9 to 8, and aligns the start of each range to be a multiple of 8. The mapping is shown in Table 1.

Table 1. Modification of ScanType mapping for Simplification 1

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 7 | 0 | 0 | 0 | 0 |
| 8 – 15 | 2 | 2 | 0 | 0 |
| 16 – 23 | 0 | 0 | 0 | 0 |
| 24 – 31 | 1 | 1 | 0 | 0 |
| 32 – 35 | 0 | 0 | 0 | 0 |

Because the range of modes has been reduced from 9 to 8 as described earlier, and because the transitions of the relevant mapping are aligned to multiples of 8, then only 3 bits of the IntraPredMode value need to be examined:

The horizontal ScanType is used if:    (IntraPredMode >> 3) ^ 0x03 is 0.

The vertical ScanType is used if:        (IntraPredMode >> 3) ^ 0x01 is 0.

## Simplification 2

This simplification is the same as Simplification 1, except the range of values is offset by 1 so that the 8 modes are the upper subset of the 9 modes currently in the HEVC specifications.

Table 2. Modification of ScanType mapping for Simplification 2

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 6 | 0 | 0 | 0 | 0 |
| 7 – 14 | 2 | 2 | 0 | 0 |
| 15 – 22 | 0 | 0 | 0 | 0 |
| 23 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

## Simplification 3

This simplification is the same as Simplification 1, except the range of values is offset by 2 so that the 8 modes are the lower subset of the 9 modes currently in the HEVC specifications.

Table 3. . Modification of ScanType mapping for Simplification 3

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 13 | 2 | 2 | 0 | 0 |
| 14 – 21 | 0 | 0 | 0 | 0 |
| 22 – 29 | 1 | 1 | 0 | 0 |
| 30 – 35 | 0 | 0 | 0 | 0 |

# Experimental results

The three simplifications were tested using the HM-7.0 software and common test conditions [2].

## Simplification 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class C | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
| Class E | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class F | 0.2% | 0.3% | 0.3% | 0.1% | 0.0% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.3% |
| Class D | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | -0.2% |
| Class E | -0.1% | 0.6% | -0.3% | 0.1% | -0.5% | -0.5% |
| Overall | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.3% | 0.2% | 0.1% | 1.2% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

## Simplification 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.5% | -0.2% |
| Class E | -0.1% | 0.5% | -0.3% | 0.0% | -0.4% | 0.3% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class F | 0.0% | 0.2% | 0.2% | 0.1% | 0.9% | 0.7% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 90% |  |  | 92% |  |  |

## Simplification 3

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.1% | 0.2% | -0.3% |
| Class E | -0.1% | 0.2% | -0.2% | 0.1% | 0.0% | -0.3% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class F | 0.0% | 0.6% | 0.5% | 0.1% | 0.8% | 0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 93% |  |  |

# Additional results in combination with JCTVC-J0093

In JCTVC-J0093 [3], “Rotation of Residual Block for Transform Skipping,” the residual transform block is rotated in the encoder prior to scanning. To see if this scan modification proposal is compatible with the rotation, we repeated the experiment for Simplification 3 above with the software modified to also perform the rotation as described in JCTVC-J0093. Only the High-Efficiency configurations have transform skip mode enabled. The results below use the results from JCTVC-J0093 (with RDOQ on) as a reference.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | -0.5% |
| Class B | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.4% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% |
| Class D | -0.1% | 0.3% | 0.4% |
| Class E | 0.0% | -0.6% | -0.2% |
| Overall | 0.0% | -0.1% | 0.1% |
|  | 0.0% | -0.2% | 0.1% |
| Class F | -0.1% | -0.2% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |

- Conclusions

The range of Intra prediction modes mapped to the horizontal and vertical coefficient scan order is reduced from 9 to 8, which makes implementation of this mapping simpler. Simplification 1 reduces the range to 8 and aligns the start of the ranges to be divisible by 8. The average BD-Rate changes for Intra configurations are 0.1% - 0.2%. Simplifications 2 and 3 offset the ranges by 1 or 2 so they are contained within the unmodified ranges specified by HEVC. The average BD-Rate changes for Intra configurations of Simplifications 2 and 3 are 0.0% to 0.1%. This proposal was also evaluated in combination with the block rotation of JCTVC-J0093, and BD-Rate changes for this proposal, as compared to JCTVC-J0093 as a reference, average to 0.0%, showing that the proposed simplification is fully compatible with JCTVC-J0093 as well. We propose adopting either Simplification 2 or 3, as they provide the simplification while minimizing changes from the existing specifications. If hardware implementers strongly favor having the range start locations be divisible by 8, Simplification 1 can be considered for adoption as well.

# References

- B. Bross et al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003_d5, Geneva, CH, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, Geneva, CH, May 2012.

- D. He et al., “Rotation of Residual Block for Transform Skipping,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0093, Stockholm, SE, July 2012.

# Changes to the HEVC text specification

The changes affect only the IntraPredMode column of Table 7-12. Additions are highlighted in yellow. Deletions are highlighted in blue.

## Simplification 1

Table 7 12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 57 | 0 | 0 | 0 | 0 |
| 68 – 1415 | 2 | 2 | 0 | 0 |
| 1516 – 2123 | 0 | 0 | 0 | 0 |
| 2224 – 3031 | 1 | 1 | 0 | 0 |
| 3132 – 35 | 0 | 0 | 0 | 0 |

## Simplification 2

Table 7 12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 56 | 0 | 0 | 0 | 0 |
| 67 – 14 | 2 | 2 | 0 | 0 |
| 15 – 2122 | 0 | 0 | 0 | 0 |
| 2223 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

## Simplification 3

Table 7 12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 1413 | 2 | 2 | 0 | 0 |
| 1514 – 21 | 0 | 0 | 0 | 0 |
| 22 – 3029 | 1 | 1 | 0 | 0 |
| 3130 – 35 | 0 | 0 | 0 | 0 |

# Patent rights declaration

Mitsubishi Electric Research laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).