| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0106 |
| --- | --- |

| Title: | AHG6/AHG5: SAO offset coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 ichong@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In HM7.0, SAO offsets are coded using truncated unary binarization with all bins being context-coded. The maximum offset value depends on the internal bit-depth: ranges 0~7/0~15/0~31 are allowed for 8/9/10 bit-depth, respectively. Therefore, the worst-case number of context-coded bins per offset is 7/15/31 bins with TU binarization. Considering that a total of 12 offsets are sent per LCU, the worst-case is allegedly too large in the current design. We propose a different binarization to reduce the worst number of bins to 5/7/9 for bit-depth 8/9/10 or higher, respectively, coding some of the bins in bypass and without coding performance loss on average.

# Technical Description

In the current HM7.0, sao_offset in Table 1 is coded with truncated unary binarization with bin0 coded with ctx0 and remaining bins with ctx1.

| Syntax element | Bin Type |
| --- | --- |
| sao_type_idx | U with Regular |
| if( sao_type_idx = = 5 ) sao_band_position | FL(5) with Bypass |
| for (i=0; i<4; i++) sao_offset_abs[i] | TU with Regular |
| for (i=0; i<4; i++) if (sao_offset_abs[i]) sao_offset_sign[i] | U(1) with Bypass |

Table 1. HM7.0 SAO syntax elements and corresponding bin type

With truncated unary sao_offset binarization, the worst-case number of bins is 7/15/31 bins for 8/9/10 or higher bit-depth. All bins are context coded: bin0 is coded with ctx0 and the rest are coded with ctx1.

In this proposal, we propose to reduce those worst-case number of bins using the binarization in Table 2, which is similar to last coefficient position coding.

## 8 bit bit-depth case, maxSymbol = 7

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 111 | XX | 0-3 |

## 9 bit bit-depth case, maxSymbol = 15

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 1110 | XX | 0-3 |
| 8-15 | 1111 | XXX | 0-7 |

## 10 bit or higher bit-depth cases, maxSymbol = 31

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 1110 | XX | 0-3 |
| 8-15 | 11110 | XXX | 0-7 |
| 16-31 | 11111 | XXXX | 0-15 |

Table 2. Proposed SAO offset binarization

In the proposed SAO offset binarization, the worst-case number of bins are 5/7/9 for 8/9/>=10 bit-depth, repectively. Also for only prefix parts are coded with contexts, i.e., bin0 is coded with ctx0 and the rest of bins with ctx1. The worst-case number of context-coded bins is 3/4/5, respectively. Table 3 summarizes the worst-case scenarios.

| Worst-case bins | HM7.0 |  | Proposal |  |
| --- | --- | --- | --- | --- |
| Bit-depth | Regular | Bypass | Regular | Bypass |
| 8 | 7 | - | 3 | 2 |
| 9 | 15 | - | 4 | 3 |
| 10 and higher | 31 | - | 5 | 4 |

Table 3. Comparison of the worst-case bins with prefix part context-coded and the suffix in bypass

We also propose further simplifications to reduce the number of context coded bins and contexts. In one approach, we used 1 bin with context-coded and others bypass-coded. In another approach, we tested all bins bypass-coded. Table 4 summarize the worst-case scenarios.

| Worst-case | HM7.0 |  | Proposal (1 ctx-coded) |  | Proposal (all bypass) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bit-depth | Regular | Bypass | Regular | Bypass | Regular | Bypass |
| 8 | 7 | - | 1 | 4 | - | 5 |
| 9 | 15 | - | 1 | 6 | - | 7 |
| 10 and higher | 31 | - | 1 | 8 | - | 9 |

Table 4. Comparison of the worst-case bins with 1 bin context-coded

As it is done for other syntax elements in HEVC (e.g., last coefficient position), the bypass bins of the offsets can be grouped together when 1 more more bins of  the offset binarization are context-coded:

| Syntax element | Bin Type |
| --- | --- |
| sao_type_idx | U with Regular |
| for (i=0; i<4; i++) sao_offset_abs_ctx[i] | TU with Regular |
| for (i=0; i<4; i++) sao_offset_abs_bypass[i] | Bypass |
| for (i=0; i<4; i++) if (sao_offset_abs_ctx[i]) sao_offset_sign[i] | U(1) with Bypass |
| if( sao_type_idx = = 5 ) sao_band_position | FL(5) with Bypass |

Table 5. Grouping of bypass coded bins

# Simulation results

The proposed methods are implemented in HM7.0 and simulated under the common test conditions. Performance shows almost no loss for both main and HE10 configurations:

- Table 6 shows the results for coding the prefix bins with context and the suffix bins in bypass.

- Table 7 shows the results for coding only 1 bin with context.

- Table 8 shows the results for coding all bins in bypass.

Source code is included in this package. Results shows that no coding loss on average for all coding configurations, even though for all bins coded with bypass case shows 0.1% loss in LP configuration.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% | 0.2% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.3% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Class D | 0.0% | 0.4% | 0.8% | 0.0% | 0.5% | -0.2% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | 0.1% | 0.4% | 0.5% | 0.1% | 0.4% | 0.7% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.1% | 0.2% | -0.1% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | 0.2% | 0.4% | 0.0% | 0.0% | -0.3% |
| Class E | 0.1% | -0.7% | 0.0% | 0.0% | 0.4% | 0.2% |
| Class F | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 6. results for proposed binarization with prefix bins context-coded

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
| Class B | 0.0% |  | 0.3% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class C | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class D | 0.0% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class E | 0.0% |  | 0.4% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Overall | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
|  | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
| Class F | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Enc Time[%] | 101% |  |  |  |  |  | 101% |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class B | 0.0% |  | 0.5% |  | 0.4% |  | 0.0% |  | 0.1% |  | 0.2% |  |  |
| Class C | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.2% |  | 0.0% |  |  |
| Class D | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.0% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% |  | 0.3% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.1% |  |  |
|  | 0.0% |  | 0.3% |  | 0.2% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class F | 0.1% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.3% |  |  |
| Enc Time[%] | 100% |  |  |  |  |  | 99% |  |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  | Low delay B Main |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% |  | 0.6% |  | 0.8% |  | 0.0% |  | 0.3% |  | 0.3% |  |  |
| Class C | 0.0% |  | 0.3% |  | 0.4% |  | 0.0% |  | 0.2% |  | 0.3% |  |  |
| Class D | 0.0% |  | 0.4% |  | 0.7% |  | 0.0% |  | 0.0% |  | -0.2% |  |  |
| Class E | 0.0% |  | 0.7% |  | 1.0% |  | 0.1% |  | -0.4% |  | 0.4% |  |  |
| Overall | 0.0% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.1% |  | 0.2% |  |  |
|  | 0.0% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.2% |  | 0.2% |  |  |
| Class F | 0.0% |  | 0.4% |  | 0.5% |  | 0.1% |  | 0.3% |  | 1.0% |  |  |
| Enc Time[%] | 103% |  |  |  |  |  | 101% |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  |  | Low delay P Main |  |  |  |  |  | Low delay P HE10 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.2% |  | 0.9% |  | 1.1% |  | 0.1% |  | 0.1% |  | 0.1% |  |
| Class C |  | 0.1% |  | 0.2% |  | 0.4% |  | 0.1% |  | 0.3% |  | 0.2% |  |
| Class D |  | 0.1% |  | 0.4% |  | 0.8% |  | 0.0% |  | 0.3% |  | -0.3% |  |
| Class E |  | 0.3% |  | 0.6% |  | 1.5% |  | 0.1% |  | 0.6% |  | 0.0% |  |
| Class F |  | 0.0% |  | 0.1% |  | 0.1% |  | -0.1% |  | -0.1% |  | -0.4% |  |
| Overall |  | 0.1% |  | 0.5% |  | 0.8% |  | 0.0% |  | 0.2% |  | 0.0% |  |
|  |  | 0.1% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.2% |  | 0.1% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  |  | 100% |  |  |  |  |  |

Table 7. results for proposed binarization with 1bin context-coded

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.4% | 0.4% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.4% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | -0.1% |
| Class F | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.7% | 0.6% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.4% | 0.4% | 0.0% | 0.1% | 0.4% |
| Class D | 0.0% | 0.4% | 0.4% | 0.0% | 0.4% | -0.1% |
| Class E | 0.0% | 0.7% | 0.7% | 0.1% | -0.1% | 0.4% |
| Overall | 0.0% | 0.5% | 0.5% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.5% | 0.5% | 0.0% | 0.2% | 0.3% |
| Class F | 0.1% | 0.8% | 0.3% | 0.2% | 0.1% | 0.8% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 1.0% | 0.8% | 0.1% | 0.2% | 0.1% |
| Class C | 0.1% | 0.3% | 0.5% | 0.0% | 0.1% | 0.3% |
| Class D | 0.1% | 0.5% | 0.3% | 0.1% | 0.4% | -0.2% |
| Class E | 0.3% | 0.8% | 1.7% | 0.0% | 0.5% | 0.5% |
| Class F | 0.0% | 0.4% | 0.1% | 0.1% | 0.3% | -0.3% |
| Overall | 0.1% | 0.6% | 0.6% | 0.1% | 0.3% | 0.0% |
|  | 0.1% | 0.6% | 0.6% | 0.1% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 8. results for proposed binarization with all bins bypass-coded

# Conclusions

A modification in the offset coding for SAO decreases the worst-case number of bins as well as the number of context coded bins in SAO offset coding significantly without coding loss. We propose to adopt the proposal into HEVC.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).