| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0110 |
| --- | --- |

| Title: | AHG9: On POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L Joshi Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com +1-858-651-8345 yekuiw@qualcomm.com +1-858-658-5804 aramasub@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document proposes the following:

- A change to the definition of picture order count to cover the missing cases

- Removal of the function DifPicOrderCnt( ) or consistently use it in the specification, particular for derivation processes for motion vector and reference indices

- Removal of one of the POC-related constraints in Annex C.4

- An SEI message conveying additional POC information for intra pictures to enable intra-picture-only trick modes

# On POC definition

The current definition of picture order count (POC) is as follows:

picture order count: A variable that is associated with each coded picture and has a value that is increasing with increasing picture position in output order relative to one of the following coded pictures:

- the previous IDR picture in decoding order, if any

- the previous BLA picture in decoding order, if any

- the previous CRA picture in decoding order, if any and if the previous CRA picture is the first coded picture in the bitstream in decoding order.

If more than one of the above coded pictures is present, the picture order count is relative to the last of such coded pictures in decoding order.

The definition covers all non-RAP pictures and all CRA pictures after the first picture in the bitstream in decoding order, but it does not cover IDR and BLA pictures themselves and the CRA picture that starts to the bitstream. Therefore, to cover these missing cases, we propose to change the definition as follows:

picture order count: A variable that is associated with each coded picture and that indicates the output order of the associated picture. If a picture is a CRA picture that is the first picture in the bitstream, an IDR picture, or a BLA picture, the picture order count is equal to the value of the least significant bits of the picture order count. Otherwise, the picture order count has a value that is increasing with increasing picture position in output order relative to one of the following coded pictures:

- the previous IDR picture in decoding order, if any

- the previous BLA picture in decoding order, if any

- the previous CRA picture in decoding order, if any and if the previous CRA picture is the first coded picture in the bitstream in decoding order.

If more than one of the above coded pictures is present, the picture order count is relative to the last of such coded pictures in decoding order.

# On the function DiffPicOrderCnt( )

Same as in AVC, the current HEVC draft includes the definition of the function DiffPicOrderCnt( ), as follows.

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-1)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that exceed the range of −215 to 215 − 1, inclusive.

NOTE 3 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt(X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

In AVC, the function DiffPicOrderCnt( ) has been used in the following places:

- Derivation process for motion vector components and reference indices

- Derivation process for prediction weights

In the current HEVC draft, the function DiffPicOrderCnt( ) is only used in the semantics of two inter-RPS prediction syntax elements (delta_rps_sign, abs_delta_rps_minus1) and the semantics of one SOP description SEI message syntax element (poc_delta[ i ]). There are other places that involve a different between two POC values but do not use this function.

We propose to either remove the definition of the functions DifPicOrderCnt( ), and in the semantics of delta_rps_sign, abs_delta_rps_minus1 and poc_delta[ i ] directly use word description as necessary, or use the function consistently in the specification, in particular in the derivation process for motion vector and reference indices, similar as in AVC.

# On two POC related constraints

Section C4 of HEVC text specification draft 7 defines conditions for bitstream conformance. There are two conditions related to POC:

- For any current picture being decoded, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the minimum, respectively, of the PicOrderCntVal values for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output". It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2

- Let the variable prevRefPic be the previous reference picture in decoding order that has temporal_id equal to or less than the temporal_id of the current picture. The absolute difference between the PicOrderCntVal values for the current picture and for prevRefPic shall be less than MaxPicOrderCntLsb / 2.

In our understanding, the rationale behind condition 1 was mainly to enable identifying a picture in the DPB with POC LSB only. At that time, this enabled signalling of long term pictures by specifying just the LSB. But it is our contention that the mandatory signalling of delta_poc_msb_present_flag when multiple pictures in the DPB have the same LSB as the LSB for the indicated LTRP makes the first condition unnecessary. See more discussions below.

Condition 2 is necessary to guarantee the correctness of POC derivation. Furthermore, for commonly used hierarchical structures, it also assures correct POC derivation under loss of one picture. For example, if the GOP size is 8, MaxPicOrderCntLsb has to be at least 32 in order to satisfy condition 2. Assuming one picture loss, the maximum POC difference between the current picture and prevRefPic is 16. So under this scenario, condition 2 becomes invalid. But since the maximum POC difference is always positive in this scenario, MaxPicOrderCntLsb of 32 is enough to correctly derive the POC. We provide examples to prove this.

Consider that picture 8 (POC) is lost. Then, for picture 16, the prevRefPic is 0. Let MaxPicOrderCntLsb be 32. Then, prevPicOrderCntLsb and prevPicOrderCntMsb are both 0. The derivation of PicOrderCntMsb, as specified in Equation 8.1 of the current HEVC draft, is reproduced below for reference:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-2)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

For picture 16, in our example, pic_order_cnt_lsb (equal to 16) is greater than prevPicOrderCntLsb. But the POC difference between the current picture and the prevRefPic is not greater than 16. Thus, PicOrderCntMsb is derived to be 0 and the PicOrderCntVal is derived as 16.

Now consider that picture 24 is lost. Then for picture 32, the prevRefPic is 16. Then, prevPicOrderCntLsb is 16 and prevPicOrderCntMsb is 0. In this case, pic_order_cnt_lsb (equal to 0) is less than prevPicOrderCntLsb and the POC difference between the current picture and the prevRefPic is less than or equal to 16. Thus, PicOrderCntMsb is set equal to (prevPicOrderCntMsb + MaxPicOrderCntLsb), which is equal to 32. Thus the PicOrderCntVal is derived as 32.

Thus condition 2 ensures correct POC derivation and is robust to one picture loss for commonly used hierarchical structures.

In the current HEVC draft, the signalling of short-term reference pictures (STRPs) is based on delta POC values. Thus the full POC is matched to identify the appropriate picture from the DPB. Even though multiple pictures in the DPB, that are eligible to be short term reference pictures, may have the same LSB values (in the absence of condition 1), their full POC values are different. Hence, the short term reference picture set can be derived correctly.

For signalling of LTRPs, if multiple pictures in the DPB have LSB values equal to the LSB value for the signalled LTRP, the current HEVC draft mandates that delta_poc_msb_present_flag is set to one for those pictures. This is followed by the delta_poc_msb_cycle_lt. In such a scenario, poc_lsb_lt and delta_poc_msb_cycle_lt together enable calculation of the full POC value for the long-term reference picture. Thus, it is possible to distinguish between multiple pictures in the DPB that have the same LSB values.

Thus, we have demonstrated that condition 2 in conjunction with the current syntax for STRP and LTRP signalling is sufficient to enable correct derviation of POC as well as short-term and long-term reference picture sets.Based on the above analysis, we think that the contraint is unnecessary. Hence we propose the removal of condition 1.

# Fast forward and rewind trick modes

It may be desirable to extract and decode only intra pictures from an HEVC conformant bitstream, to enable trick modes such as fast forward and rewind. In HEVC, intra pictures include RAP pictures (IDR/CRA/BLA) as well as non-RAP pictures containing only intra slices. An extracted bitstream consisting of only intra pictures is typically not required to be a conforming bitstream. For such a bitstream, it may no longer be possible to derive correct POC values for each picture. As an example, consider that only intra pictures from an HEVC conformant bitstream are extracted.  Let MaxPicOrderCntLsb = 256.  Let two consecutive intra pictures in the extracted bitstream be denoted by P1 and P2 (in output order) with POC values of 0 and 385, respectively.  The POC LSBs for the 2 pictures would be 0 and 129, respectively.  For the purposes of POC derivation as specified in HEVC, if P1 is treated as the prevRefPic by P2, the POC MSB of P2 would be derived as -256.  Thus a decoder would incorrectly output P2 before P1.

To enable fast forward and rewind trick modes, the decoder needs to derive the correct output order for the intra pictures. That is, the decoder needs to derive correct relative POC values. A new SEI message is proposed to accomplish this.

## SEI message for fast forward and rewind trick modes

Let prevIntraPic be defined as the previous intra picture in decoding order.  This is also the previous picture in decoding order in the extracted bitstream consisting of only the intra pictures from the original bitstream. We propose sending just enough additional LSB bits (POC LSBs) to enable correct POC derivation based on prevIntraPic. An SEI message is proposed to signal the additional POC LSBs as well as the length.

### SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 48 ) |  |
| poc_info( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| … |  |
| } |  |

### Picture order count information SEI message syntax

| poc_info( payloadSize ) { | Descriptor |
| --- | --- |
| sps_id | ue(v) |
| poc_additional_lsbs_len | ue(v) |
| if (poc_additional_lsbs_len > 0) |  |
| poc_additional_lsbs | u(v) |
| } |  |

### Picture order count information SEI message semantics

The picture order count information SEI message provides additional information to enable correct derivation of picture order count values for intra pictures when none of the non-intra pictures are present in the bitstream. A picture order count information SEI message, when present, applies to the associated picture. In a coded video sequence, either all the intra pictures have an associated picture order count information SEI message, or none of the intra pictures has an picture order count information SEI message.

sps_id specifies the sequence parameter set that contains the log2_max_pic_order_cnt_lsb used for derivation of the number of bits used to represent poc_additional_lsbs. The value of sps_id shall be equal to the value of seq_parameter_set_id in the picture parameter set referenced by the coded picture associated with the picture order count information SEI message. The value of sps_id shall be in the range of 0 to 31, inclusive.

poc_additional_lsbs_len specifies the number of additional least significant bits of the picture order count value for the associated picture. The value of poc_additional_lsbs_len shall be in the range of 0 to 64 – log2_max_pic_order_cnt_lsb – 4, inclusive.

poc_additional_lsbs specifies the additional least significant bits of the picture order count value for the associated picture when poc_additional_lsbs_len is non-zero. The length of poc_additional_lsbs is equal to poc_additional_lsbs_len bits.

In the following, "the extracted bitstream" refers to the extracted bitstream consisting of only the intra pictures in the original bitstream.

The picture order count for an intra picture from the extracted bitstream is derived as follows. The variable MaxIntraPicOrderCntLsb is derived as follows:

MaxIntraPicOrderCntLsb = 2 ( log2_max_pic_order_cnt_lsb_minus4 + 4 + poc_additional_lsbs_len )

Let prevIntraPic be the previous intra picture in decoding order. This is also the previous picture in decoding order in the extracted bitstream. Let prevIntraPicOrderCnt be equal to PicOrderCntVal of prevIntraPic. The variable prevIntraPicOrderCntMsb is set equal to MaxIntraPicOrderCountLsb × Floor( prevIntraPicOrderCnt / MaxIntraPicOrderCountLsb ), and the variable prevIntraPicOrderCntLsb is set equal to prevIntraPicOrderCnt – prevIntraPicOrderCntMsb.

Let pic_order_cnt_lsb_extended for the intra picture be derived as

pic_order_cnt_lsb_extended = ( poc_additional_lsbs << ( log2_max_pic_order_cnt_lsb_minus4 + 4) )
						  + pic_order_cnt_lsb

IntraPicOrderCntMsb of the current intra picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb_extended  <  prevIntraPicOrderCntLsb )  &&
		( ( prevIntraPicOrderCntLsb − pic_order_cnt_lsb_extended ) >= (MaxIntraPicOrderCntLsb / 2 ) ) )
	IntraPicOrderCntMsb = prevIntraPicOrderCntMsb + MaxIntraPicOrderCntLsb
else if( (pic_order_cnt_lsb_extended  >  prevIntraPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb_extended  − prevIntraPicOrderCntLsb )  >  (MaxIntraPicOrderCntLsb / 2 ) ) )
	IntraPicOrderCntMsb = prevIntraPicOrderCntMsb − MaxIntraPicOrderCntLsb
else
	IntraPicOrderCntMsb = prevIntraPicOrderCntMsb

PicOrderCntVal of the current intra picture, denoted by IntraPicOrderCntVal, is derived as

IntraPicOrderCntVal = IntraPicOrderCntMsb + pic_order_cnt_lsb_extended

The syntax element poc_additional_lsbs_len should be chosen such that the absolute difference between IntraPicOrderCntVal and prevIntraPicOrderCnt is less than MaxIntraPicOrderCntLsb / 2.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).