| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0123 |
| --- | --- |

| Title: | On tiles and wavefront parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com 1-858- 651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

There were discussions that the syntax design itself should not disallow the combination of tiles and WPP, and whether the combination is allowed should be a profile issue. This document proposes a syntax that allows for combination of parallelism tools (tiles, WPP, and entropy slices), while whether different combinations are allowed can be either restricted through semantics or through profile definitions. For the combination of tiles and WPP, a restricted co-existence is proposed, by mandating every tile to start a new slice when both of these two parallelism tools co-exist in a picture. However, the document does not intend to propose to change the fact that in the Main profile (currently the only profile defined) combination of tiles and WPP is disallowed.

# Introduction

HEVC text specification draft 7 [1] does not support co-existence of tiles and wavefront parallel processing (WPP). There were discussions that the syntax design itself should not disallow the combination of tiles and WPP, and whether the combination is allowed should be a profile issue.

If co-existence of tiles and wavefront by design is to be allowed, then a simple way to support the combination of both parallelism mechanisms would be to mandate that every tile start a new slice if both tiles and WPP exist in a picture. This simplifies the design and signaling of entry points for tiles or wavefronts. Under this scenario since tiles start new slices, the entry points signaled in the slice header would correspond to the wavefront substream entry points. This would also imply that each wavefront stays within a tile. A wavefront cannot span across tiles. If co-existence of tiles and WPP is not present, then this restriction is not imposed, and the current existing restrictions regarding tiles and WPP would hold.

The proposed scheme for co-existence of tiles and WPP can be illustrated by an example shown in the following figure:

As can be seen from the figure, in the above example, there are 4 tiles and 16 WPP substreams co-existing within a picture and every tile starting a new slice.

Our justification to allow only the above restricted co-existence of tiles and WPP in the design is as follows. Use of both tiles and WPP may only make sense to us when the spatial resolution is high, and only a few, e.g. 4 tiles are used. In this case, the overhead of a few more slice headers is negligible, not to mention that slice headers may be predicted, as discussed in JCTVC-J0109. To make the design to allow for flexible co-existence of tiles and WPP, then the relationships among slices, tiles and WPP would need to be carefully clarified. Note that the discussions on a subset of them currently specified in the specification, i.e., for the relationship between slices and tiles, and the relationship between slices and WPP, particularly the former, were both resulted from long and hard discussions. Also, the syntax and semantics of tile and WPP entropy point offset signalling would need to re-designed. While syntax wise it may be simple to just add one flag to indicate whether a particular entry point offset is for a tile or a WPP sub-stream, the semantics would not be easy to specify as herein the different combinations of slices, tiles and WPP would have to be considered.

# Proposed text changes

Basically, syntax wise the change is to replace the two-bit tiles_or_entropy_coding_sync_idc in the PPS to a three-bit parallelism_idc, wherein the first bit indicates the use of tiles, the second bit indicates the use of WPP, and the third bit indicates the use of entropy slices. This way, by syntax all possible combinations of the three parallelism tools are allowed, such that, if it is decided to certain combination in a profile (now or in the future, in a new single-layer-single-view coding profile or in a scalable or 3D profile), there is no need to change the syntax.

7.3.2.3 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. | ue(v) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| parallelism_idc | u(3) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| if( ( parallelism_idc & 0x1 ) = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| } else if( parallelism_idc = = 4 ) |  |
| cabac_independent_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| …… |  |

7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …. | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| if(parallelism_idc & 0x3 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| ….. |  |

parallelism_idc indicates the possible presence of tiles, wavefronts, or entropy slices in a picture referring to the picture parameter set. The least significant bit (bit 0) of parallelism_idc equal to 1 specifies that there may be more than one tile in the picture, and this bit equal to 0 specifies that there shall not be more than one tile in the picture. The second bit (bit 1) of parallelism_idc equal to 1 specifies that wavefronts may be present, and this bit equal to 0 specifies that wavefronts shall not be present. The third bit (bit 2) of parallelism_idc equal to 1 specifies that entropy slices may be present, and this bit equal to 0 specifies that entropy slices shall not be present.

When bit 0 or bit 1 of the field is equal to 1, bit 2 of the field shall be equal to 0.

#### 7.3.2.3 Picture parameter set RBSP semantics

dependent_slice_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. dependent_slice_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. When tiles_or_entropy_coding_sync_idc is equal to 3 parallelism_idc & 0x4 is equal to 1, the value of dependent_slice_enabled_flag shall be equal to 1.

tiles_or_entropy_coding_sync_idc parallelism_idc & 0x1 equal to 0 specifies that there shall be only one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the values of cabac_independent_flag and dependent_slice_flag for coded pictures referring to the picture parameter set shall not be both equal to 1.

[Ed.(YK): When cabac_independent_flag and dependent_slice_flag are both equal to 1 for a slice, the slice is an entropy slice.]

tiles_or_entropy_coding_sync_idc parallelism_idc & 0x1 equal to 1 specifies that there may be more than one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the values of cabac_independent_flag and dependent_slice_flag for coded pictures referring to the picture parameter set shall not be both equal to 1.

tiles_or_entropy_coding_sync_idc parallelism_idc & 0x2 equal to 2 specifies that there shall be only one tile in each picture referring to the picture parameter set, a specific synchronization process for context variables shall be invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set and a specific memorization process for context variables shall be invoked after decoding two coding tree blocks of a row of coding tree blocks in each picture referring to the picture parameter set, and the values of cabac_independent_flag and dependent_slice_flag for coded pictures referring to the picture parameter set shall not be both equal to 1.

tiles_or_entropy_coding_sync_idc equal to 3 parallelism_idc & 0x4 equal to 4 specifies that there shall be only one tile in each picture referring to the picture parameter set, there shall be no specific synchronization process for context variables invoked before decoding the first coding tree block of a row of coding tree blocks in each picture referring to the picture parameter set, and the values of cabac_independent_flag and dependent_slice_flag for coded pictures referring to the picture parameter set may both be equal to 1.

When dependent_slice_enabled_flag is shall be equal to 0, tiles_or_entropy_coding_sync_idc shall not be equal to 3 parallelism_idc & 0x4 shall not be equal to 4.

It's a requirement of bitstream conformance that the value of tiles_or_entropy_coding_sync_idc parallelism_idc shall be the same for all picture parameter sets that are activated within a coded video sequence.

For each slice referring to the picture parameter set, when tiles_or_entropy_coding_sync_idc parallelism_idc & 0x2 is equal to 2 and the first coding block in the slice is not the first coding block in the first coding tree block of a row of coding tree blocks, the last coding block in the slice shall belong to the same row of coding tree blocks as the first coding block in the slice.

For each picture referring to the picture parameter set, when parallelism_idc & 0x11 is equal to 3, the first coding tree block in each tile in the picture shall be the first coding tree block in a slice.

### 7.3.3 Slice header semantics

………

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc parallelism_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc parallelism_idc & 0x2 is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbs − 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

When tiles_or_entropy_coding_sync_idc parallelism_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of exactly one tile, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to the number of tiles in the slice.

NOTE 2 – When tiles_or_entropy_coding_sync_idc parallelism_idc is equal to 1, each slice must include either a subset of one tile (in which case signalling of entry points is unnecessary) or an integer number of complete tiles.

When tiles_or_entropy_coding_sync_idc parallelism_idc & 0x2 is equal to 2 and num_entry_point_offsets is greater than 0, each subset k with k in the range of 0 to num_entry_point_offsets − 1, inclusive, shall contain all coded bits of exactly one row of coding tree blocks (within a tile), the last subset (with subset index equal to num_entry_point_offsets) shall contain all coded bits of the remaining coding blocks included in the slice, wherein the remaining coding blocks consist of either exactly one row of coding tree blocks (within a tile) or a subset of one row of coding tree blocks (within a tile), and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree blocks (within a tile) in the slice, wherein a subset of one row of coding tree blocks (within a tile) in the slice is also counted.

NOTE 3 – When tiles_or_entropy_coding_sync_idc parallelism_idc & 0x2 is equal to 2, a slice may include a number of rows of coding tree blocks (within a tile) and a subset of a row of coding tree blocks (within a tile). For example, if a slice include two and a half rows of coding tree blocks (within a tile), the the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to 3.

9.3 CABAC parsing process for slice data

…..

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found, , the following ordered steps apply.

- The arithmetic decoding engine is initialised as follows.

  - If CtbAddrRS is equal to slice_address, dependent_slice_flag is equal to 1 and entropy_coding_reset_flag is equal to 0, the following applies.

  - The synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

  - Otherwise if tiles_or_entropy_coding_sync_idc parallelism & 0x2 is equal to 2, and CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

  - When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as input.

  - The decoding process for binary decisions before termination as specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine as specified in subclause Error: Reference source not found.

- When cabac_independent_flag is equal to 0 and dependent_slice_flag is equal to 1, or when tiles_or_entropy_coding_sync_idc parallelism & 0x2 is equal to 2, the memorization process is applied as follows.

  - When tiles_or_entropy_coding_sync_idc parallelism & 0x2 is equal to 2 and CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as output.

A.3.2 Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- …..

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc parallelism_idc in the range of 0 to 1 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- When tiles_or_entropy_coding_sync_idc parallelism_idc & 0x1 is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm. T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th meeting, Geneva, April 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).