| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0128 |
| --- | --- |

| Title: | On software complexity: decoding 720p content on a tablet |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +1-650-496-4742 bossen@docomoinnovations.com |
| Source: | DOCOMO Innovations, Inc. |  |  |

_____________________________

# Abstract

This contribution provides an update on HEVC software decoding complexity. A previously-presented real-time decoder has been further optimized such as to achieve decoding of a variety of 720p sequences at 30 fps on a single core of an ARM Cortex A9 processor clocked at 1 GHz.

# Introduction

Screen resolutions are growing rapidly on mobile devices: resolutions up to 1280x720 are available on phones, 2048x1536 on tablets, and 2880x1800 on laptops. On tablets, 1280x800 is currently a common screen resolution. To fully leverage the sharpness that these screens deliver, it is desirable to have video content that matches the resolution of the display. 720p content thus seems to be a natural fit for tablets.

Several contributions [1][2][3] have in the past reported real-time HEVC software decoding of 480p video on mobile devices. This contribution focuses on the 720p resolution (1280x720 luma samples). Considering the mobile application domain, video is further constrained to 30 frames per second and 1.5 Mbits per second.

As in the previous contribution [1], the decoding process is running on a single core of an ARM Cortex A9 processor. The player application itself is multi-threaded, with separate threads for decoding and display. OpenGL ES 2.0 is used to display video in real time. Resampling (scaling to fit screen size) and YUV to RGB color conversion are done on the GPU during shading.

# Experimental results

There are very few test sequences commonly used in standardization activities that are available in the 720p30 format. Thus common 720p sequences with higher frame rates are coded and played back at 30 fps (no frames are discarded). Additionally several 1080p sequences are rescaled to 720p and assumed to be 30fps.

The target bit rate is set to about 1.5 Mbps and the random access (RA main) configuration from the common test conditions [4] is used, where the I frame interval is set to 32 frames. Version 7.0 of HM is used.

The table below summarizes the sequences that are used, including their length and the bitrate at which they are encoded. Each sequence is decodable in real-time on a tablet featuring a 1 GHz Cortex A9 processor.

| Sequence | # frames | Bit rate [Mbps] |
| --- | --- | --- |
| Bigships | 600 | 1.41 |
| City | 600 | 1.44 |
| Crew | 600 | 1.50 |
| Jets | 550 | 1.47 |
| Night | 460 | 1.62 |
| Raven | 600 | 1.31 |
| Kimono | 240 | 1.57 |
| ParkScene | 240 | 1.73 |
| BasketballDrive | 500 | 1.55 |
| Cactus | 500 | 1.55 |
| BQTerrace | 600 | 1.56 |
| CrowdRun | 500 | 1.52 |
| ParkJoy | 500 | 1.68 |
| Sunflower | 500 | 1.55 |
| Riverbed | 250 | 1.58 |

## Battery drain

In a first test video sequences are played back-to-back continuously on a 3rd generation iPad tablet to determine how long a battery charge lasts. Starting with a full charge the screen is set to 50% brightness (auto-brightness and auto-lock off). The tablet shut down after 8 hours and 10 minutes of playback when the battery charge was exhausted. This seems to be a reasonable amount of time given that the manufacturer of the tablet claims up to 10 hours for video playback.

## Profiling

For profiling purposes, all sequences are played back-to-back. Time-based samples are recorded for 4 minutes (which corresponds the time duration of all sequences played back at 30fps). The distribution of decoding time across various modules is summarized in the table below.

| Module | Time |
| --- | --- |
| Motion compensation | 49% |
| Deblocking filter | 16% |
| Parsing | 19% |
| SAO | 5% |
| Inverse quantization and transform | 4% |
| Intra prediction / adding residual | 5% |

As in prior contributions, it is observed that motion compensation remains a significant complexity bottleneck, contributing to about half of the total decoding time.

It should be noted that with respect to results presented in [1], the implementation of the motion compensation process has been significantly improved. The table below summarizes profiling data for the RaceHorses sequence (832x480) encoded with a random-access configuration and QP=27 (bitrate is about 2 Mbps). Note that the percentages are not with respect to the same time basis, as the HM7 version of the decoder runs significantly faster than the HM4 version.

| Module | Time (HM4, G757) | Time (HM7) |
| --- | --- | --- |
| Motion compensation | 41% | 34% |
| Deblocking filter | 13% | 16% |
| Parsing | 25% | 30% |
| SAO | 8% | 5% |
| Inverse quantization and transform | 5% | 5% |
| Intra prediction / adding residual | 6% | 7% |

# Conclusion

Experiments have shown that real-time decoding and display of a variety of 720p sequences is feasible on a tablet featuring an ARM Cortex A9 processor clocked at 1 GHz. The frame and bitrate of the coded sequences were limited to 30 fps and 1.5 Mbps, respectively.

# References

- F. Bossen, “On software complexity”, JCT-VC document G757, Geneva, November 2011.

- K. McCann, J.-Y. Choi, K. Pachauri, K. P. Choi, C. Kim, Y. Park, Y. J. Kwak, S. Jun, M. Choi, H. Yang, and J. Park, “HEVC software player Demonstration on Mobile devices,” JCT-VC document G988, Geneva, November 2011.

- K. Veera, R. Ganguly, B. Zhou, N. Kamath, S. Chowdary, J. Du, I.-S. Chong, and M. Coban, “A real-time ARM HEVC decoder implementation,” JCT-VC document H0693, San Jose CA, February 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCT-VC document I1100, Geneva, May 2012.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).