| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11– 20 July 2012 | Document: JCTVC-J0131 |
| --- | --- |

| Title: | Motion estimation-based fast intra prediction(ME-FIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaohai He Guoyun Zhong Yuan Li Di Wu Linbo Qing | Tel: Email: | 86 028 85462766 nic5602@scu.edu.cn |
| Source: | Sichuan University |  |  |

_____________________________

# Abstract

In this document, a motion estimation-based fast intra prediction (ME-FIP) is presented. The reference block is found in the reference frame by motion estimation. The best intra mode of the reference block is adopted as the optimal prediction mode to reduce the number of intra prediction mode of current block. This contribution brings 1%-2%total coding time saving and 20%-24% intra coding time saving for class C,D and E.

# Introduction

In HEVC, The Prediction Unit (PU) is the basic unit used for carrying the information related to the prediction processes. In intra prediction, PU is restricted to being square in shape. Each CU may contain one or four PUs, which is shown as Figure 1. PU type NxN is allowed only when the corresponding CU size is greater than the smallest allowed CU size, which is denoted as SCUSize[1].

Figure 1 – Two types of Prediction Unit structure for intra prediction

In HM7.0, there are 35 intra directional prediction modes including DC and Planar modes for luma component of each PU. and the total number of available prediction modes is dependent on the size of the corresponding PU.

The luma intra prediction directions are shown in Figure 2 below.

Figure 2 – The 33 intra prediction directions

For bigger size of block, more directional prediction modes will bring more precise prediction, which will lead to lower bit rates. However, more prediction modes will bring higher complexity simultaneously.

# Proposed fast intra prediction mode

In this contribution, the temporal correlation is taken into consideration. The motion vector(MV) is obtained by motion estimation. Based on this MV, the reference block is found in the reference frame. The reference block is supposed to be close to current PU and the optimal intra prediction mode of the reference block is regarded as the optimal intra mode of current PU to reduce the total number of RDcost calculation for the intra prediction modes.

Based on the similarity between current PU and the reference block, the optimal intra prediction mode of current PU is similar to the one of reference block(after all the final prediction mode selection of the reference block may be inter prediction, there has searched intra prediction modes for the reference block and exists an optimal mode). Therefore, the optimal intra prediction mode of each is stored. The number of stored optimal modes is equal to the number of PUs. The optimal intra prediction mode of current PU is predicted by the optimal intra prediction mode searched by MV.

However, the area of the reference block is not necessarily be included in single PU, which means that the optimal block often covers several PUs, this phenomenon is shown as Figure.3.

To solve the this problem, the storage characteristic of HEVC coding information and the statistical methods of the optimal prediction mode histogram are used to select the optimal intra prediction mode for current PU. The matched block is found by MV. All the positions of the covered LCU by the block is pointed, then the positions of CUs and finally the PU. The optimal intra prediction modes for all 44 blocks in the PU are read. In Figure.4, the optimal reference block of current PU covers 10 CUs. Based on the pixels in the optimal block, the optimal intra prediction mode for each covered 44 block is read successively and the probability histogram chart update instantaneously.

Figure 3 –Motion estimation-based fast intra prediction(ME-FIP)

The probability histogram is defined as follows:

,(1)

Where S denotes the number of pixels the optimal block includes, NumMode denotes the frequency of the emergence of prediction mode Mode through traversing optimal block.

Figure.4 shows a probability distribution chart, which has been recorded in simulation. In Figure.4, the probability of mode 3 is far higher than other modes. Therefore, mode 3 is selected as current optimal intra prediction mode. To avoid the error spreading brought by this intra mode prediction method, apart from the temporal optimal prediction mode, DC mode, PLANAR mode and the minimal RDcost mode of the 35 modes are selected to form the candidate modes list: NewCanList. The RDcost for each mode in NewCanList is calculated successively to select the optimal intra prediction mode for current PU.

Figure 4 –Distribution chart of optimal intra prediction mode

# Experiments

## Test conditions

The common test conditions [2] are used.

## Simulation results

Table1 presents the simulation results.

Table 1 Coding efficiency of the ME-FIP compared to HM7.0

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.3% | 0.0% | 0.1% | 1.0% | 0.0% | 0.0% |
| Class D | -0.4% | -0.3% | -0.8% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.4% | 0.6% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | -0.3% | 0.2% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
| Intra Time[%] | 78% |  |  | 76% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.5% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% | 0.4% |
| Class E | 0.0% | 0.2% | 0.1% | 0.1% | -0.3% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.3% |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
| Intra Time[%] | 80% |  |  | 76% |  |  |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 0.1% | 0.4% | 0.0% | 0.2% | 0.3% | 0.2% |
| Class D | 0.2% | 0.1% | 0.8% | 0.0% | 0.6% | 0.2% |
| Class E | -0.1% | -0.2% | -0.2% | 0.2% | 0.7% | 0.9% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.3% | 0.1% | 0.5% | 0.4% |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.5% | 0.4% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
| Intra Time[%] | 78% |  |  | 76% |  |  |

# Conclusion

It has be shown how to obtain the optimal intra prediction mode based on the probability distribution histogram of the optimal intra modes for the CUs the reference block covers. The simulation results have shown that ME-FIP achieve 1%-2%total coding time saving and 20%-24% intra coding time saving for class C,D and E than HM7.0.

# Conclusions

[1] B. Bross, W.J. Han, J.R. Ohm, G. J. Sullivan, T. Wiegand. “High efficiency video coding (HEVC) text specification draft 7,”JCTVC-I1003, May, 2012.

[2] F. Bossen. “Common test conditions and software reference configurations,” JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Sichuan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).