| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0133 |
| --- | --- |

| Title: | TU Depth Clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Pieter Kapsenberg Wenhao Zhang Lidong Xu Yu Han Zhipin Deng Xiaoxia Cai | Email: Email: Email: Email: Email: Email: Email: | yi-jen.chiu@intel.com pieter.kapsenberg@intel.com wenhao.zhang@intel.com lidong.xu@intel.com yu.han@intel.com zhipin.deng@intel.com xiaoxia.cai@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

It has been observed that there are two TU depth related issues in current HEVC text and software. Firstly, there’s no limitation to the values of maximum TU depths in SPS. Secondly, the derivation scheme of context index for split_transform_flag may result in unwanted context, which was reported in ticket #533. To clarify the usage of TU depth and ensure its correctness, this contribution proposes clean-up solutions for these issues.

# Problem Statement

## Limit of max TU depth

In WD7, there is no constraint to the syntax value of max_transform_hierarchy _depth_inter and max_transform_hierarchy_depth_intra in SPS. It may result in potential problems when the decoder encounters some ill bitstream crashed by transmission error, or produced by malicious encoder. The encoder of HM7.0 software do has limitations as shown below, but the decoder of HM7.0 doesn’t check if these values are in the reasonable range.

xConfirmPara( m_uiQuadtreeTUMaxDepthInter < 1,"QuadtreeTUMaxDepthInter must be greater than or equal to 1" );

xConfirmPara( m_uiQuadtreeTUMaxDepthInter > m_uiQuadtreeTULog2MaxSize - m_uiQuadtreeTULog2MinSize + 1, "QuadtreeTUMaxDepthInter must be less than or equal to the difference between QuadtreeTULog2MaxSize and QuadtreeTULog2MinSize plus 1" );

xConfirmPara( m_uiQuadtreeTUMaxDepthIntra < 1,"QuadtreeTUMaxDepthIntra must be greater than or equal to 1" );

xConfirmPara( m_uiQuadtreeTUMaxDepthIntra > m_uiQuadtreeTULog2MaxSize - m_uiQuadtreeTULog2MinSize + 1, "QuadtreeTUMaxDepthIntra must be less than or equal to the difference between QuadtreeTULog2MaxSize and QuadtreeTULog2MinSize plus 1" );

Figure 1 Constraints to maximum TU depth in HM7.0 encoder

In addition, it’s observed that the limitation in Figure 1 also has some drawbacks. Assuming the size of current coding CU is equal to 64x64, the maximum TU size is equal to 32x32 and the minimum TU size is equal to 4x4, Table 1 shows the available TU sizes corresponding to different settings of maximum TU depth value.

Table 1 Available TU sizes for 64x64 CU under different maximum TU depth setting

| MaxTUSplitDepth in configuration parameters | max_transform_hierarchy_depth in SPS | Available TU sizes |
| --- | --- | --- |
| 1 | 0 | 32x32 |
| 2 | 1 | 32x32 |
| 3 (common test condition) | 2 | 32x32, 16x16 |
| 4 | 3 | 32x32, 16x16, 8x8 |
| 5 | 4 | 32x32, 16x16, 8x8, 4x4 |

However, in HM7.0, the MaxTUSplitDepth cannot be set to 5 as highlighted in Table 1, because it exceeds the encoder range constraint. However, this highlighted coding setting is useful to fully exploit the performance of RQT, thus it should not be abandoned by improper constraints.

## Context derivation of split_transform_flag (ticket #553)

According to WD7 and HM7.0, the context table for CABAC coding of split_transform_flag is designed based on the transform size. As shown in Figure 2, the first entry of context table is for 64x64 size, the second entry is for 32x32 size, the third entry is for 16x16 size, and etc. The split_transform_flag value of sizes equal to 64x64 and smaller than 8x8 can be inferred implicitly, and therefore the corresponding CABAC initValues are all equal to CNU.

static const UChar

INIT_TRANS_SUBDIV_FLAG[3][NUM_TRANS_SUBDIV_FLAG_CTX] =

{

{ CNU,  153,  138,  138,  CNU,  CNU,  CNU,  CNU,  CNU,  CNU, },

{ CNU,  124,  138,   94,  CNU,  CNU,  CNU,  CNU,  CNU,  CNU, },

{ CNU,  224,  167,  122,  CNU,  CNU,  CNU,  CNU,  CNU,  CNU, },

}

Figure 2 CABAC initValue table of split_transform_flag in HM7.0

Both the HM7.0 and WD7 directly use the depth value to calculate the context index as follows.

ctxIdx = ctDepth + trafoDepth 	(1)

where the ctDepth indicates the depth of current CU relative to the LCU, and trafoDepth indicates the transform depth relative to current CU. However, it’s observed by HM ticket #533 that current context index derivation scheme of split_transform_flag may result in unwanted contextwhen the LCU size is not equal to 64x64. For example, when LCU size is equal to 32x32, ctDepth is equal to 0 and trafoDepth is equal to 0, the ctxIdx will be 0, whose initial context value is equal to CNU. Thus, the derivation of context index of split_transform_flag should be corrected in order to avoid the unwanted results, and to be fit in its initial intention.

Besides, mismatch for the CABAC initValue table of split_transform_flag between WD7 and HM7.0 is also observed.

# Proposed Solutions

## Solution 1: conformance requirement of maximum TU depth

In order to solve the problems described in section 1.1, it is proposed to introduce conformance constraints to max_transform_hierarchy _depth_inter and max_transform_hierarchy_depth_intra, as show below,

7.4.2.2 Sequence parameter set RBSP semantics

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode. The value of max_transform_hierarchy_depth_intra shall not be greater than ( Log2CtbSize − Log2MinTrafoSize ).

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode. The value of max_transform_hierarchy_depth_inter shall not be greater than ( Log2CtbSize − Log2MinTrafoSize ).

Figure 3 WD changes of solution 1

## Solution 2: modified context derivation of split_transform_flag

In order to solve the problems described in section 1.2, a simplified context index derivation scheme for coding the split_transform_flag is proposed. The proposed method is based on the transform size, instead of using the depth value.

- To solve the mismatch between WD7 and HM7.0, the CABAC initValue of split_transform_flag in WD is changed to be consistent to HM7.0.

- Since the split_transform_flag is explicitly signaled only when the transform tree reaches size of 32x32, 16x16 and 8x8, only three contexts are needed. Therefore, the number of contexts used to coding split_transform_flag can be reduced to three. The modified initValue table of is shown as follows,

Table 9-1 – Values of variable initValue for split_transform_flag ctxIdx

| Initialization variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 119 | 162 | 148 | 100 | 119 | 71 | 86 | 55 | 119 | 102 | 86 | 86 |

| Initialization variable | split_transform_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 224 | 167 | 122 | 124 | 138 | 94 | 153 | 138 | 138 |

Figure 4 WD changes for solution 2: initValue table of split_transform_flag

- Corresponding derivation process for ctxIdx of split_transform_flag is also modified as follows,

| Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| split_transform_flag | Table 9-24 | 0 | ctDepth + trafoDepth | na | na | na | na |
|  |  | 4 | ctDepth + trafoDepth | na | na | na | na |
|  |  | 8 | ctDepth + trafoDepth | na | na | na | na |
| split_transform_flag | Table 9-24 | 0 | 5- log2TrafoSize | na | na | na | na |
|  |  | 3 | 5- log2TrafoSize | na | na | na | na |
|  |  | 6 | 5- log2TrafoSize | na | na | na | na |

The proposed method does not change the resulting ctxIdx of split_transform_flag, so it does not affect the coding results when LCU size equal to 64x64. Table 2 shows the BD-rate results for the comparison between HM 7.0 with the proposed technique and HM 7.0 for common test conditions and the LCU size is configured as 32x32. The BD-rate results for LCU equal to 16x16 are shown in Table 3. Under all the configuration of AI_main, LB_main and RA_main, the coding gain and coding time of the proposed method are without any change, compared to the HM7.0.

Table 2 Testing results of solution 2, when LCU size is equal to 32x32

|  | All Intra Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V |  |
| Class A | 0.0% | 0.0% | 0.0% |  |
| Class B | 0.0% | 0.0% | 0.0% |  |
| Class C | 0.0% | 0.0% | 0.0% |  |
| Class D | 0.0% | 0.0% | 0.0% |  |
| Class E | 0.0% | 0.1% | 0.0% |  |
| Overall | 0.0% | 0.0% | 0.0% |  |
|  | 0.0% | 0.0% | 0.0% |  |
| Class F | 0.0% | 0.0% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Random Access Main |  |  |  |
|  | Y | U | V |  |
| Class A | 0.0% | 0.4% | -0.2% |  |
| Class B | 0.0% | -0.1% | -0.2% |  |
| Class C | 0.0% | 0.0% | -0.1% |  |
| Class D | 0.0% | -0.2% | -0.1% |  |
| Class E |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.2% |  |
|  | 0.0% | 0.0% | -0.2% |  |
| Class F | 0.1% | 0.1% | 0.2% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Low delay B Main |  |  |  |
|  | Y | U | V |  |
| Class A |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |  |
| Class C | 0.0% | 0.3% | -0.2% |  |
| Class D | 0.0% | 1.1% | 0.4% |  |
| Class E | 0.0% | -0.4% | 0.2% |  |
| Overall | 0.0% | 0.3% | 0.2% |  |
|  | 0.0% | 0.2% | 0.1% |  |
| Class F | -0.1% | 0.6% | -1.0% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

Table 3 Testing results of solution 2, when LCU size is equal to16x16

|  | # All Intra Main |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  |
| Class A | 0.0% | 0.0% |  | 0.0% |  |
| Class B | 0.0% | 0.0% |  | 0.0% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | 0.0% |  | 0.0% |  |
| Class E | 0.0% | -0.1% |  | 0.0% |  |
| Overall | 0.0% | 0.0% |  | 0.0% |  |
|  | 0.0% | 0.0% |  | 0.0% |  |
| Class F | 0.0% | -0.2% |  | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |
|  | Random Access Main |  |  |  |  |
|  | Y | U |  | V |  |
| Class A | 0.0% | 1.7% |  | -0.3% |  |
| Class B | 0.0% | 0.1% |  | 0.3% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | -0.1% |  | 0.2% |  |
| Class E |  |  |  |  |  |
| Overall | 0.0% | 0.4% |  | 0.1% |  |
|  | 0.0% | 0.4% |  | 0.1% |  |
| Class F | 0.0% | 0.0% |  | 0.0% |  |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |
|  | Low delay B Main |  |  |  |  |
|  | Y | U |  | V |  |
| Class A |  |  |  |  |  |
| Class B | 0.0% | -0.1% |  | -0.6% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | 0.2% |  | 0.3% |  |
| Class E | 0.0% | 0.2% |  | -0.6% |  |
| Overall | 0.0% | 0.1% |  | -0.2% |  |
|  | 0.0% | 0.0% |  | -0.2% |  |
| Class F | 0.0% | -0.6% |  | 1.0% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |
| Class F |  |  |  |  |  |

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).