| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0134 |
| --- | --- |

| Title: | Cross-check of Encoding with Fixed Lagrangian Multipliers (JCTVC-J0242) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Wenhao Zhang Jun Zhang | Email: Email: Email: | yi-jen.chiu@intel.com wenhao.zhang@intel.com jun.x.zhang@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the Encoding with Fixed Lagrangian Multiplier proposed by USTC and Microsoft in JCTVC-J0242. The software and document provided are confirmed to be consistent. The cross-check coding result confirms the result reported by proponent. The decoding test also confirms the modification is non-normative. The visual quality was also checked by subjective tests, in which no subjective quality fluctuation was observed compared to HM7.0 anchor.

# Introduction

JCTVC-J0242 proposed to use fixed lambda value when encoding, while The QP value is determined as the following steps,

- The initial QP for each picture (QP0) is determined according to the coding structure.

- The λ is calculated according to QP0 by the original HM7.0 algorithm.

- A linear lambda-QP model is proposed by the proponents for the purpose of generating optimal QP from the λ of step 2, as shown in equation 1,

QP=4.2005×ln(λ)+13.7122	(1)

- The picture is encoded by using QP from step 3.

# Experimental results

The software package obtained from the proponent was successfully compiled and tested under the common test conditions [1]. Cases of RA_MAIN, RA_HE10, LB_MAIN, LB_HE10, LP_MAIN and LP_HE10 are tested. There was no mismatch between encoder and decoder. The R-D performance of encoding tests produced by this cross-check perfectly matches the results provided by the proponent. Table 1-3 show the summarized performance data and detailed results are shown in the attached excel sheets.

Table 1 Performance data of random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.3% | -6.8% | -7.7% | -1.2% | -6.9% | -7.9% |
| Class B | -1.4% | -4.8% | -5.9% | -1.5% | -5.9% | -6.4% |
| Class C | -2.2% | -5.1% | -5.0% | -2.3% | -5.4% | -5.6% |
| Class D | -1.9% | -5.0% | -5.0% | -2.2% | -5.5% | -6.1% |
| Class E |  |  |  |  |  |  |
| Overall | -1.7% | -5.4% | -5.9% | -1.8% | -5.9% | -6.5% |
|  | -1.5% | -5.4% | -5.9% | -1.6% | -6.0% | -6.5% |
| Class F | -1.7% | -3.9% | -4.2% | -2.0% | -3.9% | -3.9% |

Table 2 Performance data of low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0% | -1.8% | -2.2% | -1.0% | -2.4% | -3.2% |
| Class C | -1.3% | -1.9% | -1.7% | -1.3% | -2.9% | -2.7% |
| Class D | -1.3% | -1.1% | -1.9% | -1.2% | -2.9% | -3.2% |
| Class E | -2.5% | -3.4% | -6.5% | -2.3% | -5.9% | -5.5% |
| Overall | -1.4% | -2.0% | -2.8% | -1.4% | -3.3% | -3.5% |
|  | -1.4% | -1.9% | -2.7% | -1.4% | -3.2% | -3.4% |
| Class F | -1.9% | -2.6% | -2.4% | -1.4% | -2.9% | -1.8% |

Table 3 Performance data of low delay P (optional)

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.7% | -2.0% | -2.7% | -1.2% | -2.9% | -3.6% |
| Class C | -1.7% | -2.5% | -1.9% | -1.5% | -3.2% | -3.0% |
| Class D | -1.5% | -1.6% | -2.0% | -1.5% | -3.2% | -4.4% |
| Class E | -2.9% | -4.8% | -6.1% | -2.8% | -6.3% | -6.8% |
| Class F | -2.0% | -2.3% | -2.8% | -1.6% | -2.9% | -2.7% |
| Overall | -1.9% | -2.5% | -3.0% | -1.6% | -3.5% | -3.9% |
|  | -1.9% | -2.5% | -2.9% | -1.6% | -3.5% | -3.8% |

# Subjective tests

Subjective tests were performed to check if the results of JCTVC-J0242 have visual quality degradation compared to the HM7.0 anchor. The subjective testing condition is shown in Table 4.

Table 4 Subjective testing condition

| Sequences | Kimono1_1920x1080_24 ParkScene_1920x1080_24 Cactus_1920x1080_50 BasketballDrive_1920x1080_50 BQTerrace_1920x1080_60 BasketballPass_416x240_50 BQSquare_416x240_60 BlowingBubbles_416x240_50 RaceHorses_416x240_30 |
| --- | --- |
| Coding structure | Random access (RA_main) |
| Initial QP value | 22, 27, 32, 37 |
| Method of observation | Side by side comparison Two observers |

The resolution of testing sequences included 1920x1080 and 416x240, and the frame rate ranged from 24 to 60. During the subjective tests, no subjective quality degradation was observed in all the testing cases. Specifically, there was no visual quality fluctuation in all the tests, even though the QP values were changed compared to the HM7.0 anchor under the common test conditions.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May 2012.