| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0135 |
| --- | --- |

| Title: | Cross-verification of Simplification of coefficient scanning order (JCTVC-J0105) by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Marta Mrak Matteo Naccari | Email: | marta.mrak@bbc.co.uk matteo.naccari@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the cross-check of proposal JCTVC-J0105 on Simplification of coefficient scanning order. The proposal affects intra coded blocks therefore the tests were performed for all intra and random access configurations. The software was examined for the three different ways of simplification, as proposed by the proponent. The simulation results performed for this cross-verification match those provided by the proponents. The software and WD changes are consistent with the described simplifications.

# Introduction

In HEVC intra coding, vertical scanning is used for a range of Intra prediction directions that are oriented somewhat horizontally, and horizontal coefficient scanning is used for mostly vertically-oriented Intra prediction directions. Diagonal scans are used for the rest of the Intra prediction modes. There are 9 values of IntraPredMode that map to the horizontal scan order, and there are 9 other values that map to the vertical scan order. To realise such mapping, a decoder (and encoder) must have either a table look-up, or has to perform comparisons to match the scan order for the current intra prediction mode.

In JCTVC-J0105 3 different simplifications have been proposed whereby the range for the mapping is reduced from 9 to 8. In this way, the table lookup and/or the range check operations to derive the coefficient scanning index can be replaced by simpler binary checks on the IntraPredMode variable. The three proposed simplifications can be briefly summarised as follows:

- Simplification 1: The vertical and horizontal scan mappings are reduced from 9 to 8 and they are aligned to the indexes which are multiple of eight.

- Simplification 2: Similar to Simplification 1 except for an offset of one in the range values.

- Simplification 3: Similar to Simplification 2 except for an offset of two in the range values.

The proposed simplifications can be also pictured as in Figure 1.

Figure 1: Schematic representation of the three proposed coefficient scanning simplifications (from document JCTVC-J0105).

# Measured BD rates

## Simplification 1

Simplification 1 for coefficient scanning simplification has been tested over the common test conditions for the intra (main and he10) and random access (main and he10) configurations. The BD rates are reported in the following and match the ones declared by the proponent. Given the small changes in the source code related to this simplification, the running times are not expected to increase or decrease. The observed increments in the results reported in the following may be due to some noise in the time measurements.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class C | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
| Class E | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class F | 0.2% | 0.3% | 0.3% | 0.1% | 0.0% | 0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |

## Simplification 2

Simplification 2 for coefficient scanning has been tested over the common test conditions for the intra (main and he10) and random access (main and he10) configurations. The BD rates are reported in the following and match the ones declared by the proponent. Given the small changes in the source code related to this simplification, the running times are not expected to increase or decrease. The observed increments in the results reported in the following may be due to some noise in the time measurements.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

## Simplification 3

Simplification 3 for coefficient scanning has been tested over the common test conditions for the intra (main and he10) and random access (main and he10) configurations. The BD rates are reported in the following and match the ones declared by the proponent. Given the small changes in the source code related to this simplification, the running times are not expected to increase or decrease. The observed increments in the results reported in the following may be due to some noise in the time measurements.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |

# Source code review

The proponent has modified the HM-7.0 source by introducing three macros depending on which simplification is being tested. The three macros (i.e. the three proposed simplifications) can be selected only one at each time and their exclusive selection is controlled by a compile time check. The modifications required by the three proposed simplifications involve the TComDataCU class, namely the getCoefScanIdx method and match the proposal description.

# WD changes text review

The proponent has also provided the WD changes required by the three simplifications. These modifications involve only the range indexes in Table 7-12 of the WD. The provided modifications match with the description of all the simplifications.

# Conclusions

The proposal provides an interesting simplification for coefficient scanning order in intra prediction modes. All the proposed simplifications allow replacing operations as table lookup or range check with simpler binary checks on the prediction mode being used. The BD rate losses are negligible and the computational complexity is not increased. It is therefore desirable the adoption of this proposal.