| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0136 |
| --- | --- |

| Title: | AHG9: Improvement of HRD for sub-picture based operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes improvements of HRD for sub-picture based operation in HEVC text specification draft 7.

The proposed improvements are 1) modified definition of removal time when low_delay_hrd_flag is equal to 1, 2) new SEI as a hint of early displaying of decoded pictures, and 3) modified definition of du_cpb_removal_delay.

# Introduction

At the last 9th JCT-VC meeting, the sub-picture based CPB operation scheme was adopted and incorporated into the HEVC text specification draft 7 [1].

This sub-picture based CBP operation scheme defines the decoding time of a decoding unit which contains at least one VCL NAL unit, in addition to the decoding time of an access unit. It enables a decoder to start its decoding process earlier than in existing standards such as the AVC. As a result, interoperable ultra-low delay video transmission is realized.

After investing the latest draft, we found some items to be improved from the viewpoint of usability.

In this contribution, we propose to improve the HRD specification. The proposed improvements are 1) modified definition of removal time when low_delay_hrd_flag is equal to 1, 2) new SEI as a hint of early displaying of decoded pictures, and 3) modified definition of du_cpb_removal_delay.

# Modified definition of removal time when low_delay_hrd_flag is equal to 1

## Problem statement

At the last meeting there was a discussion on whether low_delay_hrd_flag shall be restricted to 0 or not if sub_pic_cpb_params_present_flag is 1. We think it shall be allowed to set both sub_pic_cpb_params_present_flag and low_delay_hrd_flag to 1, as allowed in the HEVC text specification draft 7 [1].

This is because there might be a rare case, such as in a severe scene change, when an encoder could not perform a proper rate control for ensuring that the arrival time of the last bit of a DU always becomes earlier than the predefined decoding time of the DU with negligible degradation in picture quality. In such the case, constraint on low_delay_hrd_flag equal to 0 when sub_pic_cpb_params_present_flag is equal to 1 is too restrictive for an encoder.

However the current derivation process of a removal time from a nominal removal time needs improvement when SubPicCpbFlag is equal to 1. The detail is explained in the following.

The current draft specifies the process of deriving the removal time of decoding unit m tr ( m ) from its nominal removal time tr,n( m ) when the arrival time of the last bit of decoding unit m taf( m ) is later than the nominal removal time tr,n( m ), that is, an underflow occurs at decoding unit m. In this process tr( m ) is shifted later than taf( m ).

Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( m ) < taf( m )), the removal time of decoding unit m is specified by tr( m ) = tr,n( m ) + Tc * Ceil( ( taf( m ) − tr,n( m ) ) c ) (C-1) NOTE 1 – The latter case indicates that the size of decoding unit m, b( m ), is so large that it prevents removal at the nominal removal time.

When SubPicCpbFlag is equal to 0 (i.e. a DU is equal to an AU), the variable Tc is equal to ( num_units_in_tick  time_scale ), which is equal to a picture interval. This means the delay of the removal time from the nominal removal time is the multiple of a picture interval. When an overflow occurs, an encoder skips the encoding process of the following picture(s) in order to avoid the propagation of the delay caused by the underflow (see Figure 1).

Figure 1 Relationship among tr,n(), tr() and display timing when SubPicCpbFlag is equal 0 and overflow occurs.

On the other hand, when SubPicCpbFlag is equal to 1 (i.e. an AU consists of multiple DUs), the propagation of the delay cannot be avoided. The reason is the following.

When SubPicCpbFlag is equal to 1, the variable Tc is defined as ( num_units_in_sub_tick  time_scale ), which is not equal to a picture interval. The removal time in case of underflow is “slightly” delayed from the arrival time of the last bit and is not aligned with the removal time of an AU.

Since a typical decoder processes a picture within a picture period, a decoder would perform DU based pipeline processing in order to reduce latency as small as possible. When such pipeline processing is applied, the delay in decoding one DU propagates to following DUs. As the result, the display times of following pictures are also delayed (see “(1) DU pipeline” in Figure 2).

Moreover, different configurations of the pipeline processing at a decoder may yield different results in display delay. If an underflow only occurs in one particular DU, AU based pipeline processing would not detect an underflow when the DU is not the last DU of a decoding unit (see “(2) AU pipeline” in Figure 2). It should be noted that the removal times of the last decoding unit in an AU is and that of the AU are equal in this example.

The specification should avoid such misalignment in output delay.

Figure 2 Relationship among tr,n(), tr() and display timing when SubPicCpbFlag is equal 1 and overflow occurs. Each access unit contains 4 decoding units.

## Proposal

It is proposed to change the definition of the removal time of decoding unit m when an underflow occurs in decoding unit m in order to avoid aforementioned misalignment. Specifically two changes are proposed.

The first change is that the variable of Tc used for deriving the removal time of a DU is set to ( num_units_in_tick  time_scale ) instead of ( num_units_in_sub_tick  time_scale ) even when SubPicCpbFlag is equal to 1. According to this change, the delay of the removal time from the nominal removal time is the multiple of a picture interval. Moreover, when an underflow occurs at decoding unit m, the removal time of each following decoding unit m’ within the same access unit n is adjusted so that it does not precede the removal time of decoding unit m.

The second change is that new constraint is added to bitstream conforming so that an underflow both at decoding unit m and at access unit n containing decoding unit m shall always occur coincidently.

Figure 3 shows the removal time of each DU/AU and the display timing according to the proposed changes.

Figure 3 Proposed relationship between tr,n() and tr(), and proposed constraint on bitstream when SubPicCpbFlag is equal 1.

The summary of the proposal is the following. Please see the attached document for detail.

On derivation of removal time tr(m) of decoding unit m

If tr,n( m ) is smaller than taf( m ) at decoding unit m, removal time tr( m ) of decoding unit m and removal time tr( m’ ) of any following decoding unit m’ ( m < m’ ) within the same access unit n are derived as follows.

tr( m ) = tr,n( m ) + tc * Ceil( ( taf( m ) − tr,n( m ) ) tc )	(C-2)

tr( m’ ) = tr,n( m’ ) + max( ( tc * Ceil( ( taf( m ) − tr,n( m ) ) tc ), max(0, tc * Ceil( ( taf( m’ ) − tr,n( m’ ) ) tc ) )

On constraint on bitstream

When low_delay_hrd_flag is equal to 1, the following constraint is added to bitstream conformance.

When an underflow occurs at decoding unit m with SubPicCpbFlag equal to 1, an underflow shall also occurs at access unit n which contains decoding unit m with SubPicCpbFlag equal to 0.

Note: An encoder could delay the arrival time of the last bit of access unit n by proper means such as lowering QP value of each coding unit in access unit n.

Furthermore, it is proposed to improve the semantics of the syntax element num_units_in_tick. In Subclause E.2.1, the following example of the values of time_scale and num_units_in_tick is described.

For example, when the picture rate of a video signal is 30 000  1001 Hz, time_scale may be equal to 60 000 and num_units_in_tick may be equal to 1001. See Equation C-1.

The variable tc specified in Equation C-1 is used for deriving the removal time of a access unit with low_delay_hrd_flag equal to 1 and for defining the constraint on to,dpb( n ), which is the difference between the output times of two succeeding pictures.

The values in the example are according to the case when the scan type of a picture is interlaced. When the scan type of a picture is progressive, the value of time_scale shall be 30 000 instead of 60 000.

The proposed improvement in this example is as follows.

For example, when the picture rate of a video signal is 30 000  1001 Hz and the scan type of a picture is progressive, time_scale may be equal to 30 000 and num_units_in_tick may be equal to 1001. See Equation C-1.

# New SEI as a hint of early displaying of a decoded picture

## Problem statement

The current HRD specification does not define the earliest time when a decoded DU becomes ready for displaying. This is because there is no simple relationship between the decoding time and the display time of each DU at various DU structure within a picture (see Figure 4).

Figure 4 Examples of relationship between DU structure and earliest display time

It is not easy for a decoder to determine the earliest display time at which all decoded pixels are properly displayed in the defined order. It may be easier when the processing order is restricted to the raster scan within a picture. But it requires parsing of all BPSEIs and PTSEIs in order to know the decoding time and the position of each decoding unit. It becomes more complicated when multiple vertical tiles exist in a picture.

## Proposal

It is proposed to introduce a new SEI message in which the earliest display time at which all decoded pixels are properly displayed in the raster scan order.

The proposed syntax of the new SEI message is the following.

| low_delay_display_hint( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_delay | u(v) |
| display_delay | u(v) |
| } |  |

The proposed semantics of the new SEI message is the following.

- A low delay display hint SEI message indicates the earliest time relative to the arrival time of the first bit of a bitstream to the CPB buffer of a decoder, when a decoder can start the output process of decoded pictures to a display device, supposing that the display order is the raster scanning.

- A low delay display hint SEI message also indicates the earliest time relative to the arrival time of the first bit to the CPB buffer of the decoder, when a decoder can start its decoding process without causing any underflow or overflow at each decoding unit, supposing that the decoder finishes decoding of each decoded picture within a picture interval and that the decoding time of each coded tree block is averaged.

- If present, a low delay display hint SEI message shall be associated to all access units associated with a buffering period SEI.

NOTE: It is recommended that a decoder referring a low delay display hint SEI message interpretates the value of syntax elements in the low delay display hint SEI message properly according to the architecture of the decoder (e.e. degree of parallelism).

- decoding_delay indicates the possible earliest time when a decoder can start decoding access units in the bitstream without causing an underflow or an overflow of the CPB, supposing that the decoder decodes one access unit within one picture period. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1.

- The possible earliest time tearliest_decode is derived by the following equation.

- tearliest_decode = decoding_delay  90 000

- display_delay indicates the possible earliest time when a decoder can start displaying decoded pixels without any loss of pixels in the display process, supposing that the display order is the raster scanning. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1.

- The possible earliest time tearliest_display is derived by the following equation.

- tearliest_display = display_delay  90 000

# Modified definition of du_cpb_removal_delay

## Problem statement

The nominal removal time of a decoding unit (hereafter referred as to DU) is calculated by adding ( tc_sub * du_cpb_removal_delay ) to that of the first DU in an access unit (hereafter referred as to AU) associated with a buffering period SEI message (hereafter referred as to BPSEI) . The values of du_cpb_removal_delay of all DUs in an AU are described in a picture timing SEI message (hereafter referred as to PTSEI) of the AU.

An encoder can choose the number of DUs in an AU and the number of VCL NAL units in a DU according to its strategy on low latency. Generally an encoder is required to set the number of coded tree blocks (hereafter referred as to CTB(s)) in each DU as small as possible and to set both bit per CTBs and the number of CTBs in each DU as equal as possible in order to achieve ultra-low latency. This is based on the fact that typical encoder needs one picture time to decode a picture and allocates an averaged decoding time to each CTB.

For example, when the vertical MB line refresh scheme introduced in JCTVC-B031 [2] is applied, the number of CTBs in each DU is set to the number of CTBs in one CTB row. An encoder can easily control the arrival time of a DU since the generated bits in each DU can be averaged without sacrificing the overall coding efficiency. The codec latency becomes 0.001 sec when the input video is a 1080p/59.97 Hz sequence.

When all CTBs are encoded in intra mode, the number of CTBs in a DU can be further reduced since the fluctuation of bits of each CTB becomes small enough.

As explained above, the number of CTBs in a DU shall be small enough in order to lower latency. In other words, the number of DUs in an AU and the codec delay are inversely proportional. Then here arises the overhead problem of du_cpb_removal_delay syntax element in the ultra-low delay case. That is, the number of bits for signalling the CPB removal delay of all DUs in an AU, which is inversely proportional to latency, becomes significant in the current specification.

The example of such overhead is explained in the following.

Two syntax elements num_nalus_in_du_minus1 and du_cpb_removal_delay are needed to describe the property of a DU. The syntax element num_nalus_in_du_minus1, which corresponds to the number of NAL units in a DU, specifies the boundary of a DU in the bitstream. The syntax element du_cpb_removal_delay specifies the offset of the CPB removal time of a DU from that of the latest AU associated with a BPSEI.

If each DU contains 5 CTBs, an AU contains about 100 DUs in a 1920x1080/59.94 Hz progressive sequence. In order to specify the CPB removal time of each DU in a meaningful manner, the unit of du_cpb_removal_delay should be smaller than the average interval of one DU, which is equal to 0.0001668 [sec]. If the interval of an AU associated with a BPSEI is 1 second, the minimum bit length of du_cpb_removal_delay is 13 bits since the maximum value of du_cpb_removal_delay is 5,995 ( = 1 / 0.0001668). If each DU contains only one VCL NAL unit, the bit length of num_nalus_in_du_minus is 1. Thus the total amount of bits for du_cpb_removal_delay and num_nalus_in_du_minus in a PTSEI reaches to 1,400 bits. The bitrate of PTSEI becomes more than 84 kbps since each picture is associated with a PTSEI.

## Proposal

It is proposed to change the definition of the value of du_cpb_removal_delay in order to reduce the number of bits. New definition is that the syntax element du_cpb_removal_delay specifies the offset of the removal time of a DU from the CPB from its preceding DU, not from the first DU in the latest AU associated with a BPSEI.

In the case discussed in subclause 4.1, the bit length of du_cpb_removal_delay becomes one, which corresponds to one DU interval. Thus the total amount of bits for num_nalus_in_du_minus1 and du_cpb_removal_delay in a PTSEI is reduced to 200 bits. The bitrate for PTSEI is also reduced to 1/7 (i.e. reduced to 12 kbps).

The bit length of du_cpb_removal_delay is signalled in a BPSEI.

Moreover, du_cpb_removal_delay of the last DU in an AU is not needed to signal in a PTSEI since its removal time is directly derived from that of the AU.

The proposed syntax is the following.

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay | u(v) |
| dpb_output_delay | u(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| num_decoding_units_minus2 | ue(v) |
| du_cpb_removal_delay_length_minus1 | u(5) |
| for( i = 0; i <= num_decoding_units_minus2; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| du_cpb_removal_delay[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The proposed semantics for newly introduced syntax element and modified syntax element are the following.

num_decoding_units_minus2 plus 2 specifies the number of decoding units in the access unit the picture timing SEI message is associated with. The value of num_decoding_units_minus2 shall be in the range of 0 to PicWidthInCtbs * PicHeightInCtbs – 2, inclusive.

du_cpb_removal_delay_length_minus1 specifies the length in bits of the du_cpb_removal_delay[ i ] syntax elements of the picture timing SEI message. The length of du_cpb_removal_delay[ i ] is du_cpb_removal_delay_length_minus1 + 1.

du_cpb_removal_delay[ i ] specifies how many sub-picture clock ticks to wait after removal from the CPB of the last decoding unit before removing from the CPB the i-th decoding unit in the access unit associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex Error: Reference source not found. The syntax element is a fixed length code whose length in bits is given by du_cpb_removal_delay_length_minus1 + 1.

In addition to the aforementioned changes, it is proposed to change definitions in Annex C. The summary is in the following. Please see the attached document for detail.

On clarification of the relationship between a decoding unit and an access unit

- Let SumDU( n ) be the function which returns the number of decoding units contained in all access units from access unit 0 to access unit n. SumDU( -1 ) returns 0.

- When SubPicCpbFlag is equal to 0, let decoding unit m be the only decoding unit in access unit n.

- Otherwise (SubPicCpbFlag is equal to 1), let decoding unit m be inside access unit n and the number of preceding decoding units in access unit n be equal to i, where n satisfies the equation SumDU( n – 1 ) <= m < ( SumDU( n ) ) and i (= [ 0, num_decoding_units_minus2 + 1 ] ) is equal to ( m – SumDU( n – 1 ) ).

- Let the i-th ( = [ 0, num_decoding_units_minus2 + 1 ] ) entries of the arrays of num_nalus_in_du_minus1 and du_cpb_removal_delay in the PTSEI of access unit n be the syntax elements num_nalus_in_du_minus1 and du_cpb_removal_delay associated with decoding unit m, where m is equal to ( SumDU( n - 1) + i ).

- Let decoding unit m be the last decoding unit of access unit n when m is equal to ( SumDU( n ) - 1 ).

On modification of the derivation process of the nominal removal time of a decoding unit when SubPicCpbFlag is equal to 1

When decoding unit m is the last decoding unit in access unit n, the nominal removal time of decoding unit m tr,n( m ) is set to tr,n( n ), which is the nominal removal time of decoding unit n in the case SubPicCpbFlag is equal to 0.

Otherwise (i.e. decoding unit m is not the last decoding unit in access unit n), the nominal removal time of decoding unit m tr,n( m ) derived from the following equation.

tr,n( m ) = min( tr,n( m - 1 ) + Tc * CpbRemovalDelay( m ), tr,n( n ) )

Figure 5 Proposed definition of duCRD (du_cpb_removal_delay)

# Proposed text

The proposed text is in the attached document.

# Conclusion

The proposed improvements can ease possible burden both at an encoder and a decoder when ultra-low delay encoding is applied.

We suggest adopting the proposed improvements into the draft.

# References

[1] "High Efficiency Video Coding (HEVC) text specification draft 7", JCTVC-I1003, May 2012, Geneva Switzerland

[1] K.Kazui et al., "Proposal of requirement on very low delay coding", JCTVC-B031, July 2010, Geneva Switzerland

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).