| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I1003_d5 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 7 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 7 of High efficiency video coding.

Ed. Notes (D7):

Changes related to 1st enhancement “alternative parameters for cpb_removal_delay and dpb_output_delay” is highlighteded in yellow.

Changes related to 2nd enhancement “new SEI message for indicating editing point” is highlighteded in green.

Changes related to 3rd enhancement “new parameter for indicating continuity of bitstream at BLA picture” is highlighteded in light blue.

- CONTENTS

Page

- List of figures

- Foreword

INTERNATIONAL  STANDARD

ITU-T  RECOMMENDATION

High efficiency video coding

# Introduction

This clause does not form an integral part of this Recommendation | International Standard.

# Scope

This document specifies High efficiency video coding.

# Normative references

# Abbreviations

# Conventions

# Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

# Syntax and semantics

# Decoding process

# Parsing process

# Specification of bitstream subsets

- Annex A

Profiles and levels

- Annex B

Byte stream format

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

  - General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex A).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks [Ed. (GJS): Fix figure numbering in annex(es)]

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause 6, Annexes D and E.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses D.1 and D.2, which is part of the sequence parameter set syntax structure.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes C, D, and E, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

Figure C-2 – HRD buffer model

[Ed. (JS): Remove "fields" from figure. (DF) Rename "frames" to "pictures"? (GJS): Yes, and I believe our current model now always stores the current picture, so the bypass around the DPB should be removed. (YK): And, "Access units" -> "decoding units"; "Reference fields or frames" -> "reference pictures"; "Fields of frames" (first instance) -> "decoded decoding units"; "Fields of frames" (second instance) -> "decoded pictures"; "Output cropped fields of frames" -> "Output cropped pictures"]

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of picture storage buffers) for temporal layer X is sps_max_dec_pic_buffering[ X ] for each X in the range of 0 to sps_max_temporal_layers_minus1, inclusive. [Ed. (GJS): Think about using MaxDpbSize versus max_dec_pic_buffering (various places in document).]

The variable SubPicCpbPreferredFlag is either specified by external means, or when not specified by external means, set to 0.

The variable SubPicCpbFlag is derived as follows:

SubPicCpbFlag = SubPicCpbPreferredFlag  &&  sub_pic_cpb_params_present_flag	(C-1)

If SubPicCpbFlag is equal to 0, the CPB operates at access unit level and each decoding unit is an access unit. Otherwise the CPB operates at sub-picture level and each decoding unit is a subset of an access unit.

The HRD operates as follows. Data associated with decoding units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB. A decoded picture is removed from the DPB at the later of the DPB output time or the time that it becomes no longer needed for inter-prediction reference.

The operation of the CPB is specified in subclause C.2. The instantaneous decoder operation is specified in clauses 2-8. The operation of the DPB is specified in subclause C.3. The output cropping is specified in subclause C.3.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses D.1.1, D.1.2, D.2.1, and D.2.2. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses D.1.1 and D.2.1. The removal timing of decoding units from the CPB and output timing of decoded pictures from the DPB are specified in the picture timing SEI message as specified in subclauses D.1.2 and D.2.1. All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses C.4 and C.5, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decoding unitis not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The variable tc_sub is derived as follows and is called a sub-picture clock tick:

tc_sub = num_units_in_sub_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the coded picture or the decoded picture of access unit n.

–	Let decoding unit m be the m-th decoding unit in decoding order with the first decoding unit being decoding unit 0.

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

- Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set as follows.

–	If either of the following conditions is true:, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message:

–	Access unit 0 is a CRA or BLA access unit for which the coded picture has nal_unit_type equal to 5 or 7, and the value of rap_cpb_params_present_flag of the associated buffering period SEI message is equal to 1;

–	SubPicCpbFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message.

The time at which the first bit of decoding unit n begins to enter the CPB is referred to as the initial arrival time tai( m ).

The initial arrival time of decoding units is derived as follows.

–	If the decoding unit is decoding unit 0, tai( 0 ) = 0,

–	Otherwise (the decoding unit is decoding unit m with m > 0), the following applies.

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decoding unit m, is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

tai( m ) = taf( m − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decoding unit m is derived by

tai( m ) = Max( taf( m − 1 ), tai,earliest( m ) )		(C-3)

where tai,earliest( m ) is derived as follows.

–	If decoding unit m is not the first decoding unit of a subsequent buffering period, tai,earliest( m ) is derived as

tai,earliest( m ) = tr,n( m ) − ( InitCpbRemovalDelay[ SchedSelIdx ] +
	InitCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( m ) being the nominal removal time of decoding unit m from the CPB as specified in subclause .

–	Otherwise (decoding unit m is the first decoding unit of a subsequent buffering period), tai,earliest( m ) is derived as

tai,earliest( m ) = tr,n( m ) − ( InitCpbRemovalDelay[ SchedSelIdx ]  90000 )	(C-5)

The final arrival time for decoding unit m is derived by

taf( m ) = tai( m ) + b( m )  BitRate[ SchedSelIdx ]		(C-6)

- where b( m ) is the size in bits of decoding unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows.

–	If the content of the active sequence parameter sets for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies.

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( m )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows.

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( m ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the last decoding unit of the access unit containing decoding unit m.

When SubPicCpbFlag is equal to 1, the initial CPB arrival time of access unit n tai( n ) is set to the initial CPB arrival time of the first decoding unit in access unit n, and the final CPB arrival time of access unit n taf( n ) is set to the final CPB arrival time of the last decoding unit in access unit n.

NOTE – When SubPicCpbFlag is equal to 0, each decoding unit is an access unit, hence the initial and final CPB arrival times of access unit n are the initial and final CPB arrival times of decoding unit n.

    - Timing of decoding unit removal and decoding of decoding unit

The variable AuCpbRemovalDelay of  access unit n is derived as follows.

- If access unit n follows an CRA access unit with nal_unit_type equal to 5 or an BLA access unit with nal_unit_type equal to 7 and with rap_cpb_params_present_flag equal to 1, AuCpbRemovalDelay is derived from cpb_removal_delay in the picture timing SEI message associated with access unit n and alt_cpb_removal_delay_offset in the buffering period SEI message associated with the CRA access unit or the BLA access unit. It is specified by

AuCpbRemovalDelay = cpb_removal_delay – alt_cpb_removal_delay_offset	 (C-7)

- Otherwise, AuCpbRemovalDelay is set to cpb_removal_delay in the picture timing SEI message associated with access unit n.

If SubPicCpbFlag is equal to 0, the variable CpbRemovalDelay( m ) is set to the value of AuCpbRemovalDelay of the access unit that is decoding unit m, and the variable Tc is set to tc. Otherwise the variable CpbRemovalDelay( m ) is set to the value of du_cpb_removal_delay[ i ] for decoding unit m in the picture timing SEI message associated with the access unit that contains decoding unit m, and the variable Tc is set to tc_sub.

When a decoding unit m is the decoding unit with n equal to 0 (the first decoding unit of the access unit that initialises the HRD), the nominal removal time of the decoding unit from the CPB is specified by

tr,n( 0 ) = InitCpbRemovalDelay[ SchedSelIdx ]  90000	(C-8)

When a decoding unit m is the first decoding unit of the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the decoding unit from the CPB is specified by

tr,n( m ) = tr,n( mb ) + Tc * CpbRemovalDelay( m )		(C-9)

where tr,n( mb ) is the nominal removal time of the first decoding unit of the previous buffering period.

When a decoding unit m is the first decoding unit of a buffering period, mb is set equal to m at the removal time tr,n( m ) of the decoding unit m.

The nominal removal time tr,n( m ) of a decoding unit m that is not the first decoding unit of a buffering period is given by

tr,n( m ) = tr,n( mb ) + Tc * CpbRemovalDelay( m )		(C-10)

where tr,n( mb ) is the nominal removal time of the first decoding unit of the current buffering period.

The removal time of decoding unit m is specified as follows.

–	If low_delay_hrd_flag is equal to 0 or tr,n( m ) >= taf( m ), the removal time of decoding unit m is specified by

tr( m ) = tr,n( m )		(C-11)

–	Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( m ) < taf( m )), the removal time of decoding unit m is specified by

tr( m ) = tr,n( m ) + Tc * Ceil( ( taf( m ) − tr,n( m ) ) c )	(C-12)

NOTE 1 – The latter case indicates that the size of decoding unit m, b( m ), is so large that it prevents removal at the nominal removal time.

When SubPicCpbFlag is equal to 1, the nominal CPB removal time of access unit n tr,n( n ) is set to the nominal CPB removal time of the last decoding unit in access unit n, the CPB removal time of access unit n tr( n ) is set to the CPB removal time of the last decoding unit in access unit n.

NOTE 2 – When SubPicCpbFlag is equal to 0, each decoding unit is an access unit, hence the nominal CPB removal time and the CPB revmoal time of access unit n are the nominal CPB removal time and the CPB removal time of decoding unit n.

At CPB removal time of decoding unit m, the decoding unit is instantaneously decoded.

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. Prior to initialization, the DPB is empty (the DPB fullness is set to zero). The following steps of the subclauses of this subclause happen in the sequence as listed below.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

If the current picture is an IDR or a BLA picture, the following applies:

- When the IDR or BLA picture is not the first IDR or BLA picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE 1 – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

All pictures k in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture k is marked as "unused for reference",

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e., to,dpb( k ) <= tr( m )

When a picture is removed from the DPB, the DPB fullness is decremented by one.

    - Picture output

The variable AuDpbOutputDelay of  access unit n is derived as follows.

- If access unit n is an CRA access unit with nal_unit_type equal to 5 or an BLA access unit with nal_unit_type equal to 7 and with rap_cpb_params_present_flag equal to 1, AuDpbOutputDelay is set to alt_dpb_output_delay in the buffering period SEI message associated with the CRA access unit or the BLA access unit.

Otherwise, AuDpbOutputDelay is set to dpb_output_delay in the picture timing SEI message associated with access unit n.

The following happens instantaneously at the CPB removal time of access unit n, tr( n ).

Picture n is considered as decoded after the last decoding unit of the picture is decoded.

When picture n has PicOutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * AuDpbOutputDelay( n )		(C-13)

where AuDpbOutputDelay( n ) is the value of AuDpbOutputDelay of  access unit n.

The output of the current picture is specified as follows.

–	If PicOutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause C.3.3.

–	Otherwise (PicOutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture is output later and will be stored in the DPB (as specified in subclause C.3.3) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

When output, the picture shall be cropped, using the cropping rectangle specified in the active sequence parameter set.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of Dto,dpb( n ) is defined as:

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-14)

where nn indicates the picture that follows after picture n in output order and has PicOutputFlag equal to 1.

    - Current decoded picture marking and storage

The following happens instantaneously at the CPB removal time of access unit n, tr( n ).

The current decoded picture is stored in the DPB in an empty picture storage buffer, and the DPB fullness is incremented by one. If the current picture is a reference picture, it is marked as "used for reference", otherwise it is marked as "unused for reference".

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard shall fulfil the following requirements.

It is a requirement of bitstream conformance that the bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

It is a requirement of bitstream conformance that the first coded picture in a bitstream shall be a RAP picture, i.e., an IDR picture or a CRA picture or a BLA picture.

It is a requirement of bitstream conformance that any TFD picture associated with a particular RAP picture shall follow in output order any another RAP picture preceding the particular RAP picture in decoding order.

NOTE 1 – This constraint guarantees decodability of a TFD picture if its associated RAP picture is a CRA picture and if that CRA picture is not the first coded picture in the bitstream.

It is a requirement of bitstream conformance that any coded picture that follows a RAP picture in decoding order and uses inter prediction either from any picture that precedes the RAP picture in decoding order or from another TFD picture associated with the RAP picture shall be a TFD picture associated with the RAP picture.

NOTE 2 – Any coded picture that follows a RAP picture in decoding order and uses inter prediction either from any picture that precedes the RAP picture in decoding order or from another TFD picture associated with the RAP picture must precede the RAP in output order.

For any current picture being decoded, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the mininum, respectively, of the PicOrderCntVal values for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output", and let the variable prevRefPic be the previous reference picture in decoding order that has temporal_id equal to or less than the temporal_id of the current picture. It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2, and that the absolute difference between the PicOrderCntVal values for the current picture and for prevRefPic shall be less than MaxPicOrderCntLsb / 2.

[Ed. (YK): Rewording of "needed for output" needed, as a picture may only be marked as "needed for output" when the output order DPB operation is applied.]

The bitstream is tested by the HRD for conformance as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 3 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a coded video sequence conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2-8, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-15)

the value of InitCpbRemovalDelay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-16)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= 	InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-17)

- NOTE 4 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which the nominal CPB removal time of decoding unit m tr,n( m ) is less than the final CPB arrival time of decoding unit m taf( m ) for any value of m. When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found through Error: Reference source not found.

- After the decoding process for reference picture set as specified in subclause Error: Reference source not found has been invoked, the number of decoded pictures in the DPB, not including the current picture, with temporal_id lower than or equal to the temporal_id of the current picture, shall be less than or equal to sps_max_dec_pic_buffering[ temporal_id ].

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in subclause C.3.

- The value of to,dpb( n ) as given by Equation C-14, which is the difference between the output time of a picture and that of the first picture following it in output order and having pic_output_flag equal to 1, shall satisfy the constraint expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream using the decoding process specified in clauses 2-8.

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard shall fulfil the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause C.4, provided that all sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified by subclause C.4 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex A for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex A. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-18)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-19)

f( r ) = InitCpbRemovalDelay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  InitCpbRemovalDelay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-20)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex A for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – InitCpbRemovalDelay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex A. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbSize. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is described below.

    - Operation of the output order DPB

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. At HRD initialization, the DPB is empty. The following steps happen in the order as listed below.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IDR or a BLA picture, the following applies.

- When the IDR or BLA picture is not the first IDR or BLA picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain.

–	Otherwise (the current picture is not an IDR or a BLA picture), picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). When one or more of the following conditions are true, the "bumping" process specified in subclause C.5.2.1 is invoked repeatedly until there is an empty picture storage buffer to store the current decoded picture.

- The number of pictures in the DPB that are marked as "needed for output" is greater than sps_num_reorder_pics[ temporal_id ],

- The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to sps_max_dec_pic_buffering[ temporal_id ].

When the current picture is an IDR or a BLA picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output).

2.	All non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause C.5.2.1.

      - "Bumping" process

The "bumping" process is invoked in the following cases.

–	The current picture is an IDR or a BLA picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause C.5.2.

–	The number of pictures in the DPB that are marked "needed for output" is greater than sps_num_reorder_pics[ temporal_id ], as specified in subclause C.5.2.

–	The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to sps_max_dec_pic_buffering[ temporal_id ], as specified in subclause C.5.2.

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

    - Picture decoding, marking and storage

The following happens instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the DPB, and the following applies.

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

- Annex D

Supplemental enhancement information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics for SEI message payloads.

SEI messages assist in processes related to decoding, display or other purposes. However, SEI messages are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex C for the specification of conformance). Some SEI message information is required to check bitstream conformance and for output timing decoder conformance.

In Annex C.5.1, specification for presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, SEI messages shall obey the syntax and semantics specified in subclauses Error: Reference source not found and this annex. When the content of an SEI message is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the SEI message is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

  - SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| full_frame_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 20 ) |  |
| deblocking_filter_display_preference( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| sop_description( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| field_indication( payloadSize ) |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = XXX) |  |
| editing_point( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

    - Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( !sub_pic_cpb_params_present_flag ) |  |
| rap_cpb_params_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag ) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if ( rap_cpb_params_present_flag ) { |  |
| alt_cpb_removal_delay_offset | ue(v) |
| alt_dpb_output_delay | u(v) |
| } |  |
| if(NalHrdBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| continuous_hrd_flag | u(1) |
| } |  |

    - Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay | u(v) |
| dpb_output_delay | u(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| du_cpb_removal_delay[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Pan-scan rectangle SEI message syntax

The syntax table is specifed in subclause D.1.3 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Filler payload SEI message syntax

The syntax table is specifed in subclause D.1.4 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - User data registered by Rec. ITU-T T.35 SEI message syntax

The syntax table is specifed in subclause D.1.5 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - User data unregistered SEI message syntax

The syntax table is specifed in subclause D.1.6 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Recovery point SEI message syntax

| recovery_point( payloadSize ) { | Descriptor |
| --- | --- |
| recovery_poc_cnt | ue(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| } |  |

[Ed. (YK): The broken_link_flag is not useful anymore after the introduction of BLA pictures, and therefore should be removed from the recovery point SEI message.]

    - Scene information SEI message syntax

The syntax table is specifed in subclause D.1.10 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Full-frame snapshot SEI message syntax

The syntax table is specifed in subclause D.1.16 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Progressive refinement segment start SEI message syntax

The syntax table is specifed in subclause D.1.17 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Progressive refinement segment end SEI message syntax

The syntax table is specifed in subclause D.1.18 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Film grain characteristics SEI message syntax

The syntax table is specifed in subclause D.1.20 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Deblocking filter display preference SEI message syntax

The syntax table is specifed in subclause D.1.21 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Post-filter hint SEI message syntax

The syntax table is specifed in subclause D.1.23 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Tone mapping information SEI message syntax

The syntax table is specifed in subclause D.1.24 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Frame packing arrangement SEI message syntax

The syntax table is specifed in subclause D.1.25 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Display orientation SEI message syntax

| display_orientation( payloadSize ) { | Descriptor |
| --- | --- |
| display_orientation_cancel_flag | u(1) |
| if( !display_orientation_cancel_flag ) { |  |
| hor_flip | u(1) |
| ver_flip | u(1) |
| anticlockwise_rotation | u(16) |
| display_orientation_repetition_period | ue(v) |
| display_orientation_extension_flag | u(1) |
| } |  |
| } |  |

    - SOP description SEI message syntax

| sop_description( payloadSize ) { | Descriptor |
| --- | --- |
| sps_id | ue(v) |
| num_pics_in_sop_minus1 | ue(v) |
| for( i = 0; i <= num_pics_in_sop_minus1; i++ ) { |  |
| sop_desc_nal_ref_flag[ i ] | u(1) |
| sop_desc_temporal_id[ i ] | u(3) |
| st_rps_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| poc_delta[ i ] | se(v) |
| } |  |
| } |  |

    - Field indication SEI message syntax

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| progressive_source_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_pic_flag ) |  |
| bottom_field_flag | u(1) |
| else if( !progressive_source_flag ) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit /* equal to 0 */ | u(1) |
| reserved_zero_4bits /* equal to 0 */ | u(4) |
| } |  |

    - Decoded picture hash SEI message syntax

| decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| for( cIdx = 0; cIdx < (chroma_format_idc = = 0) ? 1 : 3; cIdx++ ) |  |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| picture_md5[ cIdx ][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| picture_crc[ cIdx ] | u(16) |
| else if( hash_type = = 2 ) |  |
| picture_checksum[ cIdx ] | u(32) |
| } |  |

    - Editing point SEI message syntax

| editing_point( payloadSize ) { | Descriptor |
| --- | --- |
| if( NalHrBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| cpb_status_present_flag | u(1) |
| if( cpb_status_present_flag ) { |  |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Reserved SEI message syntax

| reserved_sei_message( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < payloadSize; i++ ) |  |
| reserved_sei_message_payload_byte | b(8) |
| } |  |

  - SEI payload semantics

    - Buffering period SEI message semantics

When NalHrdBpPresentFlag or VclHrdBpPresentFlag are equal to 1, a buffering period SEI message can be associated with any access unit in the bitstream, and a buffering period SEI message shall be associated with each RAP access unit, and with each access unit associated with a recovery point SEI message.

NOTE – For some applications, the frequent presence of a buffering period SEI message may be desirable.

A buffering period is specified as the set of access units between two instances of the buffering period SEI message in decoding order.

seq_parameter_set_id specifies the sequence parameter set that contains the sequence HRD attributes. The value of seq_parameter_set_id shall be equal to the value of seq_parameter_set_id in the picture parameter set referenced by the primary coded picture associated with the buffering period SEI message. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

rap_cpb_params_present_flag equal to 1 specifies the presence of the initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements. When not present, the value of rap_cpb_params_present_flag is inferred to be equal to 0. When the associated picture is neither a CRA picture nor a BLA picture, the value of rap_cpb_params_present_flag shall be equal to 0.

initial_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay[ SchedSelIdx ] specify the initial CPB removal delays for the SchedSelIdx-th CPB. The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock. The values of the syntax elements shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ]  BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cpb_removal_delay_offset[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] are used for the SchedSelIdx-th CPB to specify the initial delivery time of coded data units to the CPB. The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1 and are in units of a 90 kHz clock. These syntax elements are not used by decoders and may be needed only for the delivery scheduler (HSS) specified in Annex C.

Over the entire coded video sequence, the sum of initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx, and the sum of initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.

alt_cpb_removal_delay_offset is used in all access units which follow a CRA access unit data or a BLA access unit data and precede an access unit data associated with next buffering period SEI message. This syntax is also used in the access unit associated with next buffering period SEI message.

This syntax is to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS when a BLA access unit has nal_unit_type equal to 7 (i.e. no TFD access unit is immediately following the BLA access unit) or a CRA access unit has nal_unit_type equal to 5 (i.e. no TFD access unit is immediately following the CRA access unit) and rap_cpb_params_present_flag is equal to 1.

alt_dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit associated with the buffering period SEI from the CPB before the decoded picture is output from the DPB.

This syntax is only used when an access unit associated with a buffering period SEI message has nal_unit_type equal to 5 or 7 and rap_cpb_params_present_flag is equal to 1.

The length of the syntax element alt_dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1.

continuous_hrd_flag indicates whether a BLA picture and pictures before and after the BLA picture in the decoding order are decoded correctly and displayed continuously or not.

If a buffering period SEI message is associated with a non-BLA access unit, or both NalHrdBpPresentFlag and VclHrdBpPresentFlag are equal to 0, continuous_hrd_flag has no meaning.

Otherwise, continuous_hrd_flag equal to 1 indicates all of the following conditions are satisfied.

- The values of pic_width_in_luma_samples derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- The values of pic_height_in_luma_samples derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- The values of sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- No TFD access unit immediately follows the BLA access unit.

- No underflow or overflow of the CBP occurs at the BLA access unit and at all access units following the BLA access unit until the next BLA access unit when the CPB removal delay of the BLA picture is derived using cpb_removal_delay of the BLA picture.

- The DPB output times of all access units preceding the BLA access unit and those of all access units following the BLA access units and preceding the next BLA access unit satisfy the following constraint of this specification.

  - The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

    - Picture timing SEI message semantics

NOTE 1 – The syntax of the picture timing SEI message is dependent on the content of the sequence parameter set that is active for the coded picture associated with the picture timing SEI message. However, unless the picture timing SEI message of an IDR or BLA access unit is preceded by a buffering period SEI message within the same access unit, the activation of the associated sequence parameter set (and, for IDR or BLA pictures that are not the first picture in the bitstream, the determination that the coded picture is an IDR picture or a BLA picture) does not occur until the decoding of the first coded slice NAL unit of the coded picture. Since the coded slice NAL unit of the coded picture follows the picture timing SEI message in NAL unit order, there may be cases in which it is necessary for a decoder to store the RBSP containing the picture timing SEI message until determining the parameters of the sequence parameter that will be active for the coded picture, and then perform the parsing of the picture timing SEI message.

The presence of picture timing SEI message in the bitstream is specified as follows.

–	If CpbDpbDelaysPresentFlag is equal to 1, one picture timing SEI message shall be present in every access unit of the coded video sequence.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0), no picture timing SEI messages shall be present in any access unit of the coded video sequence.

cpb_removal_delay specifies how many clock ticks to wait after removal from the CPB of the access unit associated with the most recent buffering period SEI message in a preceding access unit before removing from the buffer the access unit data associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1. The cpb_removal_delay is the remainder of a modulo 2(cpb_removal_delay_length_minus1 + 1) counter.

NOTE 2 – The value of cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element cpb_removal_delay is the value of cpb_removal_delay_length_minus1 coded in the sequence parameter set that is active for the primary coded picture associated with the picture timing SEI message, although cpb_removal_delay specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different coded video sequence.

dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB.

NOTE 3 – A picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

NOTE 4 – Only one dpb_output_delay is specified for a decoded picture.

The length of the syntax element dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1. When sps_max_dec_pic_buffering[ max_temporal_layers_minus1 ] is equal to 0, dpb_output_delay shall be equal to 0.

The output time derived from the dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the dpb_output_delay of all pictures in any subsequent coded video sequence in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, an IDR or BLA picture with no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same coded video sequence.

num_decoding_units_minus1 plus 1 specifies the number of decoding units in the access unit the picture timing SEI message is associated with. The value of num_decoding_units_minus1 shall be in the range of 0 to PicWidthInCtbs * PicHeightInCtbs – 1, inclusive.

num_nalus_in_du_minus1[ i ] plus 1 specifies the number of NAL units in the i-th decoding unit of the access unit the picture timing SEI message is associated with. The value of num_nalus_in_du_minus1[ i ] shall be in the range of 0 to PicWidthInCtbs * PicHeightInCtbs – 1, inclusive.

The first decoding unit of the access unit consists of the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in decoding order in the access unit. The i-th (with i greater than 0) decoding unit of the access unit consists of the num_nalus_in_du_minus1[ i ] + 1 consecutive NAL units immediately following the last NAL unit in the previous decoding unit of the access unit, in decoding order. There shall be at least one VCL NAL unit in each decoding unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the same decoding unit.

du_cpb_removal_delay[ i ] specifies how many sub-picture clock ticks to wait after removal from the CPB of the first decoding unit in the access unit associated with the most recent buffering period SEI message in a preceding access unit before removing from the CPB the i-th decoding unit in the access unit associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1. The du_cpb_removal_delay[ i ] is the remainder of a modulo 2(cpb_removal_delay_length_minus1 + 1) counter.

NOTE 5 – The value of cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element du_cpb_removal_delay[ i ] is the value of cpb_removal_delay_length_minus1 coded in the sequence parameter set that is active for the coded picture associated with the picture timing SEI message, although du_cpb_removal_delay[ i ] specifies a number of sub-picture clock ticks relative to the removal time of the first decoding unit in the preceding access unit containing a buffering period SEI message, which may be an access unit of a different coded video sequence.

    - Pan-scan rectangle SEI message semantics

The semantics specifed in subclause D.2.3 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Filler payload SEI message semantics

The semantics specifed in subclause D.2.4 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - User data registered by ITU-T Rec. T.35 SEI message semantics

The semantics specifed in subclause D.2.5 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - User data unregistered SEI message semantics

The semantics specifed in subclause D.2.6 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Recovery point SEI message semantics

The recovery point SEI message assists a decoder in determining when the decoding process will produce acceptable pictures for display after the decoder initiates random access or after the encoder indicates a broken link in the coded video sequence. When the decoding process is started with the access unit in decoding order associated with the recovery point SEI message, all decoded pictures at or subsequent to the recovery point in output order specified in this SEI message are indicated to be correct or approximately correct in content. Decoded pictures produced by random access at or before the picture associated with the recovery point SEI message need not be correct in content until the indicated recovery point, and the operation of the decoding process starting at the picture associated with the recovery point SEI message may contain references to pictures unavailable in the decoded picture buffer.

In addition, by use of the broken_link_flag, the recovery point SEI message can indicate to the decoder the location of some pictures in the bitstream that can result in serious visual artefacts when displayed, even when the decoding process was begun at the location of a previous IDR access unit in decoding order.

NOTE 1 – The broken_link_flag can be used by encoders to indicate the location of a point after which the decoding process for the decoding of some pictures may cause references to pictures that, though available for use in the decoding process, are not the pictures that were used for reference when the bitstream was originally encoded (e.g., due to a splicing operation performed during the generation of the bitstream).

The recovery point is specified as a count in units of PicOrderCntVal increments subsequent to PicOrderCntVal of the current access unit at the position of the SEI message.

When random access is performed to start decoding from the access unit associated with the recovery point SEI message, picture, the associated picture is considered as the first picture in the bitstream, and the variables prevPicOrderCntLsb and prevPicOrderCntMsb used in derivation of PicOrderCntVal are both set to be equal to 0.

NOTE 2 – When HRD information is present in the bitstream, a buffering period SEI message should be associated with the access unit associated with the recovery point SEI message in order to establish initialisation of the HRD buffer model after a random access.

Any sequence, picture or adaptation parameter set RBSP that is referred to by a picture associated with a recovery point SEI message or by any picture following such a picture in decoding order shall be available to the decoding process prior to its activation, regardless of whether or not the decoding process is started at the beginning of the bitstream or with the access unit, in decoding order, that is associated with the recovery point SEI message.

recovery_frame_cnt specifies the recovery point of output pictures in output order. All decoded pictures in output order are indicated to be correct or approximately correct in content starting at the output order position of the reference picture having the frame_num equal to the frame_num of the VCL NAL units for the current access unit incremented by recovery_frame_cnt in modulo MaxFrameNum arithmetic. recovery_frame_cnt shall be in the range of 0 to MaxFrameNum − 1, inclusive.

recovery_poc_cnt specifies the recovery point of output pictures in output order. All decoded pictures in output order are indicated to be correct or approximately correct in content starting at the output order position of the first picture after the current picture in decoding order that has PicOrderCntVal equal to the PicOrderCntVal of the current picture incremented by recovery_poc_cnt. The value of recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

exact_match_flag indicates whether decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the recovery point SEI message shall be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous RAP access unit, if any, in the bitstream. The value 0 indicates that the match does not need to be exact and the value 1 indicates that the match shall be exact.

NOTE 3 – When performing random access, decoders should infer all references to unavailable reference pictures as references to pictures containing only intra coding blocks and having sample values given by Y equal to ( 1 << ( BitDepthY − 1 ) ), Cb and Cr both equal to ( 1 << ( BitDepthC − 1 ) ) (mid-level grey), regardless of the value of exact_match_flag.

When exact_match_flag is equal to 0, the quality of the approximation at the recovery point is chosen by the encoding process and is not specified by this Recommendation | International Standard.

broken_link_flag indicates the presence or absence of a broken link in the NAL unit stream at the location of the recovery point SEI message and is assigned further semantics as follows:

–	If broken_link_flag is equal to 1, pictures produced by starting the decoding process at the location of a previous RAP access unit may contain undesirable visual artefacts to the extent that decoded pictures at and subsequent to the access unit associated with the recovery point SEI message in decoding order should not be displayed until the specified recovery point in output order.

–	Otherwise (broken_link_flag is equal to 0), no indication is given regarding any potential presence of visual artefacts.

[Ed. (YK): The broken_link_flag is not useful anymore after the introduction of BLA pictures, and therefore should be removed from the recovery point SEI message.]

Regardless of the value of the broken_link_flag, pictures subsequent to the specified recovery point in output order are specified to be correct or approximately correct in content.

    - Scene information SEI message semantics

The semantics specifed in subclause D.2.10 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Full-frame snapshot SEI message semantics

The semantics specifed in subclause D.2.16 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Progressive refinement segment start SEI message semantics

The semantics specifed in subclause D.2.17 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Progressive refinement segment end SEI message semantics

The semantics specifed in subclause D.2.18 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Film grain characteristics SEI message semantics

The semantics specifed in subclause D.2.20 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Deblocking filter display preference SEI message semantics

The semantics specifed in subclause D.2.21 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Post-filter hint SEI message semantics

The semantics specifed in subclause D.2.23 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Tone mapping information SEI message semantics

The semantics specifed in subclause D.2.24 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Frame packing arrangement SEI message semantics

The semantics specifed in subclause D.2.25 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.

    - Display orientation SEI message semantics

This SEI message informs the decoder of a transformation that is recommended to be applied to the output decoded and cropped picture prior to display.

display_orientation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous display orientation SEI message in output order. display_orientation_cancel_flag equal to 0 indicates that display orientation information follows.

hor_flip equal to 1 indicates that the cropped decoded picture should be flipped horizontally for display. hor_flip equal to 0 indicates that the decoded picture should not be flipped horizontally.

When hor_flip is equal to 1, the cropped decoded picture should be flipped as follows for each colour component Z = L, Cb, and Cr, letting dZ be the final cropped array of output samples for the component Z:

for( x = 0; x < croppedWidthInSamplesZ; x++ ) [Ed. (GJS): More properly account for cropping.]
	for( y = 0; y < croppedHeightInSamplesZ; y++ )
		dZ[ x, y ] = Z[ croppedWidthInSamplesZ − x − 1, y ]

ver_flip equal to 1 indicates that the cropped decoded picture should be flipped vertically (in addition to any horizontal flipping when hor_flip is equal to 1) for display. ver_flip equal to 0 indicates that the decoded picture should not be flipped vertically.

When ver_flip is equal to 1, the cropped decoded picture should be flipped as follows for each colour component Z = L, Cb, and Cr, letting dZ be the final cropped array of output samples for the component Z:

for( x = 0; x < croppedWidthInSamplesZ; x++ )
	for( y = 0; y < croppedHeightInSamplesZ; y++ )
		dZ[ x, y ] = Z[ x, croppedWidthInSamplesZ − y − 1 ]

anticlockwise_rotation specifies the recommended anticlockwise rotation of the decoded picture (after applying horizontal and/or vertical flipping when hor_flip or ver_flip is set) prior to display. The decoded picture should be rotated by 360 * anticlockwise_rotation ÷ 216 degrees (2 * π * anticlockwise_rotation ÷ 216 radians, where π is Archimedes' Constant (3.141 592 653 589 793 …) in the anticlockwise direction prior to display. For example, anticlockwise_rotation equal to 0 indicates no rotation and anticlockwise_rotation equal to 16 384 indicates 90 degrees (π ÷ 2 radians) rotation in the anticlockwise direction.

NOTE – It is possible for equivalent transformations to be expressed in multiple ways using these syntax elements. For example, the combination of having both hor_flip and ver_flip equal to 1 with anticlockwise_rotation equal to 0 can alternatively be expressed by having both hor_flip and ver_flip equal to 1 with anticlockwise_rotation equal to 0x8000000, and the combination of hor_flip equal to 1 with ver_flip equal to 0 and anticlockwise_rotation equal to 0 can alternatively be expressed by having hor_flip equal to 0 with ver_flip equal to 1 and anticlockwise_rotation equal to 0x8000000.

display_orientation_repetition_period specifies the persistence of the display orientation SEI message and may specify a picture order count interval within which another display orientation SEI message or the end of the coded video sequence shall be present in the bitstream. The value of display_orientation_repetition_period shall be in the range 0 to 16 384, inclusive.

display_orientation_repetition_period equal to 0 specifies that the display orientation SEI message applies to the current decoded picture only.

display_orientation_repetition_period equal to 1 specifies that the display orientation SEI message persists in output order until one or more of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a display orientation SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

display_orientation_repetition_period greater than 1 specifies that the display orientation SEI message persists until one or more of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a display orientation SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + display_orientation_repetition_period.

display_orientation_repetition_period greater than 1 indicates that another display orientation SEI message shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + display_orientation_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture. [Ed. (GJS): Check POC variable use.]

display_orientation_extension_flag equal to 0 indicates that no additional data follows within the post-filter hint SEI message. The value of display_orientation_extension_flag shall be equal to 0. The value of 1 for display_orientation_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for display_orientation_extension_flag in a display orientation SEI message.

    - SOP description SEI message semantics

The SOP description SEI message indicates constraints that apply in the SOP that starts with the current access unit. [Ed.(GJS): Define SOP.]

The SOP description SEI message shall not be present in any access unit with temporal_id greater than 0.

sps_id specifies the seq_parameter_set_id value identifying a sequence parameter set.

num_pics_in_sop_minus1 + 1 specifies the number of pictures in the SOP.

sop_desc_nal_ref_flag[ i ] specifies the nal_ref_flag value of the i-th picture in decoding order within the SOP.

sop_desc_temporal_id[ i ] specifies the temporal_id value of the i-th picture in decoding order within the SOP.

st_rps_idx[ i ] specifies the short-term reference picture set included in the sequence parameter set identified by sps_id and used by the i-th picture in decoding order within the SOP.

poc_delta[ i ] specifies the value of DiffPicOrderCnt( the i-th picture in decoding order within the SOP, the (i-1)th picture in decoding order within the SOP ). [Ed. (GJS): Improve phrasing – e.g., use variables.]

The bitstream may not contain all the pictures described in a SOP description. For example, the bitstream may have been subject to temporal_id based sub-bitstream extraction, but pictures that have temporal_id values no longer existing in the extracted bitstream may still be present in the SOP description SEI message. The following constraints specify that the pictures that are present in the bitstream must match to the information given in the SOP description SEI message. [Ed. (GJS): This paragraph seems normative. Improve phrasing. Does it contradict other statements within the annex?]

The variable picOrderCntExp[ i ] is derived as follows.

picOrderCntExp[ 0 ] = PicOrderCntVal of the current picture
for( i = 1; i <= num_pics_in_sop_minus1; i++ )
	picOrderCntExp[ i ] = picOrderCntExp[ i − 1 ] + poc_delta[ i ]

Let nalRefFlag[ j ], tId[ j ], stRpsIdx[ j ], and picOrderCntVal[ j ] be the values of nal_ref_flag, temporal_id, st_rps_idx, and PicOrderCntVal that are in effect for the j-th picture in decoding order starting from j equal to 0 indicating the first picture of the SOP where this SEI message resides. Let currSeqParamSet be the previous seq_parameter_set_rbsp with seq_parameter_set_id equal to sps_id in decoding order. long_term_ref_pics_present_flag shall be equal to 0 in currSeqParamSet. [Ed. (GJS): This seems to introduce some confusion with respect to uses of the same variable names in other places in the text. I suggest using subscripts rather than array symbols here.]

It is a requirement of bitstream conformance that when the SOP description SEI message, the following constraints shall apply for each picture i from picture 0 to picture num_pics_in_sop_minus1 when picOrderCntExp[ i ] is equal to picOrderCntVal[ j ], where j is greater than 0 and picOrderCntVal[ j − 1 ] is less than or equal to picOrderCntExp[ num_pics_in_sop_minus1 ]:

–	nalRefFlag[ j ] shall be equal to sop_desc_nal_ref_flag[ i ]

–	tId[ j ] shall be equal to sop_desc_temporal_id[ i ]

–	stRpsIdx[ j ] shall be equal to st_rps_idx[ i ]

–	currSeqParamSet shall be the active sequence parameter set RBSP for picture j

    - Field indication SEI message semantics

The field indication SEI message applies to the current access unit only.

The presence of the field indication SEI message in the bitstream is specified as follows.

–	If field_seq_flag is equal to 1, one field indication SEI message shall be present in every access unit of the current coded video sequence.

–	Otherwise (field_seq_flag is equal to 0), one field indication SEI message may be present in any access unit of the current coded video sequence.

NOTE 1 – In the absence of a field indication SEI message or other indication to the contrary, the current picture should be inferred to be a progressive frame coded as a frame picture.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2, and 4:4:4 chroma formats are shown in Figure D-1, Figure D-2, and Figure D-3, respectively.

field_pic_flag equal to 0 indicates that the associated picture represents a video frame. field_pic_flag equal to 1 indicates that the associated picture represents a video field. The value of field_pic_flag shall be equal to field_seq_flag. When the field indication SEI message is not present, the value of field_pic_flag should be inferred to be equal to 0.

NOTE 2 – The specified decoding process does not treat access units conveying pictures that represent fields or frames differently. A sequence of pictures that represent fields would therefore be coded with the picture dimensions of an individual field. For example, access units containing pictures that represent 1080i fields would commonly have cropped output dimensions of 1920x540, while the sequence picture rate would commonly express the rate of the source fields (typically between 50 and 60 Hz), instead of the source frame rate (typically between 25 and 30 Hz).

progressive_source_flag equal to 1 indicates that the scan type of the associated picture should be interpreted as progressive. progressive_source_flag equal to 0 indicates that the scan type of the associated picture should be interpreted as interlaced. When the field indication SEI message is not present, the value of progressive_source_flag should be inferred to be equal to 1.

The interpretation of combinations of field_pic_flag and progressive_source_flag values is defined in Table D-10-1.

Table D-10-1 – Indicated interpretation of field_pic_flag and progressive_source_flag

| - Interpretation | - field_pic_flag | - progressive_source_flag |
| --- | --- | --- |
| Picture is progressive frame | - 0 | - 1 |
| Picture is interleaved fields coded in frame picture | - 0 | - 0 |
| Picture is field | - 1 | - 0 |
| Picture is field extracted from progressive frame | - 1 | - 1 |

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate picture.

NOTE 3 – This flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other duplication and interpolation methods. This flag would commonly be used when a video feed is encoded in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 4 – When field_pic_flag is equal to 1 and duplicate_flag is equal to 1, it is assumed the access unit contains a duplicated field of the previous field in output order with the same parity as the current field.

bottom_field_flag indicates the parity of the field contained within the access unit when field_pic_flag is equal to 1. bottom_field_flag equal to 1 indicates bottom field parity. bottom_field_flag equal to 0 indicates top field parity.

top_field_first_flag indicates the preferred field output order for display purposes when fields have been interleaved to form frames in a sequence of coded frames. If top_field_first_flag is equal to 1, the top field is indicated to be temporally first, followed by the bottom field. Otherwise (top_field_first_flag is equal to 0), the bottom field is indicated to be temporally first, followed by the top field.

NOTE 5 – The display process, when applicable, is outside the scope of this Specification.

reserved_zero_1bit shall be equal to 0. The value 1 for reserved_zero_1bit is reserved for future backward-compatible use by ITU-T | ISO/IEC. Decoders shall ignore the value of reserved_zero_1bit.

reserved_zero_4bits shall be equal to 0. Other values for reserved_zero_4bits are reserved for future backward-compatible use by ITU-T | ISO/IEC. Decoders shall ignore the value of reserved_zero_4bits.

Figure D-1 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields

Figure D-2 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields

Figure D-3 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields

    - Decoded picture hash SEI message semantics

This message provides a hash for each colour component of the decoded picture in the current access unit.

Prior to computing the hash, the decoded picture data is arranged into one or three strings of bytes called pictureData[ cIdx ] of lengths pictureDataLen[ cIdx ] according to the following pseudocode process:

for( cIdx = 0; cIdx < (chroma_format_idc  = =  0) ? 1 : 3; cIdx++ ) {
		if (cIdx  = =  0) {
			compWidth = pic_width_in_luma_samples
			compHeight = pic_height_in_luma_samples
			bitDepth = BitDepthY
		}
		else {
			compWidth = pic_width_in_luma_samples / SubWidthC
			componentWidth = pic_height_in_luma_samples / SubWidthC
			bitDepth = BitDepthC
		}
		dataLen = 0
		for( i = 0; i < compWidth * compHeight; i++ ) {
			pictureData[ cIdx ][ dataLen++ ] = component[ cIdx ][ i ] & 0xFF
			if( bitDepth > 8 ) 
				pictureData[ cIdx ][ dataLen++ ] = component[ cIdx ][ i ] >> 8
		}
		pictureDataLen[ cIdx ] = dataLen
	}

where component[ cIdx ][ i ] is an array in raster scan order of decoded sample values in two's complement representation.

[Ed. (YK): The variable SubWidthC is not defined when chroma_format_idc is equal to 3 and separate_colour_plane_flag is equal to 1.]

hash_type indicates the method used to calculate the checksum according to Table D-10-2.

Table D-10-2 – Interpretation of hash_type

| - hash_type | - Method |
| --- | --- |
| - 0 | - MD5 (RFC 1321) |
| - 1 | - CRC |
| - 2 | - Checksum |
| - 3..255 | - Reserved |

[Ed. (GJS): Add normative reference to RFC 1321, and clarify informative reference to H.271 to indicate that it is an informative reference.]

picture_md5[ cIdx ][ i ] is the 16-byte MD5 hash of the colour component cIdx of the decoded picture. [Ed. (YK): Is colour component cIdx a precise description?] The value of picture_md5[ cIdx ][ i ] shall be equal to the value of digestVal[ cIdx ] obtained by performing the following pseudocode process using the MD5 functions defined in RFC 1321:

MD5Init( context )
	MD5Update( context, pictureData[ cIdx ], pictureDataLen[ cIdx ] )
	MD5Final( digestVal[ cIdx ], context )

picture_crc[ cIdx ] is the cyclic redundancy check (CRC) of the colour component cIdx of the decoded picture. The value of picture_crc[ cIdx ] shall be equal to the value of crcVal[ cIdx ] obtained by performing the following pseudocode process:

crc = 0xFFFF
	pictureData[ cIdx ][ pictureDataLen[ cIdx ] ] = 0
	pictureData[ cIdx ][ pictureDataLen[ cIdx ] + 1 ] = 0
	for( bitIdx = 0; bitIdx < ( pictureDataLen[ cIdx ]  + 2 ) * 8; bitIdx++ ) {
		dataByte = pictureData[ cIdx ][ bitIdx >> 3 ]
		crcMsb = ( crc >> 15 ) & 1
		bitVal = ( dataByte >> (7 – ( bitIdx & 7 ) ) ) & 1
		crc = ( ( ( crc << 1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
	}
	crcVal[ cIdx ] = crc

NOTE – The same CRC specification is found in Rec. ITU-T H.271.

picture_checksum[ cIdx ] is the checksum of the colour component cIdx of the decoded picture. The value of picure_checksum[ cIdx ] shall be equal to the value of checksum[ cIdx ] obtained by performing the following pseudo code process.

sum = 0
	for( y = 0; y < componentHeight; y++ ) {
		for( x = 0; x < componentWidth; x++ ) {
			xorMask = ( x & 0xFF ) ^ ( y & 0xFF ) ^ ( x >> 8 ) ^ ( y >> 8 )
			sum = ( sum + ( ( component[ cIdx ][ y * compWidth + x ] & 0xFF ) ^ xorMask ) ) & 0xFFFFFFFF
			if( BitDepth > 8 )
				sum = ( sum + ( ( component[ cIdx ][ y * compWidth + x ] >> 8 ) ^ xorMask ) ) & 0xFFFFFFFF
		}
	}
	checksum[ cIdx ] = sum

    - Editing point SEI message semantics

An editing point SEI message indicates a possible editing point in a bitstream. When a bitstream ended with an access unit with an editing point SEI message is connected by another bitstream started with a CRA access unit without changing the values of dpb_output_delay of access units in the former bitstream, it is guaranteed that the DPB output times of the access unit with an editing point SEI message and access units preceding the access unit with an editing point SEI message fulfill the following constraint of this specification.

- The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

cpb_status_present_flag equal to 1 specifies the presence of the cpb_offset syntax element. When not present, the value of cpb_status_present_flag is inferred to be equal to 0.

cpb_offset specifies the position of the SchedSelIdx-th CPB at the moment when an access unit associated with the editing point SEI message is removed from the CPB.

The position of the SchedSelIdx-th CPB in unit of bit is calculated by Ceil( cpb_offset[ SchedSelIdx ]  90,000 * bit_rate[ SchedSelIdx ] ).

The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock.

    - Reserved SEI message semantics

This SEI message consists of data reserved for future backward-compatible use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that bitstreams shall not contain reserved SEI messages until and unless the use of such messages has been specified by ITU-T | ISO/IEC. Decoders that encounter reserved SEI messages shall discard their content without effect on the decoding process, except as specified in the future by ITU-T | ISO/IEC.

- Annex E

Video usability information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics of the VUI parameters of the sequence parameter sets.

VUI parameters are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex C for the specification of output order conformance). Some VUI parameters are required to check bitstream conformance and for output timing decoder conformance.

In Annex E, specification for presence of VUI parameters is also satisfied when those parameters (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, VUI parameters shall follow the syntax and semantics specified in subclauses Error: Reference source not found and Error: Reference source not found and this annex. When the content of VUI parameters is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the VUI parameters is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

  - VUI syntax

    - VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

    - HRD parameters syntax

| hrd_parameters( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |

  - VUI semantics

    - VUI parameters semantics

aspect_ratio_info_present_flag equal to 1 specifies that aspect_ratio_idc is present. aspect_ratio_info_present_flag equal to 0 specifies that aspect_ratio_idc is not present.

aspect_ratio_idc specifies the value of the sample aspect ratio of the luma samples. Table E-10-3 shows the meaning of the code. When aspect_ratio_idc indicates Extended_SAR, the sample aspect ratio is represented by sar_width : sar_height. When the aspect_ratio_idc syntax element is not present, aspect_ratio_idc value is inferred to be equal to 0.

Table E-10-3 – Meaning of sample aspect ratio indicator

| - aspect_ratio_idc | - Sample aspect ratio | - (informative) Examples of use |
| --- | --- | --- |
| - 0 | - Unspecified |  |
| - 1 | - 1:1 ("square") | - 1280x720 16:9 frame without horizontal overscan 1920x1080 16:9 frame without horizontal overscan (cropped from 1920x1088) 640x480 4:3 frame without horizontal overscan |
| - 2 | - 12:11 | - 720x576 4:3 frame with horizontal overscan 352x288 4:3 frame without horizontal overscan |
| - 3 | - 10:11 | - 720x480 4:3 frame with horizontal overscan 352x240 4:3 frame without horizontal overscan |
| - 4 | - 16:11 | - 720x576 16:9 frame with horizontal overscan 528x576 4:3 frame without horizontal overscan |
| - 5 | - 40:33 | - 720x480 16:9 frame with horizontal overscan 528x480 4:3 frame without horizontal overscan |
| - 6 | - 24:11 | - 352x576 4:3 frame without horizontal overscan 480x576 16:9 frame with horizontal overscan |
| - 7 | - 20:11 | - 352x480 4:3 frame without horizontal overscan 480x480 16:9 frame with horizontal overscan |
| - 8 | - 32:11 | - 352x576 16:9 frame without horizontal overscan |
| - 9 | - 80:33 | - 352x480 16:9 frame without horizontal overscan |
| - 10 | - 18:11 | - 480x576 4:3 frame with horizontal overscan |
| - 11 | - 15:11 | - 480x480 4:3 frame with horizontal overscan |
| - 12 | - 64:33 | - 528x576 16:9 frame without horizontal overscan |
| - 13 | - 160:99 | - 528x480 16:9 frame without horizontal overscan |
| - 14 | - 4:3 | - 1440x1080 16:9 frame without horizontal overscan |
| - 15 | - 3:2 | - 1280x1080 16:9 frame without horizontal overscan |
| - 16 | - 2:1 | - 960x1080 16:9 frame without horizontal overscan |
| - 17..254 | - Reserved |  |
| - 255 | - Extended_SAR |  |

NOTE 1 – For the examples in Table E-10-3, the term "without horizontal overscan" refers to display processes in which the display area matches the area of the cropped decoded pictures and the term "with horizontal overscan" refers to display processes in which some parts near the left and/or right border of the cropped decoded pictures are not visible in the display area. As an example, the entry "720x576 4:3 frame with horizontal overscan" for aspect_ratio_idc equal to 2 refers to having an area of 704x576 luma samples (which has an aspect ratio of 4:3) of the cropped decoded frame (720x576 luma samples) that is visible in the display area.

sar_width indicates the horizontal size of the sample aspect ratio (in arbitrary units).

sar_height indicates the vertical size of the sample aspect ratio (in the same arbitrary units as sar_width).

sar_width and sar_height shall be relatively prime or equal to 0. When aspect_ratio_idc is equal to 0 or sar_width is equal to 0 or sar_height is equal to 0, the sample aspect ratio shall be considered unspecified by this Recommendation | International Standard.

overscan_info_present_flag equal to 1 specifies that the overscan_appropriate_flag is present. When overscan_info_present_flag is equal to 0 or is not present, the preferred display method for the video signal is unspecified.

overscan_appropriate_flag equal to 1 indicates that the cropped decoded pictures output are suitable for display using overscan. overscan_appropriate_flag equal to 0 indicates that the cropped decoded pictures output contain visually important information in the entire region out to the edges of the cropping rectangle of the picture, such that the cropped decoded pictures output should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the cropping rectangle, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the cropped decoded pictures are not visible in the display area. The term "underscan" describes display processes in which the entire cropped decoded pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the cropped decoded pictures.

NOTE 2 – For example, overscan_appropriate_flag equal to 1 might be used for entertainment television programming, or for a live view of people in a videoconference, and overscan_appropriate_flag equal to 0 might be used for computer screen capture or security camera content.

video_signal_type_present_flag equal to 1 specifies that video_format, video_full_range_flag and colour_description_present_flag are present. video_signal_type_present_flag equal to 0, specify that video_format, video_full_range_flag and colour_description_present_flag are not present.

video_format indicates the representation of the pictures as specified in Table E-10-4, before being coded in accordance with this Recommendation | International Standard. When the video_format syntax element is not present, video_format value is inferred to be equal to 5.

Table E-10-4 – Colour primaries

| - Value | - Primaries | - Informative Remark |
| --- | --- | --- |
| - 0 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 1 | - primary x y - green 0.300 0.600 - blue 0.150 0.060 - red 0.640 0.330 - white D65 0.3127 0.3290 | Rec. ITU-R BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-4 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - primary x y - green 0.21 0.71 - blue 0.14 0.08 - red 0.67 0.33 - white C 0.310 0.316 | - Rec. ITU-R BT.470-6 System M (historical) - United States National Television System Committee 1953 Recommendation for transmission standards for colour television - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - primary x y - green 0.29 0.60 - blue 0.15 0.06 - red 0.64 0.33 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.470-6 System B, G (historical) - Rec. ITU-R BT.601-6 625 - Rec. ITU-R BT.1358 625 - Rec. ITU-R BT.1700 625 PAL and 625 SECAM |
| - 6 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.601-6 525 - Rec. ITU-R BT.1358 525 - Rec. ITU-R BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 7) |
| - 7 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Society of Motion Picture and Television Engineers 240M (1999) - (functionally the same as the value 6) |
| - 8 | - primary x y - green 0.243 0.692 (Wratten 58) - blue 0.145 0.049 (Wratten 47) - red 0.681 0.319 (Wratten 25) - white C 0.310 0.316 | - Generic film (colour filters using Illuminant C) |
| - 9..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

transfer_characteristics indicates the opto-electronic transfer characteristic of the source picture as specified in Table E-10-5 as a function of a linear optical intensity input Lc with a nominal real-valued range of 0 to 1.

When the transfer_characteristics syntax element is not present, the value of transfer_characteristics is inferred to be equal to 2 (the transfer characteristics are unspecified or are determined by the application).

Table E-10-5 – Transfer characteristics

| Value | Transfer Characteristic | Informative Remark |
| --- | --- | --- |
| 0 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 1 | V = 1.099 * Lc0.45 − 0.099 for 1 >= Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= 0 | Rec. ITU-R BT.709-5 Rec. ITU-R BT.1361 conventional colour gamut system (functionally the same as the value 6) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | Assumed display gamma 2.2 | Rec. ITU-R BT.470-6 System M (historical) United States National Television System Committee 1953 Recommendation for transmission standards for colour television United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) Rec. ITU-R BT.1700 (2007 revision) 625 PAL and 625 SECAM |
| 5 | Assumed display gamma 2.8 | Rec. ITU-R BT.470-6 System B, G (historical) |
| 6 | V = 1.099 * Lc0.45 − 0.099 for 1 >= Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= 0 | Rec. ITU-R BT.601-6 525 or 625 Rec. ITU-R BT.1358 525 or 625 Rec. ITU-R BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 1) |
| 7 | V = 1.1115 * Lc0.45 − 0.1115 for 1 >= Lc >= 0.0228 V = 4.0 * Lc for 0.0228 > Lc >= 0 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | V = Lc for 1 > Lc >= 0 | Linear transfer characteristics |
| 9 | V = 1.0 + Log10( Lc )  2 for 1 >= Lc >= 0.01 V = 0.0 for 0.01 > Lc >= 0 | Logarithmic transfer characteristic (100:1 range) |
| 10 | V = 1.0 + Log10( Lc )  2.5 for 1 >= Lc >= Sqrt( 10 )  1000 V = 0.0 for Sqrt( 10 )  1000 > Lc >= 0 | Logarithmic transfer characteristic (100 * Sqrt( 10 ) : 1 range) |
| 11 | V = 1.099 * Lc0.45 − 0.099 for Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc > −0.018 V = −1.099 * ( −Lc )0.45 + 0.099 for −0.018 >= Lc | IEC 61966-2-4 |
| 12 | V = 1.099 * Lc0.45 − 0.099 for 1.33 > Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= −0.0045 V = −( 1.099 * ( −4 * Lc )0.45 − 0.099 )  4 for −0.0045 > Lc >= −0.25 | Rec. ITU-R BT.1361 extended colour gamut system |
| 13..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

- matrix_coefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red primaries, as specified in Table E-10-6.

- matrix_coefficients shall not be equal to 0 unless one or more of the following conditions are true:

- –	BitDepthC is equal to BitDepthY,

- –	chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- matrix_coefficients shall not be equal to 8 unless one or more of the following conditions are true:

- –	BitDepthC is equal to BitDepthY,

- –	BitDepthC is equal to BitDepthY + 1 and chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- When the matrix_coefficients syntax element is not present, the value of matrix_coefficients is inferred to be equal to 2 (unspecified).

- For the interpretation of matrix_coefficients, the following is specified:

–	The range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1.

–	Otherwise (transfer_characteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361 extended colour gamut system)), E′R, E′G and E′B are real numbers with a larger range not specified in this Recommendation.

–	Nominal white is specified as having E′R equal to 1, E′G equal to 1, and E′B equal to 1.

–	Nominal black is specified as having E′R equal to 0, E′G equal to 0, and E′B equal to 0.

- The interpretation of matrix_coefficients is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, or 7, the following equations apply:

Y = Clip1Y( Round( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-1)

Cb = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-2)

Cr = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-3)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(E-4)

G = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(E-5)

B = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(E-6)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, or 8), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, or 7, the following equations apply:

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(E-7)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PB + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-8)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PR + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-9)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′R )		(E-10)

G = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′G )		(E-11)

B = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′B )		(E-12)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, or 8), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

- The variables E′Y, E′PB, and E′PR (for matrix_coefficients not equal to 0 or 8) or Y, Cb, and Cr (for matrix_coefficients equal to 0 or 8) are specified as follows:

–	If matrix_coefficients is not equal to 0 or 8, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-13)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-14)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-15)

NOTE 3 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5. When transfer_characteristics is equal to 11 (IEC 61966-2-4), or 12 (ITU-R BT.1361 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Recommendation.

–	Otherwise, if matrix_coefficients is equal to 0, the following equations apply:

Y   = Round( G )		(E-16)

Cb = Round( B )		(E-17)

Cr = Round( R )		(E-18)

–	Otherwise (matrix_coefficients is equal to 8), the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-19)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-20)

Cr = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-21)

NOTE 4 – For purposes of the YCgCo nomenclature used in Table E-10-6, Cb and Cr of Equations E-20 and E-21 may be referred to as Cg and Co, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )		(E-22)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-23)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-24)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-25)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(E-26)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-27)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )	(E-28)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-29)

NOTE 5 – For purposes of the YCgCo nomenclature used in Table E-10-6, Cb and Cr of Equations E-28 and E-26 may be referred to as Cg and Co, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-30)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-31)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-32)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-33)

Table E-10-6 – Matrix coefficients

| - Value | - Matrix | - Informative remark |
| --- | --- | --- |
| - 0 | - GBR | - Typically referred to as RGB; see Equations E-16 to E-18 |
| - 1 | - KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - KR = 0.30; KB = 0.11 | - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.470-6 System B, G (historical) - ITU-R Rec. BT.601-6 625 - ITU-R Rec. BT.1358 625 - ITU-R Rec. BT.1700 625 PAL and 625 SECAM - IEC 61966-2-4 xvYCC601 - (functionally the same as the value 6) |
| - 6 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.601-6 525 - ITU-R Rec. BT.1358 525 - ITU-R Rec. BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 5) |
| - 7 | - KR = 0.212; KB = 0.087 | - Society of Motion Picture and Television Engineers 240M (1999) |
| - 8 | - YCgCo | - See Equations E-19 to E-33 |
| - 9..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

- chroma_loc_info_present_flag equal to 1 specifies that chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are present. chroma_loc_info_present_flag equal to 0 specifies that chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are not present.

- When chroma_format_idc is not equal to 1, chroma_loc_info_present_flag should be equal to 0.

- chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field specify the location of chroma samples as follows:

–	If chroma_format_idc is equal to 1 (4:2:0 chroma format), chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field specify the location of chroma samples for the top field and the bottom field, respectively, as shown in Figure E-1.

–	Otherwise (chroma_format_idc is not equal to 1), the values of the syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be ignored. When chroma_format_idc is equal to 2 (4:2:2 chroma format) or 3 (4:4:4 chroma format), the location of chroma samples is specified in subclause Error: Reference source not found. When chroma_format_idc is equal to 0, there is no chroma sample array.

- The value of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be in the range of 0 to 5, inclusive. When the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are not present, the values of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field is inferred to be equal to 0.

NOTE 6 – When coding progressive source material, chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field should have the same value.

Figure E-1 – Location of chroma samples for top and bottom fields for chroma_format_idc equal to 1 (4:2:0 chroma format) as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field

- neutral_chroma_indication_flag equal to 1 indicates that the value of all decoded chroma samples is equal to 1 << ( BitDepthC − 1 ). neutral_chroma_indication_flag equal to 0 provides no indication of decoded chroma sample values. When neutral_chroma_indication_flag is equal to 1, it is a requirement of bitstream conformance that the value of all decoded chroma samples produced by the decoding process shall be equal to  1 << ( BitDepthC − 1 ). When neutral_chroma_indication_flag is not present, it is inferred to be equal to 0.

NOTE 7 – When neutral_chroma_indication_flag is equal to 1, it is not necessary for the decoder to apply the specified decoding process in order to determine the value of the decoded chroma samples.

- field_seq_flag equal to 1 indicates that the coded video sequence conveys pictures that represent fields, and specifies that a field indication SEI message shall be present in every access unit of the current coded video sequence. field_seq_flag equal to 0 indicates that the coded video sequence conveys pictures that represent frames and that a field indication SEI message may or may not be present in any access unit of the current coded video sequence. When field_seq_flag is not present, it is inferred to be equal to 0.

NOTE 8 – The specified decoding process does not treat access units conveying pictures that represent fields or frames differently. A sequence of pictures that represent fields would therefore be coded with the picture dimensions of an individual field. For example, access units containing pictures that represent 1080i fields would commonly have cropped output dimensions of 1920x540, while the sequence picture rate would commonly express the rate of the source fields (typically between 50 and 60 Hz), instead of the source frame rate (typically between 25 and 30 Hz).

- timing_info_present_flag equal to 1 specifies that num_units_in_tick, time_scale and fixed_pic_rate_flag are present in the bitstream. timing_info_present_flag equal to 0 specifies that num_units_in_tick, time_scale and fixed_pic_rate_flag are not present in the bitstream.

num_units_in_tick is the number of time units of a clock operating at the frequency time_scale Hz that corresponds to one increment (called a clock tick) of a clock tick counter. num_units_in_tick shall be greater than 0. A clock tick is the minimum interval of time that can be represented in the coded data when sub_pic_cpb_params_present_flag is equal to 0. For example, when the picture rate of a video signal is 30 000  1001 Hz, time_scale may be equal to 60 000 and num_units_in_tick may be equal to 1001. See Equation C-1.

time_scale is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. time_scale shall be greater than 0.

fixed_pic_rate_flag equal to 1 indicates that the temporal distance between the HRD output times of any two consecutive pictures in output order is constrained as follows. fixed_pic_rate_flag equal to 0 indicates that no such constraints apply to the temporal distance between the HRD output times of any two consecutive pictures in output order.

When fixed_pic_rate_flag is not present, it is inferred to be equal to 0.

When fixed_pic_rate_flag is equal to 1 for a coded video sequence containing picture n, the value computed for to,dpb( n ) as specified in Equation C-14 shall be equal to tc as specified in Equation C-1 (using the value of tc for the coded video sequence containing picture n) when one or more of the following conditions are true for the following picture nn that is specified for use in Equation C-14:

–	picture nn is in the same coded video sequence as picture n.

–	picture nn is in a different coded video sequence and fixed_pic_rate_flag is equal to 1 in the coded video sequence containing picture nn and the value of num_units_in_tick  time_scale is the same for both coded video sequences.

nal_hrd_parameters_present_flag equal to 1 specifies that NAL HRD parameters (pertaining to Type II bitstream conformance) are present. nal_hrd_parameters_present_flag equal to 0 specifies that NAL HRD parameters are not present.

NOTE 9 – When nal_hrd_parameters_present_flag is equal to 0, the conformance of the bitstream cannot be verified without provision of the NAL HRD parameters and all buffering period and picture timing SEI messages, by some means not specified in this Recommendation | International Standard.

When nal_hrd_parameters_present_flag is equal to 1, NAL HRD parameters (subclauses D.1.2 and D.2.2) immediately follow the flag.

The variable NalHrdBpPresentFlag is derived as follows:

–	If any of the following is true, the value of NalHrdBpPresentFlag shall be set equal to 1:

–	nal_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of buffering periods for NAL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of NalHrdBpPresentFlag shall be set equal to 0.

vcl_hrd_parameters_present_flag equal to 1 specifies that VCL HRD parameters (pertaining to all bitstream conformance) are present. vcl_hrd_parameters_present_flag equal to 0 specifies that VCL HRD parameters are not present.

NOTE 10 – When vcl_hrd_parameters_present_flag is equal to 0, the conformance of the bitstream cannot be verified without provision of the VCL HRD parameters and all buffering period and picture timing SEI messages, by some means not specified in this Recommendation | International Standard.

When vcl_hrd_parameters_present_flag is equal to 1, VCL HRD parameters (subclauses D.1.2 and D.2.2) immediately follow the flag.

The variable VclHrdBpPresentFlag is derived as follows:

–	If any of the following is true, the value of VclHrdBpPresentFlag shall be set equal to 1:

–	vcl_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of buffering periods for VCL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of VclHrdBpPresentFlag shall be set equal to 0.

The variable CpbDpbDelaysPresentFlag is derived as follows:

–	If any of the following is true, the value of CpbDpbDelaysPresentFlag shall be set equal to 1:

–	nal_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	vcl_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of CPB and DPB output delays to be present in the bitstream in picture timing SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of CpbDpbDelaysPresentFlag shall be set equal to 0.

low_delay_hrd_flag specifies the HRD operational mode as specified in Annex C. When fixed_pic_rate_flag is equal to 1, low_delay_hrd_flag shall be equal to 0. When low_delay_hrd_flag is not present, its value is inferred to be equal to 1 − fixed_pic_rate_flag.

NOTE 11 – When low_delay_hrd_flag is equal to 1, "big pictures" that violate the nominal CPB removal times due to the number of bits used by an access unit are permitted. It is expected, but not required, that such "big pictures" occur only occasionally.

sub_pic_cpb_params_present_flag equal to 1 specifies that sub-picture level CPB removal delay parameters are present and the CPB may operate at access unit level or sub-picture level. sub_pic_cpb_params_present_flag equal to 0 specifies that sub-picture level CPB removal delay parameters are not present and the CPB operates at access unit level. When sub_pic_cpb_params_present_flag is not present, its value is inferred to be equal to 0.

num_units_in_sub_tick is the number of time units of a clock operating at the frequency time_scale Hz that corresponds to one increment (called a sub-picture clock tick) of a sub-picture clock tick counter. num_units_in_sub_tick shall be greater than 0. A sub-picture clock tick is the minimum interval of time that can be represented in the coded data when sub_pic_cpb_params_present_flag is equal to 1.

bitstream_restriction_flag equal to 1, specifies that the following coded video sequence bitstream restriction parameters are present. bitstream_restriction_flag equal to 0, specifies that the following coded video sequence bitstream restriction parameters are not present.

tiles_fixed_structure_flag equal to 1 indicates that each picture parameter set that is active in the coded video sequence has the same value of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present. tiles_fixed_structure_flag equal to 0 indicates that tiles syntax elements in different picture parameter sets may or may not have the same value. When the tiles_fixed_structure_flag syntax element is not present, it is inferred to be equal to 0.

NOTE 12 – The signalling of tiles_fixed_structure_flag equal to 1 is a guarantee to a decoder that each picture in the coded video sequence has the same number of tiles distributed in the same way which might be useful for workload allocation in the case of multi-threaded decoding.

motion_vectors_over_pic_boundaries_flag equal to 0 indicates that no sample outside the picture boundaries and no sample at a fractional sample position for which the sample value is derived using one or more samples outside the picture boundaries is used for inter prediction of any sample. motion_vectors_over_pic_boundaries_flag equal to 1 indicates that one or more samples outside picture boundaries may be used in inter prediction. When the motion_vectors_over_pic_boundaries_flag syntax element is not present, motion_vectors_over_pic_boundaries_flag value is inferred to be equal to 1.

max_bytes_per_pic_denom indicates a number of bytes not exceeded by the sum of the sizes of the VCL NAL units associated with any coded picture in the coded video sequence.

The number of bytes that represent a picture in the NAL unit stream is specified for this purpose as the total number of bytes of VCL NAL unit data (i.e., the total of the NumBytesInNALunit variables for the VCL NAL units) for the picture. The value of max_bytes_per_pic_denom shall be in the range of 0 to 16, inclusive.

Depending on max_bytes_per_pic_denom the following applies:

–	If max_bytes_per_pic_denom is equal to 0, no limits are indicated.

–	Otherwise (max_bytes_per_pic_denom is not equal to 0), it is a requirement of bitstream conformance that no coded picture shall be represented in the coded video sequence by more than the following number of bytes.

( PicSizeInMinCbs * RawMinCUBits )  ( 8 * max_bytes_per_pic_denom )	(E-34)

When the max_bytes_per_pic_denom syntax element is not present, the value of max_bytes_per_pic_denom is inferred to be equal to 2.

max_bits_per_mincu_denom indicates an upper bound for the number of coded bits of coding_unit( ) data for any coding block in any picture of the coded video sequence. The value of max_bits_per_mincu_denom shall be in the range of 0 to 16, inclusive.

Depending on max_bits_per_mincu_denom the following applies:

–	If max_bits_per_mincu_denom is equal to 0, no limit is specified by this syntax element.

–	Otherwise (max_bits_per_mincu_denom is not equal to 0), it is a requirement of bitstream conformance that no coded coding_unit( ) shall be represented in the bitstream by more than the following number of bits.

( 128 + RawMinCUBits )  max_bits_per_mincu_denom * ( 2 << log2CbSize − log2MinCbSize )	(E-35)

where log2CbSize is the value of log2CbSize for the given coding block and the number of bits of coding_unit( ) data for the same coding block is given by the number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found.

When the max_bits_per_mincu_denom is not present, the value of max_bits_per_mincu_denom is inferred to be equal to 1.

log2_max_mv_length_horizontal and log2_max_mv_length_vertical indicate the maximum absolute value of a decoded horizontal and vertical motion vector component, respectively, in ¼ luma sample units, for all pictures in the coded video sequence. A value of n asserts that no value of a motion vector component shall exceed the range from −2n to 2n − 1, inclusive, in units of ¼ luma sample displacement. The value of log2_max_mv_length_horizontal shall be in the range of 0 to 16, inclusive. The value of log2_max_mv_length_vertical shall be in the range of 0 to 16, inclusive. When log2_max_mv_length_horizontal is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical is inferred to be equal to 16.

NOTE 13 – The maximum absolute value of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex A.

    - HRD parameters semantics

cpb_cnt_minus1 plus 1 specifies the number of alternative CPB specifications in the bitstream. The value of cpb_cnt_minus1 shall be in the range of 0 to 31, inclusive. When low_delay_hrd_flag is equal to 1, cpb_cnt_minus1 shall be equal to 0. When cpb_cnt_minus1 is not present, it is inferred to be equal to 0.

bit_rate_scale (together with bit_rate_value_minus1[ SchedSelIdx ]) specifies the maximum input bit rate of the SchedSelIdx-th CPB.

cpb_size_scale (together with cpb_size_value_minus1[ SchedSelIdx ]) specifies the CPB size of the SchedSelIdx-th CPB.

bit_rate_value_minus1[ SchedSelIdx ] (together with bit_rate_scale) specifies the maximum input bit rate for the SchedSelIdx-th CPB. bit_rate_value_minus1[ SchedSelIdx ] shall be in the range of 0 to 232 − 2, inclusive. For any SchedSelIdx > 0, bit_rate_value_minus1[ SchedSelIdx ] shall be greater than bit_rate_value_minus1[ SchedSelIdx − 1 ]. The bit rate in bits per second is given by

BitRate[ SchedSelIdx ] = ( bit_rate_value_minus1[ SchedSelIdx ] + 1 ) * 2(6 + bit_rate_scale)	(E-36)

When the bit_rate_value_minus1[ SchedSelIdx ] syntax element is not present, the value of BitRate[ SchedSelIdx ] is inferred as follows:

–	If profile_idc is equal to 100, BitRate[ SchedSelIdx ] is inferred to be equal to 1000 * MaxBR for VCL HRD parameters and to be equal to 1200 * MaxBR for NAL HRD parameters, where MaxBR is specified in subclause Error: Reference source not found.

–	Otherwise, BitRate[ SchedSelIdx ] is inferred to be equal to cpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

cpb_size_value_minus1[ SchedSelIdx ] is used together with cpb_size_scale to specify the SchedSelIdx-th CPB size. cpb_size_value_minus1[ SchedSelIdx ] shall be in the range of 0 to 232 − 2, inclusive. For any SchedSelIdx greater than 0, cpb_size_value_minus1[ SchedSelIdx ] shall be less than or equal to cpb_size_value_minus1[ SchedSelIdx −1 ].

The CPB size in bits is given by

CpbSize[ SchedSelIdx ] = ( cpb_size_value_minus1[ SchedSelIdx ] + 1 ) * 2(4 + cpb_size_scale)	(E-37)

When the cpb_size_value_minus1[ SchedSelIdx ] syntax element is not present, the value of CpbSize[ SchedSelIdx ] is inferred as follows:

–	If profile_idc is equal to 100, CpbSize[ SchedSelIdx ] is inferred to be equal to 1000 * MaxCPB for VCL HRD parameters and to be equal to 1200 * MaxCPB for NAL HRD parameters, where MaxCPB is specified in subclause Error: Reference source not found. [Ed. (GJS): Looks wrong, and why are there two items here? Why not just let Annex A take care of this?]

–	Otherwise, CpbSize[ SchedSelIdx ] is inferred to be equal to cpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

cbr_flag[ SchedSelIdx ] equal to 0 specifies that to decode this bitstream by the HRD using the SchedSelIdx-th CPB specification, the hypothetical stream delivery scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ SchedSelIdx ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode. When the cbr_flag[ SchedSelIdx ] syntax element is not present, the value of cbr_flag is inferred to be equal to 0.

initial_cpb_removal_delay_length_minus1 specifies the length in bits of the initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements of the buffering period SEI message. The length of initial_cpb_removal_delay[ SchedSelIdx ] and of initial_cpb_removal_delay_offset[ SchedSelIdx ] is initial_cpb_removal_delay_length_minus1 + 1. When the initial_cpb_removal_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the initial_cpb_removal_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the initial_cpb_removal_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

cpb_removal_delay_length_minus1 specifies the length in bits of the cpb_removal_delay syntax element. The length of the cpb_removal_delay syntax element of the picture timing SEI message is cpb_removal_delay_length_minus1 + 1. When the cpb_removal_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the cpb_removal_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the cpb_removal_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

dpb_output_delay_length_minus1 specifies the length in bits of the dpb_output_delay syntax element. The length of the dpb_output_delay syntax element of the picture timing SEI message is dpb_output_delay_length_minus1 + 1. When the dpb_output_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the dpb_output_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the dpb_output_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

time_offset_length greater than 0 specifies the length in bits of the time_offset syntax element. time_offset_length equal to 0 specifies that the time_offset syntax element is not present. When the time_offset_length syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the time_offset_length parameters shall be equal in both hrd_parameters( ) syntax structures. When the time_offset_length syntax element is not present, it is inferred to be equal to 24.