| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0137 |
| --- | --- |

| Title: | AHG9: New high-level syntax for simple HEVC stream editing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes enhancements of high-level syntax for realizing simple HEVC streaming editing.

The proposed enhancements are 1) alternative parameters of cpb_removal_delay and dpb_output_delay for TFD picture removal, 2) new SEI message for indicating editing point, and 3) new parameter indicating continuity of bitstream at BLA picture.

# Introduction

Editing is often used for handling bitstreams. It is highly required to enable editing a bitstream in the compressed domain, i.e. without performing decoding and re-encoding, in order to reduce the design burden and the computational complexity of an editing system. It is also required to change parameters in NAL units as less as possible (i.e. only at an editing point), particularly for real-time bitstream concatenating systems such as a splicer.

When an AVC bitstream editing is performed, changing parameters of multiple pictures after an editing point is unavoidable. Typically there is no continuity at the values of frame_num and POC between two bitstreams under concatenation. But the AVC specification requires the values of those parameters are always continuous in a bitstream except at an IDR picture. Therefore frame_num and POC-related parameters in slice headers of all coded pictures following an editing point should be modified so that the edited bitstream is still conforming to the standard.

On the other hand, the current specification of HEVC does not require such modification any more. The parameter frame_num was already removed from the draft due to the introduction of the new method to manage and identify reference pictures of each coded pictures. Regarding the POC value, JCTVC-I404 [2] at the 9th JTC-VC meeting resolved the problem of discontinuity at an editing point. JCTVC-I404 introduces a new type of an access unit, i.e. BLA (Broken Link Access) access unit, in which the POC value can be set independently from those of preceding access units. When a bitstream starting with a CRA picture is connected to the end of another bitstream, there is no need to change the POC values of all access units in the connecting bitstream. There only remains the need to change the NAL unit type of the CRA picture from “CRA” to “BLA”.

However, there still remains the need to modify HRD-related parameters in order to maintain the continuity of bitstream in terms of HRD conformance.

In this contribution we propose enhancements of the HEVC specification for realizing simple editing of HEVC bitstream.

The proposed enhancements include 1) alternative parameters of cpb_removal_delay and dpb_output_delay for TFD picture removal, 2) new SEI message for indicating editing point, and 3) new parameter indicating continuity of bitstream at BLA picture.

# Alternative parameters of cpb_removal_delay and dpb_output_delay for TFD picture removal

## Problem statement

JCTVC-I0277 [3] was adopted at the 9th JCT-VC meeting and was incorporated into the HEVC text specification draft 7 [1]. JCTVC-I0277 introduces new syntax elements alt_initial_cpb_removal_delay and alt_initial_cpb_removal_delay_offset in buffering period SEI as the alternatives of initial_cpb_removal_delay and initial_cpb_removal_delay_offset. Those two syntax elements are used in deriving the CPB removal time of the first CRA picture in a bitstream when TFD (Tagged for discard) pictures immediately following the CRA picture are removed from the bitstream.

However, JCTVC-I0277 does not mention the DPB output time of a CRA picture and the CPB removal times of pictures following the CRA picture. When TFD pictures are removed, those times shall also be changed. Figure 1 shows an example of the case.

Figure 1  Change of PTSEI parameters when TFD pictures are removed.

The example shown in Figure 1 depicts the case when a decoder starts decoding from picture I72 in bitstream BSA. It is equivalent to extract sub-bitstream BSB started with picture I72 from bitstream BSA. The CPB removal times of all pictures following picture I72 (i.e. picture P76, B74, and so on) become earlier by removing the TFD pictures B70, B69 and B71. That is, the values of cpb_removal_delay of all pictures following picture I72 shall be decremented by the number of the removed TFD pictures (i.e. three).

Similarly the DPB output time of picture I72 becomes earlier. That is, the value of cpb_output_delay of picture I72 shall be set to 2, which is the value of sps_num_reorder_pics, in order to minimize the overall output delay.

This problem occurs not only at decoding a bitstream started with a CRA picture, but also at decoding a bitstream with a BLA picture. If a bitstream with a BLA picture is intended to decode and display all pictures including the BLA picture continuously, it is the responsibility of an editing system to modify the values of cpb_removal_delay and dpb_output_delay of the BLA picture and pictures following the BLA picture. Apparently this is a burden to the editing system.

## Proposal

It is proposed to introduce two new syntax elements in which alternative values for cpb_removal_delay and dpb_output_delay in the case of TFD picture removal are specified. Those two syntax elements are included in a buffering period SEI message.

The proposed syntax is the following.

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( !sub_pic_cpb_params_present_flag ) |  |
| rap_cpb_params_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag ) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if ( rap_cpb_params_present_flag ) { |  |
| alt_cpb_removal_delay_offset | ue(v) |
| alt_dpb_output_delay | u(v) |
| } |  |
| } |  |

The proposed semantics for the newly added syntax elements is the following.

alt_cpb_removal_delay_offset is used in all access units which follow a CRA access unit data or a BLA access unit data and precede an access unit data associated with next buffering period SEI message. This syntax is also used in the access unit associated with next buffering period SEI message.

This syntax is to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS when a BLA access unit has nal_unit_type equal to 7 (i.e. no TFD access unit is immediately following the BLA access unit) or a CRA access unit has nal_unit_type equal to 5 (i.e. no TFD access unit is immediately following the CRA access unit) and rap_cpb_params_present_flag is equal to 1.

alt_dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit associated with the buffering period SEI from the CPB before the decoded picture is output from the DPB.

This syntax is only used when an access unit associated with a buffering period SEI message has nal_unit_type equal to 5 or 7 and rap_cpb_params_present_flag is equal to 1.

The length of the syntax element alt_dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1.

In Annex C, new derivation methods for obtaining the DPB output time of a CRA picture or a BLA picture using alt_dpb_output_delay when TFD pictures are inferred as removed. Also new derivation methods for obtaining the DPB output times of pictures following a CRA picture or a BLA picture using alt_cpb_removal_delay_offset when TFD pictures are inferred as removed. See the attached document for detail.

# New SEI message for indicating editing point

## Problem statement

Firstly, it is assumed that a bitstream discussed in this section has fixed_pic_rate_flag equal to 1.

When two bitstreams with the same picture size and the same picture rate are to be concatenated, many applications would require that all coded pictures in the concatenated bitstream are decoded correctly and displayed continuously. But the continuous display of all coded pictures is depended on the last coded picture in the connected bitstream. Figure 2 shows the example of discontinuous display of pictures.

Figure 2  Discontinuity of display before editing point

In the example of Figure 2, the removal of picture B15 from bitstream BSA causes discontinuity in displaying picture B14 and P16 unless the value of dpb_output_delay of P16 is changed properly (i.e. from 5 to 4). In other words, the interval between the display time of picture B14 and that of picture B16 does not fulfill the constraint when fixed_pic_rate_flag is equal to one.

In order to avoid such discontinuity without changing the value of dpb_output_delay, an editing point in a bitstream shall be restricted to such pictures that immediately precede a picture whose preceding pictures are displayed earlier than that picture. In the example of Figure 2, the editing points are B11 and B15. Although it is not difficult to find an entry point, an editing system needs to parse multiple coded pictures.

## Proposal

It is proposed to introduce a new SEI message, an editing point SEI message, in order to indicate an editing point. When the editing process concatenates a bitstream starting from a CRA picture to a bitstream ended by a picture associated with an editing point SEI message, all coded pictures in the generated bitstream are expected to output continuously.

The proposed syntax is the following.

| editing_point( payloadSize ) { | Descriptor |
| --- | --- |
| if( NalHrBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| cpb_status_present_flag | u(1) |
| if( cpb_status_present_flag ) { |  |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The proposed semantics is the following.

An editing point SEI message indicates a possible editing point in a bitstream. When a bitstream ended with an access unit with an editing point SEI message is connected by another bitstream started with a CRA access unit without changing the values of dpb_output_delay of access units in the former bitstream, it is guaranteed that the DPB output times of the access unit with an editing point SEI message and access units preceding the access unit with an editing point SEI message fulfill the following constraint of this specification

- The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

cpb_status_present_flag equal to 1 specifies the presence of the cpb_offset syntax element. When not present, the value of cpb_status_present_flag is inferred to be equal to 0.

cpb_offset specifies the position of the SchedSelIdx-th CPB at the moment when an access unit associated with the editing point SEI message is removed from the CPB.

The position of the SchedSelIdx-th CPB in unit of bit is calculated by Ceil( cpb_offset[ SchedSelIdx ]  90,000 * bit_rate[ SchedSelIdx ] ).

The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock.

# New parameter indicating continuity of bitstream at BLA picture

## Problem statement

When a bitstream started with a CRA picture is concatenated to another bitstream, the value of cpb_removal_delay of the CRA picture needs modified in order to output all pictures in the concatenated bitstream continuously even at the editing point. The proper value of cpb_removal_delay is dependent 1) on the interval between the CRA picture and its preceding picture associated with a buffering period SEI message, 2) on the values of sps_num_reorder_pic of those two bitstreams, and 3) on the continuity of those two bitstreams in terms of CPB fullness. The details of the second dependency and the third dependency are explained using Figure 3 and Figure 4 in the following.

Figure 3  Delayed CPB removal time of BLA picture due to different sps_num_reorder_pics

Firstly the second dependency is explained using Figure 3. In the example of Figure 3, the values of sps_num_reorder_pics of bitstream BSA and bitstream BSB are two and one, respectively. The decoding time of picture I72 needs delayed one picture interval because picture B15 and B16 are not displayed yet at the moment picture I72 is decoded. By delaying the decoding time of picture I72, i.e. delaying the display times of picture I72 and its following pictures, all pictures at or around the editing point can be displayed continuously without loss.

Such delay to the first picture in the connecting bitstream is only required when the value of sps_num_reorder_pics in the connected bitstream (i.e. bitstream BSA) is larger than that in the connecting bitstream (i.e. bitstream BSB). The amount of the delay is equal to the difference of the values of sps_num_reorder_pics in two bitstream.

On the other hand, when the value of sps_num_reorder_pics in the connected bitstream is smaller than that in the connecting bitstream, no such delay is required to the first picture in the connecting bitstream. However, the last picture in the connected bitstream needs displayed repeatedly for the similar reason. Again, the number of the repeat is equal to the difference of the values of sps_num_reorder_pics in two bitstream.

When the values of sps_num_reorder_pic of two bitstream is equal, no delay in decoding the first picture in the connecting bitstream or no repeat in displaying the last picture in the connected bitstream occurs.

Secondly the third dependency is explained using Figure 4.

Figure 4  Delayed CPB removal time of BLA picture due to CPB underflow

Three graphs in Figure 4, from top to bottom, depict CPB fullness of bitstream BSA and bitstream BSB, CPB fullness of the concatenated bitstream in which the first picture of bitstream BSB is decoded one picture interval after the last picture of bitstream BSA is decoded, and CPB fullness of the concatenated bitstream in which in which the first picture of bitstream BSB is decoded two picture intervals after the last picture of bitstream BSA is decoded, respectively.

As the first graph in Figure 4 shows, the CPB position of bitstream BSA just after decoding the last picture is relatively low. In other words, pictures at the end of bitstream BSA have large number of bits. On the other hand, the CPB position of bitstream BSB just before decoding the first picture is lower than the CPB position of bitstream BSA. In this case, the concatenation of two bitstream without delaying the decoding time of the first picture of bitstream BSB causes an underflow.

In other to avoid the underflow, the decoding time of the first picture of bitstream BSB shall be delayed enough as depicted in the third graph in Figure 4.

It is the responsibility of an editing system to modify the value of cpb_removal_time of the first picture of a connecting bitstream (i.e. a BLA picture) properly if the concatenated bitstream is intended to be decoded correctly and displayed continuously even at the editing point. However, the current specification does not have the mean to indicate that such modification is performed by the editing system. As the result, a decoder has to reset its decoding and display process at the editing point since the decoder is not sure whether the value of cpb_removal_time of a BLA picture is set properly or not even though the value is set properly for correct decoding and continuous displaying.

## Proposal

It is proposed to introduce a new syntax element, continuous_hrd_flag, in a buffering period SEI message.

The proposed syntax is the following.

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( !sub_pic_cpb_params_present_flag ) |  |
| rap_cpb_params_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag ) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if(NalHrdBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| continuous_hrd_flag | u(1) |
| } |  |

The proposed semantics for the new syntax elements is the following.

continuous_hrd_flag indicates whether a BLA picture and pictures before and after the BLA picture in the decoding order are decoded correctly and displayed continuously or not.

If a buffering period SEI message is associated with a non-BLA access unit, or both NalHrdBpPresentFlag and VclHrdBpPresentFlag are equal to 0, continuous_hrd_flag has no meaning.

Otherwise, continuous_hrd_flag equal to 1 indicates all of the following conditions are satisfied.

- The values of pic_width_in_luma_samples derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- The values of pic_height_in_luma_samples derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- The values of sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter sets both of the BLA access unit and the access unit immediately preceding the BLA access unit are equal.

- No TFD access unit immediately follows the BLA access unit.

- No underflow or overflow of the CBP occurs at the BLA access unit and at all access units following the BLA access unit until the next BLA access unit when the CPB removal delay of the BLA picture is derived using cpb_removal_delay of the BLA picture.

- The DPB output times of all access units preceding the BLA access unit and those of all access units following the BLA access units and preceding the next BLA access unit satisfy the following constraint of this specification.

  - The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

Alternatively, the last condition in the above definition of continuous_hrd_flag can be replaced with the following relatively relaxed condition.

- The DPB output times of all access units preceding the BLA access unit satisfy the following constraint of this specification.

  - The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

- The DPB output times of all access units following the BLA access unit preceding the next BLA access unit satisfy the following constraint of this specification.

  - The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

- The DPB output times of the BLA access unit may not satisfy the following the following constraint of this specification. That is, the following constraint is not applied to the BLA access unit exceptionally.

  - The value of Dto,dpb( n ) in Equation C-13 is always equal to tc when fixed_pic_rate_flag is equal to 1.

# Proposed text

The proposed text is in the attached document.

# Conclusion

The proposed enhancements can realize the function of simple HEVC bitstream editing with minimal burden to an editing system.

We suggest adopting the proposed enhancements into the draft.

# References

[1] "High Efficiency Video Coding (HEVC) text specification draft 7", JCTVC-I1003, May 2012, Geneva Switzerland

[2] G.Sullivan et al., "CRA pictures with broken links", JCTVC-I0404, May 2012, Geneva Switzerland

[3] S.Deshpande et al., "HRD Buffering for Bitstream Starting with CRA Picture", JCTVC-I0277, May 2012, Geneva Switzerland

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).