| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0138 |
| --- | --- |

| Title: | NSQT simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (HiSilicon), Yuan Yuan (Tsinghua), Haoping Yu (Huawei), Yun He (Tsinghua) | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies, Huawei Technologies, Tsinghua |  |  |

_____________________________

# Abstract

This document provides a simplification of NSQT. Both software implementation and text modification are provided in this contribution. The proposed solution simplifies the non-square transform quadtree split process, and merges the luma non-square transform quadtree and chroma non-square transform quadtree. As a result, NSQT has become a lot easier to describe and implement. Experimental results show that the proposed solution does not have negative impact on coding performance. Both software encoding and decoding time are slightly reduced compared to HM7.0 anchor.

# Complexity issue with the current NSQT solution in HM-7

NSQT introduces a non-square transform quadtree structure to HEVC framework. The square transform block at the root of transform quadtree would be split into non-square transform block with the smaller sizes, and the non-square transform block might be further split into non-square or square blocks. In general, the split process is controlled by the parameter interTbSplitDirectionL (for luma block) and interTbSplitDirectionC (for chroma block) whose value defines three split directions: horizontal split, vertical split and cross split. The definition of interTbSplitDirectionL / interTbSplitDirectionC is illustrated at Fig. 1.

Fig. 1: Illustration of interTbSplitDirection

The derivation of interTbSplitDirectionL / interTbSplitDirectionC is conditioned through part_mode, trafoDepth, transform block size (log2TrafoSize, log2TrafoWidth and log2TrafoHeigh) and the minimum transform block size (Log2MinTrafoSize). The derivation process is listed below:

if( ( nsrqt_enabled_flag
	&&  ( ( log2TrafoSize  = =  Log2MaxTrafoSize
				  | |  ( log2TrafoSize  <  Log2MaxTrafoSize  &&  trafoDepth  = =  0 ) ) 
			  &&  log2TrafoSize  >  ( Log2MinTrafoSize + 1 )
			  &&  ( PartMode  = =  PART_2NxN
				  | |  PartMode  = =  PART_2NxnU  | |  PartMode  = =  PART_2NxnD ) )
		  | |  ( log2TrafoSize  = =  ( Log2MinTrafoSize + 1 )
			  &&  log2TrafoWidth  <  log2TrafoHeight ) ) )
{
	interTbSplitDirectionL = 0
}
else if( ( nsrqt_enabled_flag
	&&  ( ( log2TrafoSize  = =  Log2MaxTrafoSize
				  | |  ( log2TrafoSize  <  Log2MaxTrafoSize  &&  trafoDepth  = =  0 ) ) 
			  &&  log2TrafoSize  >  ( Log2MinTrafoSize + 1 )
			  &&  ( PartMode  = =  PART_Nx2N
				  | |  PartMode  = =  PART_nLx2N  | |  PartMode  = =  PART_nRx2N ) )
		  | |  ( log2TrafoSize  = =  ( Log2MinTrafoSize + 1 )
			  &&  log2TrafoWidth  >  log2TrafoHeight ) ) )
{
	interTbSplitDirectionL = 1
}
else 
{
	interTbSplitDirectionL = 2
}

If either width or height of non-square transform block is equal to the minimum transform block size (Log2MinTrafoSize), the non-square block can be split into the smaller square block. It means that a coding unit might contain three split directions with the purpose of transformation coding. The split process is illustrated in Fig. 2. The current split process might complicate the transform block’s split control logic.

Fig. 2: The split process of a 32x32 transform block: a 32x32 block is split into 32x8 blocks; The 32x8 block is further split into 16x4 blocks, and the 16x4 block is split into four 4x4 blocks

Since square blocks with the minimum transform block size exist in the non-square transform quadtree, the current NSQT partition also causes the disparity of luma transform quadtree and chroma transform quadtree. For example, if a 16x16 luma block is split into 16x4 blocks, the corresponding chroma block size can only be 4x4 as 8x2 block size is not allowed at HEVC. This process also introduces some complexity in transform block address calculation for luma and chroma block. The transform block sizes for luma and chroma non-square quadtree are summarized in Table 1 and Table 2.

Table 1: The transform block size when partition mode is PART_2NxN, or PART_2NxnU, or PART_2NxnD

|  | 64x64 CU |  | 32x32 CU |  | 16x16 CU |  | 8x8 CU |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| trafoDepth=0 | - | - | 32x32 | 16x16 | 16x16 | 8x8 | 8x8 | 4x4 |
| trafoDepth=1 | 32x32 | 16x16 | 32x8 | 16x4 | 16x4 | 4x4 | 4x4 | - |
| trafoDepth=2 | 32x8 | 16x4 | 16x4 | 4x4 | 4x4 | - | - | - |

Table 2: The transform block size when partition mode is PART_Nx2N, or PART_nLx2U, or PART_nRx2N

|  | 64x64 CU |  | 32x32 CU |  | 16x16 CU |  | 8x8 CU |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| trafoDepth=0 | - | - | 32x32 | 16x16 | 16x16 | 8x8 | 8x8 | 4x4 |
| trafoDepth=1 | 32x32 | 16x16 | 8x32 | 4x16 | 4x16 | 4x4 | 4x4 | - |
| trafoDepth=2 | 8x32 | 4x16 | 4x16 | 4x4 | 4x4 | - | - | - |

The next section proposes a simplified solution for NSQT that simplifies the transform block’s split control logic and the transform block address calculation operation.

# NSQT simplification

First of all, to reduce the complexity of non-square quadtree’s split logic and address calculation, the split process of non-square block to square block is removed by the proposed solution. Therefore, the horizontal split (interTbSplitDirection = 0) and the vertical split (interTbSplitDirection = 1) are only used to partition the square transform block at the root of transform quadtree to the non-square transform blocks. After that, the cross split is always used to partition a non-square block into the non-square block with smaller size. Since the square block with the minimum transform block size is removed from the non-square transform quadtree, the disparity between the luma transform quadtree and chroma transform quadtree is also removed. The derivation process of interTbSplitDirection is changed as below:

if( ( nsrqt_enabled_flag
	&&  ( ( log2TrafoSize  <  Log2MaxTrafoSize  &&  log2TrafoSize  >  ( Log2MinTrafoSize + 1 ) )  && ( log2TrafoWidth  = =  log2TrafoHeight )
	&&  ( PartMode  = =  PART_2NxN	  | |  PartMode  = =  PART_2NxnU  | |  PartMode  = =  PART_2NxnD ) ) 
{
	InterTbSplitDirectionL = 0
}
else if( ( nsrqt_enabled_flag
	&&  ( ( log2TrafoSize  < =  Log2MaxTrafoSize  &&  log2TrafoSize  >  ( Log2MinTrafoSize + 1 )  && ( log2TrafoWidth  = =  log2TrafoHeight )
	&&  ( PartMode  = =  PART_Nx2N	  | |  PartMode  = =  PART_nLx2N  | |  PartMode  = =  PART_nRx2N ) ) 
{
	InterTbSplitDirectionL = 1
}
else 
{
	InterTbSplitDirectionL = 2
}

Second, to unify the description of square transform quadtree and non-square transform quadtree split process, a variable Log2MinTrafoSizePlus1 is added to the condition for the parsing process of the split flag as well as in the residual_coding process. When a coding unit allows the non-square transform block partitioning, Log2MinTrafoSizePlus1 is set to 1, which means the minimum transform block size for the current coding block is set to Log2MinTrafoSize + 1. Otherwise, Log2MinTrafoSizePlus1 is set to 0.

The proposed simplified solution also merges the luma non-square and chroma non-square transform quadtree. The chroma block’s address is always same as the corresponding luma block’s address. The transform block sizes for luma and chroma non-square quadtree, which are simplified by the proposed solution, are summarized in Table 3 and Table 4. In contrast to Table 1 and Table 2, the rectangular to square TU split is completed removed in this simplification proposal.

Table 3: The transform block size when partition mode is PART_2NxN, or PART_2NxnU, or PART_2NxnD

|  | 64x64 CU |  | 32x32 CU |  | 16x16 CU |  | 8x8 CU |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| trafoDepth=0 | - | - | 32x32 | 16x16 | 16x16 | 8x8 | 8x8 | 4x4 |
| trafoDepth=1 | 32x32 | 16x16 | 32x8 | 16x4 | 16x4 | - | 4x4 | - |
| trafoDepth=2 | 32x8 | 16x4 | 16x4 | - | - | - | - | - |

Table 4: The transform block size when partition mode is PART_Nx2N, or PART_nLx2U, or PART_nRx2N

|  | 64x64 CU |  | 32x32 CU |  | 16x16 CU |  | 8x8 CU |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| trafoDepth=0 | - | - | 32x32 | 16x16 | 16x16 | 8x8 | 8x8 | 4x4 |
| trafoDepth=1 | 32x32 | 16x16 | 8x32 | 4x16 | 4x16 | - | 4x4 | - |
| trafoDepth=2 | 8x32 | 4x16 | 4x16 | - | - | - | - | - |

# Experimental results

The proposed method is implemented on HM7.0. The coding efficiency is evaluated in RA-Main, RA-HE10, LDB-Main, and LDB-HE10 configurations. The test results are show in Table 5. The anchor data is generated by the common test conditions [1] with NSQT on. From these results we can clearly see that this simplification proposal basically has no negative impact to the coding performance. Table 6 shows the benefit of this simplified NSQT under the common test conditions.

Table 5: HM7.0 with NSQT vs. proposed NSQT simplification

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.29% | -0.20% | 0.00% | 0.03% | 0.04% |
| Class B | 0.02% | -0.03% | -0.17% | 0.00% | -0.07% | -0.15% |
| Class C | 0.01% | 0.18% | 0.17% | 0.03% | 0.23% | 0.06% |
| Class D | 0.06% | 0.27% | 0.22% | 0.05% | 0.39% | 0.18% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.00% | 0.02% | 0.14% | 0.02% |
|  | 0.00% | 0.04% | 0.00% | 0.02% | 0.15% | 0.02% |
| Class F | 0.13% | 0.17% | 0.22% | 0.10% | 0.22% | 0.30% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.30% | 0.34% | -0.05% | 0.28% | 0.16% |
| Class C | -0.04% | 0.59% | 0.41% | -0.01% | 0.37% | 0.44% |
| Class D | 0.04% | 0.64% | 0.65% | 0.05% | 0.54% | 0.34% |
| Class E | -0.20% | -0.04% | 0.16% | -0.04% | -0.82% | -0.10% |
| Overall | -0.04% | 0.39% | 0.40% | -0.01% | 0.16% | 0.23% |
|  | -0.04% | 0.38% | 0.41% | -0.01% | 0.21% | 0.35% |
| Class F | -0.04% | -0.26% | -0.41% | 0.08% | 0.14% | 1.69% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

To comparison between HM7.0 without NSQT and the proposed solution is show in Table 6.

Table 6 HM7.0 w/o NSQT vs. proposed NSQT simplification

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.0% | -0.9% | -0.4% | -0.5% | -0.7% |
| Class B | -0.5% | -1.5% | -1.5% | -0.4% | -1.4% | -1.2% |
| Class C | -0.5% | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% |
| Class D | -0.3% | -0.1% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.8% | -0.7% | -0.4% | -0.6% | -0.6% |
|  | -0.4% | -0.8% | -0.7% | -0.4% | -0.6% | -0.6% |
| Class F | -0.1% | -0.2% | 0.0% | -0.3% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0% | -3.3% | -3.3% | -0.9% | -3.0% | -3.1% |
| Class C | -1.0% | -1.5% | -1.4% | -0.8% | -1.4% | -1.1% |
| Class D | -0.7% | -0.8% | -0.2% | -0.7% | -1.3% | -0.8% |
| Class E | -1.5% | -3.0% | -2.3% | -1.3% | -3.7% | -2.8% |
| Overall | -1.0% | -2.2% | -1.9% | -0.9% | -2.3% | -2.0% |
|  | -1.0% | -2.2% | -1.8% | -0.9% | -2.3% | -1.9% |
| Class F | -1.0% | -1.5% | -1.5% | -0.9% | -1.5% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

This document proposes a solution that simplifies non-square transform quadtree split process and the address calculation for luma and chroma non-square transform units. Compared with NSQT in HM7.0, the proposed simplified solution does not have negative impact to the coding performance. Meanwhile, both encoding and decoding time is slightly reduced. More importantly, with this simplified solution, NSQT has become a lot easier to describe and implement.

NSQT introduces non-square transform to the transform quadtree structure that is fully compatible with square residual transform quadtree and is a good complementary for RQT structure. It can improve the coding performance of residual transform quadtree with 0.4% (RA-Main), 1.0% (LDB-Main), 0.4% (RA-HE) and 0.9% (LDB-HE). Meanwhile, both the encoding and decoding time is almost unchanged when NSQT is turned on. Therefore, it is suggested to adopt the proposed NSQT simplified solution to HEVC DIS text, and adopt the concept of NSQT to HEVC main profile.

# Proposed text for CD text and software

The proposed text can be found in the attachment: JCTVC-J0138-changes_for_cd.doc. The modification can be tracked by the mark “Xiaozhen ZHENG”.

The software is also attached. The changes can be tracked by the macro NSQT_SIMPLIFICATION.

# Reference

[1] F. Bossen, “Common HM test conditions and software reference configurations” JCTVC-I1100, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

[2] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7” JCTVC-I1003, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies and Tsinghua University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).