| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0139 M25462 |
| --- | --- |

| Title: | AhG6: SAO Parameter Estimation Using Non-deblocked Pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-7.0 SAO parameters can be estimated for each LCU at the encoder side. SAO processing occurs after deblock filtering process. Because the deblock filter requires to access pixels in the neighboring LCUs, certain number of bottom and right boundary pixel lines in the current LCU cannot be used for SAO parameter estimation, until processing of bottom and right LCUs is finished. For low delay processing, those unavailable pixels are not used for SAO parameter estimation in HM-7.0, which reduces coding efficiency of SAO.

This contribution is an encoder only proposal, which proposes to use non-deblocked pixels for those unavailable pixels to estimate SAO parameters, where the number of unavailable lines is exactly determined according to SAO type.

Experimental results reportedly show that the proposed method can improve coding efficiency by 0.3/0.0/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/0.5/0.6 in case of LCU size 32, and 0.1/0.6/0.7 in case of LCU size 64, respectively, on average. In case of low-delay P main, the reported gains are 1.8/0.4/0.4 for LCU size 16, 0.9/1.5/1.5 for LCU size 32, and 0.3/1.2/1.2 for LCU size 64, respectively. Note that these average values do not include Class F sequences, in which larger gains are reportedly observed.

# Introduction

In HM-7.0 [1] SAO parameters are estimated for each LCU at the encoder side. SAO processing occurs after deblock filtering process. Because the deblock filter requires to access pixels in the neighboring LCUs, certain number of bottom and right boundary pixel lines in the current LCU cannot be used for SAO parameter estimation, until processing of bottom and right LCUs is finished. For low delay processing, those unavailable pixels are not used for SAO parameter estimation, where 4 bottom lines and 5 right column lines for luma and 2 bottom line and 3 right column lines for chroma are removed for each LCU. This reduces coding efficiency of SAO as reported in [2].

For more accurate SAO parameter estimation without delay, it was proposes to use non-deblocked pixels for those unavailable pixels to estimate SAO parameters [3], where the number of unavailable lines is exactly determined according to SAO type. This contribution implemented the same method as in [2] on top of HM-7.0, of which the experimental results show similar coding gain as before.

# Proposed Method

## Number of unavailable lines according to SAO type

When deblock filter is used, 3 bottom lines and 4 right column lines are not available for SAO parameter estimation of the current LCU. In addition to these lines, one more line may not be available when edge offset is used. This depends on the direction of edge offset filter. Fig. 1 shows edge offset filter window shapes.

|  | C |  |  |  | C |  |  |  | C |  |  |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(a) 1-D 0-degree (b) 1-D 90-degree (c) 1-D 135-degree (d) 1-D 45-degree

Fig. 1. Pixel level range division into two bands and 16 sub-bands in each band.

From Fig. 1 it can be noticed one additional right column line is not available in (a), (c), and (d), and one additional bottom line is not available in (b), (c), and (d). In case of band offset, no additional line is unavailable. Table 1 shows number of unavailable lines according to SAO type.

In the proposed method, exact number of lines is set according to SAO type to use deblock-filtered pixels to estimate SAO parameters.

Table 1. Number of horizontal and vertical unavailable pixel lines for SAO parameter estimation according to SAO type.

| sao_type_idx | Luma |  | Chroma |  |
| --- | --- | --- | --- | --- |
|  | bottom lines | right column lines | bottom lines | right column lines |
| 0 | N/A | N/A | N/A | N/A |
| 1 | 3 | 5 | 1 | 3 |
| 2 | 4 | 4 | 2 | 2 |
| 3 | 4 | 5 | 2 | 3 |
| 4 | 4 | 5 | 2 | 3 |
| 5 | 3 | 4 | 1 | 2 |

## SAO parameter estimation using non-deblocked pixels

We propose to use non-deblocked pixels for unavailable pixels to estimate SAO parameters, where the number of unavailable lines is exactly determined according to SAO type as specified in section 2.1.

Fig. 2 illustrates this for luma component, where black colored pixels are already deblock-filtered when SAO estimation is performed for the current LCU. Blue colored pixels are not available due to edge offset, and red colored pixels are not available due to deblock filtering. Blue colored pixels may not exist depending on SAO type. For example, for band offset, there will be no blue pixel. In the proposed method, SAO parameter estimation is performed using deblock-filtered pixels for black colored pixels and using non-deblock-filtered pixels for blue and red colored pixels.

Fig. 2. Illustration of available and unavailable pixels in LCU for luma component.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the common test conditions [4] are used.

Results of the proposed method are summarized in Table 2. Compared to HM-7.0 anchor, bitrate reduction of 0.3/0.0/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/0.5/0.6 in case of LCU size 32, and 0.1/0.6/0.7 in case of LCU size 64, respectively, is achieved on average. In case of low-delay P main, the gains are 1.8/0.4/0.4 for LCU size 16, 0.9/1.5/1.5 for LCU size 32, and 0.3/1.2/1.2 for LCU size 64, respectively. Note that these average values do not include Class F sequences, in which larger gains are observed. Full test results can be found in the attached spread sheets.

Table 2. BD-rate results summary of the proposed method compared to HM-7.0 anchor.

|  | LCU size | average excluding class F |  |  | average of class F |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cb | Cr | Y | Cb | Cr |
| low-delay B main | 16 | -0.3% | 0.1% | -0.3% | -1.5% | -0.8% | -0.4% |
|  | 32 | -0.3% | -0.9% | -1.1% | -0.9% | -1.3% | -3.1% |
|  | 64 | -0.1% | -0.9% | -1.3% | -0.3% | -1.2% | -1.2% |
| low-delay P main | 16 | -1.8% | -0.4% | -0.4% | -1.8% | -1.8% | -1.7% |
|  | 32 | -0.9% | -1.5% | -1.5% | -0.9% | -1.5% | -1.5% |
|  | 64 | -0.3% | -1.2% | -1.2% | -0.4% | -1.6% | -1.4% |

Table 3. BD-rate results of the proposed method compared to HM-7.0 anchor.

- LCU size 16

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.8% | 0.1% | -0.1% | -0.3% | 0.2% | 0.2% |
| Enc Time[%] | 105% |  |  | 96% |  |  |
| Dec Time[%] | 107% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.1% | -0.1% | 0.0% | 0.0% | -0.4% |
| Class B | -0.3% | 0.1% | 0.3% | -0.2% | 0.0% | 0.1% |
| Class C | -0.1% | -0.1% | -0.2% | -0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | -0.1% |
|  | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class F | -0.7% | -0.5% | -1.1% | -0.3% | 0.3% | 0.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.1% | -0.2% | 0.0% | -0.2% | 0.3% |
| Class C | -0.4% | -0.2% | -1.0% | -0.1% | -0.2% | -0.1% |
| Class D | -0.2% | 0.3% | 0.0% | 0.0% | 0.2% | 0.2% |
| Class E | -0.5% | 0.1% | 0.3% | 0.0% | -0.1% | -0.3% |
| Overall | -0.3% | 0.1% | -0.3% | 0.0% | -0.1% | 0.1% |
|  | -0.3% | 0.1% | -0.2% | 0.0% | -0.1% | 0.1% |
| Class F | -1.5% | -0.8% | -0.4% | -0.9% | -0.4% | -0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.3% | -0.6% | -0.4% | -0.2% | -0.1% | -0.3% |
| Class C | -1.5% | -0.7% | -0.8% | -0.2% | 0.0% | 0.1% |
| Class D | -1.1% | -0.1% | 0.0% | -0.1% | 0.7% | 0.8% |
| Class E | -2.2% | 0.1% | -0.1% | 0.0% | 0.3% | 0.4% |
| Overall | -1.8% | -0.4% | -0.4% | -0.1% | 0.2% | 0.2% |
|  | -1.8% | -0.3% | -0.3% | -0.1% | 0.2% | 0.2% |
| Class F | -1.8% | -1.8% | -1.7% | -0.7% | 0.0% | -0.4% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  |

- LCU size 32

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.4% |
| Class B | 0.0% | -0.3% | -0.5% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.7% | -1.0% | -0.1% | -0.1% | -0.2% |
| Class D | -0.1% | -0.4% | -0.5% | -0.1% | 0.0% | 0.0% |
| Class E | -0.1% | -0.4% | -0.3% | -0.1% | 0.1% | 0.1% |
| Overall | -0.1% | -0.4% | -0.5% | -0.1% | 0.0% | -0.1% |
|  | -0.1% | -0.4% | -0.5% | -0.1% | 0.0% | -0.1% |
| Class F | -0.2% | -1.0% | -1.1% | -0.3% | -0.4% | -0.5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.7% | -0.4% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.8% | -0.8% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.9% | -1.0% | -0.1% | -0.3% | -0.4% |
| Class D | 0.1% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.7% | -0.7% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.8% | -0.7% | -0.1% | -0.2% | -0.2% |
| Class F | -0.3% | -1.1% | -1.4% | -0.3% | -0.6% | -0.8% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.8% | -0.7% | -0.2% | -0.1% | 0.1% |
| Class C | -0.3% | -1.3% | -1.9% | -0.2% | -0.3% | -0.4% |
| Class D | -0.1% | -0.5% | -0.9% | -0.2% | 0.1% | -0.3% |
| Class E | -0.4% | -1.0% | -1.1% | -0.3% | 0.0% | -0.6% |
| Overall | -0.3% | -0.9% | -1.1% | -0.2% | -0.1% | -0.2% |
|  | -0.3% | -1.0% | -1.2% | -0.2% | -0.2% | -0.3% |
| Class F | -0.9% | -1.3% | -3.1% | -0.7% | 0.0% | 0.4% |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 110% |  |  | 106% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1% | -1.4% | -1.3% | -0.4% | -0.2% | -0.5% |
| Class C | -0.8% | -1.6% | -2.0% | -0.4% | -0.2% | -0.8% |
| Class D | -0.5% | -1.5% | -1.7% | -0.3% | -0.2% | -0.4% |
| Class E | -1.2% | -1.8% | -1.1% | -0.5% | -0.1% | 0.8% |
| Overall | -0.9% | -1.5% | -1.5% | -0.4% | -0.2% | -0.3% |
|  | -0.9% | -1.6% | -1.6% | -0.4% | -0.2% | -0.4% |
| Class F | -0.9% | -1.8% | -1.7% | -0.7% | -0.2% | -2.2% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |

- LCU size 64

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.5% | 0.0% | -0.2% | -0.3% |
| Class B | 0.0% | -0.5% | -0.7% | 0.0% | -0.2% | -0.4% |
| Class C | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | -0.4% | -0.5% | 0.0% | -0.1% | -0.2% |
| Class E | 0.0% | -0.9% | -1.1% | -0.1% | -0.2% | -0.3% |
| Overall | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% |
| Class F | 0.0% | -0.4% | -0.5% | -0.1% | -0.4% | -0.5% |
| Enc Time[%] | 93% |  |  | 98% |  |  |
| Dec Time[%] | 92% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.8% | -0.9% | -0.1% | -0.4% | -0.5% |
| Class B | 0.0% | -0.6% | -0.7% | -0.1% | -0.4% | -0.4% |
| Class C | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.5% | -0.6% | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.5% | -0.6% | 0.0% | -0.3% | -0.4% |
| Class F | -0.1% | -0.4% | -0.6% | -0.1% | -0.4% | -0.6% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.9% | -1.4% | -0.1% | -0.2% | -0.5% |
| Class C | -0.1% | -0.7% | -1.0% | -0.1% | -0.6% | -1.0% |
| Class D | -0.1% | -0.5% | -0.6% | -0.1% | -0.5% | -0.3% |
| Class E | -0.2% | -1.8% | -2.3% | -0.1% | -0.7% | -1.1% |
| Overall | -0.1% | -0.9% | -1.3% | -0.1% | -0.5% | -0.7% |
|  | -0.1% | -0.9% | -1.2% | -0.1% | -0.5% | -0.6% |
| Class F | -0.4% | -1.0% | -1.6% | -0.1% | -0.6% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -1.2% | -1.3% | -0.2% | -0.4% | -0.7% |
| Class C | -0.2% | -1.1% | -1.2% | -0.1% | -0.7% | -0.8% |
| Class D | -0.1% | -0.6% | -0.3% | -0.1% | -0.5% | -1.3% |
| Class E | -0.4% | -2.3% | -2.2% | -0.3% | -0.4% | -0.8% |
| Overall | -0.3% | -1.2% | -1.2% | -0.1% | -0.5% | -0.9% |
|  | -0.3% | -1.2% | -1.1% | -0.1% | -0.5% | -0.8% |
| Class F | -0.4% | -1.6% | -1.4% | -0.3% | -0.7% | -1.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 97% |  |  |

# Conclusion

It is proposed to use non-deblocked pixels to estimate SAO parameters for LCU boundary lines, where exact number of unavailable lines is set according to SAO type. The proposed method is implemented on top of HM-7.0, and experimental results show coding efficiency improvement of 0.3/0.0/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/0.5/0.6 in case of LCU size 32, and 0.1/0.6/0.7 in case of LCU size 64, respectively, on average. In case of low-delay P main, the gains are 1.8/0.4/0.4 for LCU size 16, 0.9/1.5/1.5 for LCU size 32, and 0.3/1.2/1.2 for LCU size 64, respectively. These average values do not include Class F sequences, in which larger gains are observed.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1003, May 2012.

- W.-S. Kim, “CE1 Test 4.3 : Removal of pixel access constraints for SAO interleaving mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0195, May 2012.

- W.-S. Kim, “Non-CE1: SAO parameter estimation using non-deblocked pixels in interleaving mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0200, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).