| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0141 M25464 |
| --- | --- |

| Title: | AhG6: SAO Offset Bypass Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Madhukar Budagavi Vivienne Sze | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-7.0, SAO offset is coded using the truncated unary binarization, and all the bins are coded using a context model, where the maximum number of bin is 7 and 31 for 8 bit and 10 bit video, respectively. In this contribution, techniques that reduce number of context coded bins for coding SAO offsets are proposed. Two approaches are studied where only two bins are context coded, and the other bins are bypass coded. The first approach uses the truncated unary binarization of HM-7.0, of which the maximum number of bin is 31 for 10 bit video. In the second approach, a combination of truncated unary coding and fixed length coding is proposed, of which the maximum number of bin is 7 for 10 bit video. Experimental results are reportedly shown that there is 0.0/0.0/0.0% gain for Y/Cb/Cr in the first approach and 0.0/0.3/0.3% loss for Y/Cb/Cr in the second approach.

# Introduction

In HM-7.0, SAO offset is coded using the truncated unary binarization. As the offset range is restricted to [0, 7] in case of 8 bit video, and [0, 31] when bit-depth is larger than 9, there will be 7 bins and 31 bins, respectively, in the worst case, as shown in Fig. 1. All bins are context coded.

offset value: binarized bins

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 1111111

< 8 bit video>

offset value: binarized bins

0: 0

1: 10

2: 110

3: 1110

…

30: 1111111111111111111111111111110

31: 1111111111111111111111111111111

< 10 bit video >

Fig. 1. Generated bins to code SAO offset using truncated unary binarization in case of 8 bit video and 10 bit video in HM-7.0. All bins are context coded.

It will be desirable to reduce the number of bins to ease implementation complexity.

# Proposed methods

## Approach 1: HM-7.0 binarization with only first two bins context coded

We studied how many bins can be bypass coded without coding efficiency loss. According to our experimental results, we propose to code only the first and second bins using context model, and to code the other bins using bypass coding. The modifications are shown in Fig. 2 below. Only red colored bins are context coded. The remaining bins are bypass coded.

offset value: binarized bins

0: 0

1: 10

2: 110

3: 1110

…

6: 1111110

7: 1111111

< 8 bit video>

offset value: binarized bins

0: 0

1: 10

2: 110

3: 1110

…

30: 1111111111111111111111111111110

31: 1111111111111111111111111111111

< 10 bit video >

Fig. 2. Generated bins to code SAO offset using truncated unary binarization in case of 8 bit video and 10 bit video in HM-7.0. Only first two bins are context coded (shown in red). Remaining bins are bypass coded.

We also propose to code four SAO offsets together, and code context coded bins of all four offsets together first, followed by bypass bin coding for all four offsets, in order to avoid frequent switching between context coding and bypass coding.

## Approach 2: Truncated unary+fixed length coding

As can be seen in Fig. 2, in the worst case, too many bins have to be bypass coded. Therefore, in addition to bypass coding described in Section 2.1, we also propose a new binarization scheme for SAO offset. We provide a prefix using a truncated unary binarization followed by a fixed-length suffix. The length of the suffix is determined according to the offset range, which depends on bit-depth. When offset range is [0, 7], i.e., bit-depth is 8, the suffix length is set to 3. When offset range is [0, 15], i.e., bit-depth is 9, the suffix length is set to 4. When offset range is [0, 31], i.e., bit-depth is larger than 9, the suffix length is set to 5. Fig. 3 shows the binarized bins according to the proposed method.

offset value: binarized bins

0: 0

1: 10

2: 11000

3: 11001

…

6: 11110

7: 11111

< 8 bit video>

offset value: binarized bins

0: 0

1: 10

2: 1100000

3: 1100001

…

30: 1111110

31: 1111111

< 10 bit video >

Fig. 3. Generated bins to code SAO offset using Approach 2. Only first two bins are context coded (colored in red in figure). Remaining bins are bypass coded.

By using the proposed method, the number of bins is reduced from 7 to 5 in case of 8 bit video, and 31 to 7 in case of bit-depth larger than 9.

As same in Section 2.1, we propose to code four SAO offsets together, and code context coded bins of all four offsets together first, followed by bypass bin coding for all four offsets to avoid frequent switching between context coding and bypass coding.

# Experimental results

To evaluate the coding efficiency of the proposed methods, the common test conditions [4] are used, and HM-7.0 is used as the anchor.

## Approach 1: HM-7.0 binarization with only first two bins context coded

The proposed method in Section 2.1 is evaluated, where offset value is binarized using the truncated binarization as in HM-7.0, and the first two bins are context coded and the other bins are bypass coded.

Results of the proposed method are summarized in Table 1. Compared to HM-7.0 anchor, bitrate increment of 0.0/0.0/0.0 % (Y/Cb/Cr) is resulted on average.

Table 1. BD-rate results summary of the proposed bypass coding method.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 106% |  |  | 101% |  |  |
| Dec Time[%] | 109% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.3% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.3% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | -0.3% |
| Class E | -0.1% | 0.0% | -0.4% | 0.1% | 0.1% | -0.8% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | -0.1% | 0.2% | 0.1% | 0.3% | 0.4% | 1.3% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% |
| Class C | -0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.2% | 0.3% | 0.1% | -0.3% | -0.8% |
| Class E | 0.0% | -0.2% | 0.3% | 0.1% | 0.4% | 0.4% |
| Overall | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | -0.2% |
|  | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
| Class F | 0.0% | 0.1% | -0.1% | 0.1% | 0.3% | 0.0% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

## Approach 2: Truncated unary+fixed length coding

The proposed method in Section 2.2 is evaluated, where offset value is binarized using combination of truncated unary and fixed length binarization, and two bins are context coded and the other bins are bypass coded.

Results of the proposed method are summarized in Table 2. Compared to HM-7.0 anchor, bitrate increment of 0.0/0.3/0.3 % (Y/Cb/Cr) is resulted on average over Main configuration.

Table 2. BD-rate results summary of the proposed binarization method with bypass coding.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.3% |
| Class C | 0.0% | 0.2% | 0.3% | 0.0% | 0.2% | 0.3% |
| Class D | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.4% | 0.4% | 0.1% | 0.1% | 0.2% |
| Overall | 0.0% | 0.2% | 0.3% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.3% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.1% | 0.4% | 0.3% |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.3% | 0.4% |
| Class C | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.3% |
| Class D | 0.0% | 0.3% | 0.2% | 0.0% | 0.4% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.3% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.3% |
| Class F | 0.1% | 0.3% | 0.3% | 0.1% | 0.4% | 0.5% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% | 0.1% | 0.5% | 0.8% |
| Class C | 0.0% | 0.4% | 0.4% | 0.1% | 0.7% | 0.6% |
| Class D | 0.0% | 0.2% | 0.5% | 0.0% | 0.8% | 0.9% |
| Class E | 0.1% | 0.2% | 0.2% | 0.4% | 0.0% | 0.2% |
| Overall | 0.0% | 0.3% | 0.3% | 0.1% | 0.5% | 0.6% |
|  | 0.0% | 0.2% | 0.3% | 0.1% | 0.6% | 0.7% |
| Class F | 0.1% | -0.1% | 0.7% | 0.2% | 0.4% | 1.1% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.2% | 0.6% | 0.5% |
| Class C | 0.1% | 0.1% | 0.5% | 0.1% | 0.7% | 0.7% |
| Class D | 0.1% | 0.8% | 0.8% | 0.1% | 1.0% | 0.1% |
| Class E | 0.2% | 0.4% | 0.5% | 0.3% | 0.8% | 0.7% |
| Overall | 0.1% | 0.4% | 0.4% | 0.2% | 0.8% | 0.5% |
|  | 0.1% | 0.4% | 0.4% | 0.2% | 0.7% | 0.5% |
| Class F | 0.1% | 0.2% | 0.2% | 0.1% | 0.9% | 0.7% |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

# Conclusion

In this contribution, new SAO offset bypass coding methods are proposed. First, it is proposed to code only first two bins using context model, and bypass the other bins. Second, a new binarization method is proposed to reduce the number of bypass bins. The number of context coded bins is reduced from 31 to 2, and the total number of bins is reduced from 31 to 7 (10 bit video). Experimental results show that there is almost no coding loss from this simplification.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1003, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.

# Proposed Text Changes

#### 7.3.4.1 Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs_ctx_bins[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| if( sao_offset_abs_ctx_bins[ cIdx ][ rx][ ry ][ i ] == ‘11’ ) |  |
| sao_offset_abs_bypass_bins[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding tree block at position rx and ry for the colour component cIdx.

sao_offset_abs_ctx_bins[ cIdx ][ rx ][ ry ][ i ] and sao_offset_abs_bypass_bins[ cIdx ][ rx ][ ry ][ i ] are used to generate sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] which indicates the offset value of i-th category of current coding tree block at position rx and ry for the colour component cIdx. sao_offset_abs_ctx_bins[ cIdx ][ rx ][ ry ][ i ] is the first one or two decoded bins of binarized sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]. If it is ‘11’, sao_offset_abs_bypass_bins[ cIdx ][ rx ][ ry ][ i ] is present, and decoded bins are combined with those of sao_offset_abs_ctx_bins[ cIdx ][ rx ][ ry ][ i ] to decode sao_offset_abs[ cIdx ][ rx ][ ry ][ i ].

Table 9-1 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao_param() | sao_merge_left_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sao_offset_abs | Table 9-2 | 0..1 | 2..3 | 4..5 |

Table 9-2 – Values of variable initValue for sao_offset_abs ctxIdx

| Initialization variable | sao_offset_abs ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  | 4 |  | 5 |  |  |  |  |
| initValue | 171 | 119 | 199 | 169 |  | 119 |  | 151 |  |  |  |  |
| Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |  |  |  |  |  |  |  |

| sao_offset_abs | 0 | Prefix: TU, cMax = 2 Suffix: FL, cMax=( 1<< (bitDepth − 5)) − 1 | 1 | Table 9-2 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Table 9-2 | 2 |
|  | 2 |  | 1 | Table 9-2 | 4 |

# Patent rights declaration

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix. Additional Experimental Results

Additional experimental results when SAO offset is binarized using truncated unary binarization as in HM-7.0, and only the first 1 bin is context coded. These results show that there is a little coding efficiency loss, which suggests to increase the number of context coded bins as proposed in Section 2.1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% | 0.3% |
| Enc Time[%] | 94% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.3% |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% | 0.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.5% | 0.0% | 0.2% | 0.5% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.5% |
| Class D | 0.0% | 0.2% | 0.1% | 0.1% | 0.5% | 0.3% |
| Class E | 0.0% | 0.3% | 0.7% | 0.2% | 0.1% | 0.0% |
| Overall | 0.0% | 0.4% | 0.4% | 0.1% | 0.2% | 0.4% |
|  | 0.0% | 0.4% | 0.4% | 0.1% | 0.2% | 0.4% |
| Class F | 0.1% | 0.8% | 0.5% | 0.1% | 0.4% | 1.9% |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.9% | 0.7% | 0.0% | 0.1% | 0.3% |
| Class C | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% |
| Class D | 0.1% | 0.3% | 0.7% | 0.1% | 0.0% | 0.0% |
| Class E | 0.1% | 0.8% | 1.3% | 0.0% | 0.5% | 0.0% |
| Overall | 0.1% | 0.6% | 0.7% | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.5% | 0.6% | 0.1% | 0.2% | 0.2% |
| Class F | 0.1% | 0.4% | 0.7% | 0.2% | 1.1% | 0.7% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |