| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0142 |
| --- | --- |

| Title: | coeff_abs_level_remaining maximum codeword length reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Vivienne Sze | Email: | madhukar@ti.com, sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

HEVC HM-7.0 codes coeff_abs_level_remaining using a VLC that consists of a unary coded prefix and a fixed length suffix whose size depends on the prefix and a parameter cParam that is adaptively updated after each coeff_abs_level_remaining is coded. This VLC is basically a variant of exp-golomb (k) code. The maximum codeword length for this VLC is 37 bits. It is asserted that VLC with maximum codeword length of 32 bits or less is desirable from throughput and complexity reduction perspective especially for architectures that use 32-bit processors and/or 32-bit memories. This contribution studies several different VLCs obtained by modifying HM-7.0 coeff_abs_level_remaining VLC (denoted as U-FLC123-L37 in this contribution) so that the maximum codeword length is less than or equal to 32 bits. The VLCs are reported to result in average BD-Rate of 0.0% to 0.1% in normal and low QP range for Class A-E. For Class F sequences, the VLCs are reported to result in average BD-Rate of -0.1% to 0.2% for normal QP range and -1.7% to 0.2% for low QP range. The VLC with the smallest maximum codeword length in this contribution is TU-FLC135-L29 with maximum codeword length of 29 bits. This VLC is asserted to be a good choice from complexity reduction and BD-Rate degradation point of view. Among the VLCs with maximum codeword length equal to 32 bits, TU-FLC123-L32 is asserted to be a good choice in term of complexity reduction, closeness to HM-7.0 VLC and BD-Rate degradation. The average BD-Rate degradation of both of these VLCs is 0.0% for normal and low QP Class A-E. For Class F sequences, there is BD-Rate savings of -0.6% and -0.4% respectively for low QP value and 0.0% for normal QP range.

# Introduction

HEVC HM-7.0 codes coeff_abs_level_remaining using a VLC that consists of a unary coded prefix and a fixed length suffix whose size depends on the prefix and a parameter cParam that is adaptively updated after each coeff_abs_level_remaining is coded. The maximum codeword length for this VLC is 37 bits. It is asserted that VLCs with maximum codeword length less than or equal to 32 bits are desirable from hardware throughput and complexity perspective. Reduced maximum codeword length is also asserted to be desirable from software point of view especially on architectures with limited bit depth registers such as 16-bit and 32-bit processors. This contribution presents several modifications to HM-7.0 coeff_abs_level_remaining VLC so that the maximum codeword length is less than or equal to 32 bits.

The suffix length increases linearly with cParam for the HEVC HM=7.0 VLC. Most of VLCs proposed in this contribution behave similarly with regards to cParam, i.e. the suffix part linearly increases with cParam. Hence only the case of cParam = 0 is shown for them. For VLCs where cParam != 0 is handled differently, details of cParam != 0 are shown.

# HM-7.0 VLC (U-FLC123-L37)

HM-7.0 VLC for coeff_abs_level_remaining for cParam = 0 is shown in Table 1 below. The VLC consists of a unary coded prefix and a fixed length suffix whose size depends on the prefix and a parameter cParam that is adaptively updated after each coeff_abs_level_remaining is coded. Levels till 8 are coded using unary coding only. Levels from 9 onwards are coded using a combination of unary prefix + fixed length suffix. The fixed length suffix increases by 1 for every increment of prefix codeword length. HM-7.0 VLC for coeff_abs_level_remaining will be denoted as U-FLC123-L37. The U denotes that the prefix is unary coded, FLC123 indicates that the suffix part increments by 1 for every increase in prefix codeword length, and L37 indicates that the maximum codeword length is 37 bits.

It can be seen from Table 1 that the maximum codeword length is 37 bits for coding the range of 16391~32774.

Table 1: HM-7.0 VLC (U-FLC123-L37) for coeff_abs_level_remaining for cParam = 0.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5 | 111110 |  | 6 | 0 | 6 |
| 6 | 1111110 |  | 7 | 0 | 7 |
| 7 | 11111110 |  | 8 | 0 | 8 |
| 8 | 111111110 |  | 9 | 0 | 9 |
| 9~10 | 1111111110 | x | 10 | 1 | 11 |
| 11~14 | 11111111110 | xx | 11 | 2 | 13 |
| 15~22 | 111111111110 | xxx | 12 | 3 | 15 |
| 23~38 | 1111111111110 | xxxx | 13 | 4 | 17 |
| 39~70 | 11111111111110 | xxxxx | 14 | 5 | 19 |
| 71~134 | 111111111111110 | xxxxxx | 15 | 6 | 21 |
| 135~262 | 1111111111111110 | xxxxxxx | 16 | 7 | 23 |
| 263~518 | 11111111111111110 | xxxxxxxx | 17 | 8 | 25 |
| 519~1030 | 111111111111111110 | xxxxxxxxx | 18 | 9 | 27 |
| 1031~2054 | 1111111111111111110 | xxxxxxxxxx | 19 | 10 | 29 |
| 2055~4102 | 11111111111111111110 | xxxxxxxxxxx | 20 | 11 | 31 |
| 4103~8198 | 111111111111111111110 | xxxxxxxxxxxx | 21 | 12 | 33 |
| 8199~16390 | 1111111111111111111110 | xxxxxxxxxxxxx | 22 | 13 | 35 |
| 16391~32774 | 11111111111111111111110 | xxxxxxxxxxxxxx | 23 | 14 | 37 |

Table 2 shows the HM-7.0 VLC for coeff_abs_level_remaining for cParam = 1. Character ‘c’ in the suffix part indicates the fixed length component due to cParam. The maximum codeword length for cParam = 1 is 36 bits. The maximum codeword length for cParam = 2 is 35 bits. Therefore the worst case is Table 1 which has maximum codeword length of 37 bits.

Table 2: HM-7.0 VLC (U-FLC123-L37) for coeff_abs_level_remaining for cParam = 1. Character ‘c’ in the suffix part indicates the fixed length component due to cParam.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0~1 | 0 | c | 1 | 1 | 2 |
| 2~3 | 10 | c | 2 | 1 | 3 |
| 4~5 | 110 | c | 3 | 1 | 4 |
| 6~7 | 1110 | c | 4 | 1 | 5 |
| 8~9 | 11110 | c | 5 | 1 | 6 |
| 10~11 | 111110 | c | 6 | 1 | 7 |
| 12~13 | 1111110 | c | 7 | 1 | 8 |
| 14~15 | 11111110 | c | 8 | 1 | 9 |
| 16~17 | 111111110 | c | 9 | 1 | 10 |
| 18~21 | 1111111110 | xc | 10 | 2 | 12 |
| 22~29 | 11111111110 | xxc | 11 | 3 | 14 |
| 30~45 | 111111111110 | xxxc | 12 | 4 | 16 |
| 46~77 | 1111111111110 | xxxxc | 13 | 5 | 18 |
| 78~141 | 11111111111110 | xxxxxc | 14 | 6 | 20 |
| 142~269 | 111111111111110 | xxxxxxc | 15 | 7 | 22 |
| 270~525 | 1111111111111110 | xxxxxxxc | 16 | 8 | 24 |
| 526~1037 | 11111111111111110 | xxxxxxxxc | 17 | 9 | 26 |
| 1038~2061 | 111111111111111110 | xxxxxxxxxc | 18 | 10 | 28 |
| 2062~4109 | 1111111111111111110 | xxxxxxxxxxc | 19 | 11 | 30 |
| 4110~8205 | 11111111111111111110 | xxxxxxxxxxxc | 20 | 12 | 32 |
| 8206~16397 | 111111111111111111110 | xxxxxxxxxxxxc | 21 | 13 | 34 |
| 16398~32781 | 1111111111111111111110 | xxxxxxxxxxxxxc | 22 | 14 | 36 |

# U-FLC123-L37 + Escape coding

The first option for reducing the maximum codeword length to 32 bits is to use escape coding for levels 263~33030 (when cParam = 0) so that the maximum codeword length is 32 bits. The resulting VLC is shown in Table 3.

Table 3: HM-7.0 VLC (U-FLC123-L37) + Escape coding for coeff_abs_level_remaining for cParam = 0. Maximum codeword length is 32 bits.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5 | 111110 |  | 6 | 0 | 6 |
| 6 | 1111110 |  | 7 | 0 | 7 |
| 7 | 11111110 |  | 8 | 0 | 8 |
| 8 | 111111110 |  | 9 | 0 | 9 |
| 9~10 | 1111111110 | x | 10 | 1 | 11 |
| 11~14 | 11111111110 | xx | 11 | 2 | 13 |
| 15~22 | 111111111110 | xxx | 12 | 3 | 15 |
| 23~38 | 1111111111110 | xxxx | 13 | 4 | 17 |
| 39~70 | 11111111111110 | xxxxx | 14 | 5 | 19 |
| 71~134 | 111111111111110 | xxxxxx | 15 | 6 | 21 |
| 135~262 | 1111111111111110 | xxxxxxx | 16 | 7 | 23 |
| 263~33030 | 11111111111111110 | xxxxxxxxxxxxxxx | 17 | 15 | 32 |

Table 4 shows the corresponding VLC for cParam = 1. Note that for levels 526~3329, the suffix part is still 15 bits and does not increase with cParam.

Table 4: HM-7.0 VLC (U-FLC123-L37) + Escape coding for coeff_abs_level_remaining for cParam = 1. Character ‘c’ in the suffix part indicates the fixed length component due to cParam. Maximum codeword length is 32 bits.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0~1 | 0 | c | 1 | 1 | 2 |
| 2~3 | 10 | c | 2 | 1 | 3 |
| 4~5 | 110 | c | 3 | 1 | 4 |
| 6~7 | 1110 | c | 4 | 1 | 5 |
| 8~9 | 11110 | c | 5 | 1 | 6 |
| 10~11 | 111110 | c | 6 | 1 | 7 |
| 12~13 | 1111110 | c | 7 | 1 | 8 |
| 14~15 | 11111110 | c | 8 | 1 | 9 |
| 16~17 | 111111110 | c | 9 | 1 | 10 |
| 18~21 | 1111111110 | xc | 10 | 2 | 12 |
| 22~29 | 11111111110 | xxc | 11 | 3 | 14 |
| 30~45 | 111111111110 | xxxc | 12 | 4 | 16 |
| 46~77 | 1111111111110 | xxxxc | 13 | 5 | 18 |
| 78~141 | 11111111111110 | xxxxxc | 14 | 6 | 20 |
| 142~269 | 111111111111110 | xxxxxxc | 15 | 7 | 22 |
| 270~525 | 1111111111111110 | xxxxxxxc | 16 | 8 | 24 |
| 526~33293 | 11111111111111110 | xxxxxxxxxxxxxxx | 17 | 15 | 32 |

# Shifting up HM-7.0 VLC: U-FLC123-L32 and TU-FLC123-L32

In HM-7.0 VLC for coeff_abs_level_remaining (U-FLC123-L37), levels till 8 are coded using unary coding only for cParam = 0. This VLC can be moved up and suffix coding can be started earlier. Two VLCs in this category are proposed in this contribution: U-FLC123-L32 and TU-FLC123-L32.

## U-FLC123-L32

Table 5 shows the VLC for U-FLC123-L32. U denote that the prefix is unary coded, FLC123 denotes that the suffix part increments by 1 for every increase in prefix codeword length as in the existing VLC, L32 indicates that the maximum codeword length is 32 bits.

Table 5: U-FLC123-L32 VLC for cParam = 0. Maximum codeword length is 32 bits.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4~5 | 11110 | x | 5 | 1 | 6 |
| 6~9 | 111110 | xx | 6 | 2 | 8 |
| 10~17 | 1111110 | xxx | 7 | 3 | 10 |
| 18~33 | 11111110 | xxxx | 8 | 4 | 12 |
| 34~65 | 111111110 | xxxxx | 9 | 5 | 14 |
| 66~129 | 1111111110 | xxxxxx | 10 | 6 | 16 |
| 130~257 | 11111111110 | xxxxxxx | 11 | 7 | 18 |
| 258~513 | 111111111110 | xxxxxxxx | 12 | 8 | 20 |
| 514~1025 | 1111111111110 | xxxxxxxxx | 13 | 9 | 22 |
| 1026~2049 | 11111111111110 | xxxxxxxxxx | 14 | 10 | 24 |
| 2050~4097 | 111111111111110 | xxxxxxxxxxx | 15 | 11 | 26 |
| 4098~8193 | 1111111111111110 | xxxxxxxxxxxx | 16 | 12 | 28 |
| 8194~16385 | 11111111111111110 | xxxxxxxxxxxxx | 17 | 13 | 30 |
| 16386~32769 | 111111111111111110 | xxxxxxxxxxxxxx | 18 | 14 | 32 |

## TU-FLC123-L32

The HM-7.0 VLC can be moved up and truncated unary (TU) coding can be used instead of unary coding. Table 6 shows the resulting VLC which will be denoted as TU-FLC123-L32. TU denotes that truncated unary is used, FLC123 denotes that the suffix part increments by 1 for every increase in prefix codeword length as in the existing VLC, and L32 indicates that the maximum codeword length is 32 bits.

Table 6: TU-FLC123-L32 VLC for cParam = 0. Maximum codeword length is 32 bits.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5~6 | 111110 | x | 6 | 1 | 7 |
| 7~10 | 1111110 | xx | 7 | 2 | 9 |
| 11~18 | 11111110 | xxx | 8 | 3 | 11 |
| 19~34 | 111111110 | xxxx | 9 | 4 | 13 |
| 35~66 | 1111111110 | xxxxx | 10 | 5 | 15 |
| 67~130 | 11111111110 | xxxxxx | 11 | 6 | 17 |
| 131~258 | 111111111110 | xxxxxxx | 12 | 7 | 19 |
| 259~514 | 1111111111110 | xxxxxxxx | 13 | 8 | 21 |
| 515~1026 | 11111111111110 | xxxxxxxxx | 14 | 9 | 23 |
| 1027~2050 | 111111111111110 | xxxxxxxxxx | 15 | 10 | 25 |
| 2051~4098 | 1111111111111110 | xxxxxxxxxxx | 16 | 11 | 27 |
| 4099~8194 | 11111111111111110 | xxxxxxxxxxxx | 17 | 12 | 29 |
| 8195~16386 | 111111111111111110 | xxxxxxxxxxxxx | 18 | 13 | 31 |
| 16387~32770 | 111111111111111111 | xxxxxxxxxxxxxx | 18 | 14 | 32 |

# FLC135 based techniques: U-FLC135-L32, U-FLC135-L30, and TU-FLC135-L29

In HM-7.0 VLC (U-FLC123-L37), the suffix length increments by 1 as the prefix increases by 1. In the proposed FLC135-based techniques, the suffix length increments by 2 as the prefix increases by 1. Three VLCs in this category were tested U-FLC135-L32, U-FLC135-L30, and TU-FLC135-L29.

## U-FLC135-L32

U denotes that he prefix is unary coded, FLC135 denotes that the suffix length increments as 1, 3, 5, 7, 9, …. . Table 7 shows the U-FLC135-L32 VLC for cParam = 0. The maximum codeword length is 31 bits. However, cParam = 1 leads to worst case codeword length and the maximum codeword length for U-FLC135-L32 is 32 bits.

Table 7: U-FLC135-L32 VLC for cParam = 0.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5 | 111110 |  | 6 | 0 | 6 |
| 6 | 1111110 |  | 7 | 0 | 7 |
| 7 | 11111110 |  | 8 | 0 | 8 |
| 8~9 | 111111110 | x | 9 | 1 | 10 |
| 10~17 | 1111111110 | xxx | 10 | 3 | 13 |
| 18~49 | 11111111110 | xxxxx | 11 | 5 | 16 |
| 50~177 | 111111111110 | xxxxxxx | 12 | 7 | 19 |
| 178~689 | 1111111111110 | xxxxxxxxx | 13 | 9 | 22 |
| 690~2737 | 11111111111110 | xxxxxxxxxxx | 14 | 11 | 25 |
| 2738~10929 | 111111111111110 | xxxxxxxxxxxxx | 15 | 13 | 28 |
| 10930~43697 | 1111111111111110 | xxxxxxxxxxxxxxx | 16 | 15 | 31 |

## U-FLC135-L30

U denotes unary coding, FLC135 denotes that the suffix length increments as 1, 3, 5, 7, 9, …. . Table 8 shows the U-FLC135-L30 VLC for cParam = 0. The maximum codeword length is 29 bits. However, cParam = 1 leads to worst case codeword length and the maximum codeword length for U-FLC135-L30 is 30 bits.

Table 8: U-FLC135-L30 VLC for cParam = 0.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5 | 111110 |  | 6 | 0 | 6 |
| 6~7 | 1111110 | x | 7 | 1 | 8 |
| 8~15 | 11111110 | xxx | 8 | 3 | 11 |
| 16~47 | 111111110 | xxxxx | 9 | 5 | 14 |
| 48~175 | 1111111110 | xxxxxxx | 10 | 7 | 17 |
| 176~687 | 11111111110 | xxxxxxxxx | 11 | 9 | 20 |
| 688~2735 | 111111111110 | xxxxxxxxxxx | 12 | 11 | 23 |
| 2736~10927 | 1111111111110 | xxxxxxxxxxxxx | 13 | 13 | 26 |
| 10928~43695 | 11111111111110 | xxxxxxxxxxxxxxx | 14 | 15 | 29 |

## TU-FLC135-L29

TU denotes that prefix is coded using truncated unary, FLC135 denotes that the suffix length increments as 1, 3, 5, 7, 9, ….  Table 9 shows the TU-FLC135-L29 VLC for cParam = 0. The maximum codeword length is 28 bits. However, cParam = 1 leads to worst case codeword length and the maximum codeword length for TU-FLC135-L29 is 29 bits.

Table 9: TU-FLC135-L29 VLC for cParam = 0.

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 1 | 0 | 1 |
| 1 | 10 |  | 2 | 0 | 2 |
| 2 | 110 |  | 3 | 0 | 3 |
| 3 | 1110 |  | 4 | 0 | 4 |
| 4 | 11110 |  | 5 | 0 | 5 |
| 5 | 111110 |  | 6 | 0 | 6 |
| 6~7 | 1111110 | x | 7 | 1 | 8 |
| 8~15 | 11111110 | xxx | 8 | 3 | 11 |
| 16~47 | 111111110 | xxxxx | 9 | 5 | 14 |
| 48~175 | 1111111110 | xxxxxxx | 10 | 7 | 17 |
| 176~687 | 11111111110 | xxxxxxxxx | 11 | 9 | 20 |
| 688~2735 | 111111111110 | xxxxxxxxxxx | 12 | 11 | 23 |
| 2736~10927 | 1111111111110 | xxxxxxxxxxxxx | 13 | 13 | 26 |
| 10928~43695 | 1111111111111 | xxxxxxxxxxxxxxx | 13 | 15 | 28 |

# BD-Rate results

## Normal QP (QP = 22, 27, 32, 37)

|  | Normal QP (QP = 22, 27, 32, 37) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | U-FLC123-L32 |  | TU-FLC123-L32 |  | U-FLC135-L32 |  | U-FLC135-L30 |  | TU-FLC135-L29 |  | U-FLC123-L37 + Escape |
| Maximum codeword length | 32 bits |  | 32 bits |  | 32 bits |  | 30 bits |  | 29 bits |  | 32 bits |
| AI-Main | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RA-Main | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| LB-Main | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| LP-Main | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| AI-HE10 | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RA-HE10 | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| LB-HE10 | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| LP-HE10 | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Class F | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |

## Low QP (QP = 1, 5, 9, 13)

|  | Low QP (QP = 1, 5, 9, 13) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | U-FLC123-L32 | TU-FLC123-L32 | U-FLC135-L32 | U-FLC135-L30 | TU-FLC135-L29 | U-FLC123-L37 + Escape |
| Maximum codeword length | 32 bits | 32 bits | 32 bits | 30 bits | 29 bits | 32 bits |
| AI-Main | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| RA-Main | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LB-Main | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LP-Main | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| AI-HE10 | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| RA-HE10 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LB-HE10 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LP-HE10 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.8% | -0.6% | -0.1% | -0.4% | -0.4% | 0.1% |

# Complexity considerations for FLC123 and FLC135 codes

This section compares complexity of FLC123 (traditional EG codes) and FLC135 VLCs (modified EG codes). The HM-7.0 VLC (U-FLC123-L37) is used as an example of FLC123 VLC and U-FLC135-L30 is used as an example of FLC135 VLC.

The HM-7.0 VLC and other VLCs proposed in this contribution can be implemented in two ways. One is using compute logic (as in HM-7.0) and the other is using table look up. In the compute logic based approach, decoding happens in following three steps e.g. for U-FLC123-L37 (code extracts are from HM-7.0 software):

1. Decode prefix

UInt prefix   = 0;

UInt codeWord = 0;

do

{

prefix++;

m_pcTDecBinIf->decodeBinEP( codeWord );

}

while( codeWord);

codeWord  = 1 - codeWord;

prefix -= codeWord;

2. Check special case of codewords without suffix. Decode codewords without suffix.

codeWord=0;

if (prefix < 8 )

{

m_pcTDecBinIf->decodeBinsEP(codeWord,rParam);

rSymbol = (prefix<<rParam) + codeWord;

}

3. Decode codewords with suffix.

else

{

m_pcTDecBinIf->decodeBinsEP(codeWord,prefix-8+rParam);

rSymbol = (((1<<(prefix-8))+8-1)<<rParam)+codeWord;

}

In the table lookup based approach steps 2 and 3 can be combined into a single step without special checking. So the table look up approach is attractive in hardware. The C code for table lookup based implementation of HM-7.0 VLC (U-FLC123-L37) is provided below.

const Int TABLE_LEN = 23;

Int levelGroupStart[TABLE_LEN] = {0,1,2,3,4,5,6,7,8,9,11,15,23,39,71,135,263,519,1031,2055,4103,8199,16391};

Int suffixLenTable[TABLE_LEN] = {0,0,0,0,0,0,0,0,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14};

1. Decode prefix

UInt prefix   = 0;

UInt codeWord = 0;

do

{

prefix++;

m_pcTDecBinIf->decodeBinEP( codeWord );

}

while( codeWord);

codeWord  = 1 - codeWord;

prefix -= codeWord;

codeWord=0;

2. Decode codewords.

m_pcTDecBinIf->decodeBinsEP(codeWord, suffixLenTable[prefix]+rParam);

rSymbol = (levelGroupStart[prefix]<<rParam)+codeWord;

Compute logic based approach on the other hand could be attractive for software based implementation.

Both the approaches are analyzed in more detail in following subsections.

## Compute logic based implementation of U-FLC123-L37 and U-FLC135-L30

cParam = 0 is assumed in the analysis.

For U-FLC123-L37 (HM-7.0), the suffix length is given by (prefix_length-9). The input level values can be divided into several groups for the part of VLC that has suffix. The level groups are: 9~10, 11~14, 15~22, 23~38, 39~70, 71~134, 135~262, 263~518, 519~1030, 1031~2054, 2055~4102, 4103~8198, 8199~16390, 16391~32774 (See Table 2). The level group start values are: 9, 11, 15, 23, 39 ….  Table below shows the level group start values minus 8 and their binary representation which is all 1’s (2^M-1 where M = prefix_length-9). Hence both the suffix length and level group start values can be efficiently computed for U-FLC123-L37 once prefix_length is obtained. The decoding process involves adding suffix to level group start value, hence the decoding of U-FLC123-L37 is efficient.

| levelGroupStart-8 | Binary representation |
| --- | --- |
| 1 | 1 |
| 3 | 11 |
| 7 | 111 |
| 15 | 1111 |
| 31 | 11111 |
| 63 | 111111 |
| 127 | 1111111 |
| 255 | 11111111 |
| 511 | 111111111 |
| 1023 | 1111111111 |
| 2047 | 11111111111 |
| 4095 | 111111111111 |
| 8191 | 1111111111111 |
| 16383 | 11111111111111 |

For U-FLC135-L30, the suffix length is 1, 3, 5, 7, .. which is (2*prefix_length – 13). The level groups are: 6~7, 8~15, 16~47, 48~175, 176~687, 688~2735, 2736~10927, 10928~43695 (See Table 8).The level group start values are: 6, 8, 16, 48, 176, 688, 2736, 10928. Table below shows the level group start values minus 6 and their binary representation which is bit interleaved version of all 1’s with zero (i.e. bit interleaving of 2^M-1 where M = prefix_length-7 and zero bits). Bit interleaving can be efficiently implemented in both hardware and in software (most of the CPUs support instructions for bit interleaving). Hence both the suffix length and level group start values can be efficiently computed for U-FLC135-L30 once prefix_length is obtained. The decoding process involves adding suffix to level group start value, hence the decoding of U-FLC135-L30 is efficient.

| levelGroupStart-6 | Binary representation of levelGroupStart-6 | 2^(prefix_length-7) - 1 | Binary representation of 2^(prefix_length-7) - 1 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 2 | 10 | 1 | 1 |
| 10 | 1010 | 3 | 11 |
| 42 | 101010 | 7 | 111 |
| 170 | 10101010 | 15 | 1111 |
| 682 | 1010101010 | 31 | 11111 |
| 2730 | 101010101010 | 63 | 111111 |
| 10922 | 10101010101010 | 127 | 1111111 |

## Table lookup based implementation of U-FLC123-L37 and U-FLC135-L30

Table lookup based implementation of U-FLC123-L37 was discussed earlier in this section. The table lookup based implementation of U-FLC135-L30 is provided below. The code is identical to the U-FLC123-L37 but for the different level group start table and suffix length tables.

const Int TABLE_LEN = 13;

Int levelGroupStart[TABLE_LEN] = {0,1,2,3,4,5,6,8,16,48,176,688,2736,10928};

Int suffixLenTable[TABLE_LEN] = {0,0,0,0,0,0,1,3,5,7,9,11,13,15};

1. Decode prefix

UInt prefix   = 0;

UInt codeWord = 0;

do

{

prefix++;

m_pcTDecBinIf->decodeBinEP( codeWord );

}

while( codeWord);

codeWord  = 1 - codeWord;

prefix -= codeWord;

codeWord=0;

2. Decode codewords.

m_pcTDecBinIf->decodeBinsEP(codeWord, suffixLenTable[prefix]+rParam);

rSymbol = (levelGroupStart[prefix]<<rParam)+codeWord;

The length of the tables are smaller hence table based implementation of U-FLC135-L30 is more efficient than U-FLC123-L37.

# Conclusions

Several VLCs were studied in this contribution. The VLC with the smallest maximum codeword length in this contribution is TU-FLC135-L29 with maximum codeword length of 29 bits. This VLC is asserted to be the best choice from complexity reduction and BD-Rate degradation point of view. Among the VLCs with maximum codeword length equal to 32 bits, TU-FLC123-L32 is asserted to be the best choice in term of complexity, closeness to existing VLC and BD-Rate degradation. The average BD-Rate degradation of both of these VLCs is 0.0% for normal and low QP Class A-E. For Class F sequences, there is BD-Rate savings of -0.6% and -0.4% respectively for low QP value and 0.0% for normal QP range.

- Draft text modifications

Draft 7 text does not currently contain entropy coding adoptions from last meeting. So this section will be updated after Draft 7 text is updated.

Text modifications are highlighted in yellow below.

## Proposal 1 (U-FLC123-L32)

#### 9.2.2.8 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], and baseLevel.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows.

- If n is equal to 15, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is less than 15 ), cLastSE is set equal to baseLevel + coeff_abs_level_remaining[ n − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >= ( 3 * ( 1 << cLastRiceParam ) ) ) ? 1 : 0, 4 )	(9-1)

The variable cTRMax is derived from cRiceParam as:

cTRMax = 9 4 << cRiceParam	(9-2)

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause Error: Reference source not found for the syntax element coeff_abs_level_remaining[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived using the EGk binarization as specified in subclause Error: Reference source not found with the Exp-Golomb order k set equal to cRiceParam + 1.

## TU-FLC123-L32

#### 9.2.2.8  Binarization process for coeff_abs_level_remaining

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the truncated unary binarization process as specified in subclause Error: Reference source not found with cMax = 18 and input synElVal derived as follows:

- cLength is set equal to cParam and cVal is set equal to coeff_abs_level_remaining[ n ].

- If coeff_abs_level_remaining[ n ] is less than (4<<cParam), synElVal is set equal to (coeff_abs_level_remaining[ n ]>> cParam)

- Otherwise, the value of synElVal is specified by a process equivalent to the following pseudo-code:

cVal = cVal - (4<<cParam )

while (cVal >= (1<<cLength))

{

cVal =  cVal - (1<<cLength)

cLength = cLength+1

}

synElVal = 4 + cLength – cParam

The suffix bin string is derived by invoking the FL binarization process as specified in subclause Error: Reference source not found with cMax equal to (1<<cLength)-1 and input equal to cVal % (1<<cLength). The value of cLength and cVal is specified by the process equivalent to the above pseudo-code.

## TU-FLC135-L30

#### 9.2.2.8  Binarization process for coeff_abs_level_remaining

The binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the truncated unary binarization process as specified in subclause Error: Reference source not found with input synElVal derived as follows:

- cLength is set equal to 0 and cVal is set equal to coeff_abs_level_remaining[ n ], array  levelGroupStart[15] is set equal to {0,1,2,3,4,5,6,8,16,48,176,688,2736,10928,65536}, array suffixLenTable[15] is set equal to {0,0,0,0,0,0,1,3,5,7,9,11,13,15, 65536}.

- The value of synElVal is specified by a process equivalent to the following pseudo-code:

while (cVal >= (levelGroupStart[cLength]<<cParam))

{

cLength = cLength + 1

}

cLength = cLength – 1;

cVal = cVal - (levelGroupStart[cLength] << cParam)

synElVal = cLength + 1

The suffix bin string is derived by invoking the FL binarization process as specified in subclause Error: Reference source not found with cMax equal to suffixLenTable[cLength]+cParam and input equal to cVal. The value of cLength, suffixLenTable, and cVal is specified by the process equivalent to the above pseudo-code.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).