| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0145 M25468 |
| --- | --- |

| Title: | Simplification on spatial AMVP candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Jianhua Zheng No. 156, Rd. Beiqing Haidian District, Beijing China | Tel: Email: | +86 10 60612372 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This document proposes to simplify the derivation process of spatial AMVP candidates. Two spatial candidates are derived by checking non-scaled and scaled MV candidate based on left and above neighboring positions in HM7. It is proposed that two steps involved in the scaled MV candidate derivation are combined and only 3 neighboring positions are checked for the scaled MV candidate. It is asserted that the non-scaled MV candidate is always checked before the scaled one. The specification text is largely simplified. The test results show coding efficiency of RA-main: 0.0%, -0.1%, 0.0%, RA-HE10: 0.0%, 0.0%, 0.0%, LB-main: -0.1%, 0.0%, 0.0%, LB-HE10: 0.0%, -0.2%, -0.1% under common test conditions.

# Introduction

Derivation process of two spatial AMVP candidates in HM7 can be described as follows:[1]

- Checking non-scaled MV candidate A based on neighbor positions of A0, A1. When the non-scaled MV candidate A is available, add it into AMVP candidate list.

- If the non-scaled MV candidate A is not available, checking scaled MV candidate A based on positions of A0, A1. When the scaled MV candidate A is available, add it into the AMVP candidate list.

- Checking non-scaled MV candidate B based on neighbor positions of B0, B1, B2. When the non-scaled MV candidate B is available, add it into the AMVP candidate list.

- If candidate A is not available and candidate B is available, candidate B replaces candidate A.

- If candidate A is not available, checking scaled MV candidate B based on positions of B0, B1, B2. When the scaled MV candidate B is available, add it into the AMVP candidate list.

Fig. 1. Neighbor positions for spatial AMVP derivation in HM7

These neighbouring positions, i.e., A0, A1, B0, B1, B2, are shown in Figure 1. All the 5 positions may be checked to derive non-scaled and scaled MV candidates, thus total checking number reaches up to 10 in the worst case. It is also noted that there are two steps involved in the scaled MV candidate derivation based on left and above neighboring positions (i.e. step 2 and step 5).

In this document, the two steps involved in the scaled MV candidate derivation are combined as the last step. The combined derivation process needs to check 3 neighbouring positions (A1, B1, B2), instead of all 5 neighbouring positions. It is noted that the non-scaled MV candidate is always checked before the scaled one.

In addition, the checking condition is also modified. The proposed derivation process becomes

- Checking non-scaled MV candidate A based on neighbor positions of A0, A1. When the non-scaled MV candidate A is available, add it into AMVP candidate list.

- Checking non-scaled MV candidate B based on neighbor positions of B0, B1, B2. When the non-scaled MV candidate B is available, add it into the AMVP candidate list.

- If candidate B is not available, checking scaled MV candidate B based on positions of A0, B1, B2. When the scaled MV candidate B is available, add it into the AMVP candidate list.

# Experimental results

The proposed method is integrated into HM7.0 and tested under common test conditions specified in [2]. Test results are summarized in Table 1. It shows coding efficiency of

RA-main: 0.0%, -0.1%, 0.0%, RA-HE10: 0.0%, 0.0%, 0.0%,

LB-main: -0.1%, 0.0%, 0.0%, LB-HE10: 0.0%, -0.2%, -0.1%,

No coding efficiency loss is observed.

Table 1. Experimental results of the proposed AMVP derivation comparing to HM7.0

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.1% | 0.1% | 0.0% | 0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class D | -0.1% | 0.1% | 0.5% | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | -0.2% | -0.5% | 0.1% | -0.3% | -0.2% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | 0.4% | -0.1% | 0.0% | 0.4% | 0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |

# Conclusion and recommendation

The proposed method simplifies the derivation process of spatial AMVP candidates. The two steps involved in the scaled MV candidate derivation are combined as the last step, and the neighboring positions to be checked are reduced from 5 to 3. It is asserted that the non-scaled MV candidate is always checked before the scaled one. The specification text is largely simplified. Experimental results show no coding efficiency loss under common test condition. It is recommended to adopt the proposed simplification.

# References

- B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7,” JCT-VC Document, JCTVC-I1003, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, Geneva, Switzerland, April. 2012

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text changes

#### 8.5.2.1.6 Derivation process for motion vector predictor candidates

…

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- The sample location (xA0, yA0) is set equal to (xP − 1, yP + nPSH) and the sample location (xA1, yA1) is set equal to (xA0, yA0 − MinPuSize).

- The availability flag availableFlagLXA is set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ), the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction block covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to ListX.

- Otherwise, if the prediction block covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xAk ][ yAk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to ListY and mvLXA is set equal to mvLXA.

-

-

-

-

-

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line. [Ed. (BB): Define MinPuSize in the SPS but the derivation should depend on the use of an AMP flag ]

- When yP−1 is less than (( yC >> Log2CtbSize ) << Log2CtbSize), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-6)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-7)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-8)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction block covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to ListX.

- Otherwise, if the prediction block covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xBk ][ yBk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to ListY.

- When availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 =xA1, yB0 = yA1, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction block covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to ListX.

- Otherwise, if the prediction block covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to ListY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListB[ refIdxB ] ) is not equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and both RefPicListB[ refIdxB ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-9)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-10)

mvLXB =Clip3( −32768, 32767, Sign2( DistScaleFactor * mvLXB ) * 
		( (Abs( DistScaleFactor * mvLXB ) + 127 ) >> 8 ) )	(8-11)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListB[ refIdxB ] ) )	(8-12)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8-13)