| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0146 M25469 |
| --- | --- |

| Title: | Cross-check report of JCTVC-J0160 on LM parameter estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin No. 156, Rd. Beiqing Haidian District, Beijing China | Tel: Email: | +86 10 60612372 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Introduction

This document reports cross-check results of JCTVC-I0160 on parameter estimation of LM mode. It is proposed to calculate the parameters of LM mode at CU level instead of TU-level and apply them to every TU within the CU. As a result the number of parameter calculation can be reduced from 64 to 1 in the worst case where CU size is 64x64 and TU size is 4x4 [1].

# Test results

We got source code from proponent and independently compiled the code with VS2005, and ran in environment of Windows server 2003 R2 64bits. The common test conditions and software reference configurations are followed in our simulations [2]. HM7.0 is used as anchor. The test results are summarized in Table 1. It shows coding efficiency of 0.0%, -0.7%, -0.7% in AI-HE10, 0.0%, -0.7%, -0.6% in RA-HE10 and 0.0%, 0.1%, -0.3% in LB-HE10. It is verified that all bit-rate and PSNR values match exactly with those reported by proponent.

Table 1. Test results of the proposed method comparing to HM7.0

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -2.3% | -2.4% |
| Class B | 0.0% | -0.6% | -0.4% |
| Class C | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | -0.1% | -0.2% |
| Class E | 0.0% | -0.3% | -0.2% |
| Overall | 0.0% | -0.7% | -0.7% |
|  | 0.0% | -0.7% | -0.7% |
| Class F | 0.0% | -0.4% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | -2.2% | -1.7% |
| Class B | 0.0% | -0.5% | -0.3% |
| Class C | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | -0.7% | -0.6% |
|  | 0.0% | -0.7% | -0.6% |
| Class F | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.6% | 0.1% |
| Class E | 0.0% | -0.6% | -1.3% |
| Overall | 0.0% | 0.1% | -0.3% |
|  | 0.0% | 0.1% | -0.3% |
| Class F | 0.1% | 0.7% | 0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

# Recommendation

The test results reported in JCTVC-I0160 is confirmed.

# References

- Jungsun Kim, Byeongmoon Jeon, “Parameter derivation for intra LM chroma prediction,” JCT-VC Document, JCTVC-J0160, Stockholm, SE, July, 2012

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, Geneva, CH, April. 2012