| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0147 |
| --- | --- |

| Title: | Subjective evaluation on ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Takiue TK Tan A. Fujibayashi Y. Suzuki 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81-468-40-6833 takiue@nttdocomo.co.jp fujibayashi@nttdocomo.co.jp suzukiyos@rd.nttdocomo.co.jp +65 97993861 tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc |  |  |

_____________________________

# Abstract

This contribution reports non-experts viewing results on ALF. The subjective quality of HM7.0 was compared with that of HM7.0 enabling ALF (ALF on) in the same manner as JCTVC-I0585. The results of this contribution show HM7.0 achieved the better picture quality than HM7.0 ALF on at six test points out of forty test points, while HM7.0 ALF on is better at five test points.

# Introduction

This contribution reports subjective test results on Adaptive Loop Filer (ALF). Several subjective tests have been already conducted on ALF.

NTT DOCOMO, Inc. conducted the formal subjective test on HEVC and JCTVC-H0116 [1] reported that HM5.0 HE (with ALF) and HM5.0 HE without ALF + some tools at the same birate have the same subjective quality for the random access coding structure.

During the 9th JCTVC meeting, the expert subjective test to compare HM6.0 with ALF to HM6.0 without ALF was conducted [2]. JCTVC-I0585 reported that six test points out of forty test points showed better subjective picture quality of ALF than the HM anchor, while one test point out of forty test points showed worse subjective picture quality of ALF than the HM anchor.

In this contribution, NTT DOCOMO, Inc. conducted the non-expert subjective test to compare HM7.0 with ALF to HM7.0 without ALF in the same manner as JCTVC-I0585 [2] was done.

# Results of subjective test on ALF

The non-expert viewing was conducted based on the subjective test method used in [2]. The two coding methods were tested.

1) HM7.0 Main (anchor)

2) HM7.0 Main, ALF on (tested)

Both random access and low delay B coding structures defined in JCTVC-I1100 [3] were used for two coding methods. HM7.0 Main and HM7.0 Main ALF on bitstreams are coded at QP = 32 and 37 for test sequences shown in Table 1. Please refer to Annex A for the detailed description of this subjective test method.

For the 40 test cases, the mean opinion score (MOS) and confident interval (CI) of 22 non-expert viewers are shown in Figure 1. In this figure, positive value means ALF improves visual quality. As reported in JCTVC-I0585 [2], it is evaluated that two coding methods are comparable in subjective sense when CI is cross the value of 0. And if CI is not cross the value of 0, it is evaluated that one coding method is better than another. Figure 1 shows the following trends:

• Five test points (red circle) out of forty test points showed better subjective picture quality of HM7.0 Main ALF on than HM7.0 Main.

• Six test point (brown circle) out of forty test points showed worse subjective picture quality of HM7.0 Main ALF on than HM7.0 Main.

Figure 1: Non-expert viewing results: Sequences with red circle showed that ALF is better than the anchor. Sequences with brown circle showed that ALF is worse than the anchor.

Table 1: Test sequences

| # of test points | Test sequences for LDB condition | # of test points | Test sequences for RA codidition |
| --- | --- | --- | --- |
| 1 or 21 | KristenAndSara (1280x720) | 11 or 31 | Traffic (2560x1600) |
| 2 or 22 | Johnny (1280x720) | 12 or 32 | PeopleOnStreet (2560x1600) |
| 3 or 23 | FourPeople (1280x720) | 13 or 33 | NebutaFestival (2560x1600) |
| 4 or 23 | SpinCalendar (1280x720) | 14 or 34 | SteamLocomotive (2560x1600) |
| 5 or 25 | RiverBed (1920x1080) | 15 or 35 | RiverBed (1920x1080) |
| 6 or 26 | Kimono (1920x1080) | 16 or 36 | Kimono (1920x1080) |
| 7 or 27 | ParkScene (1920x1080) | 17 or 37 | ParkScene (1920x1080) |
| 8 or 28 | Cactus (1920x1080) | 18 or 38 | Cactus (1920x1080) |
| 9 or 29 | BasketballDrive (1920x1080) | 19 or 39 | BasketballDrive (1920x1080), |
| 10 or30 | BQTerrace (1920x1080) | 20 or 40 | BQTerrace (1920x1080) |

The following table shows bitrate of each bistream. Nine test points exceeded two percent tolerance of bitrate. Two test points (1 and 3) at which HM7.0 Main ALF on is better than HM7.0 Main are included in these test points.

Table 2: Bitrate of two methods and its ratio [kbps]

| HM7.0 Main (ALF off) vs. HM7.0 Main ALF on |  | QP=37 |  |  | QP=32 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ALF off | ALF on | ratio | ALF off | ALF on | ratio |
| LDB | KristenAndSara_1280x720_60 | 164.0 | 167.9 | 1.024 | 315.7 | 324.8 | 1.029 |
|  | Johnny_1280x720_60 | 98.9 | 102.0 | 1.032 | 186.9 | 193.8 | 1.037 |
|  | FourPeople_1280x720_60 | 222.8 | 230.0 | 1.032 | 425.7 | 435.8 | 1.024 |
|  | Spincalendar_1280x720_60fps | 374.4 | 382.1 | 1.020 | 804.3 | 807.5 | 1.004 |
|  | riverbed_1920x1080_25fps | 2855.3 | 2870.9 | 1.005 | 5859.5 | 5889.1 | 1.005 |
|  | Kimono1_1920x1080_24 | 562.3 | 574.3 | 1.021 | 1149.3 | 1169.5 | 1.018 |
|  | ParkScene_1920x1080_24 | 575.6 | 585.9 | 1.018 | 1330.1 | 1348.2 | 1.014 |
|  | cactus_1920x1080_50 | 1267.5 | 1289.1 | 1.017 | 2566.7 | 2600.5 | 1.013 |
|  | BasketballDrive_1920x1080_50 | 1581.4 | 1606.9 | 1.016 | 3112.2 | 3130.2 | 1.006 |
|  | BQTerrace_1920x1080_60 | 764.0 | 771.7 | 1.010 | 1979.0 | 1959.7 | 0.990 |
| RA | Traffic_2560x1600_30 | 1320.8 | 1339.3 | 1.014 | 2545.4 | 2569.1 | 1.009 |
|  | PeopleOnStreet_2560x1600_30 | 4622.8 | 4661.9 | 1.008 | 8255.5 | 8291.3 | 1.004 |
|  | NebutaFestival_2560x1600_60 | 7130.9 | 7179.3 | 1.007 | 29195.2 | 29141.6 | 0.998 |
|  | SteamLocomotiveTrain_2560x1600_60 | 1178.8 | 1205.0 | 1.022 | 2438.9 | 2481.0 | 1.017 |
|  | riverbed_1920x1080_25fps | 2356.4 | 2374.0 | 1.007 | 4774.8 | 4809.5 | 1.007 |
|  | Kimono1_1920x1080_24 | 542.2 | 553.3 | 1.021 | 1069.7 | 1087.0 | 1.016 |
|  | ParkScene_1920x1080_24 | 718.2 | 727.8 | 1.013 | 1539.2 | 1552.8 | 1.009 |
|  | cactus_1920x1080_50 | 1371.0 | 1386.5 | 1.011 | 2672.6 | 2699.1 | 1.010 |
|  | BasketballDrive_1920x1080_50 | 1474.5 | 1503.8 | 1.020 | 2818.4 | 2858.3 | 1.014 |
|  | BQTerrace_1920x1080_60 | 971.5 | 983.7 | 1.013 | 2248.6 | 2254.8 | 1.003 |

# Conclusions

The non-expert viewing on ALF was conducted based on the subjective test method used in JCTVC-I0585 [2]. Non-experts evaluated that ALF can improve the visual quality at five test points out of forty test points (two test points exceeded two percent tolerance of bitrate). But they also evaluated that ALF causes the subjective loss at six test points.

These results indicate that there is no clear benefit from ALF in the subjective sense of non-experts.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- TK. Tan, A. Fujibayashi, Y. Suzuki, J. Takiue, “AHG8: Objective and subjective evaluation of HM5.0”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0116, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- T. Yamagake, “Report on subjective viewing of ALF”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0585, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

- F. Bossen., “Common HM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, 9th Meeting: Geneva, CH, 27 April - 7 May, 2012.

# Annex A: Description of subjective test method.

This informal subjective test is based on the test method used in JCTVC-I0585.

For this test, following condition was used.

- Monitors for 2k sequences:

- EIZO FlexScan SX3031 (30 inch LCD, resolution: 2560x1600) and

- Viewing condition: Viewing distance was set to 2H

- Monitors for HD sequences:

- NEC LCD5220 (52 inch, 1920x1080) and

- Viewing condition: Viewing distance was set to 2H.

- Test method:

- The unit consisting of a pair decoded sequences from 2 of the codecs tested was presented to the observer as sequence A followed by sequence B

- The observer are asked to vote one of the following

  - -2: A is worse than B

  - -1: A is slightly worse than B

  - 0: Equal

  - 1: A is slightly better

  - 2: A is better

- Viewing order of A and B is randomized

  - Three randomized patterns are prepared.

  - One pattern of them is used for 7 or 8 viewers.