| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11– 20 July 2012 | Document: JCTVC-J0148 |
| --- | --- |

| Title: | AHG5/AHG6: Bypass coding for SAO syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Matsunobu Kengo Terada Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 { matsunobu.toru, terada.kengo, sasai.hisao, nishi.takahiro } @jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a modified design of CABAC is proposed to reduce the number of context models and bins for SAO syntax elements. In this proposal, sao_lcu_enable_flag is added for decoupling the SAO LCU on/off state from sao_type_idx. sao_offset_abs and the sao_type_idx are binarized using bypass models instead of using context models. sao_lcu_enable_flag is signalled at the top of bins for SAO. Average BD-rate loss by this proposal relative to HM7.0 anchors was reportedly 0.03% for Y, 0.10% for U and 0.15% for V component respectively. This proposal can reduce the worst case of the number of bins per sample for SAO, which are coded using context models, from 1.50 to 0.01.

# Introduction

SAO syntax elements in CD are shown in the Table 1. It requires a total of 8 context models. The number of context bins per sample in the worst case is 1.50 for sao_offset_abs and the second worst case is 0.07 for sao_type_idx. The order of the bins for SAO in HM7.0 is shown in Fig. 1. In the case of band offset ( sao_type_idx is equal to 5 ), context bins and bypass bins are ordered alternately. Because bypass bins do not require context fetching and adaptation, many bypass bins can be coded in a single cycle for higher throughput.Error: Reference source not foundError: Reference source not found Grouping the bypass bins together improves the throughput. In this contribution, we propose to modify a design of CABAC in order to reduce the number of context bins and group the bypass bins together.

Table 1. SAO syntax elements in CD

| SAO syntax element | Type of coding | The number of context models | Context bins per sample in the worst case |
| --- | --- | --- | --- |
| sao_merge_left_flag | Context | 3 | 0.01 |
| sao_merge_up_flag | Context | 1 | 0.01 |
| sao_type_idx | Context | 2 | 0.07 |
| sao_offset_sign | Bypass | 0 | 0 |
| sao_offset_abs | Context | 2 | 0.15 |
| sao_band_position | Bypass | 0 | 0 |

b1: sao_merge_left_flag ( coded by context )
b2: sao_merge_up_flag ( coded by context )
b3: sao_typle_idx ( coded by context )
SBP: sao_band_position ( coded by bypass )
b4: sao_offset_abs ( coded by contex )
SOS: sao_offset_sign ( coded by bypass )

Figure 1. Bins order for SAO in HM7.0

# Proposed solutions

Solution 1.

Many context bins are generated by the binarization of sao_type_idx and sao_offset_abs. In this solution, these two syntax elements are binarized using bypass models instead of context models. However, sao_lcu_enable_flag is added for decoupling SAO LCU on/off state from sao_type_idx. Because probability of SAO LCU on/off state is biased, sao_lcu_enable_flag is binarized using a context model. On the other band, sao_type_idx is binarized using bypass models instead of context models, and unary binariztion for sao_type_idx is replaced with truncated unary binarization. Table 2 and Table 3 show the bin string of sao_lcu_enable_flag and sao_type_idx in CD and this proposal respectively.

Additionally, this solution has three modifications as followings.

- sao_lcu_enable_flag is moved up to the top of bins for SAO in order to avoid signalling merge flags in the case of SAO off.

- Two context models for sao_merge_left_flag are reduced by sharing one context among all color components in order to reduce of the number of context models.

- sao_band_position is moved to the behind of sao_offset_sign in order to reduce of the number of branches on condition.

Solution 2.
In the solution 1, information about SAO LCU on/off state is collected in sao_lcu_enable_flag from sao_merge_left_flag, sao_merge_up_flag and sao_type_idx. The bias of probability of sao_merge_left_flag and sao_merge_up_flag becomes lower. In this solution, sao_merge_left_flag and sao_merge_up_flag is also binarized using bypass models instead of context models.

SAO syntax elements in proposed solutions are shown in the Table 4. The order of bins is shown in Figure 2. These proposals can reduce the number of context models and group bypass bins together.

Table 2. Bin string of sao_type_idx in CD

| Syntax element |  | Bin string |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| sao_type_idx | BinIdxCtx | 0 | 1 | 1 | 1 | 1 | 1 |
|  | off | 0 |  |  |  |  |  |
|  | EO(0) | 1 | 0 |  |  |  |  |
|  | EO(1) | 1 | 1 | 0 |  |  |  |
|  | EO(2) | 1 | 1 | 1 | 0 |  |  |
|  | EO(3) | 1 | 1 | 1 | 1 | 0 |  |
|  | BO | 1 | 1 | 1 | 1 | 1 | 0 |

Table 3. Bin string of sao_lcu_enable_flag and sao_type_idx in proposal

| Syntax element |  | Bin string |  |  |  |
| --- | --- | --- | --- | --- | --- |
| sao_lcu_enable_flag | binIdxCtx | 0 |  |  |  |
|  | Off | 0 |  |  |  |
|  | On | 1 |  |  |  |
| sao_type_idx | binIdxCtx | na | na | na | na |
|  | EO(0) | 0 |  |  |  |
|  | EO(1) | 1 | 0 |  |  |
|  | EO(2) | 1 | 1 | 0 |  |
|  | EO(3) | 1 | 1 | 1 | 0 |
|  | BO | 1 | 1 | 1 | 1 |

Table 4. Type of coding for SAO syntax elements

| SAO syntax element | CD | Solution 1 | Solution 2 |
| --- | --- | --- | --- |
| sao_lcu_enable_flag | - | Context | Context |
| sao_merge_left_flag | Context | Context | Bypass |
| sao_merge_up_flag | Context | Context | Bypass |
| sao_type_idx | Context | Bypass | Bypass |
| sao_offset_sign | Bypass | Bypass | Bypass |
| sao_offset_abs | Context | Bypass | Bypass |
| sao_band_position | Bypass | Bypass | Bypass |

b0: sao_lcu_enable_flag ( coded by context ) 
b1: sao_merge_left_flag (coded by context)
SML: sao_merge_left_flag ( coded by bypass ) 
b2: sao_merge_up_flag (coded by context )
SMU: sao_merge_up_flag ( coded by bypass )
b3: sao_typle_idx (coded by context )
STI: sao_typle_idx ( coded by bypass )
SBP: sao_band_position ( coded by bypass )
b4: sao_offset_abs (coded by contex )
SOA: sao_offset_abs ( coded by bypass )
SOS: sao_offset_sign ( coded by bypass )

Figure 2. Bins order for SAO syntax elements

Table 5. Modifications of SAO syntax in proposal

# Simulation & results

BD-rate performance

The proposed method performance relative to the HM7.0 software was checked under the common test condition described in JCTVC-I1100[4]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. Table 6 shows summary of test results of proposed solutions. And Table 10 and Table 11 in Section 7 show the BD-rate of solution 1 and solution 2 respectively.

Analysis of the number of context coded bins

Table 7 shows the ratio of context bins for SAO syntax elements based on HM7.0. Table 8 shows the number of context bins for SAO syntax elements in total context bins in each test.Analysis of isolated by-pass coded bins

Grouping the bypass bins together improves the throughput. Even if the number of context bins is reduced, it is required to avoid increasing isolated bypass bins. Table 9 shows the ratio of isolated bypass bins for SAO relative to HM7.0. In the solution 1, There is no isolated bypass bins. In the solution 2, the number of  isolated bypass bins increase considerably relative to HM7.0. Because, if sao_merge_left_flag is equal to 1, parsing process for SAO generates a context bin for sao_lcu_enable_flag and a bypass bin for sao_merge_left_flag as shown in Figure 3.

Table 6. Summary of proposed SAO syntax coding.

| test | The number of context models | Context bins per sample in the worst case | BD-rate for Y | BD-rate for U | BD-rate for V |
| --- | --- | --- | --- | --- | --- |
| Solution 1 | 8→3 | 1.50→0.01 | 0.03% | 0.10% | 0.15% |
| Solution 2 | 8→1 | 1.50→0.01 | 0.03% | 0.23% | 0.27% |

Table 7. The ratio of context bins relative to HM7.0

| test | Context bins for SAO |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | sao lcu enable flag | sao merge left flag | sao merge up flag | sao type idx | sao offset abs | Total |
| HM7.0 | - | 100% | 100% | 100% | 100% | 100% |
| Solution 1 | DIV/0 | 20.67% | 15.75% | 0.00% | 0.00% | 38.07% |
| Solution 2 | DIV/0 | 0.00% | 0.00% | 0.00% | 0.00% | 29.93% |

Table 8. The number of context bins for SAO in total context bins

| test | Context bins for SAO |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | sao lcu enable flag | sao merge left flag | sao merge up flag | sao type idx | sao offset abs | Total |
| HM7.0 | - | 0.84% | 0.25% | 0.19% | 0.31% | 2.72% |
| Solution 1 | 0.87% | 0.08% | 0.05% | 0.00% | 0.00% | 1.00% |
| Solution 2 | 0.86% | 0.00% | 0.00% | 0.00% | 0.00% | 0.86% |

b0: sao_lcu_enable_flag ( coded by context ) 
SML: sao_merge_left_flag ( coded by bypass )

Figure 3. Bins order for SAO syntax elements if sao_lcu_enable_flag is equal to 1

Table 9. The ratio of isolated bypass bins for SAO relative to HM7.0

|  | AI-Main | RA-Main | LB-Main | LP-Main | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HM7.0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Solution 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Solution 2 | 21 | 69 | 525624 | 2004 | 35 | 99 | 90 | 138 |

# Conclusions

In this contribution, we proposed the coding design for SAO syntax elements in order to reduce the number of CABAC context models and bins. The followings were confirmed in solution 1.

- 5 context models reduction in CABAC design for SAO.

- The number of context bins per sample in the worst case is reduced form 1.50 to 0.01.

- Average BD-rate loss is 0.03% for Y, 0.10% for U, 0.17% for V component.

- 62% reduction of SAO context bins relative to HM7.0 (1.7% reduction of total context coded bins).

- There is no isolated bypass bins.

From above points of view, this proposal is recommended for adoption to DIS text and HM.

# Reference

- Y.-C. Yang and J.-I. Guo, “High-Throughput H.264/AVC High-Profile CABAC Decoder for HDTV Applications,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 19, no. 9, pp. 1395 –1399, September 2009.

- B. Shi, W. Zheng, H.-S. Lee, D.-X. Li, and M. Zhang, “Pipelined Architecture Design of H.264/AVC CABAC Real-Time Decoding,” in IEEE International Conference on Circuits and Systems for Communications, May 2008, pp. 492 –496.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1003, JCT-VC 9th Meeting, Geneva, CH, 27 April -7 May, 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, JCT-VC 9th Meeting, Geneva, CH, 27 April -7 May, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Table 10. BD-rate of solution 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.10% | 0.24% | 0.21% | 0.13% | 0.14% | 0.19% |
| Class B | 0.11% | 0.25% | 0.23% | 0.10% | 0.11% | 0.15% |
| Class C | 0.07% | 0.14% | 0.18% | 0.09% | 0.14% | 0.21% |
| Class D | 0.04% | 0.05% | 0.09% | 0.05% | 0.01% | 0.01% |
| Class E | 0.14% | 0.38% | 0.47% | 0.15% | 0.12% | 0.10% |
| Overall | 0.09% | 0.21% | 0.22% | 0.10% | 0.10% | 0.14% |
|  | 0.09% | 0.20% | 0.21% | 0.10% | 0.10% | 0.13% |
| Class F | 0.07% | 0.03% | 0.08% | 0.14% | 0.26% | 0.36% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.16% | -0.22% | -0.22% | 0.13% | 0.03% | 0.05% |
| Class B | 0.05% | 0.13% | -0.06% | 0.03% | -0.01% | -0.01% |
| Class C | -0.11% | 0.01% | -0.06% | -0.04% | 0.04% | 0.01% |
| Class D | -0.28% | -0.21% | -0.15% | -0.24% | -0.13% | -0.26% |
| Class E |  |  |  |  |  |  |
| Overall | -0.04% | -0.06% | -0.12% | -0.03% | -0.02% | -0.05% |
|  | -0.04% | -0.06% | -0.13% | -0.03% | -0.02% | -0.05% |
| Class F | -0.02% | -0.05% | 0.01% | 0.00% | 0.09% | 0.21% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.13% | 0.38% | 0.25% | 0.04% | 0.02% | 0.14% |
| Class C | -0.04% | 0.14% | 0.40% | 0.03% | 0.24% | 0.62% |
| Class D | -0.17% | -0.14% | 0.43% | -0.09% | 0.50% | 0.35% |
| Class E | -0.21% | 0.24% | 0.28% | -0.13% | -0.44% | 0.12% |
| Overall | -0.05% | 0.16% | 0.34% | -0.03% | 0.11% | 0.30% |
|  | -0.05% | 0.17% | 0.36% | -0.02% | 0.12% | 0.36% |
| Class F | 0.04% | 0.10% | -0.09% | 0.29% | 0.55% | 1.35% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.44% | 0.44% | 0.20% | 0.14% | -0.05% | 0.00% |
| Class C | 0.06% | 0.05% | 0.21% | 0.12% | 0.29% | 0.60% |
| Class D | -0.08% | 0.16% | 0.25% | -0.04% | -0.01% | -0.05% |
| Class E | 0.06% | 0.00% | 0.78% | -0.26% | 0.27% | -0.68% |
| Overall | 0.14% | 0.19% | 0.32% | 0.01% | 0.10% | 0.01% |
|  | 0.14% | 0.17% | 0.31% | 0.02% | 0.14% | 0.11% |
| Class F | 0.04% | 0.17% | -0.11% | 0.22% | 0.33% | 1.32% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 11. BD-rate of solution 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.09% | 0.33% | 0.30% | 0.14% | 0.09% | 0.08% |
| Class B | 0.09% | 0.42% | 0.45% | 0.10% | 0.09% | 0.11% |
| Class C | 0.05% | 0.21% | 0.26% | 0.09% | 0.09% | 0.14% |
| Class D | 0.03% | -0.05% | -0.04% | 0.05% | -0.03% | -0.05% |
| Class E | 0.13% | 0.47% | 0.60% | 0.15% | 0.09% | 0.04% |
| Overall | 0.08% | 0.27% | 0.31% | 0.10% | 0.06% | 0.07% |
|  | 0.08% | 0.26% | 0.29% | 0.10% | 0.06% | 0.06% |
| Class F | 0.05% | 0.09% | 0.12% | 0.13% | 0.22% | 0.31% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.12% | 0.17% | 0.03% | 0.13% | -0.07% | 0.17% |
| Class B | 0.03% | 0.39% | 0.24% | 0.05% | 0.00% | 0.04% |
| Class C | -0.11% | 0.13% | 0.16% | -0.02% | 0.02% | 0.03% |
| Class D | -0.32% | -0.24% | -0.15% | -0.23% | -0.15% | -0.47% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | 0.13% | 0.08% | -0.02% | -0.05% | -0.05% |
|  | -0.06% | 0.12% | 0.06% | -0.02% | -0.05% | -0.05% |
| Class F | -0.02% | 0.10% | 0.19% | 0.02% | 0.08% | 0.21% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.13% | 0.68% | 0.69% | 0.07% | 0.18% | 0.35% |
| Class C | -0.06% | 0.59% | 0.83% | 0.08% | 0.32% | 0.58% |
| Class D | -0.16% | 0.25% | 0.52% | -0.12% | 0.28% | 0.28% |
| Class E | -0.20% | 0.70% | 0.82% | -0.19% | -0.55% | 0.05% |
| Overall | -0.05% | 0.55% | 0.71% | -0.02% | 0.10% | 0.33% |
|  | -0.05% | 0.53% | 0.70% | -0.02% | 0.08% | 0.37% |
| Class F | 0.04% | 0.24% | 0.28% | 0.33% | 0.69% | 1.90% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.46% | 1.04% | 0.72% | 0.12% | -0.04% | -0.18% |
| Class C | 0.03% | 0.14% | 0.71% | 0.12% | 0.29% | 0.47% |
| Class D | -0.07% | 0.46% | 1.07% | -0.06% | 0.14% | -0.63% |
| Class E | 0.17% | 0.54% | 1.40% | -0.15% | 0.28% | -0.70% |
| Overall | 0.16% | 0.58% | 0.93% | 0.03% | 0.15% | -0.23% |
|  | 0.16% | 0.55% | 0.91% | 0.03% | 0.16% | -0.14% |
| Class F | 0.04% | 0.63% | 0.65% | 0.13% | 0.74% | 0.64% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |