# Draft Text

#### Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| sao_lcu_enable_flag[cIdx][rx][ry] | ae(v) |
| if( sao_lcu_enable_flag[cIdx][rx][ry] == 1 ) { |  |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ]= =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 4) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Slice data semantics

#### Sample adaptive offset parameter semantics

sao_lcu_enable_flag[cIdx][rx][ry] equal to 1 sepecifies that SAO processing of current coding tree block is enabled; otherwise, there is no any additional SAO parameters for current coding tree block. The array index cIdx specifies the colour component; cIdx is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices rx and ry specify the location ( rx, ry ) of the considered coding treeblock relative to the top-left coding treeblock of the picture.

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-1. of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-1 – Specification of the SAO type

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 0 | - 1D 0-degree edge offset |
| - 2 1 | - 1D 90-degree edge offset |
| - 3 2 | - 1D 135-degree edge offset |
| - 4 3 | - 1D 45-degree edge offset |
| - 5 4 | - Band offset |

#### Initialization process for context variables

Table 9-2 – Association of ctxIdx and syntax elements for each initialisationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao_param() | sao_lcu_enable_flag | Table 9-X | 0 | 1 | 2 |
|  | sao_merge_left_flag |  | 0..2 0 | 3..5 1 | 6..8 2 |
|  | sao_merge_up_flag |  | 0 | 1 | 2 |
|  | sao_type_idx |  | 0..1 | 2..3 | 4..5 |
|  | sao_offset_abs |  | 0..1 | 2..3 | 4..5 |

Table 9-X – Values of variable initValue for sao_lcu_enable_flag ctxIdx

| Initialization variable | sao_lcu_enable_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-3 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 102 | 118 102 | 102 | 102 | 102 | 102 | 102 | 102 |

Table 9-4 – Values of variable initValue for sao_merge_up_flag ctxIdx

| Initialization variable | sao_merge_up_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

Table 9-5 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 61 | 104 | 168 | 120 | 184 | 120 |

Table 9-6 – Values of variable initValue for sao_offset_abs ctxIdx

| Initialization variable | sao_offset_abs ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 171 | 119 | 199 | 169 | 119 | 151 |

### Binarization process

| Table 9-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_lcu_enable_flag | 0 | FL, cMax = 1 | 0 | Table 9-X | 0 |
|  | 1 |  | 0 | Table 9-X | 1 |
|  | 2 |  | 0 | Table 9-X | 2 |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 |  | 0 |
|  | 1 |  | 0 |  | 3 1 |
|  | 2 |  | 0 |  | 6 2 |
| sao_merge_up_flag | 0 | FL, cMax = 1 | 0 |  | 0 |
|  | 1 |  | 0 |  | 1 |
|  | 2 |  | 0 |  | 2 |
| sao_type_idx | 0 | U TU, cMax = 4 | 1 na | na | 0 na, (uses Decode Bypass) |
|  | 1 |  | 1 na | na | 2 na, (uses Decode Bypass) |
|  | 2 |  | 1 na | na | 4 na, (uses Decode Bypass) |
| sao_band_position | 0 | FL, cMax = 5 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |
| sao_offset_abs | 0 | TU, cMax = ( 1<< (Min(bitDepth, 10) − 5)) − 1 | 1 na | na | 0 na, (uses Decode Bypass) |
|  | 1 |  | 1 na | na | 2 na, (uses Decode Bypass) |
|  | 2 |  | 1 na | na | 4 na, (uses Decode Bypass) |
| sao_offset_sign | 0 | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |

#### Derivation process for ctxIdx

| Table 9-8 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| sao_lcu_enable_flag | ’ | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| sao_merge_left_flag |  | 0 | cIdx 0 | na | na | na | na |
|  |  | 3 1 | cIdx 0 | na | na | na | na |
|  |  | 6 2 | cIdx 0 | na | na | na | na |
| sao_merge_up_flag |  | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| sao_type_idx |  | 0 na | 0 na | 1 na | 1 na | 1 na | 1 na |
|  |  | 2 na | 0 na | 1 na | 1 na | 1 na | 1 na |
|  |  | 4 na | 0 na | 1 na | 1 na | 1 na | 1 na |
| sao_offset_abs |  | 0 na | 0 na | 1 na | 1 na | 1 na | 1 na |
|  |  | 2 na | 0 na | 1 na | 1 na | 1 na | 1 na |
|  |  | 4 na | 0 na | 1 na | 1 na | 1 na | 1 na |