| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0149 |
| --- | --- |

| Title: | Signalling of Luminance Dynamic Range in Tone mapping information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-7411 Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document describes a contribution on a proposal to signal the luminance dynamic range of the images in the tone mapping information SEI message. With the improvements of capturing and displaying images in wider dynamic range, the desire to deliver such images is anticipated to increase. In order for displays to support images in various dynamic ranges, it is necessary for displays to adjust the dynamic range of the images specifically to best suit the range of the display according to its capability. The proposed information is useful in such remapping process of dynamic ranges. The contribution proposes to signal the camera setting, settings used at image production process and digital code of output image from production process in which white and black level is assigned.

# Introduction

As seen in some of the cameras today, the capability of capturing wider dynamic range is improving such that more images in future can be anticipated to be produced in wider dynamic range.

Furthermore, the displays are also improving to display wider dynamic range with higher brightness.

Considering such situations, the desire to deliver the images with wide dynamic range directly to the displays can be anticipated to increase.

On the other hand, the dynamic range supported by displays is varied. In order for displays to display images with dynamic range different from the dynamic range it supports, the remapping process of dynamic range is essential.

The contribution proposes to provide information related to luminance dynamic range of the image to be used in such remapping process.

Proposed information:

- ISO sensitivity of the camera

- Exposure index rating setting of the camera

- Exposure value setting used at image production process in relative to exposure index rating of the image

- There reference screen luminance (in units of cd/m2) for white level used at  image production process

- The maximum luminance dynamic range of produced image expressed as a percentage in reference to the nominal white level

- The digital code of luma sample in which the black level is assigned

- The digital code of luma sample in which the white level is assigned

- The digital code of luma sample in which the maximum white level is assigned

Figure: Example of proposed luminance dynamic range information

# Proposed Syntax and Semantics

E.1.24 Tone mapping information SEI syntax

| tone_mapping_info( payloadSize ) { | Descriptor |
| --- | --- |
| tone_map_id | ue(v) |
| tone_map_cancel_flag | u(1) |
| if( !tone_map_cancel_flag ) { |  |
| tone_map_repetition_period | ue(v) |
| coded_data_bit_depth | u(8) |
| target_bit_depth | u(8) |
| model_id | ue(v) |
| if( model_id = = 0 ) { |  |
| min_value | u(32) |
| max_value | u(32) |
| } |  |
| if( model_id = = 1 ) { |  |
| sigmoid_midpoint | u(32) |
| sigmoid_width | u(32) |
| } |  |
| if( model_id = = 2 ) |  |
| for( i = 0; i < ( 1 << target_bit_depth ); i++ ) |  |
| start_of_coded_interval[ i ] | u(v) |
| if( model_id = = 3 ) { |  |
| num_pivots | u(16) |
| for( i=0; i < num_pivots; i++ ) { |  |
| coded_pivot_value[ i ] | u(v) |
| target_pivot_value[ i ] | u(v) |
| } |  |
| if( model_id = = 4 ) { |  |
| camera_iso_sensitivity_idc | u(8) |
| If(camera_iso_sensitivity_idc == Extended_ISO) |  |
| camera_iso_sensitivity | u(v) |
| exposure_index_idc | u(8) |
| If(exposure_index_idc == Extended_ISO) |  |
| exposure_inedex_rating | u(v) |
| sign_image_exposure_value | u(1) |
| image_exposure_value0 | u(v) |
| image_exposure_value1 | u(v) |
| ref_screen_lw | u(v) |
| max_image_white_level | u(v) |
| black_level_code_value | u(8) |
| white_level_code_value | u(8) |
| max_white_level_code_value | u(8) |
| } |  |
| } |  |
| } |  |

- E.2.24 Tone mapping information SEI message semantics

This SEI message provides information to enable remapping of the colour samples of the output decoded pictures for customization to particular display environments. The remapping process maps coded sample values in the RGB colour space (specified in Annex Error: Reference source not found) to target sample values. All mappings are expressed in the RGB colour space and should be applied to each RGB component separately or RGB colour space expressed in luma and should be applied to luma component of decoded image.

tone_map_id contains an identifying number that may be used to identify the purpose of the tone mapping model. The value of tone_map_id shall be in the range of 0 to 232 − 2, inclusive.

Values of tone_map_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of tone_map_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all tone mapping information SEI messages containing a value of tone_map_id in the range of 256 to 511 or in the range of 231 to 232 − 2, and bitstreams shall not contain such values.

NOTE 1 – The tone_map_id can be used to support tone mapping operations that are suitable for different display scenarios. For example, different values of tone_map_id may correspond to different display bit depths.

tone_map_cancel_flag equal to 1 indicates that the tone mapping information SEI message cancels the persistence of any previous tone mapping information SEI message in output order. tone_map_cancel_flag equal to 0 indicates that tone mapping information follows.

tone_map_repetition_period specifies the persistence of the tone mapping information SEI message and may specify a picture order count interval within which another tone mapping information SEI message with the same value of tone_map_id or the end of the coded video sequence shall be present in the bitstream. The value of tone_map_repetition_period shall be in the range of 0 to 16 384, inclusive.

tone_map_repetition_period equal to 0 specifies that the tone map information applies to the current decoded picture only.

tone_map_repetition_period equal to 1 specifies that the tone map information persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a tone mapping information SEI message with the same value of tone_map_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

tone_map_repetition_period equal to 0 or equal to 1 indicates that another tone mapping information SEI message with the same value of tone_map_id may or may not be present.

tone_map_repetition_period greater than 1 specifies that the tone map information persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a tone mapping information SEI message with the same value of tone_map_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + tone_map_repetition_period.

tone_map_repetition_period greater than 1 indicates that another tone mapping information SEI message with the same value of tone_map_id shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + tone_map_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture.

coded_data_bit_depth specifies the BitDepthY of the luma component of the coded video sequence. It is used to identify the tone mapping information SEI message that is intended for use with the coded video sequence. If tone mapping information SEI messages are present that have coded_data_bit_depth that is not equal to BitDepthY, these refer to the hypothetical result of a transcoding operation performed to convert the coded video to the BitDepthY corresponding to the value of coded_data_bit_depth.

The value of coded_data_bit_depth shall be in the range of 8 to 14, inclusive. Values of coded_data_bit_depth from 0 to 7 and from 15 to 255 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all tone mapping SEI messages that contain a coded_data_bit_depth in the range of 0 to 7 or in the range of 15 to 255, and bitstreams shall not contain such values.

target_bit_depth specifies the bit depth of the output of the dynamic range mapping function (or tone mapping function) described by the tone mapping information SEI message. The tone mapping function specified with a particular target_bit_depth is suggested to be reasonable for all display bit depths that are equal to or less than the target_bit_depth.

The value of target_bit_depth shall be in the range of 1 to 16, inclusive, or 255 for unspecified value. Values of target_bit_depth equal to 0 and in the range of 17 to 254 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all tone mapping SEI messages that contain a value of target_bit_depth equal to 0 or in the range of 17 to 254, and bitstreams shall not contain such values.

model_id specifies the model utilized for mapping the coded data into the target_bit_depth range. Values greater than 4 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all tone mapping SEI messages that contain a value of model_id greater than 4, and bitstreams shall not contain such values.

NOTE 2 – A model_id of 0 corresponds to a linear mapping with clipping; a model_id of 1 corresponds to a sigmoidal mapping; a model_id of 2 corresponds to a user-defined table mapping, and a model_id of 3 corresponds to a piece-wise linear mapping, model_id of 4 corresponds to luminance dynamic range information.

min_value specifies the RGB sample value in the coded data that maps to the minimum value in the signalled target_bit_depth. It is used in combination with the max_value parameter. All values in the coded data that are less than or equal to min_value are mapped to this minimum value in the target_bit_depth representation.

max_value specifies the RGB sample value in the coded data that maps to the maximum value in the signalled target_bit_depth. It is used in combination with the min_value parameter. All values in the coded data that are larger than or equal to max_value are mapped to this maximum value in the target_bit_depth representation.

sigmoid_midpoint specifies the RGB sample value of the coded data that is mapped to the centre point of the target_bit_depth representation. It is used in combination with the sigmoid_width parameter.

sigmoid_width specifies the distance between two coded data values that approximately correspond to the 5% and 95% values of the target_bit_depth representation, respectively. It is used in combination with the sigmoid_midpoint parameter and is interpreted according to the following function:

f(i)=Round2targetbitdepth−11+exp−6∗i−sigmoid_midpoint sigmoid_width  for i=0,⋯, (2codedbitdepth−1)		(D-1)

where f( i ) denotes the function that maps an RGB sample value i from the coded data to a resulting RGB sample value in the target_bit_depth representation.

start_of_coded_interval[ i ] specifies the beginning point of an interval in the coded data such that all RGB sample values that are greater than or equal to start_of_coded_interval[ i ] and less than start_of_coded_interval[ i + 1 ] are mapped to i in the target bit depth representation. The value of start_of_coded_interval[ 2target_bit_depth ] is equal to 2coded_bit_depth. The number of bits used for the representation of the start_of_coded_interval is ( ( coded_data_bit_depth + 7 ) >> 3 ) << 3.

num_pivots specifies the number of pivot points in the piece-wise linear mapping function without counting the two default end points, (0, 0) and (2coded_data_bit_depth − 1, 2target_bit_depth − 1) .

coded_pivot_value[ i ] specifies the value in the coded_data_bit_depth corresponding to the i-th pivot point. The number of bits used for the representation of the coded_pivot_value is ( ( coded_data_bit_depth + 7 ) >> 3 ) << 3.

target_pivot_value[ i ] specifies the value in the reference target_bit_depth corresponding to the i-th pivot point. The number of bits used for the representation of the target_pivot_value is ( ( target_bit_depth + 7 ) >> 3 ) << 3.

camera_iso_sensitivity_idc specifies the camera sensitivity to light in ISO number (ISO 12232:2006).  Table X shows the meaning of the code. When camera_iso_sensitivity_idc indicates Extended_ISO, the ISO number is represented by camer_iso_sensitivity.

camera_iso_sensitivity specifies the camera sensitivity to light in ISO number (ISO 12232:2006)

exposure_index_idc specifies exposure index rating setting of the camera in ISO number (ISO 12232:2006). Table X shows the meaning of the code. When exposure_index_idc indicates Extended_ISO, the ISO number is represented by exposure_index_rating.

exposure_index_rating specifies exposure index rating setting of the camera in ISO number (ISO 12232:2006).

Table X Meaning of camera iso sensitivity indicator and exposure index rating indicator

| Indicator | ISO number |
| --- | --- |
| 0 | Unspecified |
| 1 | 10 |
| 2 | 12 |
| 3 | 16 |
| 4 | 20 |
| 5 | 25 |
| 6 | 32 |
| 7 | 40 |
| 8 | 50 |
| 9 | 64 |
| 10 | 80 |
| 11 | 100 |
| 12 | 125 |
| 13 | 160 |
| 14 | 200 |
| 15 | 250 |
| 16 | 320 |
| 17 | 400 |
| 18 | 500 |
| 19 | 640 |
| 20 | 800 |
| 21 | 1000 |
| 22 | 1250 |
| 23 | 1600 |
| 24 | 2000 |
| 25 | 2500 |
| 26 | 3200 |
| 27 | 4000 |
| 28 | 5000 |
| 29 | 6400 |
| 30 | 8000 |
| 31-254 | Reserved |
| 255 | Extended_ISO |

sign_image_exposure_value specifies the sign of exposure value of produced output image derived from image_exposure_value0 and image_exposure_value1. sign_image_exposure_value equal to 0 indicates that the exposure value is positive. sign_image_exposure_value equal to 1 indicates that the exposure value is negative.

image_exposure_value0 specifies the numerator of exposure value used at image production process in relative to exposure index rating of the camera.

image_exposure_value1 specifies the denominator of exposure value setting used at image production process in relative to exposure index rating of the camera.

The exposure value used at production stage for the output image is represented as follows:

image_exposure_value0 / image_exposure_value1

sign_image_exposure_value represents the sign of exposure value.

For example, in case the exposure value is set to +1/2 at production stage, sign_image_exposue_value is set to 0, image_exposure_value0 may be set to 1 and image_exposure_value1 may be set to 2.

screen_lw specifies reference screen luminance setting for white level used for image production process in units of cd/m2.

max_image_white_level specifies the luminance dynamic range of image expressed as an integer percentage in reference to the nominal white level.

black_level_code_value specifies the luma sample value of the coded data in which the black level is assigned.

whitle_level_code_value specifies luma sample value of the coded data in which the white level is assigned.

max_whitle_level_code_value specifies the luma sample value of the coded data in which the maximum video white level is assigned.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).