| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 |  |  | Document: JCTVC-J0150 |
| --- | --- | --- | --- |
| Title: | Removal of zigzag scan from scaling list coding |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution reports the effects of replacing zigzag scan used for scaling list coding with existing up-right diagonal scan.  In regards of scaling list coding performance, using the test matrices used in past CE4, the simulation results show that replacing zigzag scan with up-right diagonal scan will lead to about 0.6% scaling list coding bit reduction on average.  For other aspects, it is reported that the proposed changes will remove one section of HEVC draft text defining zigzag scan (about half a page) and about 50 lines of code from HM 7.0 software.

# Introduction

In most of the cases in former video coding standards, both transformed coefficients and scaling list coefficients are scanned using a zigzag scan.  However, that commonality is broken in HM6 as well as in HM7, as transformed coefficients are scanned with non-zigzag scans (horizontal, vertical, or up-right diagonal scans) while scaling list coefficients are scanned with zigzag scan.  At the 9th JCT-VC meeting, there are several contributions (JCTVC-I0102 and JCTVC-I0370) addressing the issue.  This contribution clarifies the benefits of replacing zigzag scan with existing up-right diagonal scan for scaling list coding.

# Description of the proposed methods

In this contribution, up-right diagonal scan was tested as an alternative method to zigzag scan.

## Anchor: zigzag scan used in HM7

Figure 1 shows the 4x4 and 8x8 zigzag scans used for scanning scaling list coefficients in HM7.

Figure 1: 4x4 and 8x8 zigzag scans used to scan scaling list coefficients in HM7.

## Proposed: Up-right diagonal scan

Figure 2 shows the proposed 4x4 and 8x8 up-right diagonal scans used for scanning scaling list coefficients.  It shall be noted that these two scans already exist in HM7 to scan and code “significant_coeff_group_flag” syntax element.  Accordingly, there is no need to add these scan patterns to use them.

Figure 2: 4x4 and 8x8 up-right diagonal scans used to scan scaling list coefficients

# Experimental results on scaling list coding performance

Table 1 presents the bits needed to code each scaling list in the test set used in past CE4.  In order to perform the simulation with HM7.0, the coefficients in every two rows/columns of 16x16 matrices and the coefficients in every four rows/columns of 32x32 matrices are used to create 8x8 samples.  The obtained results show that the proposed method of up-right diagonal scan requires equivalent or slightly less bits to code scaling lists comparing to zigzag scan.  The average over all the scaling lists used in past CE4 is calculated as 99.4%, which is 0.6% of scaling list coded bits saving.

Table 1: Bits needed to code scaling lists using zigzag scan and up-right diagonal scan

| Coded bits | Zigzag (Anchor, HM7.0) | Up-right diagonal (Proposed) |
| --- | --- | --- |
| Symmetry1 | 2067 | 2067 (100.0%) |
| Symmetry2 | 1255 | 1255 (100.0%) |
| Symmetry3 | 2593 | 2593 (100.0%) |
| Symmetry4 | 2565 | 2565 (100.0%) |
| Symmetry5 | 1899 | 1899 (100.0%) |
| Symmetry6 | 2089 | 2089 (100.0%) |
| Asymmetry1 | 2377 | 2345 (98.7%) |
| Asymmetry2 | 1397 | 1371 (98.1%) |
| Asymmetry3 | 2873 | 2861 (99.6%) |
| Asymmetry4 | 3859 | 3829 (99.2%) |
| Asymmetry5 | 2165 | 2127 (98.2%) |
| Asymmetry6 | 2315 | 2293 (99.0%) |
| Total | 27454 | 27294 (99.4%) |

# Other benefits of the proposed modifications

## HEVC draft text reduction

Proposed text changes are described in the appendix section of this contribution.  There are basically two changes: removal of subclause 6.5.3 “Zig-zag scanning array initialization process” and modification to subclause 7.4.2.5 “Scaling list semantics”.  The first textual change results in about half a page of text reduction.

## HM software code reduction

Accompanied two patch files show the proposed changes to HM7.0 software.  One of the patch files “JCTVC-J0150_smallest_changes_to_HM7.0.patch” indicates that changes of one line in encoder and one line in decoder, which is two lines change in total achieves proposed change of this contribution.  The other patch “ JCTVC-J0150_code_reduction_to_HM7.0.patch” indicates that the proposed changes of this contribution make the tables used for zigzag scan in HM7.0 redundant.  Accordingly, it indicates that the proposed changes of this contribution can reduce the amount of the code of HM7.0 software by about fifty lines or so by removing such redundant zigzag scan tables and related operations.

# Conclusion

This contribution clarifies the benefit of replacing zigzag scan by up-right diagonal scan for scaling list coding.  In regards of scaling list coding performance, the experimental results show that the proposed up-right diagonal scan achieves 0.6% scaling list coding bits saving while removing the zigzag scan table/operation from HEVC design.  In addition, it is demonstrated that removing zigzag scan from HEVC design can lead to the reduction of HEVC text as well as HM software.

It is recommended that the proposed method of replacing zigzag scan with up-right diagonal scan be incorporated into the next version of HM and HEVC draft text.

# Patent rights declaration(s)

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Draft 7 (JCTVC-I1003_d4.doc)

The modified or added portions are highlighted with yellow colour.  The removed portions are highlighted with purple colour.

### A.3.1. Change (removal) of subclause 6.5.3 “Zig-zag scanning array initialization process”

### 6.5.3 Zig-zag scanning array initialization process

Input to this process is a block size blkSize.

Output of this process is the array zigZag[ sPos ][ sComp ]. The array index sPos specify the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. The array zigZag is derived as follows.

zigZag[ 0 ][ 0 ]  =  0
zigZag[ 0 ][ 1 ]  =  0
i  =  1
x  =  1
y  =  0
stopLoop  =  ( blkSize  = = 1 ) ? TRUE : FALSE
while( !stopLoop ) {
	while( x  >=  0 ) {
		if(x  >=  0  &&  x  <  blkSize  &&  y  >=  0  &&  y  <  blkSize ) {
			zigZag[ i ][ 0 ]  =  x
			zigZag[ i ][ 1 ]  =  y
			i++
		}
		x− −
		y++
	}
	x  =  0
 	while( y  >=  0 ) {
		if(x  >=  0  &&  x  <  blkSize  &&  y  >=  0  &&  y  <  blkSize ) {
			zigZag[ i ][ 0 ]  =  x
			zigZag[ i ][ 1 ]  =  y
			i++
		}
		x++
		y− −
	}
	y  =  0
	if( i  >=  blkSize  *  blkSize ) 
		stopLoop  =  TRUE
}

### A.3.2. Change of subclause 7.4.2.5 “Scaling list semantics”

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 0 ][ MatrixID ][ 0 ][ y*4+x ] = ScalingList[ 0 ][ MatrixID ][ i ]	(7-1)
with i=0..15 and MatrixID=0..5

where diagScan is an array specified in subclause 6.5.4 with 16 as input, x=diagScan [ i<<4 ][ 0 ] >> 2 and y=diagScan [ i<<4 ][ 1 ] >> 2

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 0 ][ y*8+x ] = ScalingList[ 1 ][ MatrixID ][ i ]	(7-2)
with i=0..63 and MatrixID=0..5

where diagScan is an array specified in subclause 6.5.4 with 32 as input, x= diagScan [ i<<4 ][ 0 ] >> 2 and y= diagScan [ i<<4 ][ 1 ] >> 2

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (y*2+j)*16+(x*2+k) ] = ScalingList[ 2 ][ MatrixID ][ i ]	(7-3)
with i=0..63, j=0..1, k=0..1 and MatrixID=0..5

where diagScan is an array specified in subclause 6.5.4 with 32 as input, x= diagScan [ i<<4 ][ 0 ] >> 2 and y= diagScan [ i<<4 ][ 1 ] >> 2

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ MatrixID ] + 8	(7-4)
with MatrixID=0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (y*4+j)*32+(x*4+k) ] = ScalingList[ 3 ][ MatrixID ][ i ]	(7-5)
with i=0..63, j=0..3, k=0..3 and MatrixID=0..1

where diagScan is an array specified in subclause 6.5.4 with 32 as input, x= diagScan [ i<<4 ][ 0 ] >> 2 and y= diagScan [ i<<4 ][ 1 ] >> 2

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ MatrixID ] + 8	(7-6)
with MatrixID=0..1