| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0152 |
| --- | --- |

| Title: | Proposals on HEVC professional extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Paul Silcock, Karl Sharman, Nick Saunders and James Gamei Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 paul.silcock@eu.sony.com karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This contribution proposes to start more formal evaluation, CE or HM branch, of non-4:2:0 formats and n-bit video extensions to HEVC. In addition it also proposes new test sequences and test frameworks for the evaluation of coding tools for such profiles. Finally a proposal to establish a CE and to create an HM branch for further investigation is made.

# Test materials

To evaluate coding efficiency of video coding tools for non-4:2:0 formats and n-bit video, appropriate test sequences are necessary. Those test materials are not included in the current common test conditions. The followings are candidates of test sequences for such coding tools.

## EBU test sequences

Crowd Run	Flowers	Girl Flower 1	Girl Flower 2	Graphics

Horse	Kids Soccer	Rain Roses	Vegi Candle	Vegies

Waterfall	Water Rocks	Water Rocks Close

The set of 13 EBU Test Sequences was created by the EBU for a study into “HDTV Studio & Acquisition Compression Analysis”. The requirement at the time was that the portfolio of test sequences should cover different levels of spatial and temporal criticality, and should contain fine details, skin tones, different movements and graphics. The 1080 50P source material (illustrated above) was also converted to other formats (720P and 1080i). The properties of this video source are shown in Table 1. The EBU have subsequently undertaken the acquisition of additional 4k Test Sequences (http://tech.ebu.ch/news/shooting-of-4k-test-sequences-begins-in-12jun12).

Table 1 Specification of proposed EBU test sequences

| Name | Resolution | Bit depth | Frame rate | Color format | Acquisition |
| --- | --- | --- | --- | --- | --- |
| Flowers, Girl Flower 1, Girl Flower 2, Graphics, Horse, Rain Roses, Vegi Candle, Vegies, Waterfall, Water Rocks, Water Rocks Close | 1920x1080 pixels | 10 | 50fps, 500 frames | YCbCr, 4:2:2 | Sony F35 |
| Crowd Run, Kids Soccer | 1920x1080 pixels | 10 | 50fps, 500 frames | YCbCr, 4:2:2 | ArriFlex 765 System for 65mm film |

The “CoSME” test sequences were used for Call for Evidence of HEVC, and they are still allowed to be used for the development of HEVC.

The “CoSME” (Color Space Management Evaluation Material) sequences were produced by DECSDP (Digital Eizo (Video) Common Specifications Development Committee) to provide the measure to evaluate the color management system including the effect of compression [3,4]. The properties of these video sources are shown in Table 2.

Table 2 Specification of proposed CoSME test sequences

| Name | Resolution | Bit depth | Frame rate | Color format | Acquisition |
| --- | --- | --- | --- | --- | --- |
| kimono1, parkscene, tennis | 1920x1080 pixels | 10 | 24fps | RGB, 4:4:4 | Sony HDC-F950 |

Our assessment on adequacy of these sequences in terms of requirements described in the call for test materials document is shown in Table 3.

Table 3 Assessment of adequacy of CoSME sequence as test material

| Item | Requirements in [1] | Assessment |
| --- | --- | --- |
| Sequence formats and frame rates | Test material in the following sequence formats and frame rates are sought. •4Kx2K@24/30, 50/60, 100/120 frames per second •1080p@24/30, 50/60, 100/120 frames per second •VGA@30, 60 frames per second | Relevant: 1080@24fps is defined as a target video source format for HEVC work |
| Color Spaces and Color Sampling | It is desirable that the test material be available in RGB colour space at the highest colour sampling of 4:4:4. However suitable test material that have been processed to the YUV colour space and lower colour sampling such as 4:2:2 and 4:2:0 will be welcome. | Relevant: CoSME supports RGB 4:4:4 sampling, which has originally been generated to evaluate color representation fidelity of video source assumed for use in Digital Cinema application. |
| Bitdepth | Test material should have at least a bitdepth of 8 bits per component. Material with higher bitdepth such as 10, 12 or 14 bits per component are desirable, but not necessary. | Relevant: CoSME supports 10 bit per color component. |
| Scanning Methods | The test material should be progressively scanned. | Relevant: CoSME supports progressive scanning. |
| Sequence Length | The length of the test material should be at least 10 seconds but preferably more. | Relevant: CoSME consists of long and various scene cuts. It is possible to identify relevant 10 sec scene cuts quite easily by performing expert viewing. |
| Good Quality | The sequences should be captured with state of the art cameras with as low amount of noise as possible. | Relevant: The camera used for capturing CoSME content is reasonably state-of-the-art. |
| Content | The test material should cover a large variety of characteristics and content type that will be challenging for a typical video codec. The following are some possible examples of such characteristics: - irregular camera and object motion, - zoom, - non-rigid object motion, - saturated colour, - large contrast/dynamic range, - slow illumination changes, - flash, - different focus planes within the same frame, - fades/cross fades. Content examples include sports (soccer, basketball), nature (falling snow, rain, animals), people and faces, music video, performances, scenes from movies and scrolling text. | Relevant: CoSME contains various kinds of scene cuts, whose features are listed in the requirement of call for test materials. |

## ITE/ARIB test sequences

JCTVC-I0407 reported that ITU-R BT.1210-4 recommended some test materials. But ITE (The Institute of Image Information and Television Engineers) and ARIB (Association of Radio Industries and Businesses) own copyright of those materials. Sample video is available at the following site. (Japanese only)

http://www.ite.or.jp/shuppan/ippan.html

The following is the summary of image characteristics.

Table 4: Image characteristics

| Series | Materials A | Materials B |  | Materials C |
| --- | --- | --- | --- | --- |
| Video standard | Part 2 of Rec. ITU-R BT.709 |  |  |  |
| Number of pixels | 1920×1080 |  |  |  |
| Bit depth | 10-bit |  |  |  |
| Duration | 15 sec |  |  |  |
| Signal format | 4:4:4/59.94i 4:4:4/50i | 4:2:2/59.94p | 4:4:4/24p | 4:2:2/59.94i 4:2:2/50i |
| Colour mode | RGB | YCBCR | RGB | YCBCR |
| Scanning | Interlace | Progressive | Progressive | Interlace |

# Common software maintenance and test conditions

In order to conduct the evaluation of coding tools, it is better to develop and maintain an HM branch for the non-4:2:0 formats and n-bit video extensions. A reference model is required, and should be cross-verified by the JCTVC committee, using, for example proposed models such as JCTVC-I0521 and JCTVC-J0191. These are the candidate starting points of this HM branch.

During the meeting, all proposals should be evaluated and then the first HM of this extension should be created for further evaluation.

# Conclusions

In order to evaluate non-4:2:0 formats and n-bit video, this contribution proposes to establish CE and to create HM branch for further investigation.

# References

[1] Call for test materials for future high-performance video coding standards development, ISO/IEC JTC1/SC29/WG11/N10176, October 2008.

[2] Call for Test Materials for High-Performance Video Coding Standardisation, ISO/IEC JTC1/SC29/WG11/N10362, February 2009.

[3] Hiroshi Yasuda, Takuyo Kogure, Kohtaro Asai on behalf of DECSDP, “Introduction of CoSME as a candidate of test materials”, ISO/IEC JTC1/SC29/WG11/M14764, July 2007.

[4] Takuyo Kogure, Kohtaro Asai on behalf of DECSDP, “CoSME overview”, ISO/IEC JTC1/SC29/WG11/M15013, October 2007.

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.