| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0153 |
| --- | --- |

| Title: | Burst I_PCM Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Wei-Jung Chien Jaehong Chon Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com wchien@qualcomm.com jchon@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Burst I_PCM method provides sending of I_PCM sample data of successive I_PCM units without CABAC termination process in between I_PCM blocks. Burst I_PCM method sends the number of subsequent I_PCM blocks that successively follow the first I_PCM unit in the same layer of a CTB. An alternative low delay coding method that eliminates sending of number of subsequent I_PCM ahead of PCM samples is proposed. The proposed method of signalling is more efficient than the existing scheme for signalling of I_PCM blocks in terms of bit overhead.

# Introduction

In current WD7, I_PCM mode is signalled by the pcm_flag. Following the pcm_flag, CABAC is terminated and number of subsequent I_PCM blocks is signalled with num_subsequent_pcm syntax that is coded as tu(3) [1].  This is followed by byte alignment and PCM samples with sample precisions specified in SPS, pcm_bit_depth_luma_minus1 and pcm_bit_depth_chroma_minus1 parameters.  Counting of subsequent I_PCM blocks at a CTB level and signalling it before the PCM samples introduces delay and complexity [2]. Signalling of subsequent I_PCM blocks can be eliminated if the presence of next I_PCM block after the first I_PCM block is signalled by a flag after each I_PCM block except for the I_PCM block at the last CU of a CTB level. After the last I_PCM block byte alignment is performed. In terms of bit overhead proposed way of signalling is more efficient than the existing scheme for signalling of I_PCM blocks.

# Proposed WD changes

- 7.3.5 Coding tree syntax

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| ……… |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( adaptive_loop_filter_flag && alf_cu_control_flag && ctDepth <= alf_cu_control_max_depth && ( ctDepth = = alf_cu_control_max_depth ¦ ¦ split_coding_unit_flag[ x0 ][ y0 ] = = 0 ) ) { |  |
| AlfCuFlagIdx++ |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] > SliceCbAddrZS ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x0 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && x1 < pic_width_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( moreDataFlag && x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !(x0% (1<<(log2CbSize+1))) ¦ ¦ ! (y0% (1<<(log2CbSize+1))) ){ |  |
| more_pcm_flag | u(1) |
| NumPCMBlock = more_pcm_flag |  |
| } |  |
| else |  |
| NumPCMBlock = 0 |  |
| if ( NumPCMBlock = = 0 ) |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(1) |
| …….. |  |

- 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if ( log2CbSize < Log2MaxCbSize ) { |  |
| more_pcm_flag | u(1) |
| NumPCMBlock = more_pcm_flag |  |
| } |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| ……………. |  |

- 7.3.7.2  PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock-- |  |
| } |  |

more_pcm_flag specifies whether there is subsequent IPCM coding unit with the current log2CbSize that successively follow the current IPCM coding unit in the same depth of a CTB. The values of pcm_flags of the subsequent coding unit are set equal to 1. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent IPCM coding units are identical. Initial value of NumPCMBlock is 0.

The variable Log2MaxCbSize is set equal to Log2MinCbSize + log2_diff_max_min_coding_block_size.

# Results

Proposed algorithm has been implemented in HM 7.0 and tested using common test conditions [3] with PCM mode enabled. The anchor uses HM 7.0 with PCM mode enabled (PCMEnabledFlag=1, PCMLog2MaxSize=5, and PCMLog2MinSize=3). I_PCM mode is not selected under common test conditions. There is no BD-rate difference between the anchor and the proposed algorithm as shown in Table 1.

Table 1: Results for proposed algorithm with PCM mode enabled (PCMEnabledFlag=1, PCMLog2MaxSize=5, and PCMLog2MinSize=3).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 109% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |

Additional simulation results for a synthesized sequence (sandstorms, CIF, 30fps, 33 frames) [1] using lower QP values, QP={0, 4, 8, 12}, are provided Table 2. The PSNR values for the anchor and the proposed scheme are identical. As can be seen from the results, the proposed low delay I_PCM signalling scheme provides bitrate savings (1.0 – 1.7%) over the existing scheme in HM 7.0.

Table 2: Bit rate change between the HM7.0anchor and proposed method for the synthesized sequence (sandstorm CIF, 30fps 33 frames).

| Sand Storm CIF 33 frames |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Bitrate (kbps) |  |  | PSNR-Y(dB) |  |
| Config | QP | Anchor | J-0153 | %change | Anchor | J-0153 |
| AI-main | 0 | 37797.78 | 37187.07 | -1.62% | 99.99 | 99.99 |
|  | 4 | 37797.78 | 37179.93 | -1.63% | 99.99 | 99.99 |
|  | 8 | 37797.78 | 37172.52 | -1.65% | 99.99 | 99.99 |
|  | 12 | 37797.54 | 37179.69 | -1.63% | 99.99 | 99.99 |
| AI_HE10 | 0 | 37797.78 | 37187.07 | -1.62% | 99.99 | 99.99 |
|  | 4 | 37797.78 | 37179.93 | -1.63% | 99.99 | 99.99 |
|  | 8 | 37797.78 | 37172.52 | -1.65% | 99.99 | 99.99 |
|  | 12 | 37797.78 | 37179.93 | -1.63% | 99.99 | 99.99 |
| RA-Main | 0 | 37798.31 | 37245.67 | -1.46% | 99.99 | 99.99 |
|  | 4 | 37798.31 | 37241.19 | -1.47% | 99.99 | 99.99 |
|  | 8 | 37798.31 | 37238.64 | -1.48% | 99.99 | 99.99 |
|  | 12 | 37798.28 | 37231.31 | -1.50% | 99.99 | 99.99 |
| RA_HE10 | 0 | 37798.49 | 37245.85 | -1.46% | 99.99 | 99.99 |
|  | 4 | 37798.49 | 37241.37 | -1.47% | 99.99 | 99.99 |
|  | 8 | 37798.37 | 37238.70 | -1.48% | 99.99 | 99.99 |
|  | 12 | 36982.57 | 36596.41 | -1.04% | 78.02 | 78.02 |
| LB-Main | 0 | 37798.58 | 37263.16 | -1.42% | 99.99 | 99.99 |
|  | 4 | 37798.58 | 37261.01 | -1.42% | 99.99 | 99.99 |
|  | 8 | 37798.55 | 37257.05 | -1.43% | 99.99 | 99.99 |
|  | 12 | 37798.52 | 37248.09 | -1.46% | 99.99 | 99.99 |
| LB-HE10 | 0 | 37798.66 | 37263.24 | -1.42% | 99.99 | 99.99 |
|  | 4 | 37798.66 | 37261.09 | -1.42% | 99.99 | 99.99 |
|  | 8 | 37798.66 | 37257.16 | -1.43% | 99.99 | 99.99 |
|  | 12 | 37798.12 | 37247.75 | -1.46% | 99.19 | 99.19 |

# References

- K. Chono, H. Aoki “Burst transmission of I_PCM blocks”, JCTVC-H0051, San Jose, February 2012.

- M. Coban, W-J Chien, J. Chon, M. Karczewicz “Burst IPCM Coding Signalling”, JCTVC-I0300, Geneva, April 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-I1100, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th meeting, Geneva, April 2012.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).