| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0155-r1 |
| --- | --- |

| Title: | Simplification of the Scaling Process for MV prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4809 jie.dong@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications LLC |  |  |

_____________________________

# Abstract

This proposal simplifies the scaling process for MV prediction. The simplification includes two parts: 1) reducing the range of possible POC differences, and 2) combining two integer approximations into one. Under the common test conditions specified in JCTVC-I1100, the proposed method provides bit-exact results of HM-7.0.

# Introduction

In the current HEVC specification [2], the MV of the current PU is predicted from the MVs of the neighbouring PUs. The neighbourhood here includes the temporal co-located PU and the causal spatial surrounding PUs, such as the upper, left, upper-left, upper-right, and bottom-right ones. The MVs of the current and neighbouring PUs represent the displacements over different time intervals.

Fig. 1 gives an example of temporal MV prediction. The orange and blue blocks are the current and temporal co-located PUs, of which the MVs are MV2 and MV1, respectively. MV1 is over a time interval represented by the POC difference TD (TD = ColPOC - ColRefPOC); MV2 is over a time interval represented by the POC difference TB (TB = CurrPOC - CurrRefPOC). TD and TB could be different. Similar example for spatial MV prediction can be found in Fig. 2. Therefore, MV1 should be scaled, before being used to predict MV2. Assuming the motion is uniform translation within a small range of space and time, given TD and TB, one can scale MV1 as in (1).

PredMV2 = MV1 × ( TB / TD )                                                         (1)

Fig. 1 Example of temporal MV prediction

Fig. 2 Example of temporal MV prediction

The actual process of MV scaling is not as simple as in (1), because of the need to perform integer arithmetic using limited number of bits to represent a number (usually, smaller than 16 bits). In HEVC WD7 [2], the prediction of MV2 (see Fig. 1 and Fig. 2) is calculated by the following steps, taking TB and TD as inputs.

- Clip TB and TD to the range of -128 to 127, inclusive.

- Calculate iX=(214+ ( Abs(TD)>>1) )/TD

- Calculate iScale = (iX×TB+32)>>6

- Clip iScale in the range of -4096 to 4095, inclusive.

- Calculate MV2

  - Horizontal component: 	MV2_x=Sign(iScale × MV1_x) × ((|iScale × MV1_x| + 127) >> 8)

  - Vertical component: 	MV2_y=Sign(iScale × MV1_y) × ((|iScale × MV1_y| + 127) >> 8)

- Clip MV2 to the range of -32768 to 32767, inclusive. Note that the unit of MV2 is quarter-pixel.

# Proposed Options

The simplification presented in this proposal includes two parts: 1) reducing the dynamic range of TB and TD and 2) combining the two integer approximations in Steps 2 and 3 into one. They are introduced in Section 2.1 and 2.2, respectively. Compared with the first version, this simplification is more friendly to hardware implementation, according to the discussions with some hardware companies.

## Reduce the range of TB and TD

The first part of the proposed simplification reduces the dynamic range of TB and TD to -32 to 31, inclusive. The linear relationship as in (1) holds only if the motion is uniform translation, which is less likely to be true over a longer time interval. The current HEVC WD clips TB and TD’s range to -128 to 127, in order to limit the use of the MV prediction from long-term reference pictures. However, the range is still too large and unnecessary. Reducing the dynamic range of TB and TD to -32 and 31 automatically screens out the inaccurate candidates for MV prediction and at the same time is sufficient for practical use.

## Combine two integer approximations into one

iScale as the output of Step 3 is the integer approximation of the real ratio (TB/TD) scaled by 28. This is achieved by (1) Step 2: scaling 1/TD by 214 and rounding to the nearest integer, and (2) Step 3: multiplying TB, right shifting 6 bits, and rounding to the nearest integer. It is unnecessary to perform the calculation in two steps; so the second part of our simplification proposes to combine Steps 2 and 3, such that iScale can be obtained in one step, as shown in (2).

iScale = ((TB<<9) + Sign2(TB)*Abs(TD))/(TD<<1)                                             (2)

Sign2(x)=1if x> 0−1if x< 0

## Proposed scaling process for MV prediction

Combining the two simplifications in Section 2.1 and 2.2, five steps, shown below, are needed to accomplish the scaling processing for MV prediction, one step less than that specified in the current HEVC WD.

- Clip TB and TD in the range of -32 to 31, inclusive.

- Calculate iScale = ((TB<<9) + Sign2(TB)*Abs(TD))/(TD<<1).

- Clip iScale in the range of -4096 to 4095, inclusive.

- Calculate MV2

  - Horizontal component: 	MV2_x=Sign(iScale × MV1_x) × ((|iScale × MV1_x| + 127) >> 8)

  - Vertical component: 	MV2_y=Sign(iScale × MV1_y) × ((|iScale × MV1_y| + 127) >> 8)

- Clip MV2 to the range of -32768 to 32767, inclusive. Note that the unit of MV2 is quarter-pixel.

# Experimental Results

The proposed method is tested under the common test condition [1] and the results are shown in Table 1. The simplified method provides bit-exact results of HM7.0. Detailed rate distortion data can be found in the accompanying xls files. The encoding and decoding times are not reliable since our computing grid consists of cores with various speeds.

Table 1 Summary of R-D performance in terms of BD-rate

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  | 132% |  |  |
| Dec Time[%] | 100% |  |  | 124% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 129% |  |  | 118% |  |  |
| Dec Time[%] | 121% |  |  | 113% |  |  |

# Conclusion

This proposal simplifies the scaling process for MV prediction by reducing the dynamic range of TB and TD and combining two integer approximations into one. Under the common test conditions specified in JCTVC-I1100, the simplification does not cause performance loss. We suggest adopting the proposed method into HEVC WD.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations,” Document No. JCTVC-I1100, May 2012.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document No. JCTVC-I1003, May 2012.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).