| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0161 |
| --- | --- |

| Title: | Removal of the inference of CBFY at the transform depth level zero in inter CU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a mothod to remove the inference of luma CBF when both chroma CBFs are zero and the transform depth level is zero in inter predicted CUs. It simplifies the CBF coding and decoding with BD-rate change of 0.0%, 0.1%, 0.1% and 0.1%, 0.1%, 0.1% for RAMAIN and RAHE10 respectively. It also shows 0.1%, 0.3%, 0.3% and 0.1%, 0.4%, 0.4% BD-rate change for LBMAIN and LBHE10.

# Introduction

In the current HEVC, a luma CBF is inferred 1 when the transform depth level is zero and both chroma CBFs are zero in inter predicted CUs. If none of these three CBFs is 1, the parsing/encoding process will end before parsing/encoding any CBFs since there is no_residula_data_flag which indicates that every TU including chromas does not have non-zero coefficients in the inter predicted CU. Fig. 1 depicts the process of parsing/encoding of no_residual_data_flag and CBFs.

Fig. 1 the current process of coding and derivation of CBFs

# Proposed Methods

We propose a method to remove the inference process of luma CBF when transform depth is zero and chroma CBFs are zero. It simplifies the parsing/encoding process of CBFs.

Fig. 2 the proposed process of coding of CBFs

# Experimental Results

The experimental result is shown in Table 1. The proposed method is implemented on top of HM7.0. The detailed results are described in the accompanied excel file. It removes the parsing dependency between chroma CBFs and luma CBFs with BD-rate change of 0.0%, 0.1%, 0.1% and 0.1%, 0.1%, 0.1% for RAMAIN and RAHE10 respectively. It also shows 0.1%, 0.3%, 0.3% and 0.1%, 0.4%, 0.4% BD-rate change for LBMAIN and LBHE10.

Table 1 the BD rate of the proposed method under the common condition

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.4% | 0.4% | 0.1% | 0.4% | 0.3% |
| Class C | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.3% |
| Class D | 0.0% | 0.3% | 0.6% | 0.1% | 0.8% | 0.4% |
| Class E | 0.2% | 0.4% | -0.2% | 0.1% | 0.2% | 0.6% |
| Overall | 0.1% | 0.3% | 0.3% | 0.1% | 0.4% | 0.4% |
|  | 0.1% | 0.4% | 0.3% | 0.1% | 0.4% | 0.4% |
| Class F | 0.0% | 0.5% | 0.6% | 0.1% | 0.4% | 0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Working Draft

### Transform tree syntax

| transform_tree( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( split_transform_flag[ x0L ][ y0L ][ trafoDepth ] ) { |  |
| … |  |
| transform_tree( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

cbf_luma[ x0L ][ y0L ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0L, y0L specify the location ( x0L, y0L ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to 1.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, 9th JCT-VC Meeting, Geneva, Switzerland, May. 2012.

# Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).