| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0163 |
| --- | --- |

| Title: | On DPBsize reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Lu Shuo Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2820 teruhikos@jp.sony.com Shuo.Lu@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

DPB size is critical for decoder memory size especially with high resolution sequences. This contribution proposes to reduce DPB size value in the level definition from 6 to 5. In current Random access common conditions, DPB size of 6 is needed. Coding performance with DPB size 5 is studied in the two different coding structures .It is reported that the simulation results show the DPB size reduction affects the coding performance maximally 0.1%.

# Introduction

In current Random access common conditions, DPB size of 6 is needed which consumes rather big memory size especially with high resolution sequences.

Table 1-1: DPB buffer status for current GOP size 8 (encoder_randomaccess_main.cfg)

| Decoding order | pic type | RPS | DPB | Output | RefPicList0 | RefPicList1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | - |  |  |  |  |
| 8 | B | 0 | 0, 8 |  | 0 | 0 |
| 4 | B | 0, 8 | 0, 4, 8 |  | 0 8 | 8 0 |
| 2 | B | 0, 4, 8 | 0, 2, 4, 8 | 0 | 0 4 | 4 8 |
| 1 | b | 0, 2, 4, 8 | 0, 2, 4, 8 | 1 | 0 2 | 2 4 |
| 3 | b | 0, 2, 4, 8 | 0, 2, 3, 4, 8 | 2 | 2 0 | 4 8 |
| 6 | B | 0, 2, 4, 8 | 0, 2, 3, 4, 6, 8 | 3 | 4 2 | 8 4 |
| 5 | b | 0, 4, 6, 8 | 0, 4, 5, 6, 8 | 4 | 4 0 | 6 8 |
| 7 | b | 0, 4, 6, 8 | 0, 4, 5, 6, 7, 8 | 5 | 6 4 | 8 6 |
| 16 | B | 0, 4, 6, 8 | 0, 4, 6, 7, 8, 16 | 6 | 8 6 4 0 | 8 6 4 0 |
| 12 | B | 6, 8, 16 | 6, 7, 8, 12, 16 | 7 | 8 6 | 16 8 |
| 10 | B | 6, 8, 12, 16 | 6, 8, 10, 12, 16 | 8 | 8 6 | 12 16 |
| 9 | b | 8, 10, 12, 16 | 8, 9, 10, 12, 16 | 9 | 8 10 | 10 12 |
| 11 | b | 8, 10, 12, 16 | 8, 10, 11, 12, 16 | 10 | 10 8 | 12 16 |
| 14 | B | 8, 10, 12, 16 | 8, 10, 11, 12, 14, 16 | 11 | 12 10 | 16 12 |
| 13 | b | 8, 12, 14, 16 | 8, 12, 13, 14, 16 | 12 | 12 8 | 14 16 |
| 15 | b | 8, 12, 14, 16 | 8, 12, 13, 14, 15, 16 | 13 | 14 12 | 16 14 |
| … | … | … | … | … | … | … |

Note: The pictures are not used as reference pictures but waiting for output are highlighted in yellow.

# Proposal

It is proposed to reduce the DPB size with the following GOP structures. As shown in Table 2-1 and Table 2-2, DPB size of 5 is enough for both GOP structures.

Comparing with Table 1-1, the proposed changes are highlighted in red.

## Coding structure-1 for DPB size 5

This structure is always selecting the closest pictures in play order as reference. It will be beneficial to apply this structure for sequences with fast and violent movement, in which close pictures are more related.

Figure2-1: config parameters for coding structure-1

Table 2-1: DPB buffer status for Coding Structure-1 GOP size 8

| Decoding order | pic type | RPS | DPB | Output | RefPicList0 | RefPicList1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | - |  |  |  |  |
| 8 | B | 0 | 0, 8 |  | 0 | 0 |
| 4 | B | 0, 8 | 0, 4, 8 |  | 0 8 | 8 0 |
| 2 | B | 0, 4, 8 | 0, 2, 4, 8 | 0 | 0 4 | 4 8 |
| 1 | B | 0, 2, 4, 8 | 0, 2, 4, 8 | 1 | 0 2 | 2 4 |
| 3 | B | 0, 2, 4, 8 | 0, 2, 3, 4, 8 | 2 | 2 0 | 4 8 |
| 6 | B | 2, 4, 8 | 2, 3, 4, 6, 8 | 3 | 4 2 | 8 4 |
| 5 | B | 2, 4, 6, 8 | 2, 4, 5, 6, 8 | 4 | 4 2 | 6 8 |
| 7 | B | 4, 6, 8 | 4, 5, 6, 7, 8 | 5 | 6 4 | 8 6 |
| 16 | B | 4, 6, 8 | 4, 6, 7, 8, 16 | 6 | 8 6 4 | 8 6 4 |
| 12 | B | 6, 8, 16 | 6, 7, 8, 12, 16 | 7 | 8 6 | 16 8 |
| 10 | B | 6, 8, 12, 16 | 6, 8, 10, 12, 16 | 8 | 8 6 | 12 16 |
| 9 | B | 8, 10, 12, 16 | 8, 9, 10, 12, 16 | 9 | 8 10 | 10 12 |
| 11 | B | 8, 10, 12, 16 | 8, 10, 11, 12, 16 | 10 | 10 8 | 12 16 |
| 14 | B | 10, 12, 16 | 10, 11, 12, 14, 16 | 11 | 12 10 | 16 12 |
| 13 | B | 10,12, 14, 16 | 10, 12, 13, 14, 16 | 12 | 12 10 | 14 16 |
| 15 | B | 12, 14, 16 | 12, 13, 14, 15, 16 | 13 | 14 12 | 16 14 |
| … | … | … | … | … | … | … |

## Coding structure-2 for DPB size 5

This coding structure is designed to keep pictures with lower QP as references, since pictures with lower QP contain accurate information and better image quality. It will be beneficial to apply this structure for sequences without violent movement.

Figure2-2: config parameters for Coding Structure-2

Table 2-2: DPB buffer status for Coding Structure-2 GOP size 8

| Decoding order | pic type | RPS | DPB | Output | RefPicList0 | RefPicList1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | - |  |  |  |  |
| 8 | B | 0 | 0, 8 |  | 0 | 0 |
| 4 | B | 0, 8 | 0, 4, 8 |  | 0 8 | 8 0 |
| 2 | B | 0, 4, 8 | 0, 2, 4, 8 | 0 | 0 4 | 4 8 |
| 1 | B | 0, 2, 4, 8 | 0, 2, 4, 8 | 1 | 0 2 | 2 4 |
| 3 | B | 0, 2, 4, 8 | 0, 2, 3, 4, 8 | 2 | 2 0 | 4 8 |
| 6 | B | 0, 4, 8 | 0, 3, 4, 6, 8 | 3 | 4 0 | 8 4 |
| 5 | B | 0, 4, 6, 8 | 0, 4, 5, 6, 8 | 4 | 4 0 | 6 8 |
| 7 | B | 0, 6, 8 | 0, 5, 6, 7, 8 | 5 | 6 0 | 8 6 |
| 16 | B | 0, 6, 8 | 0, 6, 7, 8, 16 | 6 | 8 6 0 | 8 6 0 |
| 12 | B | 6, 8, 16 | 6, 7, 8, 12, 16 | 7 | 8 6 | 16 8 |
| 10 | B | 6, 8, 12, 16 | 6, 8, 10, 12, 16 | 8 | 8 6 | 12 16 |
| 9 | B | 8, 10, 12, 16 | 8, 9, 10, 12, 16 | 9 | 8 10 | 10 12 |
| 11 | b | 8, 10, 12, 16 | 8, 10, 11, 12, 16 | 10 | 10 8 | 12 16 |
| 14 | B | 8, 12, 16 | 8, 11, 12, 14, 16 | 11 | 12 8 | 16 12 |
| 13 | b | 8, 12, 14, 16 | 8, 12, 13, 14, 16 | 12 | 12 8 | 14 16 |
| 15 | b | 8, 14, 16 | 8, 13, 14, 15, 16 | 13 | 14 8 | 16 14 |
| … | … | … | … | … | … | … |

# Results

Sequences from common test conditions are tested with the proposed coding structures respectively.

As mentioned above, the proper coding structures should be selected considering the sequences’ features. For high resolution sequences in test class A(4k), since A.Nebuta/A. SteamLocomotive involves fast movement, Coding structure-1 will be used, while Coding structure-2 will be applied to A.Traffic/A.PeopleOnStreet. For the other low resolution sequences in Class B/C/D, Coding structure-2 does not bring notable impact as shown in Table 3-2. The results are shown in Table 3-3 with this structure selection.

The results show that reduction of DPB size from 6 to 5 has negligible influence on BD-rate.

Table 3-1: BD-rate with Coding Structure-1

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.5% | 0.2% | 0.3% | 0.6% |
| Class B | 0.5% | 0.7% | 0.7% | 0.5% | 0.6% | 0.5% |
| Class C | 1.1% | 1.2% | 1.2% | 1.0% | 1.0% | 1.0% |
| Class D | 0.9% | 0.9% | 0.9% | 0.7% | 0.8% | 0.8% |
| Class E |  |  |  |  |  |  |
| Overall | 0.6% | 0.8% | 0.8% | 0.6% | 0.7% | 0.7% |
|  | 0.6% | 0.8% | 0.8% | 0.6% | 0.7% | 0.7% |

Table 3-2: BD-rate with Coding Structure-2

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | -0.4% | -0.4% | 0.4% | 0.2% | 0.1% |
| Class B | 0.1% | -0.1% | 0.0% | 0.2% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |

Table 3-3: BD-rate with selection from Coding Structure-1 and Coding Structure-2

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% | 0.5% |
| Class B | 0.1% | -0.1% | 0.0% | 0.2% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |

# Conclusion

The impact on the coding efficiency to reduce DPB size is investigated in this contribution. Since the coding loss is small, this contribution proposes to reduce MaxDpbSize from 6 to 5.

# Reference

[1] “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, April 2012

- [2] “Common test conditions and software reference configurations”, JCTVC-I1100,

- April, 2012

- [3] “Common conditions for reference picture marking and list construction proposals”, JCTVC-H0725,

- February, 2012

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).