| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0164 |
| --- | --- |

| Title: | AHG 13: Simplified signalling of MSB cycle |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Current signalling method of the MSB cycle information for long-term reference pictures is as the follow: delta_poc_msb_present_flag is signalled in slice header; if the flag is set, then delta_poc_msb_cycle_lt is signalled. It is assessed that delta_poc_msb_present_flag is not necessary as it actually can be inferred by inspecting the value of poc_lsb_lt such as the flag shall equal 1 if poc_lsb_lt[i] equals to poc_lsb_lt [j] for any possible j value (i.e. j is not equal i). Otherwise, the flag shall equal 0.

This contribution proposes to:

- Remove delta_poc_msb_present_flag

- Rename delta_poc_msb_cycle_lt to delta_poc_msb_cycle_idc

# Introduction and Problem Statement

Current signalling of long-term reference picture (LTRP) only uses least significant bits (LSB) of the POC to indicate the LTRP that must present in DPB (decoded picture buffer) prior to decoding the slice. Thus, it is necessary to signal most significant bit (MSB) information (i.e., in term of delta MSB cycle) to deal with case when there are more than one LTRP with the same POC LSB.

Current signalling method of the MSB cycle is as the following:

- A flag is signalled in Slice Header to indicate whether or not MSB cycle is present

- If the flag is set, MSB cycle is signalled in term of delta MSB cycle relative to MSB cycle of the current slice.

The syntax element for the above signaling method is shown in the following table. The related syntax elements are shown in red color font:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | e(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

The following example shows the use of MSB cycle information. Assume that current slice has POC 300 and there are two LTRPs, which are reference picture with POC 0 and reference picture with POC 256, which must exist in DPB. For such situation, the signaling is done as the following. Note that both reference pictures have the same POC LSB, which is 0, so that it is necessary to signal delta MSB cycle information.

- num_long_term_pics = 2

- poc_lsb_lt [0] = 0

- delta_poc_msb_present_flag [0] = 1

- delta_poc_msb_cycle_lt [0] = 1

- used_by_curr_pic_lt_flag [0] = 1

- poc_lsb_lt [1] = 0

- delta_poc_msb_present_flag [1] = 1

- delta_poc_msb_cycle_lt [1] = 0

- used_by_curr_pic_lt_flag [1] = 1

In our assessment, current signaling method may have redundancy since the delta_poc_msb_present_flag [i] might not be necessary to be signaled. This flag actually can be inferred by inspecting the value of poc_lsb_lt such as the flag shall equal 1 if poc_lsb_lt[i] equals to poc_lsb_lt [j] for any possible j value (i.e. j is not equal i). Otherwise, the flag shall equal 0.

# Proposed simplification

We propose to simplify the method of signalling MSB cycle information from two stages signalling (i.e, signal a flag first and then signal the actual delta MSB cycle if the flag is set) to only one stage signalling by removing the flag indication.

With this modification, the syntax table of elements related to LTRPs is as the following:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | e(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| delta_poc_msb_cycle_idc[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

delta_poc_msb_cycle_idc[ i ] to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. When poc_lsb_lt[ i ] is not equal to poc_lsb_lt [ j ] for any value of j in the range of 0 to num_long_term_pics − 1, inclusive, and not being equal to i, delta_poc_msb_cycle_idc [i] shall be 0.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | | poc_lsb_lt [ i − 1 ]  ! = poc_lsb_lt [ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]

else

DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The benefit of such modification is as follow:

- Simplified signaling and parsing process from two stages into one stage only.

- Required less bit count, particularly when there are same POC LSB present.

With such modification, for the example given above, the signaling can be done as the following:

- num_long_term_pics = 2

- poc_lsb_lt [0] = 0

- delta_poc_msb_cycle_idc [0] = 0

- used_by_curr_pic_lt_flag [0] = 1

- poc_lsb_lt [1] = 0

- delta_poc_msb_cycle_idc [1] = 1

- used_by_curr_pic_lt_flag [1] = 1

With such modification, for the example given above, the signalling can be done as the following:

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).