| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0165 |
| --- | --- |

| Title: | LCU-based framework with zero pixel line buffers for non-local means filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

In this contribution, non-local means (NLM) filter is applied to HM7.0 after SAO and before ALF; and LCU-based framework that allows reconstructing the decoded picture in LCU order at encoder and decoder, which offers low-delay capability, is proposed. With picture-based RDO, the average BD-rate for luma component and chroma component improves 0.38-1.79% and 0.05-1.46% respectively. With LCU-based RDO, those are 0.21-1.46% and 0.62-2.08% respectively. Subjective quality improvements were also observed.

# Introduction

In-loop filter that supports to reduce the distortion of a decoded picture is one of important coding tools for improving coding efficiency and visual quality. Reference software of HEVC HM7.0 [1] employs two kinds of in-loop filter deblocking filter and SAO. As an additional filter, ALF is also available on high coding efficiency setting. One of the important factors for in-loop filter of video codec is low-delay capability for encoding and decoding. To achieve this low-delay capability, SAO and ALF employ virtual boundary approach [2, 3] that avoids pixel reference to lower un-reconstructed LCUs.

When NLM filter [4] is utilized as one of the in-loop filters of HEVC, coding efficiency improvements and visible improvements of subjective picture quality are reported in [5, 6, 7, 8]. However, these works permit the pixel reference between the LCU belonging to the target pixel to be filtered and other LCUs. Since HEVC encodes and decodes each LCU in raster scan order, the pixel reference to un-reconstructed LCUs such as lower LCU and right neighboring LCU causes the delay of decoded picture reconstruction for decoder. Furthermore, since these works generate filter strength picture by picture, these encoders have to wait for the reconstruction of decoded picture.

In this contribution, NLM filter is applied to HM7.0 after SAO and before ALF; and LCU-based framework that allows reconstructing the decoded picture in LCU order at encoder and decoder, which offers low-delay capability, is proposed.

# Syntax and semantics

In this section, syntax and semantics for proposed NLM filter are described. The modified syntax and additional syntax are specified in Table 1 to 4. In Table 1 and 2, the difference between syntax for HM7.0 and proposal set highlighted in yellow.

slice_non_local_means_filter_flag[ cIdx ] equal to 1 specifies that NLM filter is enabled for the colour component cIdx in the current slice; equal to 0 specifies that NLM filter is disabled for the colour component cIdx in the current slice. When slice_non_local_means_filter_flag[ cIdx ] is set always equal to 1 at the encoder, the encoder can execute low-delay NLM filtering without waiting for reconstruction of neighboring LCUs. Therefore, this optimization is named LCU-based RDO. When the encoder optimizes slice_non_local_means_filter_flag[ cIdx ], the encoder have to wait for the reconstruction of decoded picture. Therefore, this optimization is named picture-based RDO.

| Table 1: Slice header syntax |  |
| --- | --- |
| slice_header( ) { | Descriptor |
| ... |  |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| slice_non_local_means_filter_flag[ i ] | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| ... |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

| Table 2: Slice data syntax |  |
| --- | --- |
| slice_data( ) { | Descriptor |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| NlmFilterStrengthSum[ 0 ] = 0 |  |
| NlmFilterStrengthSum[ 1 ] = 0 |  |
| NlmFilterStrengthSum[ 2 ] = 0 |  |
| NlmFilterCount[ 0 ] = 0 |  |
| NlmFilterCount[ 1 ] = 0 |  |
| NlmFilterCount[ 2 ] = 0 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_nlm_local_means_filter_flag[ cIdx ] ) |  |
| nlm_param( xCtb, yCtb, cIdx, NlmFilterStrengthSum[ cIdx ], NlmFilterCount[ cIdx ]) |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc == 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc == 2 && CtbAddrTS % PicWidthInCtbs == 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| }while( moreDataFlag ) |  |
| } |  |

lcu_non_local_means_filter_flag[ cIdx ][ rx ][ ry ] equal to 1 specifies that NLM filter is enabled for the current LCU; equal to 0 specifies that NLM filter is disabled for the current LCU.

lcu_non_local_means_filter_reference_mode specifies reference mode for filter strength. If the value is set equal to 0, the filter strength NlmFilterStrength[ cIdx ][ rx ][ ry ] is set equal to  NlmFilterStrength[ cIdx ][ rx − 1 ][ ry ]. Otherwise, if the value is set equal to 1, NlmFilterStrength[ cIdx ][ rx ][ ry ] is set equal to NlmFilterStrength[ cIdx ][ rx ][ ry − 1 ]. Otherwise, if the value is set equal to 2, NlmFilterStrength[ cIdx ][ rx ][ ry ] is set equal to mean value FilterStrengthSum ÷ FilterCount. Otherwise, NlmFilterStrength[ cIdx ][ rx ][ ry ] is signaled in lcu_non_local_means_filter_strength.

lcu_non_local_means_filter_strength_index specifies the index of filter strength for the current LCU. If the colour component cIdx is luma, lcu_non_local_means_filter_strength shall be in the range of 0 to 31, inclusive. Otherwise, lcu_non_local_means_filter_strength shall be in the range of 0 to 15, inclusive.

NlmFilterStrength[cIdx][rx][ry] specifies the filter strength for the current LCU. The value shall be in the range of 0.03125 to 1.0.

| Table 3: NLM filter parameter syntax |  |
| --- | --- |
| nlm_param ( rx, ry, cIdx, FilterStrengthSum, FilterCount) { | Descriptor |
| lcu_non_local_means_filter_flag[cIdx][rx][ry] | ae(v) |
| if(lcu_non_local_means_filter_flag) { |  |
| lcu_non_local_means_filter_reference_mode | ae(v) |
| if (lcu_non_local_means_filter_reference_mode == 0) |  |
| NlmFilterStrength[cIdx][rx][ry] = NlmFilterStrength[cIdx][rx−1][ry] |  |
| else if (lcu_non_local_means_filter_reference_mode == 1) |  |
| NlmFilterStrength[cIdx][rx][ry] = NlmFilterStrength[cIdx][rx][ry−1] |  |
| else if (lcu_non_local_means_filter_reference_mode == 2) |  |
| NlmFilterStrength[cIdx][rx][ry] = FilterStrengthSum  FilterCount |  |
| else{ |  |
| lcu_non_local_means_filter_strength_index | ae(v) |
| NlmFilterStrength[cIdx][rx][ry] = (lcu_non_local_means_filter_strength + 1) cIdx == 0 ? 32 : 16) |  |
| } |  |
| if(cIdx == 0) |  |
| nlm_tree ( rx, ry, CtbSize ) |  |
| FilterStrengthSum += NlmFilterStrength[cIdx][rx][ry] |  |
| FilterCount++ |  |
| } |  |
| } |  |

non_local_means_filter_split_flag specifies whether a block is split into coding units with half horizontal and vertical size.

non_local_means_filter_block_flag equal to 1 specifies that NLM filter is enabled for the block; equal to 0 specifies that NLM filter is disabled for the block.

MinSize specifies the minimum size of the quadtree block. The value is set to 16.

| Table 4: NLM tree syntax |  |
| --- | --- |
| nlm_tree ( rx, ry, blockSize ) { | Descriptor |
| non_local_means_filter_split_flag = 0 |  |
| if( blockSize > MinSize ) |  |
| non_local_means_filter_split_flag | ae(v) |
| if( non_local_means_filter_split_flag ){ |  |
| lowerBlockSize = blockSize >> 1 |  |
| nlm_tree ( rx, ry, lowerBlockSize ) |  |
| nlm_tree ( rx + lowerBlockSize, ry, lowerBlockSize ) |  |
| nlm_tree ( rx, ry + lowerBlockSize, lowerBlockSize ) |  |
| nlm_tree ( rx + lowerBlockSize, ry + lowerBlockSize, lowerBlockSize ) |  |
| } |  |
| else |  |
| non_local_means_filter_block_flag |  |
| } |  |

# Decoding description

In this section, the decoding process of proposed NLM filter is described. .

Inputs of this process are the reconstructed picture buffers prior to non-local means filter recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture buffers after non-local means filter nlmPictureL, nlmPictureCb and nlmPictureCr.

For the derivation of nlmPictureL, nlmPictureCb and nlmPictureCr, following ordered steps apply:

- All samples of nlmPictureL, nlmPictureCb and nlmPictureCr are set equal to recPictureL, recPictureCb and recPictureCr, respectively.

- The setting process for boundaries as specified in subclause 3.1 is invoked with boundary information of LCUs, slices and tiles as the inputs and nlmBoundaryMapY and nlmBoundaryMapC as the outputs.

- For every coding tree unit with coding tree block location ( rx, ry ), where rx = 0..PicWidthInCtbs − 1 and ry = 0..PicHeightInCtbs − 1, the following applies:

- The setting process for reference picture as specified in 3.2 is invoked with colour component index cIdx set equal to 0, the location ( xL, yL ) set equal to ( rx, ry ), coding tree block size nS set equal to ( 1 << Log2CtbSize ), sampleWidth set equal to pic_width_in_luma_samples, sampleHeight set equal to pic_height_in_luma_samples, and reconstructed picture buffers recPicture set equal to recPictureL as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedL and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfL as the outputs.

- The setting process for reference picture as specified in 3.2 is invoked with colour component index cIdx set equal to 1, the location ( xL, yL ) set equal to ( rx >> 1, ry >> 1 ), coding tree block size nS set equal to ( 1 << ( Log2CtbSize − 1) ), sampleWidth set equal to ( pic_width_in_luma_samples >> 1 ), sampleHeight set equal to ( pic_height_in_luma_samples >> 1 ), and reconstructed picture buffers recPicture set equal to recPictureCb as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedCb and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCb as the outputs.

- The setting process for reference picture as specified in 3.2 is invoked with colour component index cIdx set equal to 2, the location ( xL, yL ) set equal to ( rx >> 1, ry >> 1 ), coding tree block size nS set equal to ( 1 << ( Log2CtbSize − 1) ), sampleWidth set equal to ( pic_width_in_luma_samples >> 1 ), sampleHeight set equal to ( pic_height_in_luma_samples >> 1 ), and reconstructed picture buffers recPicture set equal to recPictureCr as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedCr and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCr as the outputs.

- If lcu_non_local_means_filter_flag[ 0 ][ rx ][ ry ] is equal to 1, the following applies:

  - The derivation process for SSD table of pixel template as specified in subclause 3.3.1 and the filterStrength set equal to 1.5 * NlmFilterStrength[ 2 ][ rx ][ ry ], and internalBitDepth set equal to BitDepthY as the inputs and ssdTableP as the output.

  - The derivation process for SSD table of cross-shaped template as specified in subclause 3.3.2 and the filterStrength set equal to 5.0 * NlmFilterStrength[ 0 ][ rx ][ ry ], and internalBitDepth set equal to BitDepthY as the inputs and ssdTableX as the output.

  - The filtering process for luma samples as specified in subclause 3.4.1 is invoked with colour component index cIdx, the location ( xL, yL ) set equal to ( rx, ry ), coding tree block size nS set equal to ( 1 << Log2CtbSize ), sampleWidth set equal to pic_width_in_luma_samples, sampleHeight set equal to pic_height_in_luma_samples, reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedL, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfL, and boundary map nlmBoundaryMapL as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureL as the output.

- If lcu_non_local_means_filter_flag[ 1 ][ rx ][ ry ] is equal to 1, the following applies:

  - The derivation process for SSD table of pixel template as specified in subclause 3.3.1 and the filterStrength set equal to 1.5 * NlmFilterStrength[ 1 ][ rx ][ ry ], and internalBitDepth set equal to BitDepthC as the inputs and ssdTableP as the output.

  - The filtering process for chroma samples as specified in subclause 3.4.2 is invoked with colour component index cIdx, the location ( xL, yL ) set equal to ( rx >> 1, ry >> 1 ), coding tree block size nS set equal to ( 1 << ( Log2CtbSize − 1) ), sampleWidth set equal to ( pic_width_in_luma_samples >> 1 ), sampleHeight set equal to ( pic_height_in_luma_samples >> 1 ), reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedCb, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCb, and boundary map nlmBoundaryMapC as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureCb as the output.

- If lcu_non_local_means_filter_flag[2][rx][ry] is equal to 1, the following applies:

  - The derivation process for SSD table of pixel template as specified in subclause 3.3.1 and the filterStrength set equal to 1.5 * NlmFilterStrength[2][rx][ry], and internalBitDepth set equal to BitDepthC as the inputs and ssdTableP as the output.

  - The filtering process for chroma samples as specified in subclause 3.4.2 is invoked with colour component index cIdx, the location ( xL, yL ) set equal to ( rx >> 1, ry >> 1 ), coding tree block size nS set equal to ( 1 << ( Log2CtbSize − 1) ), sampleWidth set equal to ( pic_width_in_luma_samples >> 1 ), sampleHeight set equal to ( pic_height_in_luma_samples >> 1 ), reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedCr, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCr, and boundary map nlmBoundaryMapC as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureCr as the output.

[Ed. (WJ): recPicturePaddedL, recPicturePaddedCb and recPicturePaddedCr are now a picture-size array, but actually CU size + appropriate margin is enough]

## Setting process for boundaries

Inputs to this process are boundary information of LCUs, slices and tiles.

Output of this process is boundary information nlmBoundaryMapY and nlmBoundaryMapC.

For the derivation of nlmBoundaryMapY and nlmBoundaryMapC, following ordered steps apply:

- All the location of nlmBoundaryMapY and nlmBoundaryMapC are set equal to 0.

- The location ( x, y ) of nlmBoundaryMaps belonging to lower 2-line or right 2-line from the LCU boundary is set equal to 2, and the location ( x, y ) of nlmBoundaryMapY belonging to the lower 3 to 4-line or right 3 to 4-line from the LCU boundary is set equal to 1.

- For each LCU, when the LCU is located at the boundary of slice or tile, the location ( x, y ) of nlmBoundaryMaps belonging to the LCU that is located at 2-line from slice or tile boundary is set equal to 2, and the location ( x, y ) of nlmBoundaryMapY belonging to 3 to 4-line from slice or tile boundary is set equal to 1.

Figure 1 and 2 illustrate the boundary settings for nlmBoundaryMapY and nlmBoundaryMapC, respectively.

Figure 1: Boundary setting for luma component.

Figure 2: Boundary setting for chroma component.

## Setting process for reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx.

- picture buffers recPicture for the colour component cIdx.

Outputs of this process are:

- padded picture buffers recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx.

When slice_non_local_means_filter_flag[ cIdx ] of the current slice is equal to 1, The generation process for padded reference picture as specified in subclause 3.2.1 is invoked with cIdx, ( xL, yL ), nS, sampleWidth, sampleHeight, and recPicture as the inputs and recPicturePadded as the output; and the generation process for half-shrunk reference picture as specified in subclause 3.2.2 is invoked with cIdx, ( xL, yL ), nS, sampleWidth, sampleHeight, and recPicturePadded as the inputs and recPictureHalf as the output.

### Generation process for padded reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx.

- picture buffer recPicture for the colour component cIdx,

Output of this process is padded reference picture recPicturePadded for the colour component cIdx.

The padded reference picture recPicturePadded is specified in the follows:

- All samples of the location ( x, y ) recPicturePadded[ x ][ y ] with x = xL..xE and y = yL..yE are set equal to recPicture [ x ][ y ]; where ex and ey are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1,				(3-1)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(3-2)

- If xL is equal to 0, for all samples of the vertical location y = yL..yE, recPicturePadded[ −1.. −4 ][ y ] are set equal to recPicture [ 0 ][ y ].

- If xL + nS is equal to or larger than sampleWidth, for all samples of the vertical location y = yL..yE, recPicturePadded[ sampleWidth..sampleWidth+3 ][ y ] are set equal to recPicture[ sampleWidth−1 ][ y ].

- If yL is equal to 0, for all samples of the horizontal location x = xL’..xE’, recPicturePadded[ x ][ −1..−4 ] are set equal to recPicture[ x ][ 0 ]; where xL’ and yE’ are set to:

xL’ = xL == 0 ? −4 : 0, 									(3-3)
xE’ = xL + nS <= sampleWidth ? xL + nS − 1 : sampleWidth + 3. 			(3-4)

- If yL + nS is equal to or larger than sampleHeight, for all samples of the horizontal location x = xL’..xE’, recPicturePadded[ x ][ sampleHeight..sampleHeight+3 ] are set equal to recPicture[ x ][ sampleHeight−1 ].

Figure 3 illustrates the setting of padded area corresponding to LCU location.

Figure 3: The setting of padded area corresponding to LCU location.

### Generation process for half-shrunk padded reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

Output of this process is half-shrunk padded reference picture buffer recPictureHalf for the colour component cIdx.

The half-shrunk padded reference picture buffer recPictureHalf[ x2 ][ y2 ] with x2 = bx2..ex2 and y2 = by2..ey2 are derived by:

bx2 = xL == 0 ? −1 : xL >> 1, 								(3-5)
by2 = yL == 0 ? −1 : yL >> 1, 								(3-6)
ex2 = xL + nS <= sampleWidth  ? ( ( xL + nS ) >> 1 ) − 1 : sampleWidth  >> 1, 		(3-7)
ey2 = yL + nS <= sampleHeight ? ( ( yL + nS ) >> 1 ) − 1 : sampleHeight >> 1, 		(3-8)

x = x2 << 1 												(3-9)
y = y2 << 1 												(3-10)
recPictureHalf [ x2 ][ y2 ] = recPicture[ x ][ y   ] + recPicture[ x+1 ][ y   ]

recPicture[ x ][ y+1 ] + recPicture[ x+1 ][ y+1 ] 		(3-11)

## Derivation process for SSD table

### Derivation process for SSD table of pixel template

Inputs to this process are:

- a variable filterStrength specifying the filter strength of the LCU,

- a variable internalBitDepth specifying the additional bit-depth by internal bit depth increase.

Output of this process is ssdTable specifying the SSD lookup table.

For each element of ssdTable[ n ] with n = 0..( 1 << ( internalBitDepth + 1 ) ) − 1 is set to

div = filterStrength * filterStrength,								(3-12)
ssdTable[ n ] = Min( Floor( ( n − ( 1 << internalBitDepth ) − 1 )2 ÷ div ), 1023 ). 		(3-13)

### Derivation process for SSD table of cross-shaped template

Inputs to this process are:

- a variable filterStrength specifying the filter strength of the LCU,

- a variable internalBitDepth specifying the additional bit-depth by internal bit depth increase.

Output of this process is ssdTable specifying the SSD lookup table.

For each element of ssdTable[ n ] with n = 0..( 1 << ( internalBitDepth + 1 ) ) − 1 is set to

div = filterStrength * filterStrength,								(3-14)
ssdTable[ n ] = Floor( ( ( n − ( 1 << internalBitDepth ) − 1 )2 << 16 ) ÷ div ).			(3-15)

## Filtering process

### Filtering process for luma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapY.

Output of this process is filtered picture buffer nlmPicture for the colour component cIdx.

The filtered picture buffer nlmPicture is derived in the following ordered steps.

- All the samples of the location ( x, y ) nlmPicture[ x ][ y ] with x = xL..xE and y = yL..yE are set equal to recPicturePadded[ x ][ y ]; where ex and ey are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1, 			(3-16)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(3-17)

- The filter block derived by non_local_means_filter_split_flag and non_local_means_filter_block_flag is split into 2x2 sub-blocks. For each 2x2 sub-block with the location ( xS, yS ) specifying the top-left the sample the following steps apply:

- The derivation process for the index of activity and gradient as specified in 3.5.1 is invoked with recPictureHalf, ( xS, yS ), and nlmBoundaryMapY as the inputs and the activity index ActivityIdx and DirectionIdx as the outputs.

- If ActivityIdx is equal to 1, for each location of the sample ( xP, yP ) in 2x2 sub-block, the following steps apply:

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[ xP ][ yP ].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to DirectionIdx are illustrated in Figure X), sumWeights and sumSamples are derived as follows.

ibdiMax = ( 1 << internalBitDepth ) −1,						(3-18)
tableIdx = recPicturePadded[ xP2 ][ yP2 ]

− recPicturePadded[ xS ][ yS ] + ibdiMax,				(3-19)

weight = expTable[ ssdTable[ expIdx ] ],						(3-20)
sumWeights += weight,								(3-21)
sumSamples += weight * recPicturePadded[ xP2 ][ yP2 ]. 			(3-22)

- nlmPicture[ xP ][ yP ] is derived by nlmPicture[ xP ][ yP ] = sumSamples / sumWeights.

- Otherwise, if ActivityIdx is equal to 2, for each location of the sample ( xP, yP ) in 2x2 sub-block, the following steps apply:

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[ xP ][ yP ].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to DirectionIdx are illustrated in Figure 4), sumWeights and sumSamples are derived as follows.

ibdiMax = ( 1 << internalBitDepth ) −1,						(3-23)
tableIdx0 = recPicturePadded[ xP2 − 1 ][ yP2 − 1 ]

− recPicturePadded[ xS − 1 ][ yS − 1 ] + ibdiMax ), 			(3-24)

tableIdx1 = recPicturePadded[ xP2 + 1 ][ yP2 − 1 ]

− recPicturePadded[ xS + 1 ][ yS − 1 ] + ibdiMax ), 			(3-25)

tableIdx2 = recPicturePadded[ xP2 ][ yP2 ]

− recPicturePadded[ xS ][ yS ] + ibdiMax ), 				(3-26)

tableIdx3 = recPicturePadded[ xP2 − 1 ][ yP2 + 1 ]

− recPicturePadded[ xS − 1 ][ yS + 1 ] + ibdiMax ), 			(3-27)

tableIdx4 = recPicturePadded[ xP2 + 1 ][ yP2 + 1 ]

− recPicturePadded[ xS + 1 ][ yS + 1 ] + ibdiMax ), 			(3-28)

weight = expTable[ Min( ( ssdTable[ tableIdx0 ] + ssdTable[ tableIdx1 ]

+ ssdTable[ tableIdx2 ] + ssdTable[ tableIdx3 ]

+ ssdTable[ tableIdx4 ] ) >> 16, 1023 ) ], 		(3-29)

sumWeights += weight,								(3-30)
sumSamples += weight * recPicturePadded[ xP2 ][ yP2 ]. 			(3-31)

- nlmPicture[ xP ][ yP ] is derived by nlmPicture[ xP ][ yP ] = sumSamples / sumWeights.

NOTE –expTable[i] ( i = 0..1023 ) is assigned by

expTable[ i ]= 0;i=1023Floor( 255*exp( −i÷198.0 )+0.5 );otherwise

Figure 4: Relative locations (xP2, yP2) and the location (xP, yP) corresponding to GradientIdx.

### Filtering process for chroma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapC.

Output of this process is filtered picture buffer nlmPicture for the colour component cIdx.

The filtered picture buffer nlmPicture is derived in the following ordered steps.

- All the samples of the location ( x, y ) nlmPicture[ x ][ y ] with x = xL..xE and y = yL..yE are set equal to recPicturePadded[ x ][ y ]; where ex and ey are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1, 			(3-32)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(3-33)

- The filter block derived by non_local_means_filter_split_flag and non_local_means_filter_block_flag is split into 2x2 sub-blocks. For each 2x2 sub-block with the location ( xS, yS ) specifying the top-left the sample the following steps apply:

- The derivation process for activity index of chroma samples as specified in 3.5.2 is invoked with recPictureHalf, ( xS, yS ), and nlmBoundaryMapC as the inputs and the activity index ActivityIdx as the output.

- If ActivityIdx is equal to 1, for each location of the sample ( xP, yP ) in 2x2 sub-block, the following steps apply:

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[ xP ][ yP ].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) are illustrated in Figure 5), sumWeights and sumSamples are derived as follows.

bitDepthMax = ( 1 << internalBitDepth ) −1, 					(3-34)
tableIdx = recPicturePadded[ xP2 ][ yP2 ]

− recPicturePadded[ xS ][ yS ] + ibdiMax ), 				(3-35)

weight = expTable[ ssdTable[ tableIdx ] ], 					(3-36)
sumWeights += weight, 							(3-37)
sumSamples += weight * recPicturePadded[ xP2 ][ yP2 ]. 			(3-38)

- nlmPicture[ xP ][ yP ] is derived by nlmPicture[ xP ][ yP ] = sumSamples / sumWeights.

Figure 5: Relative locations (xP2, yP2) and the location (xP, yP) corresponding to GradientIdx.

## Derivation process for activity index

### Derivation process for activity index and direction index of luma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xS, yS ) specifying the 2x2 sub-block location,

- a variables internalBitDepth specifying the bit-depth for the chroma component,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapY.

Outputs of this process are:

- a variable activity index ActivityIdx,

- a variable direction index DirectionIdx.

If nlmBoundaryMapY[ xS ][ yS ] is equal to 2, the index of activity ActivityIdx is assigned to 0. Otherwise, the following steps apply:

- The index of activity index buffer ActivityIdxBuf is assigned to:

x2 = xS >> 1,										(3-39)
y2 = yS >> 1,										(3-40)
dx = recPictureHalf[cIdx][ x2+1 ][ y2 ] − recPictureHalf[ cIdx ][ x2−1 ][ y2 ], 		(3-41)
dy = recPictureHalf[cIdx][ x2 ][ y2+1 ] − recPictureHalf[ cIdx ][ x2] [ y2−1 ], 		(3-42)
ActivityIdxBuf = ( Abs( dx ) + Abs( dy )) >> ( internalBitDepth – 8 ) < 4 ? 0 : 1. 		(3-43)

- If ActivityIdxBuf is equal to 0, ActivityIdx is assigned to 0.

- Otherwise, if ActivityIdxBuf is larger than or equal to 50 and if nlmBoundaryMapY[ xS ][ yS ] is equal to 1, ActivityIdxBuf is assigned to 1,and activity index ActivityIdx and direction index DirectionIdx are derived by:

ActivityIdx = ActivityIdxBuf,								(3-44)
Dir = 1024 * dx / dy,									(3-45)
DirectionIdx= 556789012345;;;;;;;;;;;;dy==0Dir<−5120Dir<−1707Dir<−1024Dir<−614Dir<−205Dir<205Dir<614Dir<1024Dir<1707Dir<5120otherwise.							(3-46)

### Derivation process for activity index of chroma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xS, yS ) specifying the 2x2 sub-block location,

- a variables internalBitDepth specifying the bit-depth for the chroma component,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapC.

Output of this process is the index of activity ActivityIdx for the colour component cIdx.

If nlmBoundaryMapC[ xS ][ yS ] is equal to 2, the index of activity ActivityIdx is assigned to 0. Otherwise, ActivityIdx is assigned to:

x2 = xS >> 1, 										(3-47)
y2 = yS >> 1, 										(3-48)
dx = recPictureHalf[cIdx][ x2+1 ][ y2 ] − recPictureHalf[ cIdx ][ x2−1 ][ y2 ], 		(3-49)
dy = recPictureHalf[cIdx][ x2 ][ y2+1 ] − recPictureHalf[ cIdx ][ x2] [ y2−1 ], 		(3-50)
ActivityIdx = ( Abs( dx ) + Abs( dy )) >> ( internalBitDepth – 8 ) < 4 ? 0 : 1. 		(3-51)

# Experimental results

In this section, objective/subjective evaluations are described.

## Objective evaluation

The source code was compiled by Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i5 2500K (4.2GHz) and 16GB RAM. The experimental results described in Table 5 to 8 present coding efficiencies calculated by BD-rate [9] and computational complexities compared to HM7.0 anchors. The results of picture-based RDO described in Table 5 and 7, and the results of LCU-based RDO described in Table 6 and 8, respectively.

| Table 5: BD-Rate and complexity results for picture-based RDO (mandatory). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.3% | 0.0% | 0.0% |
| Class B | -0.1% | 0.1% | -0.3% | -0.2% | 0.0% | -0.4% |
| Class C | -0.2% | -0.5% | -0.9% | -0.5% | -0.9% | -1.4% |
| Class D | -0.1% | -0.4% | -1.0% | -0.2% | -0.5% | -1.0% |
| Class E | -0.2% | -1.2% | -1.0% | -0.9% | 0.5% | 0.4% |
| Overall | -0.1% | -0.4% | -0.7% | -0.4% | -0.2% | -0.5% |
|  | -0.1% | -0.4% | -0.7% | -0.4% | -0.2% | -0.4% |
| Class F | -1.6% | -0.8% | -1.5% | -0.6% | -0.8% | -1.0% |
| Enc Time[%] | 110% |  |  | 109% |  |  |
| Dec Time[%] | 110% |  |  | 118% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.2% | -0.1% | -0.9% | -0.8% | -0.1% |
| Class B | -0.3% | 0.1% | -0.1% | -0.7% | -0.5% | -0.5% |
| Class C | -0.4% | -0.3% | -0.5% | -0.8% | -1.5% | -2.0% |
| Class D | -0.2% | 0.1% | -0.4% | -0.4% | -0.8% | -1.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | 0.0% | -0.3% | -0.7% | -0.9% | -1.0% |
|  | -0.4% | -0.1% | -0.3% | -0.7% | -0.8% | -1.0% |
| Class F | -1.4% | -0.9% | -1.4% | -0.6% | -1.0% | -1.2% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 108% |  |  | 111% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.1% | -0.4% | -0.8% | -1.2% | -0.9% |
| Class C | -0.6% | -0.9% | -1.0% | -0.9% | -2.7% | -3.2% |
| Class D | -0.5% | -0.1% | -0.8% | -0.6% | -1.7% | -2.3% |
| Class E | -0.7% | -1.0% | -1.2% | -1.2% | -1.0% | -0.4% |
| Overall | -0.5% | -0.5% | -0.8% | -0.8% | -1.6% | -1.7% |
|  | -0.5% | -0.5% | -0.8% | -0.8% | -1.6% | -1.6% |
| Class F | -1.4% | -1.1% | -0.8% | -0.6% | -1.0% | -1.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 0% |  |  | 111% |  |  |

| Table 6: BD-Rate and complexity results for LCU-based RDO (mandatory). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.8% | -1.0% | -0.2% | -0.3% | -0.3% |
| Class B | 0.1% | -0.6% | -1.0% | -0.1% | -0.4% | -0.7% |
| Class C | -0.1% | -0.6% | -1.0% | -0.5% | -1.0% | -1.5% |
| Class D | -0.1% | -0.6% | -1.1% | -0.2% | -0.6% | -1.0% |
| Class E | -0.1% | -1.7% | -2.0% | -0.6% | -0.3% | -0.2% |
| Overall | 0.0% | -0.8% | -1.2% | -0.3% | -0.5% | -0.8% |
|  | 0.0% | -0.8% | -1.2% | -0.3% | -0.5% | -0.7% |
| Class F | -1.6% | -1.3% | -2.0% | -0.5% | -1.1% | -1.4% |
| Enc Time[%] | 110% |  |  | 109% |  |  |
| Dec Time[%] | 111% |  |  | 119% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.3% | -1.3% | -0.7% | -1.4% | -0.7% |
| Class B | 0.0% | -0.6% | -1.2% | -0.5% | -1.2% | -1.1% |
| Class C | -0.2% | -0.6% | -1.0% | -0.6% | -1.7% | -2.3% |
| Class D | 0.0% | 0.1% | -0.6% | -0.3% | -1.0% | -1.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.6% | -1.0% | -0.5% | -1.3% | -1.5% |
|  | -0.2% | -0.6% | -1.0% | -0.5% | -1.3% | -1.4% |
| Class F | -1.2% | -1.4% | -2.2% | -0.4% | -1.4% | -1.6% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 110% |  |  | 113% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.2% | -2.4% | -0.4% | -2.3% | -2.3% |
| Class C | -0.3% | -1.1% | -1.2% | -0.7% | -2.8% | -3.4% |
| Class D | -0.3% | -0.3% | -1.3% | -0.4% | -2.2% | -2.6% |
| Class E | 0.5% | -0.8% | -2.1% | -0.1% | -2.8% | -2.0% |
| Overall | -0.1% | -0.9% | -1.8% | -0.5% | -2.5% | -2.6% |
|  | -0.1% | -0.9% | -1.7% | -0.4% | -2.5% | -2.4% |
| Class F | -1.0% | -1.1% | -1.1% | -0.1% | -0.9% | -1.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 0% |  |  | 114% |  |  |

| Table 7: BD-Rate and complexity results for picture-based RDO (all). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.15% | -0.29% | -0.31% | -0.28% | -0.03% | -0.02% |
| Class B | -0.07% | 0.05% | -0.30% | -0.24% | 0.01% | -0.37% |
| Class C | -0.16% | -0.51% | -0.93% | -0.49% | -0.93% | -1.43% |
| Class D | -0.09% | -0.42% | -1.02% | -0.23% | -0.53% | -0.96% |
| Class E | -0.21% | -1.21% | -1.05% | -0.91% | 0.48% | 0.45% |
| Class F | -1.62% | -0.84% | -1.50% | -0.56% | -0.79% | -0.97% |
| Overall | -0.38% | -0.49% | -0.82% | -0.39% | -0.22% | -0.51% |
|  | -0.37% | -0.51% | -0.83% | -0.39% | -0.19% | -0.44% |
| Enc Time[%] | 110.33% |  |  | 108.88% |  |  |
| Dec Time[%] | 111.35% |  |  | 117.77% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.80% | -0.17% | -0.08% | -0.92% | -0.80% | -0.07% |
| Class B | -0.30% | 0.13% | -0.14% | -0.75% | -0.49% | -0.48% |
| Class C | -0.39% | -0.32% | -0.51% | -0.75% | -1.52% | -2.02% |
| Class D | -0.17% | 0.12% | -0.42% | -0.44% | -0.83% | -1.56% |
| Class E |  |  |  |  |  |  |
| Class F | -1.38% | -0.86% | -1.45% | -0.61% | -1.00% | -1.21% |
| Overall | -0.41% | -0.05% | -0.28% | -0.72% | -0.89% | -1.00% |
|  | -0.39% | -0.08% | -0.31% | -0.73% | -0.84% | -0.96% |
| Enc Time[%] | 102.33% |  |  | 102.68% |  |  |
| Dec Time[%] | 109.00% |  |  | 111.14% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.32% | -0.11% | -0.40% | -0.76% | -1.17% | -0.89% |
| Class C | -0.56% | -0.93% | -0.98% | -0.88% | -2.72% | -3.21% |
| Class D | -0.48% | -0.12% | -0.84% | -0.61% | -1.65% | -2.34% |
| Class E | -0.71% | -0.97% | -1.23% | -1.21% | -1.01% | -0.38% |
| Class F | -1.42% | -1.06% | -0.82% | -0.56% | -1.01% | -1.22% |
| Overall | -0.68% | -0.59% | -0.81% | -0.78% | -1.52% | -1.63% |
|  | -0.68% | -0.59% | -0.78% | -0.78% | -1.45% | -1.55% |
| Enc Time[%] | 101.56% |  |  | 101.72% |  |  |
| Dec Time[%] | 109.72% |  |  | 111.62% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.25% | -0.95% | -0.91% | -1.36% | -1.39% | -1.41% |
| Class C | -1.46% | -1.55% | -1.67% | -1.28% | -3.11% | -3.70% |
| Class D | -0.91% | -0.68% | -1.46% | -0.87% | -2.69% | -3.30% |
| Class E | -2.40% | -1.98% | -1.91% | -2.66% | -1.53% | -1.01% |
| Class F | -1.95% | -2.02% | -1.60% | -0.92% | -1.76% | -1.60% |
| Overall | -1.79% | -1.38% | -1.46% | -1.35% | -2.09% | -2.22% |
|  | -1.78% | -1.43% | -1.48% | -1.36% | -2.02% | -2.15% |
| Enc Time[%] | 102.41% |  |  | 102.55% |  |  |
| Dec Time[%] | 112.84% |  |  | 114.27% |  |  |

| Table 8: BD-Rate and complexity results for LCU-based RDO (all). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.84% | -1.00% | -0.20% | -0.27% | -0.30% |
| Class B | 0.08% | -0.58% | -0.96% | -0.08% | -0.43% | -0.72% |
| Class C | -0.14% | -0.61% | -1.04% | -0.47% | -1.01% | -1.51% |
| Class D | -0.05% | -0.57% | -1.10% | -0.20% | -0.65% | -1.04% |
| Class E | -0.09% | -1.73% | -1.97% | -0.63% | -0.30% | -0.23% |
| Class F | -1.59% | -1.27% | -2.01% | -0.52% | -1.09% | -1.36% |
| Overall | -0.30% | -0.88% | -1.30% | -0.29% | -0.54% | -0.78% |
|  | -0.30% | -0.90% | -1.31% | -0.28% | -0.51% | -0.74% |
| Enc Time[%] | 109.81% |  |  | 108.55% |  |  |
| Dec Time[%] | 112.24% |  |  | 118.78% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.66% | -1.31% | -1.28% | -0.72% | -1.41% | -0.69% |
| Class B | -0.04% | -0.61% | -1.18% | -0.49% | -1.15% | -1.10% |
| Class C | -0.18% | -0.61% | -0.99% | -0.60% | -1.70% | -2.30% |
| Class D | 0.01% | 0.06% | -0.61% | -0.30% | -1.00% | -1.83% |
| Class E |  |  |  |  |  |  |
| Class F | -1.22% | -1.43% | -2.17% | -0.37% | -1.44% | -1.59% |
| Overall | -0.21% | -0.62% | -1.02% | -0.52% | -1.31% | -1.46% |
|  | -0.20% | -0.64% | -1.04% | -0.53% | -1.27% | -1.42% |
| Enc Time[%] | 102.19% |  |  | 102.55% |  |  |
| Dec Time[%] | 110.62% |  |  | 112.89% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.10% | -1.19% | -2.38% | -0.44% | -2.27% | -2.30% |
| Class C | -0.32% | -1.08% | -1.24% | -0.71% | -2.85% | -3.36% |
| Class D | -0.26% | -0.27% | -1.28% | -0.45% | -2.15% | -2.57% |
| Class E | 0.50% | -0.76% | -2.06% | -0.12% | -2.82% | -1.97% |
| Class F | -0.97% | -1.08% | -1.14% | -0.05% | -0.85% | -1.13% |
| Overall | -0.26% | -0.90% | -1.64% | -0.37% | -2.16% | -2.28% |
|  | -0.25% | -0.93% | -1.61% | -0.37% | -2.16% | -2.12% |
| Enc Time[%] | 101.53% |  |  | 101.87% |  |  |
| Dec Time[%] | 111.53% |  |  | 113.72% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.06% | -2.08% | -3.36% | -1.02% | -2.74% | -2.83% |
| Class C | -1.29% | -1.75% | -1.96% | -1.03% | -3.37% | -3.88% |
| Class D | -0.79% | -0.58% | -1.33% | -0.69% | -2.25% | -3.26% |
| Class E | -1.59% | -2.38% | -2.57% | -1.69% | -4.20% | -3.23% |
| Class F | -1.46% | -1.73% | -0.96% | -0.53% | -2.38% | -2.82% |
| Overall | -1.46% | -1.69% | -2.08% | -0.96% | -2.92% | -3.18% |
|  | -1.45% | -1.70% | -2.12% | -0.96% | -2.84% | -3.04% |
| Enc Time[%] | 102.34% |  |  | 102.63% |  |  |
| Dec Time[%] | 114.55% |  |  | 116.44% |  |  |

## Subjective evaluation

Figure 6 to 9 demonstrate the frames where subjective picture quality differences are distinguished.

Figure 6: BasketballDrive (class B), LD-P (Main Profile), QP=37, POC #335,
Anchor: 17880 [bits], PSNR-Y 32.92 [dB], PSNR-U 40.25 [dB], PSNR-V 39.68 [dB],
Proposed: 17328 [bits], PSNR-Y 33.58 [dB], PSNR-U 40.30 [dB], PSNR-V 39.82 [dB].

Figure 7: BasketballDrill (class C), LD-P (Main Profile), QP=37, POC #270,
Anchor: 5952 [bits], PSNR-Y 31.28 [dB], PSNR-U 36.13 [dB], PSNR-V 36.06 [dB],
Proposed: 6624 [bits], PSNR-Y 31.53 [dB], PSNR-U 36.27 [dB], PSNR-V 36.28 [dB].

Figure 8: KristenAndSara (class E), LD-P (Main Profile), QP=37, POC #141,
Anchor: 17880 [bits], PSNR-Y 32.92 [dB], PSNR-U 40.25 [dB], PSNR-V 39.68 [dB],
Proposed: 17328 [bits], PSNR-Y 33.58 [dB], PSNR-U 40.30 [dB], PSNR-V 39.82 [dB].

Figure 9: SlideEditing (class F), LD-B (Main Profile), QP=37, POC #001, 
Anchor: 368 [bits], PSNR-Y 32.43 [dB], PSNR-U 37.81 [dB], PSNR-V 37.22 [dB],
Proposed: 320 [bits], PSNR-Y 33.04 [dB], PSNR-U 37.88 [dB], PSNR-V 37.30 [dB].

# Conclusion

In this contribution, NLM filter was applied to HM7.0 after SAO and before ALF, and LCU-based framework that allows reconstructing the decoded picture in LCU order at encoder and decoder, which offers low-delay capability, was proposed. With picture-based RDO, the average BD-rate for luma component and chroma component improved 0.38-1.79% and 0.05-1.46% respectively. With LCU-based RDO, those were 0.21-1.46% and 0.62-2.08% respectively. Subjective quality improvements were also observed. Thanks to KDDI’s cross-verification [10] for confirming the coding results.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803_d5.doc ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, and S. Lei, “Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding,” JCTVC-F054, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, and S. Lei, “Sample Adaptive Offset with Zero Pixel Line Buffers for LCU-based Decoding,” JCTVC-F055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- A. Buades, B. Coll, and J. M. Morel, “A Non Local Algorithm for Image Denoising,” Proc. IEEE Int. Conf. on Computer Vision and Pattern Recognition, vol. 2, pp. 60-65, June 2005.

- M. Matsumura, Y. Bandoh, S. Takamura, H. Jozawa, “In-loop filter based on non-local means filter,” JCTVC-E206, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- M. Matsumura, Y. Bandoh, S. Takamura, H. Jozawa, “Modifications of in-loop filter based on non-local means filter,” JCTVC-F047, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- M. Matsumura, S. Takamura, H. Jozawa, “CE8.h: CU-based ALF with non-local means filter,” JCTVC-G235, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- M. Matsumura, S. Takamura, A. Shimizu, “Picture quality evaluation of non-local means post filtering by SSIM,” JCTVC-I0055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” VCEG-AI11, ITU-T SG16 Q.6, 35th VCEG meeting, Berlin, Germany, July 2008.

- T. Yoshino, K. Kawamura, S. Naito, “Cross-check of LCU-based framework for non-local means filter (JCTVC-J0165),” JCTVC-J0190, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).