| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0168 |
| --- | --- |

| Title: | AHG11: Syntax harmonisation of the I_PCM and TransQuantBypass modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS, Patrice ONNO, Guillaume LAROCHE, Tangi POIRIER Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France Masato SHIMA 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Research Centre France, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution proposes several syntax harmonisations of the I_PCM and TransQuantBypass (also known as Lossless) modes in HM7.  Three points are addressed: 1) control of the modes at the SPS and PPS levels, 2) granularity of the TransQuantBypass mode, 3) location of the pcm_flag signaling. In terms of coding efficiency, the changes only impact the Lossless coding. Simulations reportedly show that limiting the usage of the TransQuantBypass mode to CU size from 8x8 to 32x32 leads to minor losses in between 0% and 0.2% for Y, U and V, depending on the configuration, while noticeably reducing the encoding run time (16% in pure lossless scenario, from 2 to 8% in mixed lossy-lossless scenario).

# Introduction

The HM7.0 specifies two specific modes, I_PCM and TransQuantBypass modes.

- I_PCM mode is an INTRA mode consisting of skipping the prediction, quantization and transform, and entropy coding steps. This mode is relevant for blocks for which compression turns out to be more costly in rate than just doing no compression.

- The TransQuantBypass mode, actually emulating Lossless coding, skips the quantization, transform and loop filtering. It enables to code the signal losslessly.

These two modes have therefore several important conceptual commonalities. There are usable for very high quality video applications. They both apply bypassing of the transform and quantization. But they also differ in several aspects that could be harmonized.

The following table summarizes the design of these modes.

| Type | I_PCM | TransQuantBypass |
| --- | --- | --- |
| Activation | SPS pcm_enabled_flag | PPS transquant_bypass_enable_flag |
| Granularity | 2Nx2N PU min/max size in SPS log2_min_pcm_coding_block_size_minus3 log2_diff_max_min_pcm_coding_block_size | CU No restriction |
| Location | first SE of intra PU pcm_flag | first SE of CU cu_transquant_bypass_flag |
| Loop filter | On/Off flag in SPS pcm_loop_filter_disable_flag | Bypassed |
| Transform | Bypassed | Bypassed |
| Quantization | Bypassed | Bypassed |
| Prediction | No | Intra / Inter |

The goal of this proposal is to harmonize part of the design of these two modes with the 3 proposals described hereafter.

# Proposals

## Proposal 1: control of the modes at the SPS and PPS levels

Currently the activation of the I_PCM mode is made at the SPS level, while the TransQuantBypass mode is activated at the PPS level. It is proposed to harmonize this design for both modes, by using a flag in the SPS and in the PPS for each mode. This enables controlling the usage of the modes at the Sequence and Picture levels.

Proposal:

- Use of one flag in the SPS to activate/de-activate the TransQuantBypass mode per sequence  and of one flag in the PPS to activate/de-activate the TransQuantBypass mode per picture

  - sps_transquant_bypass_enabled_flag

  - pps_transquant_bypass_enabled_flag

- Use of one flag in the SPS to activate/de-activate the I_PCM mode per sequence  and of one flag in the PPS to activate/de-activate the I_PCM mode per picture

  - sps_pcm_enabled_flag

  - pps_pcm_enabled_flag

- Constraints on the value of these flags:

  - The flag is PPS shall be equal to 0 when the flag is SPS is equal to 0

## Proposal 2: granularity of the TransQuantBypass mode

Currently the TransQuantBypass mode applies to any CU, whatever its size. It is proposed to control the size of the CUs supporting the TransQuantBypass mode, as currently done for the I_PCM mode.

Proposal:

- Use of two syntax elements in the SPS to control the min and max sizes of CUs supporting the TransQuantBypass mode.

2 options:

  - different syntax elements for the I_PCM and for the TransQuantBypass modes

    - log2_min_pcm_coding_block_size_minus3 and log2_diff_max_min_pcm_coding_block_size

    - log2_min_transquant_bypass_coding_block_size_minus3 and log2_diff_max_min_transquant_bypass_coding_block_size

  - preferred: common syntax elements applying for both modes

    - generalization of current pcm syntax elements

## Proposal 3: location of the pcm_flag signaling

pcm_flag is currently signaled in the prediction_unit syntax. In order to have a consistent signaling of pcm_flag and transquant_bypass_enable_flag, it is proposed to displace the pcm_flag into the coding_unit syntax.

Proposal:

- Move the signaling of pcm_flag from the prediction_unit() syntax to the coding_unit() syntax

# Experiments

The proposed changes may have an impact on the performance of the Lossless mode. The performance of I_PCM is not at all impacted by the proposed changes. The following experiments have been made, based on the test conditions specified in the AHG11. The reference software is the HM7.1.

Test Scenarios:

- Frame level Lossless coding: All frames are lossless coded.

  - QP setting: QP = 0

- Region based lossless coding: In each frame, three regions with the following top-left and bottom right coordinates are lossless coded.

  - Region 1: (272, 32) , (1119, 143)

  - Region 2: (208, 352), (495, 623)

  - Region 3: (1152,  192+16*floor(POC/100)),  (1231, 223+16*floor(POC/100))

Two F-class 720p sequences, SlideShow and SlideEditing.

QP setting: QP=22, 27, 32, 37

In addition, the following settings have been used:

- LosslessCuEnabled                 : 1

- TransquantBypassEnableFlag		: 1

- CUTransquantBypassFlagValue		: 1

Notes:

- A next revision will also report results based on HM7.0-dev-r2461, with parameter LosslessCuEnabled set to 0, as lately recommended by the AHG11.

- For Scenario 1, a next revision will also report additional results for classes A-E, as lately recommended by the AHG11.

For both scenarios, the TransQuantBypass mode has been tested with CU size from 8x8 to 64x64, versus CU size from 8x8 to 32x32.

## Scenario 1

The following table evaluates the impact of limiting the TransQuantBypass mode to CUs of size 8x8 to 32x32 in the pure lossless scenario. An average rate variation of +0.11% is observed, with an average encoding time of 84% compared to HM7.1.

Note: A next revision will also incorporate results for classes A to E.

| configuration | sequence | Delta rate | Enc Time |
| --- | --- | --- | --- |
| AI Main | S14 - SlideEditing | 0.01% | 88% |
|  | S15 - SlideShow | 0.12% | 89% |
| RA Main | S14 - SlideEditing | 0.20% | 80% |
|  | S15 - SlideShow | 0.28% | 83% |
| LB Main | S14 - SlideEditing | 0.20% | 81% |
|  | S15 - SlideShow | 0.13% | 82% |
| Average Main |  | 0.15% |  |
| AI HE10 | S14 - SlideEditing | 0.00% | 88% |
|  | S15 - SlideShow | 0.07% | 91% |
| RA HE10 | S14 - SlideEditing | 0.14% | 81% |
|  | S15 - SlideShow | 0.02% | 82% |
| LB HE10 | S14 - SlideEditing | 0.13% | 80% |
|  | S15 - SlideShow | 0.09% | 81% |
| Average HE10 |  | 0.07% |  |
| Average All |  | 0.11% | 84% |

## Scenario 2

The following table evaluates the impact of limiting the TransQuantBypass mode to CUs of size 8x8 to 32x32 in the region-based lossless scenario. An average BDR variation of 0.2% for the Main configuration is observed and of 0% for the HE10 configuration. A noticeable encoder time decrease is observed, from 2 to 8% depending on the configurations.

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| Class F | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 92% |  |  | 92% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| Class F | 0.3% | 0.4% | 0.4% | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| Average | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |

# Conclusion

Three changes, aiming at harmonizing the design of I_PCM and TransQuantBypass modes, are proposed in this contribution:

- Use of flags at the SPS and PPS levels to control the activation of the I_PCM and TransQuantBypass modes

- Control of the size of the CUs supporting the TransQuantBypass mode, as already done for the I_PCM mode. This has a minor impact on coding performance, while giving more flexibility to the usage of the TransQuantBypass mode.

- Displacing the pcm_flag signaling from PU to CU level

It is proposed to adopt these 3 proposals in the HEVC DIS.

# Syntax and semantics changes

We consider in the following syntax changes for only the preferred options of common syntax elements for I_PCM and TransQuantBypass modes.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_pcm_enabled_flag | u(1) |
| sps_transquant_bypass_enabled_flag | u(1) |
| … |  |
| if( sps_pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| … |  |
| if( sps_pcm_enabled_flag ¦ ¦ sps_transquant_bypass_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| … |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_pcm_bypass_enable_flag | u(1) |
| pps_transquant_bypass_enable_flag | u(1) |
| … |  |
| } |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode = = MODE_INTRA && PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| … |  |
| } |  |
| } |  |

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| … |  |
| } else { /* MODE_INTER */ |  |
| … |  |
| } |  |
| } |  |

7.4.2.2	Sequence parameter set RBSP semantics

…

sps_pcm_enabled_flag equal to 0 specifies that PCM data shall not be present in the video sequence.

sps_transquant_enabled_flag equal to 0 specifies that no CU with the syntax element  cu_transquant_bypass_flag equal to 1 shall be present in the video sequence.

…

log2_min_pcm_coding_block_size_minus3 + 3 specifies the minimum size of I_PCM coding blocks and of coding blocks with the syntax element  cu_transquant_bypass_flag equal to 1.

The variable Log2MinIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3. The variable Log2MinIPCMCUSize shall be equal or less than Min( Log2CtbSize, 5 ).

log2_diff_max_min_pcm_coding_block_size specifies the difference between the maximum and minimum size of I_PCM coding blocks and of coding blocks with the syntax element  cu_transquant_bypass_flag equal to 1.

The variable Log2MaxIPCMCUSize is set equal to log2_min_pcm_coding_block_size_minus3 + 3 + log2_diff_max_min_pcm_coding_block_size. The variable Log2MaxIPCMCUSize shall be equal or less than Min( Log2CtbSize, 5 ).

…

7.4.2.3	Picture parameter set RBSP semantics

…

pps_pcm_enable_flag specifies whether a pcm_flag syntax element is coded or not. If pps_pcm_enable_flag is equal to 1, pcm_flag is coded, otherwise it is not coded. When the syntax element sps_pcm_enable_flag is equal to 0, pps_pcm_enable_flag shall be equal to 0.

pps_transquant_bypass_enable_flag specifies whether a cu_transquant_bypass_flag syntax element is coded or not. If transquant_bypass_enable_flag is equal to 1, cu_transquant_bypass_flag is coded, otherwise it is not coded. When the syntax element sps_transquant_bypass_enable_flag is equal to 0, pps_transquant_bypass_enable_flag shall be equal to 0.

…

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).