| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0169r2 |
| --- | --- |

| Title: | AHG11: Loop filtering control for I_PCM and TransQuantBypass modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS, Patrice ONNO, Guillaume LAROCHE, Tangi POIRIER Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France Masato SHIMA 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Research Centre France, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution proposes one change regarding the loop filtering process in the current HEVC draft 7 specifications. It consists in deactivating the loop filtering (SAO and ALF) process at the LCU level when the LCU contains Lossless blocks or I_PCM blocks and that loop filtering is disabled for IPCM blocks.

# Introduction

Following the discussions from the BoG on alternative coding modes, reported in JCTVC-J0512, only one of the different proposals from the initial version of J0169 was considered as relevant. This release version thus focuses on this single relevant proposal, for further discussion with the JCTVC experts.

# Proposal: Enabling loop filtering at the LCU (CTB) level

In the current HM 7.0 specifications, the loop filtering is not performed on a given CU if the

    - pcm_loop_filter_disable_flag is equal to 1 and the pcm_flag is equal to 1  or

    - cu_transquant_bypass_flag is equal to 1

This current design contains one important drawback since it is necessary to check for each CU if the CU is coded with the I_PCM or Lossless mode or not. This increases the complexity of loop-filtering operating at the CTB level, that is, SAO and ALF.

In our proposal, we change the loop filtering behaviour in order to turn off the (SAO and the ALF) loop filtering process for the entire CTB if this CTB contains at least one I_PCM or Lossless CU.

This is illustrated on the following figure. The left case corresponds to the current design. If a CTB contains IPCM blocks and that pcm_loop_filter_disable_flag is equal to 1, it is necessary to check for each pixel of the LCU if it belongs to an IPCM block or not to decide if it is filtered or not. Our proposal corresponds to the left case, which disables the loop filtering for the entire CTB if it contains IPCM blocks.

In the initial proposal, it was additionally proposed to introduce a new flag at the CTB level to indicate the presence of I_PCM CU in the current CTB to avoid checking one by one if the current CU contains one I_PCM CU.

This flag was named pcm_or_tqb_present_flag.

In addition, when a CTB contains Lossless blocks or IPCM blocks and that pcm_loop_filter_disable_flag is equal to 1, we propose to skip the encoding of any SAO and ALF parameters for this LCU since no loop filtering will be applied as shown in the syntax change in section 5 regarding the slice_data syntax element.

# Results

## Experiments on I_PCM mode

This section presents the results for the I_PCM mode..

### Common test conditions

Tests were made with and without the flag pcm_or_tqb_present_flag in normal test conditions with I_PCM activated, but under these conditions there is no I_PCM CU which are coded due to the QP value which are quite high for this mode. Full results are in file JCTVC-J0169-IPCM_LcuFlag_CommonTestConditions.xls. There is no impact on coding performance. The flag is of minor cost, and similar results would be obtained without coding this flag.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.3% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.3% |
| Class D | 0.0% | 0.4% | 0.2% | 0.0% | 0.3% | -0.1% |
| Class E | 0.1% | -0.7% | -0.7% | 0.3% | 0.6% | 0.0% |
| Overall | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |
| Class F | 0.0% | 0.6% | 0.5% | 0.1% | -0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] |  |  |  |  |  |  |

### Test conditions with low QPs

Tests were also made on common test sequences with and without pcm_or_tqb_present_flag with low QPs (0, 4, 8 12) and with I_PCM activated. This triggers higher probability that I_PCM CUs are encoded. Again no impact on coding performance is observed. The cost of the flag is negligible and not coding ti would lead to same performance.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] |  |  |  |  |  |  |

The following tables give additional results for a synthesized sequence (SandStorm, a synthesized sequence is Gaussian noise) are also provided. The synthesized sequence is generated by using JCTVC-E057-NEC-Sandstorms-Software available from:

http://phenix.int-evry.fr/jct/doc_end_user/documents/5_Geneva/wg11/JCTVC-E057-v5.zip

Full results are in file JCTVC-J0169-IPCM_SandStorm.xls. Again no impact on coding performance is observed.

# Syntax modification

Syntax changes are given in this section. Yellow highlighted lines correspond to proposed modifications.

8.7.3.1	Coding tree block modification process (SAO filtering)

…

For i = 0..nS−1 and j = 0..nS−1, depending on the value of saoTypeIdx, pcm_loop_filter_disable_flag, pcm_flag pcmPresentFlag, and cu_transquant_bypass_flag transQuantBypassPresentFlag of the coding unit which includes the coding block covering recPicture[ xC + i, yC + j ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_loop_filter_disable_flag and pcm_flag pcmPresentFlagare both equal to 1.

- cu_transquant_bypass_flag transQuantBypassPresentFlag is equal to 1.

- saoTypeIdx is equal to 0.

…

8.7.4.3	Coding tree block filtering process for luma samples (ALF filtering)

…

For each luma coding block in the current luma coding tree block with location ( xC, yC ) specifying the top-left luma sample of the coding block relative to the top left luma sample of the current picture and size cbSize equal to ( 1 << log2CbSize ), depending on pcm_flag pcmPresentFlag, pcm_loop_filter_disable_flag, and cu_transquant_bypass_flag transQuantBypassPresentFlag of the respective coding unit, the following applies:

- If one or more of the following conditions are true, the adaptive loop filter process is not applied for the current coding block.

  - pcm_loop_filter_disable_flag is equal to 1 and pcm_flag pcmPresentFlagis equal to 1.

  - cu_transquant_bypass_flag transQuantBypassPresentFlag is equal to 1.

…

8.7.4.4	Coding tree block filtering process for chroma samples (ALF filtering)

…

For each chroma coding block of colour component cIdx in the current chroma coding tree block with location ( xC, yC ) specifying the top-left chroma sample of the coding block relative to the top left chroma sample of the current picture for colour component cIdx and size cbSize equal to ( 1 << (log2CbSize−1) ), depending on pcm_flag pcmPresentFlag, pcm_loop_filter_disable_flag, and cu_transquant_bypass_flag transQuantBypassPresentFlag of the respective coding unit, the following applies:

- If one or more of the following conditions are true, the adaptive loop filter process is not applied for the current coding block.

  - pcm_loop_filter_disable_flag is equal to 1 and pcm_flag pcmPresentFlag is equal to 1.

  - cu_transquant_bypass_flag transQuantBypassPresentFlag is equal to 1.

…

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] >= SliceCbAddrZS && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if ( log2CbSize = = Log2CtbSize ) { |  |
| pcmPresentFlag = 0 |  |
| transQuantBypassPresentFlag = 0 |  |
| } |  |
| … |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| if ( cu_transquant_bypass_flag ) |  |
| transQuantBypassPresentFlag = 1 |  |
| } |  |
| … |  |
| if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| pcmPresentFlag = 1 |  |
| num_subsequent_pcm | tu(3) |
| … |  |

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| CuQpDelta = 0 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize, cIdx ) |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_alf_flag[ cIdx ] ) |  |
| ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| if ( !( pcm_loop_filter_disable_flag && pcmPresentFlag ) && !transQuantBypassPresentFlag ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize, cIdx ) |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_alf_flag[ cIdx ] ) |  |
| ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| } |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

# Conclusion

This contribution is about the loop filtering control process related to the I_PCM and Lossless modes. The proposed modification avoids a systematic check for each CU of a CTB to know if it is needed or not to apply the loop filtering (SAO and ALF) process.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).