| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0169 |
| --- | --- |

| Title: | AHG11: Loop filtering control for I_PCM and TransQuantBypass modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS, Patrice ONNO, Guillaume LAROCHE, Tangi POIRIER Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France Masato SHIMA 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Research Centre France, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two main changes regarding the loop filtering process in the current HEVC draft 7 specifications. The first one consists in harmonizing the loop filtering de-activation in a similar way for the I_PCM modes and for the TransQuantBypass mode. The second one consists in introducing a flag at the LCU level indicating if it contains I_PCM and TransQuantBypass CUs. This flag is used to activate/deactivate the loop filtering (SAO and ALF) process at the LCU level for both I_PCM and TransQuantBypass (also named Lossless) modes.

# Introduction

I_PCM and TransQuantBypass modes have several important conceptual commonalities. They are usable for very high quality video applications. They both apply bypassing of the transform and quantization. However in HM7.0, two distinct loop filtering behaviors are specified for the I_PCM and TransQuantBypass modes.

- For the I_PCM mode, the loop filtering (deblocking filter, SAO and ALF) is disabled for a given CU if the flag called pcm_loop_filter_disable_flag located in the Sequence Parameter Set (SPS) is equal to 1. Otherwise if this flag is equal to 0, loop filtering is performed.

- For the TransQuantBypass mode, the loop filtering is never authorized.

The goal of this proposal is to harmonize part of the design of these two modes with the 2 proposals described hereafter.

# Proposal 1: Control of the loop filtering for both the I_PCM and the TransQuantBypass mode

As it was noticed in JCTVC-E192, when original uncompressed samples are coded as I_PCM samples, they are distortion free. Therefore in-loop filtering processes should conceptually be skipped. However, some encoder implementations may code non-original (e.g. reconstructed) samples as I_PCM samples. Performing in-loop filtering may be useful/necessary in such implementations. Therefore, it was decided to add a flag pcm_loop_filter_disable_flag located in the SPS in order to enable or disable the loop filtering for I_PCM blocks.

In this contribution, we propose to generalize this idea applied for the I_PCM mode to the Lossless mode. This implies to separate the loop filtering control from the Transform and Quantization bypass control.

Proposal:

- Change the semantic of cu_transquant_bypass_flag: this flag now only relates to the bypassing of Transform and Quantization processes.

- Use of a flag in the SPS to control the loop filtering for the TransQuantBypass mode. To emulate lossless coding, the loop filtering must be disabled at the SPS level for TransQuantBypass CUs.

2 options:

  - different loop filtering control flags for the I_PCM and for the TransQuantBypass modes

    - pcm_loop_filter_disable_flag

    - transquant_bypass_loop_filter_disable_flag

  - Preferred: common flag applying for both modes

    - generalization of current pcm syntax element and rename it to cover both I_PCM and TransQuantBypass modes

    - use of a common flag: pcm_tqb_loop_filter_disable_flag

# Proposal 2: Enabling loop filtering at the LCU (CTB) level

In the current HM 7.0 specifications, the loop filtering is not performed on a given CU if the

    - pcm_loop_filter_disable_flag is equal to 1 and the

    - pcm_flag is equal to 1.

This current design contains one important drawback since it is necessary to check for each CU if the CU is coded with the I_PCM mode or not. This increases the complexity of loop-filtering operating at the CTB level, that is, SAO and ALF.

In our proposal, we first change the loop filtering behaviour in order to turn off the (SAO and the ALF) loop filtering process for the entire CTB if this CTB contains at least one I_PCM CU.

Secondly, we introduce a new flag at the CTB level to indicate the presence of I_PCM CU in the current CTB to avoid checking one by one if the current CU contains one I_PCM CU.

Finally, in combination with the first proposal, we are extending the meaning of this flag so that it indicates both the presence of I_PCM or TransQuant CU. This flag will be named pcm_or_tqb_present_flag and is introduced in the slice_data syntax element as described in the section 5.

Consequently, in our proposal, the loop filtering (SAO and ALF) process will be not performed on an entire CTB[ xCtb ][ yCtb ] if the

- the pcm_tqb_loop_filter_disable_flag is equal to 1 and the

- pcm_or_tqb_present_flag  is equal to 1.

By using this flag, this will prevent the check of each CU if it has been coded with the I_PCM mode or the TransQuant mode. In addition, under the conditions above, we will not encode any SAO parameters for this LCU since no loop filtering will be applied as shown in the syntax change in section 5 regarding the slice_data syntax element.

# Results

The proposed changes have been evaluated in two sets of experiments, one related to the I_PCM mode, one related to the Lossless mode. The experiments evaluate the impact of the signaling of the new CTB flag pcm_or_tqb_present_flag indicating if the CTB contains or not I_PCM or Lossless CUs.

## Experiments on I_PCM mode

This section presents the results for the I_PCM mode. Two sets of tests are made.

Notes:

- The following results only check the impact of the flag pcm_or_tqb_present_flag when IPCM is on. In these tests SAO is disabled. Additional results with SAO enabled will be provided is a revised version.

- 2 tests with and without the flag pcm_or_tqb_present_flag in normal conditions with I_PCM activated, but under these conditions there is no I_PCM CU which are coded due to the QP value which are quite high for this mode. There is no impact on coding performance.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] |  |  |  |  |  |  |

- 2 tests with and without pcm_or_tqb_present_flag with low QPs (0, 4, 8 12) and with I_PCM activated, this triggers higher probability that I_PCM CUs are encoded. Again no impact on coding performance is observed.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] |  |  |  |  |  |  |

## Experiments on TransQuantBypass mode

This section presents the results of the two Lossless tests. The reference software is the HM7.1. Test conditions are those specified in AHG11:

- Scenario 1: with Lossless mode activated everywhere

- Scenario 2: with mixed Lossy/Lossless mode per region

with the following settings:

- LosslessCuEnabled                 : 1

- TransquantBypassEnableFlag		: 1

- CUTransquantBypassFlagValue		: 1

Notes:

- A next revision will also report results based on HM7.0-dev-r2461, with parameter LosslessCuEnabled set to 0, as lately recommended by the AHG11.

- For Scenario 1, a next revision will also report additional results for classes A-E, as lately recommended by the AHG11.

The results depicted below show that the new flag has a minor impact on the performance in both scenarios.

Scenario 1

|  |  |  | delta R | Enc time |
| --- | --- | --- | --- | --- |
| AIMain | S14 | SlideEditing | 0.00% | 101% |
|  | S15 | SlideShow | 0.00% | 98% |
| RAMain | S14 | SlideEditing | 0.03% | 101% |
|  | S15 | SlideShow | 0.02% | 101% |
| LBMain | S14 | SlideEditing | 0.06% | 101% |
|  | S15 | SlideShow | 0.02% | 100% |
| Avg Main |  |  | 0.02% |  |

Scenario 2

|  | Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -0.02% | -0.03% | -0.04% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] |  |  |  |
| Class F | 0.13% | 0.09% | 0.08% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] |  |  |  |
| Class F | 0.12% | 0.15% | 0.02% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] |  |  |  |
| Average | 0.08% | 0.07% | 0.02% |

# Syntax modification

Syntax changes are given in this section. Yellow highlighted lines correspond to modification of proposal 1 while purple highlighted lines correspond to modification of proposal 2.

7.4.6	Coding unit semantics

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause 8.6 and the in-loop filter process as specified in subclause 8.7 are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

8.7.1.4.7	Filtering process for a luma sample (deblocking filter)

…

When nDp is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample qj is equal to

…

8.7.1.4.8	Filtering process for a chroma sample (deblocking filter)

…

When one or more of the following conditions are true, the filtered sample value, p0’ is substituted by the corresponding input sample value p0.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample pi is equal to 1.

When one or more of the following conditions are true, the filtered sample value, q0’ is substituted by the corresponding input sample value q0.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

–	pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag of the coding unit which includes the coding block containing the sample qj is equal to 1.

…

8.7.2.1	Coding tree block modification process (SAO filtering)

…

For i = 0..nS−1 and j = 0..nS−1, depending on the value of saoTypeIdx, pcm_tqb_loop_filter_disable_flag, pcm_flag, and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xC + i, yC + j ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xC + i, yC + j ] is not modified.

- pcm_or_tqb_present_flag[rx][ry] is equal to 1.

- pcm_tqb_loop_filter_disable_flag and pcm_flag are both equal to 1.

- pcm_tqb_loop_filter_disable_flag is equal to 1and cu_transquant_bypass_flag is equal to 1.

- saoTypeIdx is equal to 0.

…

8.7.3	Adaptive loop filter process (ALF filtering)

…

When interpreting the luma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_tqb_loop_filter_disable_flag, alf_lcu_enable_flag[ 0 ][ ry ][ rx ] and alf_cu_control_flag, the following applies.

- If pcm_or_tqb_present_flag[rx][ry] is equal to 1.

- If pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the luma samples are not filtered.

- Otherwise, if pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag is equal to 1, luma samples are not filtered,

- Otherwise, if alf_lcu_enable_flag[ 0 ][ ry ][ rx ] is equal to 0, the luma samples are not filtered,

- Otherwise, if alf_cu_control_flag is equal to 0, the luma samples are filtered,

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise, the luma samples are not filtered.

When interpreting the Cb chroma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_tqb_loop_filter_disable_flag alf_lcu_enable_flag[1][ry][rx], and alf_cb_enable_flag, the following applies.

- If pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the samples of the colour component 1 are not filtered.

- Otherwise, if pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag is equal to 1, the samples of the colour component 1 are not filtered.

- Otherwise, if alf_cb_enable_flag is equal to 0, the samples in the colour component 1 are not filtered.

- Otherwise, if alf_lcu_enable_flag[1][ry][rx] is equal to 1, the samples of the colour component 1 are filtered.

- Otherwise (alf_lcu_enable_flag[1][ry][rx] is equal to 0), the samples of the colour component 1 are not filtered.

When interpreting the Cr samples of the coding unit as to be filtered, depending on pcm_flag, pcm_tqb_loop_filter_disable_flag, alf_lcu_enable_flag[2][ry][rx], and alf_cr_enable_flag, the following applies.

- If pcm_tqb_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the samples of the colour component 2 are not filtered.

- Otherwise, if pcm_tqb_loop_filter_disable_flag is equal to 1 and cu_transquant_bypass_flag is equal to 1, the samples of the colour component 2 are not filtered.

- Otherwise, if alf_cr_enable_flag is equal to 0, the samples in the colour component 2 are not filtered.

- Otherwise, if alf_lcu_enable_flag[2][ry][rx] is equal to 1, the samples of the colour component 2 are filtered.

- Otherwise (alf_lcu_enable_flag[2][ry][rx] is equal to 0), the samples of the colour component 2 are not filtered.

…

7.3.4	Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| if( (transquant_bypass_enable_flag ¦¦pcm_enabled_flag) && pcm_tqb_loop_filter_disable_flag ) |  |
| pcm_or_tqb_present_flag[xCtb][yCtb] | ae(v) |
| if ( ! pcm_or_transquant_present_flag ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| } |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

9.2.1.1	Initialization process for context variables

…

Table 9 4 – Association of ctxIdx and syntax elements for each initialisationTypeslice type in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| … |  |  |  |  |  |
| slice_data() | pcm_or_tqb_present_flag | Table 9-xx | 0 | 1 | 2 |
| … |  |  |  |  |  |

…

Table 9-xx – Values of variable initValue for pcm_or_tqb_present_flag ctxIdx

| Initialization variable | pcm_or_tqb_present_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 102 | 102 |

9.2.2	Binarization process

…

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ... |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| pcm_or_tqb_present_flag | 0 | FL, cMax = 1 | 0 | Table 9-xx | 0 |
|  | 1 |  | 0 | Table 9-xx | 1 |
|  | 2 |  | 0 | Table 9-xx | 2 |
| ... |  |  |  |  |  |

9.2.3.1	Derivation process for ctxIdx

…

Table 9-39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| … |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pcm_or_tqb_present_flag | Table 9-xx | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| … |  |  |  |  |  |  |  |

# Conclusion

This contribution is about the loop filtering process related to the I_PCM mode and TransQuantBypass modes. Two modifications are presented. The first one is about the harmonization of the 3 loop filterings (Deblocking filter, SAO and ALF) signalling to activate or deactivate the loop filtering process for these both modes. The second one is about the introduction of a flag at the CTB level to avoid a systematic check for each CU to know if we need or not to apply the loop filtering (SAO and ALF) process. Furthermore, this additional flag enables to save some bits in signalling SAO parameters which will be not used for a CTB where SAO is disabled.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).