| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0171r1 |
| --- | --- |

| Title: | On transform skip mode for chroma TUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS, Patrice ONNO, Christophe GISQUET , Guillaume LAROCHE Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution aims at simplifying the Transform Skip (TS) mode for chroma. The proposal consists in disabling the TS mode of 4x4 chroma TUs 1) when the corresponding 8x8  luma TU is not split into four 4x4 TUs;  2) when the 8x8  luma TU is not split into four 4x4 TUs or when none of the four 4x4 luma TUs uses the Transform Skip mode. It is reported that the proposal 1 has no impact on coding efficiency while reducing the encoding time by up to 5% (in All intra configuration). For proposal 2, it is also reported a minor impact on coding efficiency, including for class F, with an encoding time decrease up to 7% (in All intra configuration). In both proposals, the decoding time is not impacted.

# Introduction

The Transform Skip mode has been proposed in documents JCTVC-H0361 and JCTVC-I0408. It has been adopted in HM7 (JCTVC-I1003) in May 2012.

This mode applies to the decoded prediction residual samples of 4x4 luma or chroma blocks, coded in Intra mode. This mode consists in skipping the transform process, which is replaced by a scaling process to keep a same signal range as when transform applies.

A high-level flag, named transform_skip_enabled_flag, placed in the SPS, is used to enable or disable the Transform Skip mode for the images of the sequence. In addition, a flag transform_skip_flag is inserted in the syntax of the prediction residual signal decoding of 4x4 blocks, to signal if the Transform Skip mode applies or not to the block.

It turns out that statistically, chroma TUs  use the Transform Skip mode in specific configurations, related to their corresponding luma TU.

The goal of this proposal is to therefore to exploit this behavior to simplify the design of Transform Skip mode  for chroma.

# Proposals

Two simplifications are proposed:

- disabling the TS mode for 4x4 chroma TUs  when the corresponding 8x8  luma TU is of size 8x8 and is not split into four 4x4 TUs

- disabling the TS mode of 4x4 chroma TUs when the corresponding 8x8  luma TU is not split into four 4x4 TUs  OR  when the 8x8  luma TU is split into four 4x4 TUs but none of these TUs uses the TS mode

## Proposal 1: TS mode enabled when the luma TU is split into four 4x4 TUs

It turns out that for class F, in 95% of time, TS mode applies for 4x4 chroma TUs when the corresponding 8x8  luma TU is split into four 4x4 TUs. It is therefore logical to disable the TS mode for chroma TUs when the corresponding 8x8 luma TU is not split into four 4x4 TUs.

This corresponds to the following syntax changes. Changes are indicated in highlighted font.

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0, 1) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1, 0 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2, 0 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1, 1 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2, 1 ) |  |
| } |  |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx, useTS ) { | Descriptor |
| --- | --- |
| … |  |
| if( transform_skip_enabled_flag && useTS && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| … | ae(v) |

## Proposal 2: TS mode enabled when at least one 4x4 luma TU uses TS

Similarly, it turns out that for class F, in more than 90% of time, TS mode applies for 4x4 chroma TUs when the corresponding 8x8  luma TU is split into four 4x4 TUs and that at least one of the 4x4 luma TU uses the TS mode.

In proposal 2, we disable TS mode for the 4x4 chroma TUs when:

- The 8x8 luma TU is not split into four 4x4 TUs			OR

- The 8x8 luma TU is split into four 4x4 TUs but none of these TUs uses the TS mode

This corresponds to the following syntax changes. Changes are indicated in highlighted font. The syntax changes for residual_coding() in the same as in proposal 1.

| transform_unit( x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( cbf_luma[ x0L ][ y0L ][ trafoDepth ] ) |  |
| residual_coding( x0L, y0L, log2TrafoWidth, log2TrafoHeight, scanIdx, 0, 1 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 1, 0 ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth − 1, log2TrafoHeight − 1, scanIdxC, 2, 0 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| useTS = ( transform_skip_flag [ x0L - 1 ][ y0L - 1 ][ 0 ] ¦ ¦ transform_skip_flag [ x0L ][ y0L - 1 ][ 0 ] ¦ ¦ transform_skip_flag [ x0L - 1 ][ y0L ][ 0 ] ¦ ¦ transform_skip_flag [ x0L ][ y0L ][ 0 ] ) |  |
| if( cbf_cb[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 1, useTS ) |  |
| if( cbf_cr[ x0C ][ y0C ][ trafoDepth ] ) |  |
| residual_coding( x0C, y0C, log2TrafoWidth, log2TrafoHeight, scanIdxC, 2, useTS ) |  |
| } |  |
| } |  |
| } |  |

# Experiments

For the experiments, the reference is the HM7.0 with the TS mode enabled for both the Main and HE10 configurations (which differs from the common test conditions in which the TS mode in only enabled for HE10).

## Proposal 1

The following table summarizes the performance of disabling the TS mode for 4x4 chroma TUs, when the corresponding luma TU is 8x8. It can be seen that there is almost no impact on coding efficiency, with an encoding time reduction of up to 5% in the All Intra configurations. The decoding time is not changed. Full results are provided in attached file JCTVC-J0171_8x8Luma.xls.

|  |  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra | Class A-E | -0.01% | 0.03% | 0.02% | -0.01% | 0.00% | 0.01% |
|  | Class F | -0.01% | -0.05% | -0.04% | -0.02% | -0.07% | 0.03% |
|  | Enc Time[%] | 95% |  |  | 95% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |
| Random Access | Class A-D | 0.00% | 0.01% | 0.03% | 0.01% | 0.01% | 0.04% |
|  | Class F | -0.03% | 0.01% | -0.02% | 0.01% | 0.07% | -0.09% |
|  | Enc Time[%] | 100% |  |  | 99% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |
| Low Delay B | Class B-E | -0.01% | -0.04% | -0.30% | -0.03% | 0.14% | 0.06% |
|  | Class F | 0.03% | -0.16% | -0.22% | 0.07% | 0.66% | 0.20% |
|  | Enc Time[%] | 99% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Average | Class A-E | -0.01% | 0.00% | -0.08% | -0.01% | 0.05% | 0.04% |
|  | Class F | 0.00% | -0.07% | -0.09% | 0.02% | 0.22% | 0.05% |

## Proposal 2

The following table summarizes the performance of disabling the TS mode for 4x4 chroma TUs, when the corresponding luma TU is 8x8 or when none of the four 4x4 TUs uses TS mode. Compared to proposal 1, the impact on coding efficiency is slightly more important on chroma for class F, with an average loss from 0.27% to 0.50%. Luma coding efficiency is almost not impacted. The encoding time decrease is of 7% in the All Intra configurations and of 1% for the other configurations. The decoding time is not changed. Full results are provided in attached file JCTVC-J0171_8x8LumaAndNbTS.xls.

|  |  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra | Class A-E | -0.05% | 0.12% | 0.18% | -0.04% | 0.07% | 0.12% |
|  | Class F | -0.05% | 0.20% | 0.26% | -0.05% | 0.10% | 0.25% |
|  | Enc Time[%] | 93% |  |  | 93% |  |  |
|  | Dec Time[%] | 99% |  |  | 99% |  |  |
| Random Access | Class A-D | -0.04% | 0.16% | 0.16% | -0.02% | 0.23% | 0.19% |
|  | Class F | -0.08% | 0.53% | 0.49% | -0.06% | 0.44% | 0.37% |
|  | Enc Time[%] | 99% |  |  | 99% |  |  |
|  | Dec Time[%] | 100% |  |  | 99% |  |  |
| Low Delay B | Class B-E | -0.02% | -0.19% | 0.02% | -0.01% | 0.05% | 0.03% |
|  | Class F | 0.06% | 0.64% | 0.06% | 0.23% | 0.69% | 0.87% |
|  | Enc Time[%] | 99% |  |  | 100% |  |  |
|  | Dec Time[%] | 101% |  |  | 99% |  |  |
| Average | Class A-E | -0.03% | 0.03% | 0.12% | -0.02% | 0.12% | 0.11% |
|  | Class F | -0.02% | 0.46% | 0.27% | 0.04% | 0.41% | 0.50% |

## Proposal 1 applied only at encoder

We report in this section simulation results when applying the proposal 1 only at the encoder (non-normative change). The performance is slightly worse than the one of the normative case. Full results are provided in attached file JCTVC-J0171_8x8Luma_EncoderOnly.xls. Some simulations are still running and full results will be given in a next release version.

|  |  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra | Class A-E | -0.01% | 0.03% | 0.04% | 0.00% | 0.04% | 0.03% |
|  | Class F | 0.00% | 0.05% | 0.08% | -0.01% | 0.03% | 0.09% |
|  | Enc Time[%] | 97% |  |  | 96% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |
| Random Access | Class A-D |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |
|  | Enc Time[%] |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |
| Low Delay B | Class B-E | 0.02% | -0.15% | -0.11% | 0.00% | 0.02% | 0.22% |
|  | Class F | 0.08% | 0.34% | 0.23% | 0.02% | 0.76% | 1.31% |
|  | Enc Time[%] | 99% |  |  | 100% |  |  |
|  | Dec Time[%] | 99% |  |  | 99% |  |  |
| Average | Class A-E |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |

## Proposal 2 applied only at encoder

We report in this section simulation results when applying the proposal 2 only at the encoder (non-normative change). The performance is slightly worse than the one of the normative case. Full results are provided in attached file JCTVC-J0171_8x8LumaAndNbTS_EncoderOnly.xls. Some simulations are still running and full results will be given in a next release version.

|  |  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra | Class A-E | -0.03% | 0.17% | 0.21% | -0.02% | 0.14% | 0.17% |
|  | Class F | -0.04% | 0.41% | 0.49% | -0.05% | 0.33% | 0.39% |
|  | Enc Time[%] | 94% |  |  | 92% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Random Access | Class A-D |  |  |  |  |  |  |
|  | Class F | -0.04% | 0.67% | 0.74% | -0.05% | 0.68% | 0.52% |
|  | Enc Time[%] |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |
| Low Delay B | Class B-E | -0.01% | 0.10% | -0.01% | -0.01% | 0.19% | 0.30% |
|  | Class F | 0.09% | 0.81% | 0.55% | 0.16% | 0.53% | 1.46% |
|  | Enc Time[%] | 100% |  |  | 101% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Average | Class A-E |  |  |  |  |  |  |
|  | Class F | 0.00% | 0.63% | 0.59% | 0.02% | 0.51% | 0.79% |

## Proposal 2 combined with J0238 Proposal 1

The following table reports the results when combining proposal 2 with the proposal 1 of J0238. Proposal 1 of J0265 consists of restricting transform skipping for 4x4 intra TUs further to the SCU level. There is a slight additional encoding runtime decrease, while the BDR performance is close to those of proposal 2 only. Full results are provided in attached file JCTVC-J0171_8x8LumaAndNbTS_WithJ0265Proposal1.xls.

|  |  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra | Class A-E | -0.04% | 0.14% | 0.19% | -0.03% | 0.10% | 0.14% |
|  | Class F | 0.02% | 0.33% | 0.40% | 0.02% | 0.08% | 0.40% |
|  | Enc Time[%] | 92% |  |  | 92% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Random Access | Class A-D | -0.01% | 0.24% | 0.25% | -0.02% | 0.14% | 0.15% |
|  | Class F | -0.01% | 0.56% | 0.52% | 0.04% | 0.42% | 0.27% |
|  | Enc Time[%] | 100% |  |  | 99% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Low Delay B | Class B-E | -0.02% | -0.13% | -0.07% | -0.01% | 0.11% | 0.25% |
|  | Class F | 0.09% | 0.19% | 0.10% | 0.25% | 0.29% | 0.83% |
|  | Enc Time[%] | 99% |  |  | 99% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
| Average | Class A-E | -0.03% | 0.08% | 0.12% | -0.02% | 0.12% | 0.18% |
|  | Class F | 0.03% | 0.36% | 0.34% | 0.10% | 0.26% | 0.50% |

# Conclusion

It is recommended to adopt proposal 2 in the HM design.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).