| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0175 |
| --- | --- |

| Title: | AHG7: Memory bandwidth limitation for higher level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan Motoharu Ueda Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81 50 3750 2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com +81-46-836-9945 ueda.motoharu@jvckenwood.com fukushima.shigeru@jvckenwood.com |
| Source: | Sony Corporation and JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to restrict the bi-prediction 8x8 with the control flag in SPS. In current HM 7.0 design, bi-prediction 8x4 and 4x8 is replaced to uni-prediction for all levels. The limitation for memory bandwidth is no problem for HD sequences, but it may not be enough for higher level in the future. So this contribution presents that the encoder send a flag whether bi-prediction 8x8 is replaced or not, and the decoder replaces the inter direction for also 8x8 as according to the flag. The flag is sent in sequence parameter sets (SPS). In this experiment, it is shown that with this technique the coding efficiency loss is decreased.

# Introduction

The memory bandwidth of motion compensation (MC) could often be bottleneck for throughput of hardware implementation. It will be problem for especially high resolution image and high frame rate. In Geneva meeting, the restriction for bi-prediction 8x4, 4x8 and inter 4x4 was adopted [3].

In current HM7.0 design, the worst case for memory bandwidth is that the bi-prediction 8x8 is continually chosen. And the next worst case is uni-prediction 4x8 and 8x4. Here, uni-prediction 8x4 and 4x8 as 2nd worst is that the encoder can restrict 8x4 and 4x8 although it brings the coding efficiency loss. On the other hand, for bi-prediction 8x8, there is chance to prevent some coding efficiency loss by replacing bi to uni prediction. So, this contribution proposes to replace not only 8x4 and 4x8 also bi-prediction 8x8. But, it will be unnecessary for not high resolution application such as less than HD or 720p. To not use the replacement for lower resolution application, this proposal technique sends a flag whether inter direction is replaced or not in SPS.

## Proposed method

This basic idea was shown in Geneva meeting [4]. The change at this time is the variation of block size. In [4], it could choose a block size within (8x4 and 4x8) or (8x8) for restriction because the bi-prediction 8x4 and 4x8 were not restricted at previous version HM 6.0. In current HM 7.0, bi-prediction 8x4 and 4x8 restriction is already restricted. So this proposal proposes to replace the bi-prediction 8x8 to uni-prediction with the flag.

### High level syntax

The encoder sends disable_bi8x8_flag as follow in SPS:

- disable_bi8x8_flag:

  - 0: no modification (bi-prediction 8x4 and 4x8 are replaced)

  - 1: The bi-prediction 8x4, 4x8 and 8x8 are replaced to L0 prediction.

The decoder parses the disable_bi8x8_flag in SPS. If the flag is equal to 1, when PU is bi-prediction 8x4, 4x8 or 8x8, the inter direction is replaced as follow process of [3]. If the flag is equal to 0, the process is the same as current HM 7.0 design.

# Experiment

For experiment, this proposal technique as above was implemented in HM7.0 software. The common test conditions and reference configurations specified in [1] are followed. The simulation environment is cluster with some Linux 64 bit OS machines that has the Intel Xeon W3670.

In this experiment, the BD% is compared between the restrictions without this proposal (encoder issue) and with this proposal method.

## The BD% performance comparison of proposal

The summary BD% for RA-Main and LD-Main is shown in Table 1 and Table 2. As we can see, the BD% loss decreases with this proposal method. For this phenomenon, we can understand that the replacement of inter direction when PU size is 8x8, it helps to use merge/skip mode at PU 8x8. Unfortunately, for lower resolution such as Class C and/or D, the coding efficiency loss is not small. However, it usually does not become problem because the memory bandwidth limitation is not required for lower resolution sequence.

For simulation time, we can see that the proposal method is almost the same as anchor. It means that there is no additional complexity.

Table 1: The summary BD% for RA-Main

|  | Without the proposal |  |  | With the proposal |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.60% | 0.20% | 0.30% | 0.30% | 0.10% | 0.30% |
| Class B | 0.80% | 0.40% | 0.40% | 0.50% | 0.20% | 0.30% |
| Class C | 0.90% | 1.10% | 1.20% | 0.60% | 0.80% | 0.90% |
| Class D | 1.10% | 1.00% | 1.10% | 0.90% | 0.80% | 1.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.80% | 0.60% | 0.70% | 0.60% | 0.50% | 0.60% |
|  | 0.90% | 0.60% | 0.70% | 0.60% | 0.50% | 0.60% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 96% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2: The summary BD% for LD-Main

|  | Without the proposal |  |  | With the proposal |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.00% | 0.00% | 0.00% | 0.60% | -0.20% | -0.10% |
| Class C | 1.30% | 1.10% | 1.40% | 0.70% | 0.50% | 0.60% |
| Class D | 1.30% | 0.70% | 1.40% | 0.70% | 0.30% | 0.80% |
| Class E | 0.70% | 0.40% | 0.00% | 0.20% | 0.10% | 0.00% |
| Overall | 1.10% | 0.50% | 0.70% | 0.60% | 0.20% | 0.30% |
|  | 1.10% | 0.40% | 0.70% | 0.60% | 0.10% | 0.30% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## The restriction of bi-prediction 8x8 in encoder issue

The simulation results are shown in Table 3.

Table 3: The simulation results for encoder restriction

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.6% | 0.2% | 0.3% | 0.5% | 0.2% | 0.5% |
| Class B | 0.8% | 0.4% | 0.4% | 0.8% | 0.4% | 0.4% |
| Class C | 0.9% | 1.1% | 1.2% | 1.0% | 1.1% | 1.1% |
| Class D | 1.1% | 1.0% | 1.1% | 1.1% | 1.3% | 1.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.8% | 0.6% | 0.7% | 0.8% | 0.7% | 0.7% |
|  | 0.9% | 0.6% | 0.7% | 0.8% | 0.7% | 0.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.0% | 0.0% | 0.0% | 0.9% | -0.1% | 0.1% |
| Class C | 1.3% | 1.1% | 1.4% | 1.3% | 1.1% | 1.4% |
| Class D | 1.3% | 0.7% | 1.4% | 1.3% | 0.7% | 1.3% |
| Class E | 0.7% | 0.4% | 0.0% | 0.7% | 0.0% | 0.2% |
| Overall | 1.1% | 0.5% | 0.7% | 1.1% | 0.4% | 0.8% |
|  | 1.1% | 0.4% | 0.7% | 1.1% | 0.5% | 0.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 95% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

## The restriction of bi-prediction 8x8 with proposal

The simulation results are shown in Table 4.

Table 4: The simulation results for encoder restriction

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.1% | 0.3% | 0.3% | 0.2% | 0.4% |
| Class B | 0.5% | 0.2% | 0.3% | 0.6% | 0.3% | 0.2% |
| Class C | 0.6% | 0.8% | 0.9% | 0.7% | 0.8% | 0.8% |
| Class D | 0.9% | 0.8% | 1.0% | 0.9% | 0.9% | 0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.6% | 0.5% | 0.6% | 0.6% | 0.5% | 0.6% |
|  | 0.6% | 0.5% | 0.6% | 0.6% | 0.5% | 0.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.6% | -0.2% | -0.1% | 0.5% | -0.3% | 0.1% |
| Class C | 0.7% | 0.5% | 0.6% | 0.7% | 0.5% | 0.7% |
| Class D | 0.7% | 0.3% | 0.8% | 0.9% | 0.7% | 0.6% |
| Class E | 0.2% | 0.1% | 0.0% | 0.3% | 0.2% | 0.3% |
| Overall | 0.6% | 0.2% | 0.3% | 0.6% | 0.2% | 0.4% |
|  | 0.6% | 0.1% | 0.3% | 0.6% | 0.2% | 0.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, the restriction for bi-prediction 8x8 was proposed to limit memory bandwidth for higher resolution and/or higher frame rate application. And to send disable_bi8x8_flag is proposed. If the bandwidth limitation is not required in applications such as lower resolution image, the inter direction replacement for 8x8 can be disabled with disable_bi8x8_flag sets to 0. In experiment, it was shown that the proposed method can reduce coding efficiency loss. The average BD% was 0.8 and 1.1 for RA-Main and LD-Main without proposed method. With proposed method, it was reduced 0.6% and 0.6% for RA-Main and LD-Main.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7,” JCT-VC Document, JCTVC-I1003, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012.

[3] S. Fukushima, M. Ueda and H. Takehara, “AHG7: Bi-pred restriction for small PUs,”  JCT-VC Document,  JCTVC-I0297, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012

[4] S. Fukushima, M. Ueda, K. Kondo and T. Suzuki, “AHG7: High-level syntax for explicit memory bandwidth restriction”,  JCT-VC Document,  JCTVC-I0558, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).