| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0176 |
| --- | --- |

| Title: | AHG5: Reference index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method for encoding reference indices that introduces bypass coding into the binarisation of the reference index and removes two contexts from the context model.  Under common conditions the simulation results indicate 0.0%, 0.0%, 0.0% in AI Main, AI HE10, RA Main, RA HE10 configurations, 0.0%, 0.1%, 0.0% in LDB Main, 0.0%, 0.0%, 0.1% in LDB HE10 configuration.

# Problem Statement

In HM-7.0, the reference picture list indices ref_idx_l0 and ref_idx_l1 are encoded as truncated unary bin strings, using only arithmetic coding.  Three contexts are used, allocated as shown in Table 1.

Table 1. ref_idx_l0, ref_idx_l1 binarisation (reference picture list length = 6).

| binIdx | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| ctxIdx | 0 | 1 | 2 | 2 | 2 |

In JCTVC-I0352 [1], it is proposed to use bypass coding from binIdx 2 and above, which enables removing two contexts from the context model.  When reference picture list length is larger than 4, throughput improvement can be achieved via the adjacent bypass coded bins.

A drawback of this approach is that accessing reference pictures at high list indices (possible when the reference picture list length is longer than specified in the common conditions), requires long strings of bypass coded bins, yet bypass coded bins are preferred for use when coding equiprobable bin values.

# Description of the proposed change

The proposed binarisation introduces a bypass coded bin at the final bin in the binarisation for reference picture list lengths of 4 and above and removes the third context, illustrated in Table 2 and Table 3.  The bypass bin is coded when either of the last two reference pictures in the reference picture list is accessed.

Table 2. ref_idx_l0, ref_idx_l1 binarisation (reference picture list length = 6).

| binIdx | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| ctxIdx | 0 | 1 | 1 | 1 | B |

Table 3. ref_idx_l0, ref_idx_l1 binarisation (reference picture list length = 4).

| binIdx | 0 | 1 | 2 | N/A |
| --- | --- | --- | --- | --- |
| ctxIdx | 0 | 1 | B |  |

By using bypass coding when accessing only the last two reference pictures, the statistics indicate a distribution closer to the equiprobable case.

Note that the binarisations for ref_idx_l0/l1 when reference picture list length = 4 is the same as for I0352.

# Experiment Results

Results are presented in Table 4 below. Simulations were performed under the common conditions [2]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 4. Simulation results in HM-7.0 (without bypass concatenation).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | -0.3% | 0.1% | -0.3% | 0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class F | -0.1% | 0.2% | 0.1% | 0.1% | 0.4% | 0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Working Draft changes

Table 9-5 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_flag | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 0..1 | 3..5 2..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-6 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 32 | 43 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

| Table 9-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 21 | Error: Reference source not found | prefix: 0 suffix: na, (uses Decode Bypass) |
|  | 2 |  | 21 | Error: Reference source not found | prefix: 23 suffix: na, (uses Decode Bypass) |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 21 | Error: Reference source not found | prefix: 0 suffix: na, (uses Decode Bypass) |
|  | 2 |  | 21 | Error: Reference source not found | prefix: 23 suffix: na, (uses Decode Bypass) |

| Table 9-8 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |
| --- | --- | --- | --- |
| ref_idx_l0 | Error: Reference source not found | 0 | 0 1 2 2 2 0..1 (subclause 9.3.3.1.7) |
| ref_idx_l0, ref_idx_l1 | Error: Reference source not found | 32 | # 1 2 2 2 0..1 (subclause 9.3.3.1.7) |

9.3.3.1.7 Derivation process of ctxIdxInc and bypassFlag for the syntax element ref_idx_l0 and ref_idx_l1

Inputs to this process are the binIdx, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1.

Outputs of this process are ctxIdxInc and bypassFlag.

- If binIdx is less than or equal to 1, ctxIdxInc is derived as follows.

ctxIdxInc  = binIdx		(9-1)

bypassFlag = 0

- Otherwise (binIdx is greater than 1), ctxIdxInc is derived as follows.

- If binIdx plus 1 is less than num_ref_idx_l0_active_minus1 for list 0 or num_ref_idx_l1_active_minus for list 1, the following applies.

ctxIdxInc  =  1		(9-2)

bypassFlag = 1

- Otherwise (binIdx plus 1 is greater than or equal to num_ref_idx_l0_active_minus1 for list 0 or num_ref_idx_l1_active_minus1 for list 1), the following applies.

bypassFlag = 1		(9-3)

# Conclusion

This contribution presents a binarisation for ref_idx_l0 and ref_idx_l1 with two contexts removed. Experimental results show no noticeable impact on coding efficiency, while context coding is retained for the majority of cases.

# References

- V. Seregin, X. Wang, J. Chen,M. Karczewicz, “Bypass bins for reference index coding”, Joint Collaborative Team on Video Coding, JCTVC-I0352, April 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-I1100, May 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).